# 3GPP TSG RAN WG1 Meeting #92bis                                   R1-1803609

# Sanya, China, Apr. 16th – 20th, 2018

# Source:      	ZTE, Sanechips

Title:         	Remaining details of PRACH Resource Configuration

Agenda Item:  	7.1.1.4.1

Document for: 	Discussion and Decision

## Introduction

After the RAN1# 92 meeting, the configuration table design of the short sequence format for unpaired spectrum in FR1 and FR2 has been partly finished. In this meeting, some issues will be revisited regarding RACH design of FR1, either the priority rule on the collision between the DL parts and RACH in FR1 may use the same rule of FR2, or the configuration table design of the short sequence format for unpaired spectrum in FR1 may be revised based on the original working assumption of priority rule. And also the configuration table design in FR2 has not totally finished as only format A1 has some outcome in the Athens meeting.

In this contribution, chapter 2 discusses the handling rule on the collision between the DL parts and RACH in FR1 and the gap necessary in between SS/PBCH block or DL part and PRACH transmission. Chapter 3 discusses the possible revision to configuration table in FR1 and the total design on the configuration table in FR2. Chapter 4 discusses the question on indication redundancy between the parameter restriction set and parameter SCS of Msg1 which needs to be clarified.

## Handling rule on the collision between the DL parts and RACH

The agreement in RAN1#92 meeting on the handling of Semi-static UL/DL configuration in RMSI and collision between RACH occasion and DL part is:

- For the previous working assumption:

  - If the Semi-static UL/DL configuration is in RMSI, only PRACH occasions within the UL part is transmitted

    - UE assumes that RACH occasions configured in RMSI are not collided with DL transmission

      - Introducing start symbol(s) larger than 2 for a limited number of entries in the Configuration table.

- For FR2, it is replaced with the following working assumption (no change of the previous working assumption regarding FR1)

  - For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot

- UE is not expected to receive DL signals during any valid RACH occasion

- Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

- FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx

In the working assumption on FR2, the UL and X part provides more RACH occasions to mapping the SSBs than the UL part only in one configuration period especially when the number of SSB is large. For example, the maximum number of SSB in one SS burst set period is 64, the whole mapping period may need more than one configuration period, the UL and X part for RACH occasions can let the mapping period more short and reduce the access latency.

From UE’s perspective, when UE follows the handling rule for FR2, it is easy and natural to replace the handling rule for FR1 with the working assumption of FR2. Also the working assumption for FR1 in last meeting highly restricts the flexibility of the configuration of frame structure and slot format. For example, in the PRACH configuration table for FR1, there are 3 choices {0,2,7} for starting symbol for format A1. This will limit the slot format to leave the whole slot for the RACH starting symbol equal 0, or only 1 DL symbol in slot for the RACH starting symbol equal 2, or only less than 7 DL symbols in slot for the RACH starting symbol equal 7.

Proposal 1: Replace the working assumption of priority rule for FR1 with the working assumption of FR2.

In TDD network, the interference from DL to UL between different gNBs can’t be ignored as the interference will strongly affect the uplink performance if the received interference power is big enough. The typical solution for this problem is setting some blank symbols or slots as guard period between the DL parts and UL parts to avoid that DL transmissions from the nearby gNBs interfere with the local cell UL signals reception. The distant gNBs DL transmission power will fade to an acceptable interference level for local cell UL reception.

We build a simplified numerical calculation model to estimate the guard period length for FR1 and FR2. The judgement of the interference level is based on the ratio between the received power of PRACH sent in local serving cell edge and the received power of other gNB DL part within the distance of signal traveling through the guard period.

Figure 1 The interference from adjacent gNB DL to local UL

For PRACH short sequence in FR2, the typical cell radius is from about 73 m (B1@120KHz) to 1150 m (C2@120KHz). For conservative estimation of the interference, we use 1150m and format C2@120KHz to minimize the received PRACH power.

The frequency of the band is 30 GHz. The typical pathloss model is PL=32.4+20log10d+20log10(fc) in Uma LOS scenario. gNB maximum power is 38 dBm, UE maximum power is 23 dBm.

Table 1 Light distance travelling through the guard period @120KHz

| Guard period | Length of guard period | Light distance travelling through the guard period |
| --- | --- | --- |
| 1 symbol@120KHz | 0.125 ms/14=0.008929 ms | 2678 m |
| 2 symbol@120KHz | 0.125 ms*2/14=0.017857 ms | 5357 m |
| 3 symbol@120KHz | 0.125 ms*3/14=0.026786 ms | 8036 m |

Table 2 simplified numerical calculation on received power offset between the RACH and interference @120KHz

|  | Received power offset between the RACH and interference from one gNB | Received power offset between the RACH and interference from the multiple gNBs in the layer |
| --- | --- | --- |
| First interference layer | [23dBm -20lg(dRACH)]-[38dBm-20lg(dinf)] =20lg(4*1150)-20lg(1150)-15dBm=-2.96dB | -2.96-10lg(12)=-13.75dB Note: there are at least 12 interfering gNBs |
| Second interference layer | [23dBm -20lg(dRACH)]-[38dBm-20lg(dinf)] =20lg(6*1150)-20lg(1150)-15dBm=0.56dB | 0.56-10lg(18)=-11.99dB Note: there are at least 18 interfering gNBs |
| Third interference layer | [23dBm -20lg(dRACH)]-[38dBm-20lg(dinf)] =20lg(8*1150)-20lg(1150)-15dBm=3.06dB | 3.06-10lg(24)=-10.74dB Note: there are at least 24 interfering gNBs |
| Fourth interference layer | [23dBm -20lg(dRACH)]-[38dBm-20lg(dinf)] =20lg(10*1150)-20lg(1150)-15dBm=5dB | 5-10lg(30)=-9.77dB Note: there are at least 24 interfering gNBs |
| Fifth interference layer | [23dBm -20lg(dRACH)]-[38dBm-20lg(dinf)] =20lg(12*1150)-20lg(1150)-15dBm=6.58dB | 6.58-10lg(36)=-8.98dB Note: there are at least 36 interfering gNBs |
| sixth interference layer | [23dBm -20lg(dRACH)]-[38dBm-20lg(dinf)] =20lg(14*1150)-20lg(1150)-15dBm=7.92dB | 7.92-10lg(42)=-8.31dB Note: there are at least 42 interfering gNBs |

The first interference lap is actually the second layer around the target gNB. The first layer around the target gNB can’t interfere the target gNB as the 2*(distance to cell-edge) is smaller than light travelling distance in one symbol.

For 1 symbol guard period case, the interference of first interference layer only interfere part of the first symbol of PRACH preamble, if we assume the PRACH receiving power is unit 1, the average interference counted on the whole 4-symbol preamble is 14*1013.7510. The interference from the second interference layer is 12*1011.9910, as two symbols of PRACH preamble are interfered only. The interference from the third interference layer is 34*1010.7410 , as three symbols of PRACH preamble are interfered. The interference from the fourth interference layer is 109.7710 , as four symbols of PRACH preamble are interfered. The larger interference layer is neglected as the far away interference power will be very small compared with the first interference layer as the pathloss model changing to the NLOS. Then the rough received power offset between the RACH and the total interference is about -10lg14*1013.7510+12*1011.9910+34*1010.7410+109.7710=-15.08dB.

For 2 symbol guard period case, the interference of first interference layer has no interference on the PRACH preamble. If we assume the PRACH receiving power is unit 1, the interference from the second interference layer is 14*1011.9910. The interference from the third interference layer is 12*1010.7410 . The interference from the fourth interference layer is 34*109.7710. The interference from the fifth interference layer is 108.9810. Then the rough received power offset between the RACH and the total interference is about -10lg14*1011.9910+12*1010.7410+34*109.7710+108.9810=-13.96dB.

For 3 symbol guard period case, the rough received power offset between the RACH and the total interference is about -10lg14*1010.7410+12*109.7710+34*108.9810+108.3110=-13.10dB.

The minimum SINR requirement of 2 symbol RACH sequence @120KHz is about -11dB derived from [1]. From the reasonable linear derivation, the SINR requirement for 4 symbol RACH sequence format C2 @120KHz is about -14dB.

Comparing the received power offset between the RACH and the total interference with the PRACH minimum SINR requirement, the 1 symbol guard period is not enough, 2 symbols guard period can keep the marginal performance. 3 or more symbols is enough safe.

This numerical calculation is the most severe scenario and very pessimistic, when the cell radius is not so large, the antenna tilts down and the pathloss model of the inference from farther gNB to local cell is more close to NLOS, 2 symbols as guard period is sufficient for most scenarios. Regarding the patterns of SSB in slot, there are at least 2 symbols after SSB in the SSB slot, then the 2 symbols after the SSB are nature to be the guard period and won’t enlarge the overhead waste and resource inflexibility.

From the receiving power offset calculation equation, we can see the power offset is not related to the absolute value of cell radius but to the relative value of the light distance travelling through the guard period divided by the cell radius, so if the RACH SCS changes to 60KHz, the cell radius and the light distance travelling through the guard period will increase proportionally, then the above conclusion doesn’t need to change.

Observation 1: 2 symbols as guard period between the SSB and RACH is sufficient for most scenarios in FR2.

Proposal 2: The minimum gap necessary in between SS/PBCH block or DL part and PRACH tx can be set to 2 symbols in FR2.

If guard period of 2 symbols is not enough in some scenario, for example, the 100km cell radius, gNB can configure more DL symbols but not transmit anything in the additional DL symbols, these symbols can be treated as the additional guard period just like below figure 2. It can solve the concern of the variable time gap. The disadvantage is the sacrifice of the flexibility of X part in cell specific configuration, but this method is the final remedy for some special cases, the flexibility loss can be accepted.

Figure 2 Additional guard period

## Discussion on configuration of short sequence format for unpaired spectrum

## Revision on the short sequence format for unpaired spectrum in FR1

If the agreement in RAN1#92 meeting on the handling of Semi-static UL/DL configuration in RMSI and collision between RACH occasion and DL part in FR2 keeps the original working assumption, some rows in the configuration table for FR1 and unpaired spectrum should be revised to avoid the collision between the SSB and RACH. For the collision avoidance, an assumption of SS burst set period equals 20ms is reasonable, as it is the default period in initial access.

To avoid the collision between SSB and RACH, we have two types of revision below:

First revision: The subframe index of {1,3,5,7,9} should be reduce to {5,7,9} at least to avoid the SSB in the first half frame.

Second revision: The subframe index of {0,1,2,3,4,5,6,7,8,9} should be reduce to {4,5,6,7,8,9} at least to avoid the SSB in the first half frame.

To bring more access RACH occasion for DL/UL configuration period = 2ms, we suggest revising the starting symbol {6,7,8,9}of subframe index {7,9} in 10ms RACH configuration period changes to 0 or 2 accordingly. The revision can bring more access RACH occasion and be comparable to subframe index {4,9}. It is noteworthy that this revision will not affect the case that starting symbol from the middle of slot in 20ms RACH configuration period

The entire text proposal for this table is listed in the appendix 1.

Proposal 3: The below revisions for configuration table for FR1 and unpaired spectrum are proposed and the text proposal in appendix 1 are adopted if proposal 1 is not agreed:

- The subframe index of {1,3,5,7,9} should be reduce to {5,7,9} at least to avoid the SSB in first half frame.

- The subframe index of {0,1,2,3,4,5,6,7,8,9} should be reduce to {4,5,6,7,8,9} at least to avoid the SSB in first half frame.

Proposal 4: Changing the starting symbol {6,7,8,9}of subframe index {7,9} in 10ms RACH configuration period to 0 or 2 accordingly.

## Configuration of the short sequence format for unpaired spectrum in FR2

From the agreement of RAN1#92, we have a primitive RACH configuration table for FR2 and unpaired spectrum below, values in [brackets] are still up for discussion and other format except A1 should be decided in this meeting:

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| PRACH Config. Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | Number of time-domain PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | [2 or 1] | 6 |
| 1 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 3 |
| 2 | A1 | 1 | 0 | 24,29,34,39 | 0 | [2 or 1] | 6 |
| 3 | A1 | 1 | 0 | 24,29,34,39 | 7 | 1 | 3 |
| 4 | A1 | 1 | 0 | 18,19,38,39 | 0 | [2 or 1] | 6 |
| 5 | A1 | 1 | 0 | 18,19,38,39 | 7 | 1 | 3 |
| 6 | A1 | 1 | 0 | 0,1,2,…,39 | 0 | [2 or 1] | 6 |
| 7 | A1 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 3 |
| 8 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | [2 or 1] | 6 |
| 9 | A1 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 3 |
| 10 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | [2 or 1] | 6 |
| 11 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 3 |
| 12 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | [2 or 1] | 6 |
| 13 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | [2 or 1] | 6 |
| 14 | A1 | 1 | 0 | 7,15,23,31,39 | 7 | 1 | 3 |
| 15 | A1 | 16 | [0] | 4,9,14,19,24,29,34,39 | 0 | [2 or 1] | 6 |
| 16 | A1 | 16 | [1] | [3],7,[11],15,[19],23,[27],31,[35],39 | [0 or 7] | 1 | [6 or 3] |
| 17 | A1 | 8 | [1] | 4,9,14,19,24,29,34,39 | 0 | [2 or 1] | 6 |
| 18 | A1 | 8 | [1] | [3],7,[11],15,[19],23,[27],31,[35],39 | [0 or 7] | 1 | [6 or 3] |
| 19 | A1 | 4 | [1] | 4,9,14,19,24,29,34,39 | 0 | [2 or 1] | 6 |
| 20 | A1 | 4 | [1] | [3],7,[11],15,[19],23,[27],31,[35],39 | [0 or 7] | 1 | [6 or 3] |
| 21 | A1 | 2 | [1] | 4,9,14,19,24,29,34,39 | 0 | [2 or 1] | 6 |
| 22 | A1 | 2 | [1] | [3],7,[11],15,[19],23,[27],31,[35],39 | [0 or 7] | 1 | [6 or 3] |
| 23 | A1 | [1] | [0] | 23,31,39 | 0 | 1 | 6 |
| 24 | A1 | [1] | [0] | 1,3,5,7,…,37,39 | 7 | 1 | 3 |

The index 23,24 which added at the end of last meeting should be kept in this table. Index 24 is the companion configuration with index 12, the difference is the starting symbol.

In index 15,16,17,18,19,20,21,22, for the values “y”, should be 1 as we follow the same principle for FR1 that, in the longer configuration period, the ROs have better to start from the second frame.

For index 0,2,4,13,15,17,19,21, the “Number of PRACH slots within a 60kHz slot” can down select to 2, as the possible DL/UL configuration can allow consecutive 2 120KHz slots as RACH slots.

For index 6,8,10,12, the “Number of PRACH slots within a 60kHz slot” can down select to 1. The reason is for this RACH density and possible DL/UL ratio 3:1, we should leave space for other UL channel in SCS=120KHz.

For index 16,18,20,22, we select the larger density {3,7,11,15,19,23,27,31,35,39} set and the starting symbol is 0. The “Number of time-domain PRACH occasions within a RACH slot” will be selected accordingly.

As we have free space for format A1, we suggest to add two rows in the end, which the density is the integral multiple of 4:

| 25 | A1 | 1 | 0 | 7,11,15,19,27,31,35,39 | 0 | 1 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | A1 | 1 | 0 | 27,31,35,39 | 0 | 1 | 6 |

For other format except A1, we can reuse as much as possible from the format A1 but delete the 160/80/40 ms configuration period in format A1/B1, A2/B2, A3/B3.

The entire text proposal for this table is listed in the appendix 2.

Proposal 5: Adopt the text proposal in appendix 2 for FR2 and unpaired spectrum.

## Redundancy between the indication of parameter Restriction set and parameter SCS of Msg1

We all know the parameter restriction set is only valid in the case of long preamble sequence and the parameter SCS of Msg1 is only valid in the case of short preamble sequence. The two parameters cannot be valid at the same time. This means that there is redundancy between the indication of parameter Restriction set and parameter SCS of Msg1.

How to indicate them in the RMSI or RRC signaling will bring UE two different understanding way. But it is unclear now. There may be two alternatives on the indication way:

Alt1: indicate only one parameter in signaling according whether the long or short sequence is used for RACH in the cell. (This can save one bit in signaling);

Alt2: indicate both the two parameters in signaling, and let UE itself to distinguish which one is valid based on the RACH format configuration. The invalid parameter is neglected by UE and the invalid parameter can be set to any value in gNB.

Proposal 6: Only one parameter between the parameter of restriction set and parameter of SCS of Msg1 is preferred to save overhead in RMSI.

## Conclusions

The following proposals have been made:

Proposal 1: Replace the working assumption of priority rule for FR1 with the working assumption of FR2.

Proposal 2: The minimum gap necessary in between SS/PBCH block or DL part and PRACH tx can be set to 2 symbols in FR2.

Proposal 3: The below revisions for configuration table for FR1 and unpaired spectrum are proposed and the text proposal in appendix 1 are adopted if proposal 1 is not agreed:

- The subframe index of {1,3,5,7,9} should be reduce to {5,7,9} at least to avoid the SSB in first half frame.

- The subframe index of {0,1,2,3,4,5,6,7,8,9} should be reduce to {4,5,6,7,8,9} at least to avoid the SSB in first half frame.

Proposal 4: Changing the starting symbol {6,7,8,9}of subframe index {7,9} in 10ms RACH configuration period to 0 or 2 accordingly.

Proposal 5: Adopt the text proposal in appendix 2 for FR2 and unpaired spectrum.

Proposal 6: Only one parameter between the parameter of restriction set and parameter of SCS of Msg1 is preferred to save overhead in RMSI.

References

- R1-1707048 “PRACH Design”, ZTE

Appendix 1

============ Text proposal 38.211, section 6.3.3.2 =======================

Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | Number of time-domain PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 16 | 1 | 9 | 0 | - | - |
| 1 | 0 | 8 | 1 | 9 | 0 | - | - |
| 2 | 0 | 4 | 1 | 9 | 0 | - | - |
| 3 | 0 | 2 | 0 | 9 | 0 | - | - |
| 4 | 0 | 2 | 1 | 9 | 0 | - | - |
| 5 | 0 | 2 | 0 | 4 | 0 | - | - |
| 6 | 0 | 2 | 1 | 4 | 0 | - | - |
| 7 | 0 | 1 | 0 | 9 | 0 | - | - |
| 8 | 0 | 1 | 0 | 8 | 0 | - | - |
| 9 | 0 | 1 | 0 | 7 | 0 | - | - |
| 10 | 0 | 1 | 0 | 6 | 0 | - | - |
| 11 | 0 | 1 | 0 | 5 | 0 | - | - |
| 12 | 0 | 1 | 0 | 4 | 0 | - | - |
| 13 | 0 | 1 | 0 | 3 | 0 | - | - |
| 14 | 0 | 1 | 0 | 2 | 0 | - | - |
| 15 | 0 | 1 | 0 | 1 | 0 | - | - |
| 16 | 0 | 1 | 0 | 4,9 | 0 | - | - |
| 17 | 0 | 1 | 0 | 3,8 | 0 | - | - |
| 18 | 0 | 1 | 0 | 2,7 | 0 | - | - |
| 19 | 0 | 1 | 0 | 8,9 | 0 | - | - |
| 20 | 0 | 1 | 0 | 4,8,9 | 0 | - | - |
| 21 | 0 | 1 | 0 | 3,4,9 | 0 | - | - |
| 22 | 0 | 1 | 0 | 3,4,8 | 0 | - | - |
| 23 | 0 | 1 | 0 | 7,8,9 | 0 | - | - |
| 24 | 0 | 1 | 0 | 3,4,8,9 | 0 | - | - |
| 25 | 0 | 1 | 0 | 6,7,8,9 | 0 | - | - |
| 26 | 0 | 1 | 0 | 1,4,6,9 | 0 | - | - |
| 27 | 0 | 1 | 0 | 1,6 | 0 |  |  |
| 28 | 0 | 1 | 0 | 1,6 | 7 | - | - |
| 29 | 0 | 1 | 0 | 5,7,9 | 0 | - | - |
| 30 | 1 | 16 | 1 | 7 | 0 | - | - |
| 31 | 1 | 8 | 1 | 7 | 0 | - | - |
| 32 | 1 | 4 | 1 | 7 | 0 | - | - |
| 33 | 1 | 2 | 0 | 7 | 0 | - | - |
| 34 | 1 | 2 | 1 | 7 | 0 | - | - |
| 35 | 1 | 1 | 0 | 7 | 0 | - | - |
| 36 | 2 | 16 | 1 | 6 | 0 | - | - |
| 37 | 2 | 8 | 1 | 6 | 0 | - | - |
| 38 | 2 | 4 | 1 | 6 | 0 | - | - |
| 39 | 2 | 2 | 0 | 6 | 7 | - | - |
| 40 | 2 | 2 | 1 | 6 | 7 | - | - |
| 41 | 2 | 1 | 0 | 6 | 7 | - | - |
| 42 | 3 | 16 | 1 | 9 | 0 | - | - |
| 43 | 3 | 8 | 1 | 9 | 0 | - | - |
| 44 | 3 | 4 | 1 | 9 | 0 | - | - |
| 45 | 3 | 2 | 0 | 9 | 0 | - | - |
| 46 | 3 | 2 | 1 | 9 | 0 | - | - |
| 47 | 3 | 2 | 0 | 4 | 0 | - | - |
| 48 | 3 | 2 | 1 | 4 | 0 | - | - |
| 49 | 3 | 1 | 0 | 9 | 0 | - | - |
| 50 | 3 | 1 | 0 | 8 | 0 | - | - |
| 51 | 3 | 1 | 0 | 7 | 0 | - | - |
| 52 | 3 | 1 | 0 | 6 | 0 | - | - |
| 53 | 3 | 1 | 0 | 5 | 0 | - | - |
| 54 | 3 | 1 | 0 | 4 | 0 | - | - |
| 55 | 3 | 1 | 0 | 3 | 0 | - | - |
| 56 | 3 | 1 | 0 | 2 | 0 | - | - |
| 57 | 3 | 1 | 0 | 1 | 0 | - | - |
| 58 | 3 | 1 | 0 | 1,6 | 0 | - | - |
| 59 | 3 | 1 | 0 | 1,6 | 7 | - | - |
| 60 | 3 | 1 | 0 | 4,9 | 0 | - | - |
| 61 | 3 | 1 | 0 | 3,8 | 0 | - | - |
| 62 | 3 | 1 | 0 | 2,7 | 0 | - | - |
| 63 | 3 | 1 | 0 | 8,9 | 0 | - | - |
| 64 | 3 | 1 | 0 | 4,8,9 | 0 | - | - |
| 65 | 3 | 1 | 0 | 3,4,9 | 0 | - | - |
| 66 | 3 | 1 | 0 | 7,8,9 | 0 | - | - |
| 67 | 3 | 1 | 0 | 3,4,8,9 | 0 | - | - |
| 68 | 3 | 1 | 0 | 6,7,8,9 | 0 | - | - |
| 69 | 3 | 1 | 0 | 1,4,6,9 | 0 | - | - |
| 70 | 3 | 1 | 0 | 5,7,9 | 0 | - | - |
| 71 | A1 | 16 | 1 | 9 | 0 | 2 | 6 |
| 72 | A1 | 8 | 1 | 9 | 0 | 2 | 6 |
| 73 | A1 | 4 | 1 | 9 | 0 | 1 | 6 |
| 74 | A1 | 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 6 |
| 75 | A1 | 2 | 1 | 8,9 | 0 | 2 | 6 |
| 76 | A1 | 2 | 1 | 7,9 | 0 | 1 | 6 |
| 77 | A1 | 2 | 1 | 7,9 | 7 | 1 | 3 |
| 78 | A1 | 2 | 1 | 4,9 | 7 | 1 | 3 |
| 79 | A1 | 2 | 1 | 4,9 | 0 | 2 | 6 |
| 80 | A1 | 2 | 1 | 9 | 0 | 1 | 6 |
| 81 | A1 | 1 | 0 | 9 | 0 | 2 | 6 |
| 82 | A1 | 1 | 0 | 9 | 7 | 1 | 3 |
| 83 | A1 | 1 | 0 | 9 | 0 | 1 | 6 |
| 84 | A1 | 1 | 0 | 8,9 | 0 | 2 | 6 |
| 85 | A1 | 1 | 0 | 4,9 | 0 | 1 | 6 |
| 86 | A1 | 1 | 0 | 7,9 | 0 | 1 | 6 |
| 87 | A1 | 1 | 0 | 3,4,8,9 | 0 | 1 | 6 |
| 88 | A1 | 1 | 0 | 3,4,8,9 | 0 | 2 | 6 |
| 89 | A1 | 1 | 0 | 4,5,6,7,8,9 | 7 | 1 | 3 |
| 90 | A1 | 1 | 0 | 5,7,9 | 0 | 1 | 6 |
| 91 | A2 | 16 | 1 | 9 | 0 | 2 | 3 |
| 92 | A2 | 16 | 1 | 4,9 | 0 | 2 | 3 |
| 93 | A2 | 8 | 1 | 9 | 0 | 2 | 3 |
| 94 | A2 | 8 | 1 | 4,9 | 0 | 2 | 3 |
| 95 | A2 | 4 | 1 | 9 | 0 | 1 | 3 |
| 96 | A2 | 2 | 1 | 8,9 | 0 | 2 | 3 |
| 97 | A2 | 2 | 1 | 7,9 | 9 | 1 | 1 |
| 98 | A2 | 2 | 1 | 4,9 | 9 | 1 | 1 |
| 99 | A2 | 2 | 1 | 4,9 | 0 | 2 | 3 |
| 100 | A2 | 2 | 1 | 9 | 0 | 1 | 3 |
| 101 | A2 | 1 | 0 | 9 | 0 | 2 | 3 |
| 102 | A2 | 1 | 0 | 9 | 9 | 1 | 1 |
| 103 | A2 | 1 | 0 | 9 | 0 | 1 | 3 |
| 104 | A2 | 1 | 0 | 8,9 | 0 | 2 | 3 |
| 105 | A2 | 1 | 0 | 4,9 | 0 | 1 | 3 |
| 106 | A2 | 1 | 0 | 7,9 | 0 | 1 | 3 |
| 107 | A2 | 1 | 0 | 3,4,8,9 | 0 | 1 | 3 |
| 108 | A2 | 1 | 0 | 3,4,8,9 | 0 | 2 | 3 |
| 109 | A2 | 1 | 0 | 4,5,6,7,8,9 | 9 | 1 | 1 |
| 110 | A2 | 1 | 0 | 5,7,9 | 0 | 1 | 3 |
| 111 | A3 | 16 | 1 | 9 | 0 | 2 | 2 |
| 112 | A3 | 8 | 1 | 9 | 0 | 2 | 2 |
| 113 | A3 | 4 | 1 | 9 | 0 | 1 | 2 |
| 114 | A3 | 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 2 |
| 115 | A3 | 2 | 1 | 8,9 | 0 | 2 | 2 |
| 116 | A3 | 2 | 1 | 7,9 | 0 | 1 | 2 |
| 117 | A3 | 2 | 1 | 7,9 | 7 | 1 | 1 |
| 118 | A3 | 2 | 1 | 4,9 | 7 | 1 | 1 |
| 119 | A3 | 2 | 1 | 4,9 | 0 | 2 | 2 |
| 120 | A3 | 2 | 1 | 9 | 0 | 1 | 2 |
| 121 | A3 | 1 | 0 | 9 | 0 | 2 | 2 |
| 122 | A3 | 1 | 0 | 9 | 7 | 1 | 1 |
| 123 | A3 | 1 | 0 | 9 | 0 | 1 | 2 |
| 124 | A3 | 1 | 0 | 8,9 | 0 | 2 | 2 |
| 125 | A3 | 1 | 0 | 4,9 | 0 | 1 | 2 |
| 126 | A3 | 1 | 0 | 7,9 | 0 | 1 | 2 |
| 127 | A3 | 1 | 0 | 3,4,8,9 | 0 | 1 | 2 |
| 128 | A3 | 1 | 0 | 3,4,8,9 | 0 | 2 | 2 |
| 129 | A3 | 1 | 0 | 4,5,6,7,8,9 | 7 | 1 | 1 |
| 130 | A3 | 1 | 0 | 5,7,9 | 0 | 1 | 2 |
| 131 | B1 | 16 | 1 | 9 | 2 | 2 | 6 |
| 132 | B1 | 8 | 1 | 9 | 2 | 2 | 6 |
| 133 | B1 | 4 | 1 | 9 | 2 | 1 | 6 |
| 134 | B1 | 2 | 1 | 2,3,4,7,8,9 | 2 | 1 | 6 |
| 135 | B1 | 2 | 1 | 8,9 | 2 | 2 | 6 |
| 136 | B1 | 2 | 1 | 7,9 | 2 | 1 | 6 |
| 137 | B1 | 2 | 1 | 7,9 | 8 | 1 | 3 |
| 138 | B1 | 2 | 1 | 4,9 | 8 | 1 | 3 |
| 139 | B1 | 2 | 1 | 4,9 | 2 | 2 | 6 |
| 140 | B1 | 2 | 1 | 9 | 2 | 1 | 6 |
| 141 | B1 | 1 | 0 | 9 | 2 | 2 | 6 |
| 142 | B1 | 1 | 0 | 9 | 8 | 1 | 3 |
| 143 | B1 | 1 | 0 | 9 | 2 | 1 | 6 |
| 144 | B1 | 1 | 0 | 8,9 | 2 | 2 | 6 |
| 145 | B1 | 1 | 0 | 4,9 | 2 | 1 | 6 |
| 146 | B1 | 1 | 0 | 7,9 | 2 | 1 | 6 |
| 147 | B1 | 1 | 0 | 3,4,8,9 | 2 | 1 | 6 |
| 148 | B1 | 1 | 0 | 3,4,8,9 | 2 | 2 | 6 |
| 149 | B1 | 1 | 0 | 4,5,6,7,8,9 | 8 | 1 | 3 |
| 150 | B1 | 1 | 0 | 5,7,9 | 2 | 1 | 6 |
| 151 | B4 | 16 | 1 | 9 | 0 | 2 | 1 |
| 152 | B4 | 8 | 1 | 9 | 0 | 2 | 1 |
| 153 | B4 | 4 | 1 | 9 | 2 | 1 | 1 |
| 154 | B4 | 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 1 |
| 155 | B4 | 2 | 1 | 8,9 | 0 | 2 | 1 |
| 156 | B4 | 2 | 1 | 7,9 | 2 | 1 | 1 |
| 157 | B4 | 2 | 1 | 4,9 | 2 | 1 | 1 |
| 158 | B4 | 2 | 1 | 4,9 | 0 | 2 | 1 |
| 159 | B4 | 2 | 1 | 9 | 2 | 1 | 1 |
| 160 | B4 | 1 | 0 | 9 | 0 | 2 | 1 |
| 161 | B4 | 1 | 0 | 9 | 2 | 1 | 1 |
| 162 | B4 | 1 | 0 | 8,9 | 0 | 2 | 1 |
| 163 | B4 | 1 | 0 | 4,9 | 2 | 1 | 1 |
| 164 | B4 | 1 | 0 | 7,9 | 2 | 1 | 1 |
| 165 | B4 | 1 | 0 | 3,4,8,9 | 2 | 1 | 1 |
| 166 | B4 | 1 | 0 | 3,4,8,9 | 0 | 2 | 1 |
| 167 | B4 | 1 | 0 | 4,5,6,7,8,9 | 2 | 1 | 1 |
| 168 | B4 | 1 | 0 | 5,7,9 | 2 | 1 | 1 |
| 169 | A1/B1 | 2 | 1 | 8,9 | 2 | 2 | 6 |
| 170 | A1/B1 | 2 | 1 | 7,9 | 2 | 1 | 6 |
| 171 | A1/B1 | 2 | 1 | 7,9 | 8 | 1 | 3 |
| 172 | A1/B1 | 2 | 1 | 4,9 | 8 | 1 | 3 |
| 173 | A1/B1 | 2 | 1 | 4,9 | 2 | 2 | 6 |
| 174 | A1/B1 | 2 | 1 | 9 | 2 | 1 | 6 |
| 175 | A1/B1 | 1 | 0 | 9 | 2 | 2 | 6 |
| 176 | A1/B1 | 1 | 0 | 9 | 8 | 1 | 3 |
| 177 | A1/B1 | 1 | 0 | 9 | 2 | 1 | 6 |
| 178 | A1/B1 | 1 | 0 | 8,9 | 2 | 2 | 6 |
| 179 | A1/B1 | 1 | 0 | 4,9 | 2 | 1 | 6 |
| 180 | A1/B1 | 1 | 0 | 7,9 | 2 | 1 | 6 |
| 181 | A1/B1 | 1 | 0 | 3,4,8,9 | 2 | 2 | 6 |
| 182 | A1/B1 | 1 | 0 | 4,5,6,7,8,9 | 8 | 1 | 3 |
| 183 | A1/B1 | 1 | 0 | 5,7,9 | 2 | 1 | 6 |
| 184 | A2/B2 | 2 | 1 | 8,9 | 0 | 2 | 3 |
| 185 | A2/B2 | 2 | 1 | 7,9 | 6 | 1 | 2 |
| 186 | A2/B2 | 2 | 1 | 4,9 | 6 | 1 | 2 |
| 187 | A2/B2 | 2 | 1 | 4,9 | 0 | 2 | 3 |
| 188 | A2/B2 | 2 | 1 | 9 | 0 | 1 | 3 |
| 189 | A2/B2 | 1 | 0 | 9 | 0 | 2 | 3 |
| 190 | A2/B2 | 1 | 0 | 9 | 6 | 1 | 2 |
| 191 | A2/B2 | 1 | 0 | 9 | 0 | 1 | 3 |
| 192 | A2/B2 | 1 | 0 | 8,9 | 0 | 2 | 3 |
| 193 | A2/B2 | 1 | 0 | 4,9 | 0 | 1 | 3 |
| 194 | A2/B2 | 1 | 0 | 7,9 | 0 | 1 | 3 |
| 195 | A2/B2 | 1 | 0 | 3,4,8,9 | 0 | 1 | 3 |
| 196 | A2/B2 | 1 | 0 | 3,4,8,9 | 0 | 2 | 3 |
| 197 | A2/B2 | 1 | 0 | 4,5,6,7,8,9 | 6 | 1 | 2 |
| 198 | A2/B2 | 1 | 0 | 5,7,9 | 0 | 1 | 3 |
| 199 | A3/B3 | 2 | 1 | 8,9 | 0 | 2 | 2 |
| 200 | A3/B3 | 2 | 1 | 7,9 | 0 | 1 | 2 |
| 201 | A3/B3 | 2 | 1 | 7,9 | 2 | 1 | 2 |
| 202 | A3/B3 | 2 | 1 | 4,9 | 2 | 1 | 2 |
| 203 | A3/B3 | 2 | 1 | 4,9 | 0 | 2 | 2 |
| 204 | A3/B3 | 2 | 1 | 9 | 0 | 1 | 2 |
| 205 | A3/B3 | 1 | 0 | 9 | 0 | 2 | 2 |
| 206 | A3/B3 | 1 | 0 | 9 | 2 | 1 | 2 |
| 207 | A3/B3 | 1 | 0 | 9 | 0 | 1 | 2 |
| 208 | A3/B3 | 1 | 0 | 8,9 | 0 | 2 | 2 |
| 209 | A3/B3 | 1 | 0 | 4,9 | 0 | 1 | 2 |
| 210 | A3/B3 | 1 | 0 | 7,9 | 2 | 1 | 2 |
| 211 | A3/B3 | 1 | 0 | 3,4,8,9 | 0 | 2 | 2 |
| 212 | A3/B3 | 1 | 0 | 4,5,6,7,8,9 | 2 | 1 | 2 |
| 213 | A3/B3 | 1 | 0 | 5,7,9 | 0 | 1 | 2 |
| 214 | C0 | 16 | 1 | 9 | 2 | 2 | 6 |
| 215 | C0 | 8 | 1 | 9 | 2 | 2 | 6 |
| 216 | C0 | 4 | 1 | 9 | 2 | 1 | 6 |
| 217 | C0 | 2 | 1 | 2,3,4,7,8,9 | 2 | 1 | 6 |
| 218 | C0 | 2 | 1 | 8,9 | 2 | 2 | 6 |
| 219 | C0 | 2 | 1 | 7,9 | 2 | 1 | 6 |
| 220 | C0 | 2 | 1 | 7,9 | 8 | 1 | 3 |
| 221 | C0 | 2 | 1 | 4,9 | 8 | 1 | 3 |
| 222 | C0 | 2 | 1 | 4,9 | 2 | 2 | 6 |
| 223 | C0 | 2 | 1 | 9 | 2 | 1 | 6 |
| 224 | C0 | 1 | 0 | 9 | 2 | 2 | 6 |
| 225 | C0 | 1 | 0 | 9 | 8 | 1 | 3 |
| 226 | C0 | 1 | 0 | 9 | 2 | 1 | 6 |
| 227 | C0 | 1 | 0 | 8,9 | 2 | 2 | 6 |
| 228 | C0 | 1 | 0 | 4,9 | 2 | 1 | 6 |
| 229 | C0 | 1 | 0 | 7,9 | 2 | 1 | 6 |
| 230 | C0 | 1 | 0 | 3,4,8,9 | 2 | 1 | 6 |
| 231 | C0 | 1 | 0 | 3,4,8,9 | 2 | 2 | 6 |
| 232 | C0 | 1 | 0 | 4,5,6,7,8,9 | 8 | 1 | 3 |
| 233 | C0 | 1 | 0 | 5,7,9 | 2 | 1 | 6 |
| 234 | C2 | 16 | 1 | 9 | 2 | 2 | 2 |
| 235 | C2 | 8 | 1 | 9 | 2 | 2 | 2 |
| 236 | C2 | 4 | 1 | 9 | 2 | 1 | 2 |
| 237 | C2 | 2 | 1 | 2,3,4,7,8,9 | 2 | 1 | 2 |
| 238 | C2 | 2 | 1 | 8,9 | 2 | 2 | 2 |
| 239 | C2 | 2 | 1 | 7,9 | 2 | 1 | 2 |
| 240 | C2 | 2 | 1 | 7,9 | 8 | 1 | 1 |
| 241 | C2 | 2 | 1 | 4,9 | 8 | 1 | 1 |
| 242 | C2 | 2 | 1 | 4,9 | 2 | 2 | 2 |
| 243 | C2 | 2 | 1 | 9 | 2 | 1 | 2 |
| 244 | C2 | 1 | 0 | 9 | 2 | 2 | 2 |
| 245 | C2 | 1 | 0 | 9 | 8 | 1 | 1 |
| 246 | C2 | 1 | 0 | 9 | 2 | 1 | 2 |
| 247 | C2 | 1 | 0 | 8,9 | 2 | 2 | 2 |
| 248 | C2 | 1 | 0 | 4,9 | 2 | 1 | 2 |
| 249 | C2 | 1 | 0 | 7,9 | 2 | 1 | 2 |
| 250 | C2 | 1 | 0 | 3,4,8,9 | 2 | 1 | 2 |
| 251 | C2 | 1 | 0 | 3,4,8,9 | 2 | 2 | 2 |
| 252 | C2 | 1 | 0 | 4,5,6,7,8,9 | 8 | 1 | 1 |
| 253 | C2 | 1 | 0 | 5,7,9 | 2 | 1 | 2 |
| 254 | C2 | 8 | 1 | 9 | 8 | 2 | 1 |
| 255 | C2 | 4 | 1 | 9 | 8 | 1 | 1 |

Appendix 2

============ Text proposal 38.211, section 6.3.3.2 =======================

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60KHz slot | Number of PRACH occasions within a RACH slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | [2] | 6 |
| 1 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 3 |
| 2 | A1 | 1 | 0 | 24,29,34,39 | 0 | [2] | 6 |
| 3 | A1 | 1 | 0 | 24,29,34,39 | 7 | 1 | 3 |
| 4 | A1 | 1 | 0 | 18,19,38,39 | 0 | [2] | 6 |
| 5 | A1 | 1 | 0 | 18,19,38,39 | 7 | 1 | 3 |
| 6 | A1 | 1 | 0 | 0,1,2,…,39 | 0 | [1] | 6 |
| 7 | A1 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 3 |
| 8 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | [1] | 6 |
| 9 | A1 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 3 |
| 10 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | [1] | 6 |
| 11 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 3 |
| 12 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | [1] | 6 |
| 13 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | [2] | 6 |
| 14 | A1 | 1 | 0 | 7,15,23,31,39 | 7 | 1 | 3 |
| 15 | A1 | 16 | [1] | 4,9,14,19,24,29,34,39 | 0 | [2] | 6 |
| 16 | A1 | 16 | [1] | 3,7, 11,15, 19,23, 27,31, 35,39 | [0] | 1 | [6] |
| 17 | A1 | 8 | [1] | 4,9,14,19,24,29,34,39 | 0 | [2] | 6 |
| 18 | A1 | 8 | [1] | 3,7,11,15,19,23,27,31,35,39 | [0] | 1 | [6] |
| 19 | A1 | 4 | [1] | 4,9,14,19,24,29,34,39 | 0 | [2] | 6 |
| 20 | A1 | 4 | [1] | 3,7,11,15,19,23,27,31,35,39 | [0] | 1 | [6] |
| 21 | A1 | 2 | [1] | 4,9,14,19,24,29,34,39 | 0 | [2] | 6 |
| 22 | A1 | 2 | [1] | 3,7,11,15,19,23,27,31,35,39 | [0] | 1 | [6] |
| 23 | A1 | [1] | [0] | 23,31,39 | 0 | 1 | 6 |
| 24 | A1 | [1] | [0] | 1,3,5,7,…,37,39 | 7 | 1 | 3 |
| 25 | A1 | 1 | 0 | 7,11,15,19,27,31,35,39 | 0 | 1 | 6 |
| 26 | A1 | 1 | 0 | 27,31,35,39 | 0 | 1 | 6 |
| 27 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 28 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 9 | 1 | 1 |
| 29 | A2 | 1 | 0 | 24,29,34,39 | 0 | 2 | 3 |
| 30 | A2 | 1 | 0 | 24,29,34,39 | 9 | 1 | 1 |
| 31 | A2 | 1 | 0 | 18,19,38,39 | 0 | 2 | 3 |
| 32 | A2 | 1 | 0 | 18,19,38,39 | 9 | 1 | 1 |
| 33 | A2 | 1 | 0 | 0,1,2,…,39 | 0 | 1 | 3 |
| 34 | A2 | 1 | 0 | 0,1,2,…,39 | 9 | 1 | 1 |
| 35 | A2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 3 |
| 36 | A2 | 1 | 0 | 23,27,31,35,39 | 9 | 1 | 1 |
| 37 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 38 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 9 | 1 | 1 |
| 39 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 3 |
| 40 | A2 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 3 |
| 41 | A2 | 1 | 0 | 7,15,23,31,39 | 9 | 1 | 1 |
| 42 | A2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 43 | A2 | 16 | 1 | 3,7, 11,15, 19,23, 27,31, 35,39 | 0 | 1 | 3 |
| 44 | A2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 45 | A2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 46 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 47 | A2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 48 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 |
| 49 | A2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 |
| 50 | A2 | 1 | 0 | 23,31,39 | 0 | 1 | 3 |
| 51 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 9 | 1 | 1 |
| 52 | A2 | 1 | 0 | 7,11,15,19,27,31,35,39 | 0 | 1 | 3 |
| 53 | A2 | 1 | 0 | 27,31,35,39 | 0 | 1 | 3 |
| 54 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 55 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 1 |
| 56 | A3 | 1 | 0 | 24,29,34,39 | 0 | 2 | 2 |
| 57 | A3 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 |
| 58 | A3 | 1 | 0 | 18,19,38,39 | 0 | 2 | 2 |
| 59 | A3 | 1 | 0 | 18,19,38,39 | 7 | 1 | 1 |
| 60 | A3 | 1 | 0 | 0,1,2,…,39 | 0 | 1 | 2 |
| 61 | A3 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 |
| 62 | A3 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 |
| 63 | A3 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 1 |
| 64 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 65 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 |
| 66 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 |
| 67 | A3 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 2 |
| 68 | A3 | 1 | 0 | 7,15,23,31,39 | 7 | 1 | 1 |
| 69 | A3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 70 | A3 | 16 | 1 | 3,7, 11,15, 19,23, 27,31, 35,39 | 0 | 1 | 2 |
| 71 | A3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 72 | A3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 73 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 74 | A3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 75 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 76 | A3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 77 | A3 | 1 | 0 | 23,31,39 | 0 | 1 | 2 |
| 78 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 7 | 1 | 1 |
| 79 | A3 | 1 | 0 | 7,11,15,19,27,31,35,39 | 0 | 1 | 2 |
| 80 | A3 | 1 | 0 | 27,31,35,39 | 0 | 1 | 2 |
| 81 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 82 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 |
| 83 | B1 | 1 | 0 | 24,29,34,39 | 2 | 2 | 6 |
| 84 | B1 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 |
| 85 | B1 | 1 | 0 | 18,19,38,39 | 2 | 2 | 6 |
| 86 | B1 | 1 | 0 | 18,19,38,39 | 8 | 1 | 3 |
| 87 | B1 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 6 |
| 88 | B1 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 |
| 89 | B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 |
| 90 | B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 |
| 91 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 92 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 |
| 93 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 |
| 94 | B1 | 1 | 0 | 7,15,23,31,39 | 2 | 2 | 6 |
| 95 | B1 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 3 |
| 96 | B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 97 | B1 | 16 | 1 | 3,7, 11,15, 19,23, 27,31, 35,39 | 2 | 1 | 6 |
| 98 | B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 99 | B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 100 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 101 | B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 102 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 103 | B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 104 | B1 | 1 | 0 | 23,31,39 | 2 | 1 | 6 |
| 105 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 8 | 1 | 3 |
| 106 | B1 | 1 | 0 | 7,11,15,19,27,31,35,39 | 2 | 1 | 6 |
| 107 | B1 | 1 | 0 | 27,31,35,39 | 2 | 1 | 6 |
| 108 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 109 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 1 |
| 110 | B4 | 1 | 0 | 24,29,34,39 | 0 | 2 | 1 |
| 111 | B4 | 1 | 0 | 24,29,34,39 | 2 | 1 | 1 |
| 112 | B4 | 1 | 0 | 18,19,38,39 | 0 | 2 | 1 |
| 113 | B4 | 1 | 0 | 18,19,38,39 | 2 | 1 | 1 |
| 114 | B4 | 1 | 0 | 0,1,2,…,39 | 0 | 1 | 1 |
| 115 | B4 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 1 |
| 116 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 1 |
| 117 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 1 |
| 118 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 |
| 119 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 |
| 120 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 1 |
| 121 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 1 |
| 122 | B4 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 1 |
| 123 | B4 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 124 | B4 | 16 | 1 | 3,7, 11,15, 19,23, 27,31, 35,39 | 0 | 1 | 1 |
| 125 | B4 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 126 | B4 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 |
| 127 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 128 | B4 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 |
| 129 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 |
| 130 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 |
| 131 | B4 | 1 | 0 | 23,31,39 | 0 | 1 | 1 |
| 132 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 1 |
| 133 | B4 | 1 | 0 | 7,11,15,19,27,31,35,39 | 0 | 1 | 1 |
| 134 | B4 | 1 | 0 | 27,31,35,39 | 0 | 1 | 1 |
| 135 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 136 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 |
| 137 | C0 | 1 | 0 | 24,29,34,39 | 2 | 2 | 6 |
| 138 | C0 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 |
| 139 | C0 | 1 | 0 | 18,19,38,39 | 2 | 2 | 6 |
| 140 | C0 | 1 | 0 | 18,19,38,39 | 8 | 1 | 3 |
| 141 | C0 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 6 |
| 142 | C0 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 |
| 143 | C0 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 |
| 144 | C0 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 |
| 145 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 146 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 |
| 147 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 |
| 148 | C0 | 1 | 0 | 7,15,23,31,39 | 2 | 2 | 6 |
| 149 | C0 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 3 |
| 150 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 151 | C0 | 16 | 1 | 3,7, 11,15, 19,23, 27,31, 35,39 | 2 | 1 | 6 |
| 152 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 153 | C0 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 154 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 155 | C0 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 156 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 157 | C0 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 158 | C0 | 1 | 0 | 23,31,39 | 2 | 1 | 6 |
| 159 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 8 | 1 | 3 |
| 160 | C0 | 1 | 0 | 7,11,15,19,27,31,35,39 | 2 | 1 | 6 |
| 161 | C0 | 1 | 0 | 27,31,35,39 | 2 | 1 | 6 |
| 162 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 163 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 1 |
| 164 | C2 | 1 | 0 | 24,29,34,39 | 2 | 2 | 2 |
| 165 | C2 | 1 | 0 | 24,29,34,39 | 8 | 1 | 1 |
| 166 | C2 | 1 | 0 | 18,19,38,39 | 2 | 2 | 2 |
| 167 | C2 | 1 | 0 | 18,19,38,39 | 8 | 1 | 1 |
| 168 | C2 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 2 |
| 169 | C2 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 1 |
| 170 | C2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 2 |
| 171 | C2 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 1 |
| 172 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 173 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 1 |
| 174 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 2 |
| 175 | C2 | 1 | 0 | 7,15,23,31,39 | 2 | 2 | 2 |
| 176 | C2 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 1 |
| 177 | C2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 178 | C2 | 16 | 1 | 3,7, 11,15, 19,23, 27,31, 35,39 | 2 | 1 | 2 |
| 179 | C2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 180 | C2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 181 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 182 | C2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 183 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 |
| 184 | C2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 185 | C2 | 1 | 0 | 23,31,39 | 2 | 1 | 2 |
| 186 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 8 | 1 | 1 |
| 187 | C2 | 1 | 0 | 7,11,15,19,27,31,35,39 | 2 | 1 | 2 |
| 188 | C2 | 1 | 0 | 27,31,35,39 | 2 | 1 | 2 |
| 189 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 190 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 |
| 191 | A1/B1 | 1 | 0 | 24,29,34,39 | 2 | 2 | 6 |
| 192 | A1/B1 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 |
| 193 | A1/B1 | 1 | 0 | 18,19,38,39 | 2 | 2 | 6 |
| 194 | A1/B1 | 1 | 0 | 18,19,38,39 | 8 | 1 | 3 |
| 195 | A1/B1 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 6 |
| 196 | A1/B1 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 |
| 197 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 |
| 198 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 |
| 199 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 200 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 |
| 201 | A1/B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 |
| 202 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 2 | 2 | 6 |
| 203 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 8 | 1 | 3 |
| 204 | A1/B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 |
| 205 | A1/B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 |
| 206 | A1/B1 | 1 | 0 | 23,31,39 | 2 | 1 | 6 |
| 207 | A1/B1 | 1 | 0 | 1,3,5,7,…,37,39 | 8 | 1 | 3 |
| 208 | A1/B1 | 1 | 0 | 7,11,15,19,27,31,35,39 | 2 | 1 | 6 |
| 209 | A1/B1 | 1 | 0 | 27,31,35,39 | 2 | 1 | 6 |
| 210 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 3 |
| 211 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 6 | 1 | 2 |
| 212 | A2/B2 | 1 | 0 | 24,29,34,39 | 2 | 2 | 3 |
| 213 | A2/B2 | 1 | 0 | 24,29,34,39 | 6 | 1 | 2 |
| 214 | A2/B2 | 1 | 0 | 18,19,38,39 | 2 | 2 | 3 |
| 215 | A2/B2 | 1 | 0 | 18,19,38,39 | 6 | 1 | 2 |
| 216 | A2/B2 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 3 |
| 217 | A2/B2 | 1 | 0 | 0,1,2,…,39 | 6 | 1 | 2 |
| 218 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 3 |
| 219 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 6 | 1 | 2 |
| 220 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 |
| 221 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 6 | 1 | 2 |
| 222 | A2/B2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 3 |
| 223 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 2 | 2 | 3 |
| 224 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 6 | 1 | 2 |
| 225 | A2/B2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 3 |
| 226 | A2/B2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 |
| 227 | A2/B2 | 1 | 0 | 23,31,39 | 2 | 1 | 3 |
| 228 | A2/B2 | 1 | 0 | 1,3,5,7,…,37,39 | 6 | 1 | 2 |
| 229 | A2/B2 | 1 | 0 | 7,11,15,19,27,31,35,39 | 2 | 1 | 3 |
| 230 | A2/B2 | 1 | 0 | 27,31,35,39 | 2 | 1 | 3 |
| 231 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 232 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 |
| 233 | A3/B3 | 1 | 0 | 24,29,34,39 | 0 | 2 | 2 |
| 234 | A3/B3 | 1 | 0 | 24,29,34,39 | 2 | 1 | 2 |
| 235 | A3/B3 | 1 | 0 | 18,19,38,39 | 0 | 2 | 2 |
| 236 | A3/B3 | 1 | 0 | 18,19,38,39 | 2 | 1 | 2 |
| 237 | A3/B3 | 1 | 0 | 0,1,2,…,39 | 0 | 1 | 2 |
| 238 | A3/B3 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 2 |
| 239 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 |
| 240 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 2 |
| 241 | A3/B3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 242 | A3/B3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 |
| 243 | A3/B3 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 |
| 244 | A3/B3 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 2 |
| 245 | A3/B3 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 2 |
| 246 | A3/B3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 |
| 247 | A3/B3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 |
| 248 | A3/B3 | 1 | 0 | 23,31,39 | 0 | 1 | 2 |
| 249 | A3/B3 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 2 |
| 250 | A3/B3 | 1 | 0 | 7,11,15,19,27,31,35,39 | 0 | 1 | 2 |
| 251 | A3/B3 | 1 | 0 | 27,31,35,39 | 0 | 1 | 2 |