3GPP TSG RAN WG1 Meeting #92bis	 		R1-1804355

# Sanya, China, 16th – 20th April 2018

Agenda item:	7.1.2.1.2

Source: 	Samsung

Title: 	Issues on codebook based UL transmission

Document for:	Discussion and Decision

# Introduction

The following agreement was made in RAN1 NR-AH #1801 [1].

Agreement

- For codebook based uplink transmission, SRS resources in the same resource set should have the same time domain behavior on periodic, aperiodic and semi-persistent SRS

- Send the above as part of LS to RAN2

In RAN1#92, there were further discussions about time domain behavior of SRS resources in a SRS resource set. This contribution provides our view on this issue.

# SRS time domain behavior

Based on the offline summary [2], the following alternatives were discussed during RAN1#92.

- Alt 1: UE can be configured with one SRS resource set for codebook based UL transmission with only one type of time-domain behaviour (agreement made in RAN1 NR-AH #1801)

- Alt 2: UE can be configured with multiple SRS resource sets for codebook based UL transmission, each with only one type of time-domain behaviour

- Alt 3: UE can be configured with one SRS resource set for codebook based UL transmission, each SRS resource can have a SRS-SlotConfig parameter for periodic/semi-persistent transmission and can be triggered by SRS request field.

Alt 1 was agreed because it is simple, hence preferred over a more complex solution such as Alt 2 or Alt 3.

Both Alt 2 and Alt 3, though provide more flexibility, are unclear to provide any significant performance gain over Alt 1. Also, the following issues were identified at least for Alt 3 (they may also be applicable to Alt 2):

- If Alt 3 is also used for non-codebook based UL transmission, then it can complicate the association of CSI-RS resource to a SRS resource.

- It is unclear why aperiodic SRS is not included.

Alt 2, on the other hand, can be argued to be simpler than Alt 3 (since similar functionality can be achieved by allowing, e.g., up to two SRS resource sets) with less specification impact on the existing parameters such as SRS-SlotConfig. But again, it is not clear how much benefit it can bring when compared with Alt 1.

In summary, in our view, there is no critical issue with Alt 1, and there is no clear understanding about how much benefit can be achieved with either Alt 2 or Alt 3 over Alt 1. Such improvements require more discussion, hence can be discussed in future release.

Proposal: Further improvements to time domain behaviour of SRS resources for codebook-based UL transmission is not supported in Rel. 15; such improvements can be discussed in future release.

# 3	Conclusion

In this contribution, the following proposal is made regarding codebook-based UL transmission:

- Further improvements to time domain behaviour of SRS resources for codebook-based UL transmission is not supported in Rel. 15; such improvements can be discussed in future release.

# Reference

- 3GPP, RAN1 NR-AH#1801, Chairman’s Notes

- R1-1803321, Offline Discussion Summary on Codebook Based UL Transmission, Intel