3GPP TSG RAN WG1 Meeting #93 	R1-1805907

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.4.1

Source:	Huawei, HiSilicon

Title:	Discussion on the design of SCMA

Document for:	Discussion and decision

- Introduction

In Rel-15 NR Non-orthogonal Multiple Access (NOMA) SID, the following objectives have been identified for the study of NoMA transmitter side signal processing [1]:

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]:

  - Modulation and symbol level processing, including spreading, repetition, interleaving, new constellation mapping, etc.

  - Coded bit level processing including interleaving and/or scrambling, etc.

  - Symbol to resource element mapping, sparse or not, etc.

  - Demodulation reference signal. Other signal is not excluded.

In this contribution, we discuss the design of NoMA transmitter design. In particular, we introduce the design of Sparse Code Multiple Access (SCMA), and explain the motivation and benefit in the design.

- SCMA Transmitter Side Design

A NoMA transmitter maps a stream of coded binary bits of a UE to the available transmission resources by some user-specific operations to facilitate decoding of the superposed multi-user data at the receiver side with reasonable complexity.

For the bit level operations, since NR has already supported  a UE specific scrambling that can be reused in any NoMA scheme, we focus on the symbol level NoMA design that can be generalized into two logical mapping functions: bits-to-symbols and symbols-to-REs.

SCMA design aims to create a more “structured” symbol dependency among a sequence of symbols via joint modulation and spreading in the bits-to-symbols mapping, and introduce sparsity over REs (resource element) allocation via spreading and layering in the symbols-to-REs mapping for suppressing the ISI (inter-user interference). Thereby, a NoMA receiver can benefit from both the symbol-level dependency and the sparse mapping for UE separation, especially when the NoMA overloading ratio is high.

A SCMA transmitter strictly follows the unified framework agreed in RAN1 #86bis [2][3].

Figure 1. Transmit side design of SCMA based on the unified framework agreed in RAN1 #86bis

  - Bits-to-Symbols Mapping of SCMA

The symbol-level dependency is attributed to the flexibility to optimize the constellations across multiple symbols. Figure 2 shows an example of 16-point SCMA constellation over two symbols. By adjusting the labeling of the input binary bits to the constellation points for each RE (resource element), a NoMA transmitter can optimize the overall distances (Euclidean/product).

In contrast, a linear spreading with symbol-independent modulation (sequence-based nor repetition based spreading) has no means to improve the overall distance property. This explains the link-level performance gain of the 16-point SCMA modulation against linear spreading over 16QAM over two REs in Figure 3 (More details about the simulation parameters can be found in table A-1 in the appendix). Although the gain stands regardless of the spreading factor, larger spreading factor would bring about more degrees of freedom to optimize the distances, thereby leading to larger gain over linear spreading.

Figure 2. Example of 2-symbol mapping for 16-point constellation

| Figure 3. Example of single user performance comparison between the 2-symbol mapping and linear spreading (repetition or sequence based) in both AWGN and fading channels |  |
| --- | --- |

Observation 1: The structured symbol dependency in the bits-to-symbols mapping of SCMA provides more flexibility and better distance properties than sequence or repetition based linear spreading.

The M-bit to N-symbol mapping of SCMA transmitter can be represented by a N×2M table in which each column represents the symbol sequence in term of an index of the input bit stream. These tables for 8-point, 16-point and 64-point modulations with spreading length of 2 are given in tables A-2 to A-4 in the appendix.

The same mapping function can also be presented by a formula expressing the relation between the input bit stream b and the output symbol sequence x. For example, the formula of 8-point table in table A-2 in the appendix is

x=23-12j0-120j(1-2b).

Compared with a linear spreading over one symbol QAM modulation, the 8-point constellation modulation of SCMA maps three binary bits over two symbols, as shown in Figure 4 (correspondent table A-2). This equips NoMA scheme with a finer granularity on the optimization and configuration over the modulation sizes and FEC coding rates, the benefit of which is shown in Figure 5: an 8-point SCMA modulation outperforms a linear spreading over both QPSK and 16QAM modulations.  Details about the simulation parameters are in Table A-5.

Figure 4. Example of joint mapping of three bits to two REs

| Figure 5. Example of single user performance of 8-point joint mapping vs linear spreading (sequence or repetition based) in AWGN and fading channels |  |
| --- | --- |

Observation 2: The bits-to-symbols mapping design in SCMA provides more modulation granularity than the sequence or repetition based linear spreading.

For further user separation, more choices of the joint mapping sequences for a given bit stream (equivalently more M-bits to N-symbols mapping matrices) can be generated by casting some N-by-N transform matrix. For instance of N = 2, this 2-by-2 transform matrix can be one of

1001, 100-1, 100j, 100-j.

  - Symbols-to-REs mapping of SCMA

SCMA utilizes the symbols-to-REs mapping to further mitigate the inter-user interference and facilitate the low complexity implementation of the NoMA receivers. UE specific sparse patterns at configurable sparsity level are introduced as follows:

- Sparsity level is designated as the ratio of the non-zero elements to all available REs in a giving spreading block. The SCMA transmitter configures it for optimal performance by adjusting the channel coding gain and the MUD mitigation capability.

- Sparse pattern defines the locations of the non-zero elements in the group of REs of a spreading block.  The pattern can be repeated throughout the available bandwidth. For example, the sparse patterns of sparse level 50% over the spreading block of size 4 are

1100,0011,1010,0101,1001,0110.

In practice, the symbols-to-REs mapping can also be integrated with the bits-to-symbols mapping to generate the symbol sequence (including zeros) directly from a sequence of coded bits and then map them to the available REs sequentially from an input bit stream.

Observation 3: UE-specific sparse spreading of SCMA provides good tradeoff between the coding gain and the multi-user interference mitigation capability.

  - UE Separation

As discussed above, SCMA can adopt UE-specific operations in the two mapping operations in favor of UE separation:

- UE-specific bits-to-symbols mapping: UE-specific transform matrix

- UE specific symbols-to-REs mapping: UE-specific sparse pattern

The UE separation herein mainly facilitates the MUD receiver, and is different from the UE identification and activity detection that are jointly performed with other procedures such as DMRS based UE detection. More details can be found in [4].

- Features and Benefits of SCMA Transmitter

  - Flexibility and Scalability

SCMA transmitter provides sufficient flexibility and configurability for various NR scenarios such as mMTC, URLLC, and eMBB, specifically in the three following dimensions:

- Bits-to-Symbols mapping dimension: SCMA comes up with the constellations with larger distances and finer granularity than symbol-independent one. It offers the flexibility to extend the spreading length above 2.

- Symbols-to-REs mapping dimension: SCMA sparse spreading comes up with the optimal tradeoff between the coding gain and multi-user interference mitigation capability by adjusting the spreading length, and the UE-specific sparsity levels and patterns.

- Layered transmission dimension: SCMA accommodates multiple layers (rate splitting) for a single UE to achieve higher SE (spectrum efficiency) more efficiently. The diagram of multi-layer SCMA is given in Figure 6, where each layer is configured with layer-specific bits-to-symbols mapping and symbols-to-REs mapping.

Figure 6. Diagram of SCMA transmitter side design with multi-layer transmission.

Moreover, SCMA design supports both waveform types of CP-OFDM and DFT-s-OFDM. For DFT-s-OFDM, one example of design is to have the transform precoding (DFT precoding matrix) between the modulated symbol sequence generator and symbol-to-RE mapping blocks in Figure 1.

In summary, by configuring the design parameters, one can adapt the SCMA transmission scheme based on the system requirements in each scenario.

Observation 4: SCMA provides flexible and scalable solution for NoMA transmission to address versatile KPIs in different application scenarios.

  - Performance

The structured symbol dependency in SCMA offers the best and finest tradeoffs between the coding gain and spreading gain, and between the good distance properties and the inter-user interference mitigation capability, which can be utilized by the NoMA receiver (which includes multi-user detector and channel decoder as described in [5]) to improve the overall performance. Moreover, SCMA design allows for near-optimal multi-user decoding such as MPA with reasonable implementation cost and using low-complexity receivers (like EPA [5]) even for high overloading scenarios. The following curves in Figure 7 show some example of SCMA performance compared to some other NoMA schemes.

More detailed simulation results capturing the performance of SCMA in link-level can be found in [6]-[8]. As a summary of the features and benefits described above, SCMA is a suitable candidate for NoMA transmission in NR to satisfy different performance metrics for all application scenarios.

Proposal 1: Further study and evaluation on SCMA should be performed during the Rel-15 NoMA SI.

| (a). mMTC, TDLA-30ns, 1T2R, ICE, Equal SNR | (b). mMTC, TDLA-30ns, 1T4R, ICE, Equal SNR |
| --- | --- |
| (c). mMTC, TDLA-30ns, 1T2R, ICE, Unequal SNR | (d). mMTC, TDLA-30ns, 1T4R, ICE, Unequal SNR |
| (e). URLLC, TDLA-30ns, 1T2R, ICE, Equal SNR | (f). URLLC, TDLA-30ns, 1T4R, ICE, Equal SNR |
| (g). eMBB, TDLA-30ns, 1T4R, ICE, Equal SNR | (h). eMBB, TDLA-30ns, 1T4R, ICE, Unequal SNR |

Figure 7. Performance comparison of SCMA and some other NoMA schemes

Conclusions

In this contribution, we introduce the detailed transmitter design for SCMA and made the following observations and proposals.

Observation 1: The structured symbol dependency in the bits-to-symbols mapping of SCMA provides more flexibility and better distance properties than sequence or repetition based linear spreading.

Observation 2: The bits-to-symbols mapping design in SCMA provides more modulation granularity than the sequence or repetition based linear spreading.

Observation 3: UE-specific sparse spreading of SCMA provides good tradeoff between the coding gain and the multi-user interference mitigation capability.

Observation 4: SCMA provides flexible and scalable solution for NoMA transmission to address versatile KPIs in different application scenarios.

Proposal 1: Further study and evaluation on SCMA should be performed during the Rel-15 NoMA SI.

References

RP-170829, “New SID on Non-Orthogonal Multiple Access (NoMA) for NR”, RAN#75, Dubrovnik, Croatia, March 6 - 9, 2017.

RAN1 Chairman’s notes, RAN1#86b, Lisbon, Portugal, October, 10-15, 2016.

3GPP TR 38.802: “Study on New Radio (NR) Access Technology; Physical Layer Aspects”.

R1-1805909, “Discussion on NoMA related procedures”, Huawei, HiSilicon, RAN1#93, Busan, Korea, May 21st – 25th, 2018.

R1-1805908, “Discussion on the design of NoMA receiver”, RAN1#93, Busan, Korea, May 21st – 25th, 2018.

R1-1805911, “Preliminary LLS evaluation results for mMTC scenario”, Huawei, HiSilicon, RAN1#93, Busan, Korea, May 21st – 25th, 2018.

R1-1805912, “Preliminary LLS evaluation results for uRLLC scenario”, Huawei, HiSilicon, RAN1#93, Busan, Korea, May 21st – 25th, 2018.

R1-1805913, “Preliminary LLS evaluation results for eMBB scenario”, Huawei, HiSilicon, RAN1#93, Busan, Korea, May 21st – 25th, 2018.

Appendix

Table A-1. Simulation Assumptions for Figure 3.

| Parameter | Value |
| --- | --- |
| Evaluated schemes | Two-dimensional 16-point shown in Figure 3 vs. linear spreading of 16QAM over two symbols |
| Channel model | SISO AWGN and SIMO 1x2 TDLA-30 channel |
| Channel coding | Turbo code with rate 0.5 |
| Number of data tones | 864 (equivalent to 6 LTE RBs, including the DM-RS overhead) |
| Transport Block size | 864 (including CRC bits, Spectral Efficiency = 1) |

Table A-2. Mapping function for the 8-point modulated symbol sequence with length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| Output symbol sequence | -a8+jb8-a8+jb8 | a8+jb8a8+jb8 | -a8-jb8-a8+jb8 | a8-jb8a8+jb8 | -a8+jb8-a8-jb8 | a8+jb8a8-jb8 | -a8-jb8-a8-jb8 | a8-jb8a8-jb8 |

with a8=0.5774 and b8=0.8165.

Table A-3. Mapping function for the 16-point modulated symbol sequence with length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 0000 | 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 |
| Output Symbol sequence | 3+3j1+j | -1+3j3+j | 1+3j-3+j | -3+3j-1+j | 3-j1+3j | -1-j3+3j | 1-j-3+3j | -3-j-1+3j |
| Sequence index | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Corresponding bit sequence | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| Output Symbol sequence | 3+j1-3j | -1+j3-3j | 1+j-3-3j | -3+j-1-3j | 3-3j1-j | -1-3j3-j | 1-3j-3-j | -3-3j-1-j |

Table A-4. Mapping function for the 64-point modulated symbol sequence with length 2

| Sequence index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding bit sequence | 000 000 | 000 001 | 000 010 | 000 011 | 000 100 | 000 101 | 000 110 | 000 111 |
| Output symbol sequence | 1+j1+j | 1+3j1+3j | 1-j1+j | 1-3j1+3j | 3+j3+j | 3+3j3+3j | 3-j3+j | 3-3j3+3j |
| Sequence index | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Corresponding bit sequence | 001 000 | 001 001 | 001 010 | 001 011 | 001 100 | 001 101 | 001 110 | 001 111 |
| Output symbol sequence | -1+j1+j | -1+3j1+3j | -1-j1+j | -1-3j1+3j | -3+j3+j | -3+3j3+3j | -3-j3+j | -3-3j3+3j |
| Sequence index | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Corresponding bit sequence | 010 000 | 010 001 | 010 010 | 010 011 | 010 100 | 010 101 | 010 110 | 010 111 |
| Output symbol sequence | 1+j1-j | 1+3j1-3j | 1-j1-j | 1-3j1-3j | 3+j3-j | 3+3j3-3j | 3-j3-j | 3-3j3-3j |
| Sequence index | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| Corresponding bit sequence | 011 000 | 011 001 | 011 010 | 011 011 | 011 100 | 011 101 | 011 110 | 011 111 |
| Output symbol sequence | -1+j1-j | -1+3j1-3j | -1-j1-j | -1-3j1-3j | -3+j3-j | -3+3j3-3j | -3-j3-j | -3-3j3-3j |
| Sequence index | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| Corresponding bit sequence | 100 000 | 100 001 | 100 010 | 100 011 | 100 100 | 100 101 | 100 110 | 100 111 |
| Output Symbol sequence | 1+j-1+j | 1+3j-1+3j | 1-j-1+j | 1-3j-1+3j | 3+j-3+j | 3+3j-3+3j | 3-j-3+j | 3-3j-3+3j |
| Sequence index | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| Corresponding bit sequence | 101 000 | 101 001 | 101 010 | 101 011 | 101 100 | 101 101 | 101 110 | 101 111 |
| Output Symbol sequence | -1+j-1+j | -1+3j-1+3j | -1-j-1+j | -1-3j-1+3j | -3+j-3+j | -3+3j-3+3j | -3-j-3+j | -3-3j-3+3j |
| Sequence index | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
| Corresponding bit sequence | 110 000 | 110 001 | 110 010 | 110 011 | 110 100 | 110 101 | 110 110 | 110 111 |
| Output Symbol sequence | 1+j-1-j | 1+3j-1-3j | 1-j-1-j | 1-3j-1-3j | 3+j-3-j | 3+3j-3-3j | 3-j-3-j | 3-3j-3-3j |
| Sequence index | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |
| Corresponding bit sequence | 111 000 | 111 001 | 111 010 | 111 011 | 111 100 | 111 101 | 111 110 | 111 111 |
| Output Symbol sequence | -1+j-1-j | -1+3j-1-3j | -1-j-1-j | -1-3j-1-3j | -3+j-3-j | -3+3j-3-3j | -3-j-3-j | -3-3j-3-3j |

Table A-5. Simulation Assumptions for Figure 5.

| Parameter | Value |
| --- | --- |
| Evaluated schemes | Single-UE performance for Joint mapping of 8-point shown in Figure 6 to two tones with two zero tones vs. linear spreading of QPSK and 16QAM and over 4 symbols |
| Channel model | SISO AWGN and SIMO 1x2 TDLA-30 channel |
| Spectral Efficiency | 0.25 per tone |
| Number of data tones | 864 (equivalent to 6 LTE RBs, including the DM-RS overhead) |
| Transport Block size | 216 (including the CRC bits) |