3GPP TSG RAN WG1 Meeting #93	R1-1805927

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.8.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for eMBB spectral efficiency

Document for:	Discussion

Introduction

In this contribution, the initial evaluation results for eMBB spectral efficiency (average and the 5th percentile user spectral efficiency) are provided.

Test environments for eMBB spectral efficiency

According to Report ITU-R M.2410, average spectral efficiency and 5th percentile user spectral efficiency are used to evaluate the achievement for three times spectral efficiency improvement in ITU-R vision (see Recommendation ITU-R M.2083). The eMBB test environments and their related evaluation configurations are employed in Report ITU-R M.2412 to test the fulfillment of the spectral efficiency requirements. In this contribution, a subset of the eMBB test environments and the related evaluation configurations as given in Table 1 is considered for initial spectral efficiency evaluation.

TABLE 1 Evaluation configuration for initial spectral efficiency evaluation

| Test env. | Evaluation configuration | Carrier frequency | ISD | Remark |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB | Config. A | 4 GHz | 20 m |  |
| Dense Urban – eMBB | Config. A | 4 GHz | 200 m | Macro layer only |
| Rural - eMBB | Config. A | 700 MHz | 1 732 m |  |
|  | Config. C (LMLC) | 700 MHz | 6 000 m |  |

Evaluated technical features

The IMT-2020 eMBB spectral efficiency requirement is three times higher compared to IMT-Advanced. Therefore, it is a challenging requirement and the NR features that could contribute to the improvement of spectral efficiency should be selected.

On the other hand, it is recalled that for LTE-Advanced evaluation towards IMT-Advanced submission, the evaluation is basically applied based on duplexing schemes, i.e., to FDD and TDD, respectively. This is due to the fact that, duplexing scheme is one of the fundamental features among the other features that impact spectral efficiency performance.

Duplexing scheme

In NR design, the flexible duplexing scheme is available, e.g.,

- Different transmission directions in either part of a paired spectrum,

- TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing.

In this document, the following duplexing schemes are considered

- FDD (for paired spectrum): NR features that can be applied to FDD operation are invoked.

- TDD (for unpaired spectrum): NR features that can be applied to TDD operation are invoked.

Spectral efficiency calculation for duplexing scheme

The spectral efficiency of different duplexing schemes can be calculated according to Report ITU-R M.2412.

For DL average spectral efficiency and 5th percentile spectral efficiency,

- In case of FDD, the simulation bandwidth is 10 MHz for DL and 10 MHz for UL. The DL average spectral efficiency is given by

(1)

where W is the DL bandwidth of 10 MHz; Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control and DL reference signals on the DL bandwidth of 10 MHz is taken into account when deriving Ri (T); and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

- For TDD, the simulation bandwidth is 20 MHz for DL and UL. The DL average spectral efficiency is given by (1), where W is the effective DL bandwidth that accounts for the time-frequency resource used for DL transmission (including GP symbols); Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control, DL reference signal on the DL effective bandwidth is taken into account; and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

For UL average spectral efficiency and 5th percentile spectral efficiency, similar way is employed to derive the evaluation results for these two metrics.

Guard band ratio for different bandwidth

To reflect the benefit of reduced guard band ratio for larger bandwidth, i.e. when the system bandwidth is larger than simulation bandwidth (10 MHz in FDD and 20 MHz in TDD), the spectral efficiency can be derived from Eq. (2) [1]

(2)

where  is the guard band ratio at given number of RB N and  is calculated by Eq. (1). For FDD,  for 10 MHz simulation bandwidth and 15 kHz subcarrier spacing. For TDD,  for 20 MHz simulation bandwidth and 30 kHz subcarrier spacing. The number of RB for larger bandwidth for FR1 and FR2 is shown in Table 2 and Table 3, respectively.

TABLE 2 Maximum number of RBs and guard band ratio per SCS and bandwidth for FR1

| SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
|  | 10.00% | 6.40% | 5.20% | 4.60% | 4.24% | [4.00%] | 2.80% | 2.80% | N.A | N.A | N.A | N.A | N.A |
| 30 | 11 | 24 | 38 | 51 | 65 | [78] | 106 | 133 | 162 | [189] | 217 | [245] | 273 |
|  | 20.80% | 13.60% | 8.80% | 8.20% | 6.40% | [6.40%] | 4.60% | 4.24% | 2.80% | [2.80%] | 2.35% | [2.00%] | 1.72% |
| 60 | N.A | 11 | 18 | 24 | 31 | [38] | 51 | 65 | 79 | [93] | 107 | [121] | 135 |
|  | N.A | 20.80% | 13.60% | 13.60% | 10.72% | [8.80%] | 8.20% | 6.40% | 5.20% | [4.34%] | 3.70% | [3.20%] | 2.80% |

TABLE 3 Maximum number of RBs and guard band ratio per SCS and bandwidth for FR2

| SCS [kHz] | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
|  | 4.96% | 4.96% | 4.96% | 4.96% |
| 120 | 32 | 66 | 132 | 264 |
|  | 7.84% | 4.96% | 4.96% | 4.96% |

PDCCH overhead for larger bandwidth

For the overhead reduction in the case of larger bandwidth, more discussions are needed to derive the spectral efficiency, especially for PDCCH overhead reduction. One example calculation is given below. By assuming M0 OFDM symbols for PDCCH at the bandwidth BW0, the number of OFDM symbol for PDCCH at bandwidth BW could be

M= BW0/ BW×M0

For example, if we assume M0 = 2 for 20 MHz bandwidth system, then M=1 for 40 MHz bandwidth system. The value of M could be a non-integer since NR supports PDCCH sharing with PDSCH. According to [1], this can be considered for NR evaluation.

In this evaluation, the same overhead is assumed to be the same for different bandwidth in the same duplexing mode.

Initial evaluation results

Based on the above discussion, initial evaluation results for average spectral efficiency and 5th percentile spectral efficiency is evaluated. For the evaluation, non-ideal channel estimation is considered. The detailed evaluation assumptions can be found in Appendix.

DL spectral efficiency evaluation results

The initial DL spectral efficiency evaluation results for NR are given in Table 4 and Table 5.

TABLE 4 Initial FDD DL spectral efficiency evaluation for different system bandwidth

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | BW=20 MHz | BW=50 MHz | Req. | BW=10 MHz | BW=20 MHz | BW=50 MHz | Req. |
| Indoor Hotspot | Config. A (4 GHz); 32T4R | 10.24 | 10.44 | 10.84 | 9 | 0.33 | 0.34 | 0.35 | 0.3 |
| Dense Urban | Config. A (4 GHz); 32T4R | 10.37 | 10.57 | 10.98 | 7.8 | 0.34 | 0.35 | 0.36 | 0.225 |
| Rural | Config. A (700 MHz; 8T2R | 6.18 | 6.30 | 6.54 | 3.3 | 0.14 | 0.14 | 0.15 | 0.12 |
|  | Config. C (700 MHz); 8T4R | 6.96 | 7.09 | 7.37 | 3.3 | 0.17 | 0.17 | 0.18 | 0.12 |

TABLE 5 Initial TDD DL spectral efficiency evaluation for different system bandwidth

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. |
| Indoor Hotspot | Config. A (4 GHz); 32T4R | 11.6 | 12.05 | 12.91 | 9 | 0.34 | 0.35 | 0.38 | 0.3 |
| Dense Urban | Config. A (4 GHz); 32T4R | 11.62 | 12.08 | 12.93 | 7.8 | 0.33 | 0.34 | 0.37 | 0.225 |
| Rural | Config. A (700 MHz; 8T2R | 6.90 | 7.17 | 7.68 | 3.3 | 0.15 | 0.16 | 0.17 | 0.12 |
|  | Config. C (700 MHz); 8T4R | 6.92 | 7.19 | 7.70 | 3.3 | 0.16 | 0.17 | 0.18 | 0.12 |

Observation 1: NR with sub 6 GHz can fulfill DL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

UL spectral efficiency evaluation results

The initial UL spectral efficiency evaluation results for NR are given in Table 6 and Table 7. Observation is made based on these initial evaluation results.

TABLE 6 Initial FDD UL spectral efficiency evaluation

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=10 MHz | BW=20 MHz | BW=40 MHz | Req. | BW=10 MHz | BW=20 MHz | BW=40 MHz | Req. |
| Indoor Hotspot | (Config. A 4 GHz; 2T16R) | 6.13 | 6.25 | 6.49 | 6.75 | 0.29 | 0.3 | 0.31 | 0.21 |
|  | (Config. A 4 GHz; 2T32R) | 7.99 | 8.14 | 8.46 | 6.75 | 0.50 | 0.51 | 0.53 | 0.21 |
| Dense Urban | (Config. A 4 GHz; 2T16R) | 5.86 | 5.97 | 6.20 | 5.4 | 0.24 | 0.24 | 0.25 | 0.15 |
| Rural | (Config. A 700 MHz; 1T8R) | 3.72 | 3.79 | 3.94 | 1.6 | 0.12 | 0.12 | 0.13 | 0.045 |

TABLE 7 Initial TDD UL spectral efficiency evaluation

| Test env. | Evaluation Config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. | BW=20 MHz | BW=40 MHz | BW=100 MHz | Req. |
| Indoor Hotspot | (Config. A 4 GHz; 2T32R) | 6.89 | 7.16 | 7.67 | 6.75 | 0.39 | 0.41 | 0.43 | 0.21 |
| Dense Urban | (Config. A 4 GHz; 2T32R) | 5.97 | 6.20 | 6.64 | 5.4 | 0.28 | 0.29 | 0.31 | 0.15 |
| Rural | (Config. A 700 MHz; 2T8R) | 4.61 | 4.79 | 5.13 | 1.6 | 0.10 | 0.10 | 0.11 | 0.045 |

Observation 2: NR with sub 6 GHz can fulfill UL spectral efficiency requirements in all test environments.

Conclusion

In this document, the initial evaluation of eMBB spectral efficiency for NR operating on frequency range 1 (FR1, i.e., below 6 GHz) is provided. Based on the evaluation, the following observations are made.

Observation 1: NR with sub 6 GHz can fulfill DL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

Observation 2: NR with sub 6 GHz can fulfill UL spectral efficiency requirements in all test environments.

- Reference

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei, February 2018.

- Appendix: Evaluation assumptions

Table A-1 Evaluation assumptions for DL

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |
| Evaluation configuration |  | Configuration A | Configuration A | Configuration A, Configuration C |
| Channel model |  | InH_A | UMa_A | RMa_A |
| ISD |  | 20 m (12TRxP) | 200 m | Configuration A: 1732 m Configuration C: 6000 m |
| TDD frame structure |  | DDDSU | DDDSU | DDDSU |
| Carrier Frequency |  | 4 GHz | 4 GHz | 700 MHz |
| System bandwidth |  | TDD: 20MHz | TDD: 20MHz | TDD: 20MHz |
|  |  | FDD:10MHz | FDD:10MHz | FDD:10MHz |
| Subcarrier spacing |  | FDD: 15 kHz TDD: 30 kHz | FDD: 15 kHz TDD: 30 kHz | FDD: 15 kHz TDD: 30 kHz |
| Symbols number per slot |  | 14 | 14 | 14 |
| Number of antenna elements per TRxP |  | 32Tx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 64Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP |  | 32TXRU | For 32TXRU: Vertical 2-to-8 For 16TXRU: Vertical 1-to-8 | 8TXRU Vertical 1-to-8 |
| Number of antenna elements per UE |  | 4Rx with 0°,90° polarization | 4Rx or 2Rx with 0°,90° polarization | Configuration A: 2Rx Configuration C: 4Rx with 0°,90° polarization |
| Transmit power per TRxP |  | TDD: 24 dBm | TDD: 44 dBm | TDD: 49 dBm |
|  |  | FDD: 21 dBm | FDD: 41 dBm | FDD: 46 dBm |
| TRxP number per site |  | 1 | 3 | 3 |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 90deg in LCS | 105deg in LCS | Configuration A: 100deg in LCS Configuration C: 92deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF | MU-PF | MU-PF |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 4 adaptation per user; Maximum MU layer = 12 | MU-MIMO with rank 4 adaptation per user Maximum MU layer = 12 | MU-MIMO with rank 2/4 adaptation per user Maximum MU layer = 8 |
| Guard band ratio |  | TDD: 8.2% (for 20 MHz) | TDD: 8.2% (for 20 MHz) | TDD: 8.2% (for 20 MHz) |
|  |  | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) |
| BS receiver type |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| CSI feedback |  | 5 slots period based on CSI-RS with delay | 5 slots period based on CSI-RS with delay | 5 slots period based on CSI-RS with delay |
| Precoder derivation |  | TDD: SRS based | TDD: SRS based | TDD: SRS based |
|  |  | FDD: NR Type II codebook based | FDD: NR Type II codebook based | FDD: NR Type II codebook based |
| Overhead | PDCCH | 2 complete symbols | 2 complete symbols | 2 complete symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) | Type II, based on MU-layer (dynamic in simulation) | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | FDD: 32 ports per 5 slots | FDD: 32 ports per 5 slots | FDD: 8 ports per 5 slots |
|  |  | TDD: 4 ports per UE per 5 slots | TDD: 4 ports per UE per 5 slots | TDD: 2/4 ports per UE per 5 slots |
|  | SSB | [1 SSB per 20 ms] | [1 SSB per 20 ms] | [1 SSB per 20 ms] |
|  | TRS | 2 consecutive slots per 20ms, 1 port, 50PRB | 2 consecutive slots per 20ms, 1 port, 50PRB | 2 consecutive slots per 20ms, 1 port, 50PRB |
| Channel estimation |  | Non-ideal | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM | OFDM |

Table A-2 Evaluation assumptions for UL

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Test environment |  | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |
| Evaluation configuration |  | Configuration A | Configuration A | Configuration A |
| Channel model |  | InH_A | UMa_A | RMa_A |
| Subcarrier spacing |  | FDD: 15 kHz TDD: 30 kHz | FDD: 15 kHz TDD: 30 kHz | FDD: 15 kHz TDD: 30 kHz |
| TDD frame structure |  | DDDSU | DDDSU | DDDSU |
| Symbols number per slot |  | 14 | 14 | 14 |
| Number of antenna elements per TRxP |  | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 64Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP |  | 32TXRU Vertical 1-to-1 16TXRU, Vertical 1-to-2 | 32TXRU Vertical 2-to-8 16TXRU Vertical 1-to-8 | 8TXRU Vertical 1-to-8 |
| Number of antenna elements per UE |  | 2Tx with 0°,90° polarization | 2Tx with 0°,90° polarization | FDD: 1Tx TDD: 2Tx with 0°,90° polarization |
| UE power class |  | 23 dBm | 23 dBm | 23 dBm |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 90deg in LCS | 105deg in LCS | 100deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | MU-PF | MU-PF | SU-PF |
| MIMO mode |  | SU-MIMO with rank 2/4 adaptation | SU-MIMO with rank 2/4 adaptation | SIMO for FDD; SU-MIMO with rank 4 adaptation for TDD |
| BS receiver type |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| UE precoder scheme |  | Codebook based | Codebook based | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay |
| Power control |  | α=0.9， P0=-86 dBm | α=0.6， P0=-60 dBm | α=0.8， P0=-76 dBm |
| Power backoff model |  | Continuous RB allocation: follow TS 38.101; Non-continuous RB allocation: additional 2 RB reduction | Continuous RB allocation: follow TS 38.101; Non-continuous RB allocation: additional 2 RB reduction | Continuous RB allocation: follow TS 38.101; Non-continuous RB allocation: additional 2 RB reduction |
| Overhead | PUCCH | 2 RBs, 14 OFDM symbols | 2 RBs, 14 OFDM symbols | 2 RBs, 14 OFDM symbols |
|  | DMRS | Type II, 1 additional DMRS symbol, and no FDM with PUSCH | Type II, 1 additional DMRS symbol, and no FDM with PUSCH | Type II, 1 additional DMRS symbol, and no FDM with PUSCH |
|  | SRS | 2 symbols per 5 slots, 8 RBs per symbol | 2 symbols per 5 slots, 8 RBs per symbol | 2 symbols per 5 slots, 8 RBs per symbol |
| Channel estimation |  | Non-ideal | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM | OFDM |