3GPP TSG RAN WG1 Meeting #93	R1-1805928

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.8.2

Source:	Huawei, HiSilicon

Title:	IMT-2020 initial evaluation results for user experienced data rate

Document for:	Discussion and decision

Introduction

In this contribution, the consideration on self evaluation for user experienced data rate is provided.

Definition of user experienced data rate

In Report ITU-R M.2410 (see [1]), the following definition and requirements of user experienced data rate are defined in subsection 4.3.

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time. In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by: Ruser = W × SEuser (3) In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

Evaluation configuration and technical features

It is observed from the ITU definition that user experienced data rate is used to test the proposal’s capability on providing the 5th percentile user throughput. Single-band or multiple-band configuration can be used for evaluation.

For NR deployment, it would be good that NR can reach high user experienced data rate (i.e., the 5th percentile user throughput) by using the same inter-site distance (ISD) as existing systems (e.g., LTE). However, if NR is deployed only on higher frequency band, it might be challenging to use the same ISD to provide the desired user experienced data rate.

It is noted that in Rel-15 NR WI, NR-LTE spectrum sharing using a TDD band and a supplementary uplink (SUL) band is enabled to enhance the NR uplink coverage to guarantee good UL user experienced data rate under the same ISD as LTE. By this means, NR is expected to appropriately use the two bands to provide the target user experienced data rate using the same ISD.

The above mechanism can be tested using Configuration C of Dense Urban – eMBB test environment as defined in Report ITU-R M.2412 (see [2]), where the macro layer only layout (ISD = 200m) with two operating bands can be used. As comparison, Configuration A (macro layer only with low frequency band) and Configuration B (macro layer only with high frequency band) are also considered for evaluation.

Therefore, it is considered to evaluate the configurations and technical features as shown in Table 1 for UL user experienced data rate.

TABLE 1

Evaluation configuration and technical feature for UL user experienced data rate

| Test environment | Evaluation configuration | Network layout | Carrier frequency | Evaluated feature for NR | Remarks |
| --- | --- | --- | --- | --- | --- |
| Dense Urban – eMBB | Configuration A | Macro layer only ISD=200m | 4 GHz | NR FDD / TDD | NR UEs are operated on 4 GHz. |
|  | Configuration B | Macro layer only ISD=200m | 30 GHz | NR TDD | NR UEs are operated on 30 GHz. |
|  | Configuration C | Macro layer only ISD=200m | 4 GHz (SUL band) and 30 GHz (TDD band) | SUL+TDD band | Part of NR UEs are operated on 4 GHz, and part of NR UEs are operated on 30 GHz. |

Specifically, in NR/LTE uplink sharing, the NR UEs can transmit UL data on either TDD band (higher frequency band) or SUL band (lower frequency band). By this means, the NR users that are relatively far from NR TRxP can benefit from the lower frequency band to achieve good channel condition and therefore keep good throughput (data rate) under the same ISD as previously deployed systems (e.g., LTE).

Proposal 1: NR multi-band operation with SUL+TDD band is evaluated for UL user experienced data rate in addition to NR FDD/TDD with a single band.

Proposal 2: Configuration C with macro layer operating on 4 GHz (SUL band) and 30 GHz (TDD band) is employed for SUL+TDD evaluation, in addition to Configuration A and B for NR FDD/TDD evaluation.

Evaluation methodology

According to Report ITU-R M.2412, the following evaluation methodology is defined.

For one frequency band and one TRxP layer, user experienced data rate is derived analytically from the 5th percentile user spectral efficiency according to equation (3) defined in Report ITU-R M.2410. The bandwidth used should be reported by the proponent. In case of multi-layer configuration, system-level simulation is used. In this case, the single user data rate may be aggregated over layers and/or bands. The user experienced data rate is derived from the 5th percentile point of the CDF of single user data rate.

Evaluation method for NR FDD/TDD with single band

From the above description we can see that for one frequency band and one TRxP layer, user experienced data rate can be derived from equation (3) using the supported system bandwidth and the 5th percentile user spectral efficiency.

Evaluation method for SUL+TDD band

For NR multi-band operation with SUL+TDD band, the following evaluation method is considered in accordance with the evaluation methodology defined in Report ITU-R M.2412.

It is assumed that an NR UE can camp on either frequency band of a macro TRxP. Denote the k-th UE’s throughput (data rate) as Rk, and it camps on frequency band fi (i=1 or 2) on uplink. The simulation bandwidth for frequency band fi is SBWi. The simulated user throughput on the simulation bandwidth is SRk for the k-th UE. Assume the supported system bandwidth on frequency band fi is BWi, then, similar to equation (3), the k-th UE’s throughput Rk is given by

Rk = SRk×Ci 									(2)

where Ci = BWi / SBWi with SBWi being the simulation bandwidth for frequency band fi and BWi being the supported system bandwidth on frequency band fi (i=1 or 2).

After the k-th user’s throughput Rk is derived, we can collect all the users’ throughput to form the CDF of the user throughput, and get the 5th percentile point of the CDF as the user experienced data rate.

The detailed evaluation procedure of user experienced data rate for the NR multi-band operation with SUL+TDD band can be summarized as below.

Proposal 3: For evaluation of NR multi-band operation with SUL+TDD band, use the following procedure as evaluation method

- Step1: Drop users in the network and each user selects its serving cell and frequency band fi (i=1 or 2) based on a predefined criteria. gNB configures UE’s frequency band fi (i=1 or 2). - Step2: Data transmission simulation on each frequency band. - Step3: Collect each user’s simulated throughput SRk on specific band. - Step4: For specific frequency band, calculate each user’s throughput Rk based on the assumption of supported system bandwidth according to equation (2). - Step5: Generate the CDF of user throughput from all users on frequency band f1 and f2, and take the 5th percentile point of the user throughput CDF as the user experienced data rate.

Initial evaluation results

Based on the above discussion, the initial evaluation results for UL user experienced data rate are presented. Evaluation assumptions can be found in Appendix.

In Table 1, the 5%-tile simulated user throughput for NR FDD, NR TDD and NR SUL+TDD case are presented. It is seen that the NR multi-band operation with SUL+TDD band demonstrate good  performance compared to NR FDD and NR TDD on a single band. Specifically, the initial evaluation shows that it would be challenging for NR to provide good coverage (“cell edge” user throughput) in Configuration B. (Note that further optimization is possible for Configuration B.)

TABLE 1

User throughput under simulation bandwidth

| Evaluated features | Evaluation configuration | Carrier frequency for fi (i=1 or 2) | Simulation bandwidth SBWi on fi | Simulated user throughput SRk |
| --- | --- | --- | --- | --- |
| NR FDD | Configuration A | f1=4 GHz | 10 MHz | 2.46 Mbps |
| NR TDD | Configuration B | f1=30 GHz | 80 MHz | 0.12 Mbps |
| NR SUL+TDD band | Configuration C | f1=4 GHz (SUL band) and f2=30 GHz (TDD band) | 4GHz:10 MHz 30GHz:80 MHz | 6.57 Mbps |

In Table 2, the initial evaluation results of UL user experienced data rate are derived from the evaluation method as given in Section 4. It shows that UL user experienced data rate can reach 50Mbps requirement with NR multi-band operation by using a SUL band (4 GHz) with 80 MHz bandwidth and a TDD band (30 GHz) with 600 MHz bandwidth. Compared to NR single band operation, it reduces the required bandwidth that achieves 50 Mbps target from 200 MHz to 80 MHz for 4 GHz, and it improves the UL experienced data rate for 30 GHz from 1 Mbps to 50 Mbps under the bandwidth of 600 MHz.

TABLE 2

Initial UL user experienced data rate evaluation

| Evaluated features | Evaluation configuration | Carrier frequency for fi (i=1 or 2) | Supported bandwidth SBWi on fi | UL user experienced data rate (Mbps) |
| --- | --- | --- | --- | --- |
| NR FDD | Configuration A | f1=4 GHz | 200 MHz | 49.2 |
| NR TDD | Configuration B | f1=30 GHz | 600 MHz | 0.9 |
| NR SUL+TDD band | Configuration C | f1=4 GHz (SUL band) and f2=30 GHz (TDD band) | 4GHz:80MHz 30GHz:600MHz | 50.3 |

Observation 1: NR multi-band operation with SUL+TDD band can fulfill the UL user experienced data rate with reasonable bandwidth at 30GHz (TDD band) and much smaller bandwidth for 4GHz (SUL band) compared with NR single band operation.

Observation 2: NR multi-band operation with SUL+TDD band can improve the user experienced data rate by using less bandwidth on 4 GHz and 30 GHz compared to NR single band operation.

Conclusion

In this paper, consideration of user experienced data rate evaluation is presented. Based on the discussion, the following proposals are made.

Proposal 1: NR multi-band operation with SUL+TDD band is evaluated for UL user experienced data rate in addition to NR FDD/TDD with a single band.

Proposal 2: Configuration C with macro layer operating on 4 GHz (SUL band) and 30 GHz (TDD band) is employed for SUL+TDD evaluation, in addition to Configuration A and B for NR FDD/TDD evaluation.

Proposal 3: For evaluation of NR multi-band operation with SUL+TDD band, use the following procedure as evaluation method

- Step1: Drop users in the network and each user selects its serving cell and frequency band fi (i=1 or 2) based on a predefined criteria. gNB configures UE’s frequency band fi (i=1 or 2). - Step2: Data transmission simulation on each frequency band. - Step3: Collect each user’s simulated throughput SRk on specific band. - Step4: For specific frequency band, calculate each user’s throughput Rk based on the assumption of supported system bandwidth according to equation (2). - Step5: Generate the CDF of user throughput from all users on frequency band f1 and f2, and take the 5th percentile point of the user throughput CDF as the user experienced data rate.

Based on the initial evaluation results, it is observed that,

Observation 1: NR multi-band operation with SUL+TDD band can fulfill the UL user experienced data rate with reasonable bandwidth at 30GHz (TDD band) and much smaller bandwidth for 4GHz (SUL band) compared with NR single band operation.

Observation 2: NR multi-band operation with SUL+TDD band can improve the user experienced data rate by using less bandwidth on 4 GHz and 30 GHz compared to NR single band operation.

- References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- Appendix: Evaluation assumptions

Table A-1 Evaluation assumptions for UL user experienced data rate

| Parameters | Value |  |  |
| --- | --- | --- | --- |
| Test environment | Dense Urban -eMBB |  |  |
| Evaluation configuration | Configuration A: Macro layer with 4 GHz (FDD) Configuration B: Macro layer with 30 GHz (TDD with DL:UL = 4:1) Configuration C: Macro layer with 4 GHz (SUL) + 30 GHz (TDD with DL:UL = 4:1) |  |  |
| Network layout | Macro layer only |  |  |
| Carrier frequency for evaluation | 4 GHz | 30 GHz |  |
| Simulation bandwidth | FDD:10MHz | TDD: 80MHz |  |
| Subcarrier spacing | 30 kHz | 60kHz |  |
| Symbols number per slot | 14 |  |  |
| Channel model | UMa_B |  |  |
| Inter-site distance | 200 m |  |  |
| BS antenna height | 25 m |  |  |
| UE power class | 23 dBm |  |  |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss |  |  |
| Inter-site distance | 200 m |  |  |
| Number of antenna elements per TRxP | 128Tx/Rx = (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.5)λ. +45°, -45° polarization | 256Tx/Rx = (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |  |
| Number of TXRU per TRxP | 16TXRU=(Mp,Np,P,Mg,Ng) =(1,8,2,1,1) | 8TXRU=(Mp,Np,P,Mg,Ng) =(1,1,2,2,2) |  |
| Number of UE antenna elements | 2Tx/Rx = (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°,90° polarization | 32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |  |
| Number of TXRU per UE | 2TXRU=(Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) | 4TXRU=(Mp,Np,P,Mg,Ng)=(1,1,2,1,2) |  |
| Device deployment | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |  |  |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |  |  |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |  |  |
| Inter-site interference modeling | Explicitly modelled |  |  |
| BS noise figure | 5 dB | 7 dB |  |
| UE noise figure | 7 dB | 10 dB |  |
| BS antenna element gain | 8 dBi |  |  |
| UE antenna element gain | 0 dBi | 5 dBi |  |
| Thermal noise level | -174 dBm/Hz |  |  |
| Traffic model | Full buffer |  |  |
| UE density | 10 UEs per TRxP |  |  |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |  |
| TRP number per site | 3 |  |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |  |  |
| Electronic tilt | 105deg in LCS |  | (According to Zenith angle in "Beam set at TRxP") |
| TRxP boresight | 30degree/150degree/270degree |  |  |
| UT attachment | Based on RSRP (formula) from CRS port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. For SUL+TDD evaluation, around 30% of the users (the edge users) are offloaded to 4GHz layer |  |  |
| Wrapping around method | Geographical distance based wrapping |  |  |
| Minimum distance of TRxP and UE | Follow corresponding channel model variant |  |  |
| Polarized antenna model | Model-2 in TR36.873 |  |  |
| Beam set at TRxP for 30 GHz (Constraints for the range of selective analog beams per TRxP) | - | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. 2D DFT beam for beam at (φi, θj) |  |
| Beam set at UE for 30 GHz (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. 2D DFT beam for beam at (φi, θj) |  |
| Criteria for selection for serving TRxP for 30 GHz | - | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |  |
| Criteria for analog beam selection for serving TRxP for 30 GHz | - | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. |  |
| Scheduler | SU-PF |  |  |
| MIMO mode | SU-MIMO with rank 2 adaptation per user |  |  |
| BS receiver type | MMSE-IRC |  |  |
| UE precoder scheme | Codebook based |  |  |
| UL CSI derivation | SRS (ideal) based, with delay |  |  |