3GPP TSG RAN WG1 Meeting #93	                                                                 R1-1806072

Busan, Korea, May 21st – 25th, 2018

Source:	vivo

Title:	Remaining issues on bandwidth part switching

Agenda Item:	7.1.3.4.1

Document for:	Discussion and Decision

- Introduction

In RAN1 92bis, followings were agreed.

Agreements:

Confirm the following working assumption with updates:

- Sizes of all DCI bitfields in DCI formats 0-1 and 1-1 in USS determined by current BWP. Data transmitted on the BWP indicated by the BWP index. If the BWP index activates another BWP, transform as follows:

  - Zero-pad too small bitfields to match the new BWP

  - Truncate too large bitfields to match the new BWP

  - The truncation is done from MSB (including the bit indicating the resource allocation type)

  - Zero-padding is done for MSB

Although the working assumption was confirmed, some further clarification is still needed.

- Discussion

  - BWP dependent DCI field with BWP dependency

Based on the latest endoresed version of  TS38.212 and updated RRC parameters, the analysis on BWP dependency for UL and DL non-fallback DCI is shown in the following table 1 and 2, respectively

Table 1- BWP dependency for DCI format 0_1

| DCI field | Field size | BWP dependency | Comment |
| --- | --- | --- | --- |
| Carrier indicator | 0 or 3 bits | No | Same size across BWPs |
| UL/SUL indicator | 0 or 1bit | No | Same size across BWPs |
| Identifier for DCI formats | [1] bit | No | Same size across BWPs |
| Bandwidth part indicator | 0, 1 or 2 bits | No | Same size across BWPs |
| Frequency domain resource assignment | X bits | Yes | Size determined by the active DL BWP, size can be different across BWPs |
| Time domain resource assignment | 0, 1, 2, 3, or 4 bits | Yes | Configured per BWP, size can be different across BWPs |
| VRB-to-PRB mapping | 0 or 1 bit | Yes | Has dependency on the resource allocation type which is configured per BWP, size can be different across BWPs |
| Frequency hopping flag | 0 or 1 bit | Yes | Has dependency on the resource allocation type which is configured per BWP, size can be different across BWPs |
| Modulation and coding scheme | 5 bits | No | Fixed size across BWPs |
| New data indicator | 1 bit | No | Fixed size across BWPs |
| Redundancy version | 2 bits | No | Fixed size across BWPs |
| HARQ process number | 4 bits | No | Fixed size across BWPs |
| 1st downlink assignment index | 1 or 2 bits | No | Has dependency on the HARQ-ACK codebook type (semi-static or dynamic) which is configured per UE, size is the same across BWPs |
| 2nd downlink assignment index | 0 or 2 bits | No | Has dependency on the HARQ-ACK codebook type (semi-static or dynamic) which is configured per UE, size is the same across BWPs |
| TPC command for scheduled PUSCH | 2 bits | No | Same size across BWPs |
| SRS resource indicator | [0 or] or bits | Yes | Size determined by the number of SRS resource, which is configured per BWP, size can be different across BWPs, up to 4 bits difference |
| Precoding information and number of layers | 0, 1, 2, 4, 5, or 6bits | Yes | Size determined by the following configurations, which are all per BWP configuration, size can be size can be different across BWPs, up to 6 bits difference - ulTxConfig -PUSCH-tp - ULmaxRank |
| Antenna ports | 2, 3, 4 or 5 bits | Yes | Size determined by the following configurations, which are all per BWP configuration, size can be size can be different across BWPs, up to 3 bits difference - PUSCH-tp - dmrs-Uplink |
| SRS request | 2 or 3bits | No | 2bits if UE is not configured with SUL, 3bits otherwise. |
| CSI request | 0, 1, 2, 3, 4, 5, or 6 bits | Yes | Size determined by the configuration of ReportTriggerSize, which is per BWP configured. Size can be different across BWPs, up to 6 bits difference |
| CBG transmission information (CBGTI) | 0, 2, 4, 6, or 8 bits | FFS | It is under discussion whether CBG is configured per BWP or per cell |
| PTRS-DMRS association | 0 or 2bits | Yes | Size determined by the following configurations, which are all per BWP configuration, size can be size can be different across BWPs, up to 2bits difference - UL-PTRS-present -PUSCH-tp |
| beta_offset indicator | 0 or 2bits | No | Configured per serving cell, same size across BWPs |
| DMRS sequence initialization | 0 or 1bit | Yes | Size determined by the configuration of PUSCH-tp, which is per BWP configured. Size can be different across BWPs, up to 1 bit difference |

Table 2- BWP dependency for DCI format 1_1

| DCI field | Field size | BWP dependency | Comment |
| --- | --- | --- | --- |
| Carrier indicator | 0 or 3 bits | No | Configured per serving cell, same size across BWPs |
| Identifier for DCI formats | [1] bits | No | Same size across BWPs |
| Bandwidth part indicator | 0, 1 or 2 bits | No | Configured per serving cell, same size across BWPs |
| Frequency domain resource assignment | Y bits | Yes | Size determined by the active DL BWP , size can be different across BWPs |
| Time domain resource assignment | 0, 1, 2, 3, or 4 bits | Yes | Configured per BWP, size can be different across BWPs |
| VRB-to-PRB mapping | 0 or 1 bit | Yes | Has dependency on the resource allocation type which is configured per BWP, size can be different across BWPs |
| PRB bundling size indicator | 0 or 1 bit | Yes | Configured per BWP, size can be different across BWPs |
| Rate matching indicator | 0, 1, or 2bits | Yes | Configured per BWP, size can be different across BWPs |
| ZP CSI-RS trigger | 0, 1, or 2 bits | Yes | Configured per BWP, size can be different across BWPs |
| Modulation and coding scheme for TB1 | 5bits | No | Fixed size across BWPs |
| New data indicator for TB1 | 1bit | No | Fixed size across BWPs |
| Redundancy version for TB1 | 2bits | No | Fixed size across BWPs |
| Modulation and coding scheme for TB2 | 5bits | No | Fixed size across BWPs |
| New data indicator for TB2 | 1bit | No | Fixed size across BWPs |
| Redundancy version for TB2 | 2bits | No | Fixed size across BWPs |
| HARQ process number | 4bits | No | Fixed size across BWPs |
| Downlink assignment index | 0, 2,or 4bits | No | Configured per UE, Fixed size across BWPs |
| TPC command for scheduled PUCCH | 2bits | No | Fixed size across BWPs |
| PDSCH-to-HARQ_feedback timing indicator | 3 bits | No | Fixed size across BWPs 0bit under discussion |
| Antenna port(s) | 4, 5, or 6 bits | Yes | Configured per BWP, size can be different across BWPs |
| Transmission configuration indication | 0 or 3 bits | Yes | Configured per BWP, size can be different across BWPs |
| SRS request | 2 or 3bits | No | 2bits if UE is not configured with SUL, 3bits otherwise. |
| CBG transmission information (CBGTI) | 0, 2, 4, 6, or 8 bits | FFS | It is under discussion whether CBG is configured per BWP or per cell |
| CBG flushing out information (CBGFI) | 0 or 1 bit | FFS | It is under discussion whether CBG is configured per BWP or per cell |
| DMRS sequence initialization | 1 bit | No | Fixed size across BWPs |

Instead of per DCI field handling, the DCI field mismatch issue can be handled in a generic framework which is applicable to all related DCI fields. As agreed in RAN1#92bis, zero is padded from the MSB of each DCI field when more bits in a field is required for UE to interpret the DCI field in the new BWP than the current BWP, or bit truncating is performed  from the MSB of each DCI field when less bits in a field is required in the new BWP than the current BWP. In this method, for each DCI field that has BWP dependency, the network shall carefully configure the DCI state for each BWP such that no undesired behavior at the UE side during BWP switch. In addition, a default state shall be specified for each BWP dependent field and the default  bahavior is performed when UE switches from a BWP with zero bit field length to a new BWP with non-zero bit field length.

Proposal 1:

- The ‘zero-padding or truncating from MSB’ is performed in a per DCI field basis, when necessary.

Proposal 2 :

- Default states are specified for each potential BWP dependent DCI fields, which is used when UE switches from a BWP with zero bit field length to a new BWP with non-zero bit field length.

In case both type 0 and type 1 resource allocaiton are configured, there is one bit in DCI to dynamicly switch between type 0 and 1. It was agreed that the zero-padding or truncating from MSB also applies to the bit indicating the resource allocation type, therefore the UE behavior regarding the interpretation of RA type should be clarified. One simple solution is to preclude the possibility of simultaneous usage of dynamic BWP switch and dynamic RA type switching from specification. Alternatively, it can be considered to use a default RA type, e.g. type 0 when the RA type indicator bit is to be truncated or padded.

Proposal 3 :

- Adopt one of the following for dynamic RA type switching

  - Do not allow simutanoeous use of dynamic BWP switching and dynamic RA type switching

  - Type 0 RA is assumed when the RA type indicator bit is truncated or padded.

- Conclusion

In this contributions, we discuss some remaining details for bandwidth part switching based on the agreement in RAN1#92bis, and have following proposals :

Proposal 1:

- The ‘zero-padding or truncating from MSB’ is performed in a per DCI field basis, when necessary.

Proposal 2 :

- Default states are specified for each potential BWP dependent DCI fields, which is used when UE switches from a BWP with zero bit field length to a new BWP with non-zero bit field length.

Proposal 3 :

- Adopt one of the following for dynamic RA type switching

  - Do not allow simutanoeous use of dynamic BWP switching and dynamic RA type switching

  - Type 0 RA is assumed when the RA type indicator bit is truncated or padded.

- References