3GPP TSG-RAN WG1 Meeting #93	R1-1806148

Busan, Korea, May 21st – 25th 2018                                            (Revision of R1-1804632)

Agenda Item:	6.2.5.2

Source:	Ericsson

Title:	Remaining aspects of 64QAM

Document for:	Discussion, Decision

# 1			Introduction

In RAN1#92 [1], the following working assumption was made on the principle defining MCS/TBS tables.

Working assumption - TBS scaling (<1) is applied with additional MCS indices in ‘Modulation and TBS index table’ - Number of additional MCS indices is three - Additional TBS values which will be down-selected from Table 7.1.7.2.1-1 in 36.213 - FFS downselected TBS values - Select the scaling factor <1 so as to avoid reducing the peak SE (after adding additional MCS values above 28) compared to MCS 28 with scaling factor 1 - FFS the exact scaling factor.

Based on the above working assumption, the following progress was made in RAN1#92bis [2].

Agreement The following working assumption is confirmed - Single TBS scaling factor value is applied (i.e. independently of ITBS/IMCS values) to all numbers in the table. Working Assumption - Scaling factor is applied to the number of PRBs derived from SCI - Actual TBS value is derived by using scaled number of PRBs defined by the following equation , - where NPRB' is the original total number of allocated PRBs according to 7.1.6 from 3GPP 36.213 LTE R14. - No new TBS values are introduced. Agreement - TBS scaling factor value is equal to =0.8 Agreement - Agree to use reserved bit(s) in SCI format to indicate R15 PSSCH transmission format/features - Further discuss whether it is needed to separately indicate specific features (e.g. rate-matching, 64-QAM support) or transmission format

In this contribution, we present our views on the remaining issues to support 64QAM on PC5.

# 2			Discussion

- In RAN1#92, a compromising principle was discussed based on difference in opinions of companies when it comes to optimization of peak spectral efficiency. Based on this, a working assumption to use reserved MCS values (i.e. IMCS > 28) is made which allows higher peak spectral efficiency. We suggest confirming the working assumption.

- Proposal 1	Confirm the working assumption of RAN1#92.

To keep the monotonic spectral efficiency, the switching points between QPSK-16QAM and 16QAM-64QAM need to be chosen according to the selected scaling factor. As a generic design principle, it is important to design the MCS table so that most of the MCS values are usable given the new/modified TBS values. Therefore, for the scaling factor of 0.8, the modulation switching point (QPSK-16QAM) should be IMCS = 11 (corresponding to ITBS = 10) and the modulation switching point (16-QAM to 64QAM) should be IMCS = 20 (corresponding to ITBS = 18).

For scaling factor of 0.8, the modulation switching points of 16QAM to 64QAM is changed from IMCS = 21 to IMCS = 20.

- Furthermore, to make use of reserved MCS values for achieving higher peak spectral efficiency, we propose to use TBS values (30, 32, 33) corresponding to 256QAM from Table 7.1.7.2.1-1 in 36.213. Based on this, the modified MCS-TBS mapping is given in Table 1. Furthermore, IMCS ≥ 28 is used only in case of two transmissions.

- Proposal 2	Use the new mapping in Table 1 for Rel-15 transmissions with a condition that IMCS ≥ 28 is chosen for two transmission case.

Table 1. Legacy and new mappings between MCS index, TBS index, and modulation order.

| MCS Index | Legacy Mapping |  | New Mapping |  |
| --- | --- | --- | --- | --- |
|  | Modulation Order | TBS Index | Modulation Order | TBS Index |
| 0 | 2 | 0 | 2 | 0 |
| 1 | 2 | 1 | 2 | 1 |
| 2 | 2 | 2 | 2 | 2 |
| 3 | 2 | 3 | 2 | 3 |
| 4 | 2 | 4 | 2 | 4 |
| 5 | 2 | 5 | 2 | 5 |
| 6 | 2 | 6 | 2 | 6 |
| 7 | 2 | 7 | 2 | 7 |
| 8 | 2 | 8 | 2 | 8 |
| 9 | 2 | 9 | 2 | 9 |
| 10 | 2 | 10 | 2 | 10 |
| 11 | 4 | 10 | 4 | 10 |
| 12 | 4 | 11 | 4 | 11 |
| 13 | 4 | 12 | 4 | 12 |
| 14 | 4 | 13 | 4 | 13 |
| 15 | 4 | 14 | 4 | 14 |
| 16 | 4 | 15 | 4 | 15 |
| 17 | 4 | 16 | 4 | 16 |
| 18 | 4 | 17 | 4 | 17 |
| 19 | 4 | 18 | 4 | 18 |
| 20 | 4 | 19 | 6 | 18 |
| 21 | 6 | 19 | 6 | 19 |
| 22 | 6 | 20 | 6 | 20 |
| 23 | 6 | 21 | 6 | 21 |
| 24 | 6 | 22 | 6 | 22 |
| 25 | 6 | 23 | 6 | 23 |
| 26 | 6 | 24 | 6 | 24 |
| 27 | 6 | 25 | 6 | 25 |
| 28 | 6 | 26 | 6 | 26 |
| 29 | Reserved |  | 6 | 30 |
| 30 |  |  | 6 | 32 |
| 31 |  |  | 6 | 33 |

# 3			Receiver requirements

The current Rel. 14 specification captures the minimum decoding requirements for a UE under the assumption that only QPSK and 16QAM are supported.

- Proposal 3	RAN1 to revise the minimum decoding requirements, including soft buffer size, etc.

# 4			Conclusion

In this paper, we have discussed the introduction of support for 64QAM for V2X, and observed and proposed the following:

- Observation 1	For scaling factor of 0.8, the modulation switching points of 16QAM to 64QAM is changed from IMCS = 21 to IMCS = 20.

Proposal 1		Confirm the working assumption of RAN1#92.

- Proposal 2		Use the new mapping in Table 1 for Rel-15 transmissions with a condition that IMCS ≥ 28 is chosen for two transmission case.

- Proposal 3			RAN1 to revise the minimum decoding requirements, including soft buffer size, etc.

# 5			References

- RAN1#92, Chairman’s notes.

- RAN1#92bis, Chairman’s notes.