3GPP TSG-RAN WG1 Meeting #93	Tdoc R1-1806153

Busan, Korea, May 21st – 25th 2018	(Resubmission of R1-1805090)

Agenda Item:	7.5.3

Source:	Ericsson

Title:	Discussion on eV2X use cases

Document for:	Discussion, Decision

# Introduction

In the context of the SI on evaluation assumptions for eV2X, there have been discussions about the suitability and sufficiency of the urban and highway deployment models in TR 36.885 [1] for NR V2X. In parallel, there have been discussions on the data traffic models required for correct evaluation of the eV2X use cases identified by SA1 and captured in TR 22.886 [2].

In this paper, we consider the eV2X use cases listed in TR 22.886 [2] and present our views on the deployment and traffic models required for their evaluation.

# eV2X use cases

SA1 identified 25 use cases that are relevant for eV2X evaluations [2]. We have listed in Table 1-Table 4 the use cases associated with consolidated requirements together with the remaining use cases in Table 5. We have also listed the deployment and traffic models to be used for evaluating each of them.

Table 1. Use cases associated with platooning requirements.

| Use case |  | Deployment | Traffic model |
| --- | --- | --- | --- |
| 1 | eV2X support for vehicle platooning | Hw(1) | Periodic Set1: 300-400 bytes @ 30 Hz Set2: 50-1200 bytes @ 30 Hz |
| 2 | Information exchange within platoon | Hw(1) | Event-triggered 50-1200 byte |
| 5 | Automated cooperative driving for short distance grouping | Hw(1) | Periodic Set1: 300-400 bytes Set2: Up to 1200 bytes |
| 12 | Information sharing for partial/ conditional automated platooning | Hw(1) | Periodic (or event-triggered, MBB) Coop. perception (2.5 Mbps/link) Coop. maneuver (0.25 Mbps/link) |
| 13 | Information sharing for high/full automated platooning | Hw(1) | Periodic (or event-triggered, MBB) Coop. perception (50 Mbps/link) Coop. maneuver (15 Mbps/link) |

Notes:

- We consider highway as the typical model for platooning. In our view, platooning in urban scenarios is not reasonable, although this is not explicitly stated in [2].

Table 2. Use cases associated with advanced driving requirements.

| Use case |  | Deployment | Traffic model |
| --- | --- | --- | --- |
| 9 | Cooperative collision avoidance (CoCA) of connected automated vehicles | Hw, Urb | 10Mbps Event-triggered, MBB |
| 10 | Information sharing for partial/ conditional automated driving | Hw, Urb | Periodic (or event-triggered, MBB) Coop. perception (0.5 Mbps) Coop. maneuver (0.05 Mbps) |
| 11 | Information sharing for high/full automated driving | Hw, Urb | Periodic (or event-triggered, MBB) Coop. perception (50 Mbps) Coop. maneuver (3 Mbps) |
| 20 | Emergency trajectory alignment | Hw, Urb | 30Mbps Event-triggered, MBB |
| 22 | Intersection safety information provisioning for urban driving | Urb(2) | DL: 0.5Mbps, UL: 50Mbps Periodic |
| 23 | Cooperative lane change (CLC) of automated vehicles | Hw, Urb | Set 1: 300-400bytes Set 2: 12000 bytes Event-triggered |
| 25 | 3D video composition for V2X scenario | Hw, Urb | Video (UL: 10 Mbps) Periodic |

Notes:

- Restricted in the definition of the use case.

Table 3. Use case associated with extended sensors requirements.

| Use case |  | Deployment | Traffic model |
| --- | --- | --- | --- |
| 3 | Automotive: sensor and state map sharing | Hw, Urb | MBB 25 Mbps |
| 6 | Collective perception of environment | Hw, Urb | Set1: Periodic, 1600bytes Set 2: Event triggered, MBB, 1Gbps |
| 16 | Video data sharing for assisted and improved automated driving (VaD) | Hw, Urb | Set1: 10 Mbps Set2: 100-700 Mbps Periodic |

Table 4. Use case associated with remote driving requirements.

| Use case |  | Deployment | Traffic model |
| --- | --- | --- | --- |
| 21 | Teleoperated support (TeSo) | Hw, Urb | Periodic UL: 25 Mbps DL: 1 Mbps |

Table 5. Use cases not associated with any consolidated requiremement.

| Use case |  | Deployment | Traffic model |
| --- | --- | --- | --- |
| 4 | eV2X support for remote driving | Hw, Urb | Periodic DL: 1 Mbps UL: 20 Mbps |
| 7 | Communication between vehicles of different 3GPP RATs |  | Not relevant |
| 8 | Multi-PLMN environment |  | Not relevant |
| 14 | Dynamic ride sharing |  | Periodic, MBB |
| 15 | Use case on multi-RAT |  | Not relevant |
| 17 | Changing driving-mode |  | Not relevant |
| 18 | Tethering via Vehicle |  | MBB |
| 19 | Use case out of 5G coverage |  | Not relevant |
| 24 | Proposal for secure software update for electronic control unit |  | MBB |

Given the lists of use cases, we observe the following:

The urban and highway deployment models are sufficient to evaluate all the use cases identified by SA1.

Periodic, aperiodic, and MBB traffic models are sufficient to evaluate all the use cases identified by SA1.

SA1 use cases involves all types of transmissions i.e. unicast, broadcast or multicast.

- Based on this, we propose the following:

Mixed scenario considering multiple use cases are not considered as a baseline for decisions of technical solutions.

Coexistence of different traffic types is to be considered.

# Conclusion

In this paper we have observed and proposed the following:

- The urban and highway deployment models are sufficient to evaluate all the use cases identified by SA1.

Periodic, aperiodic, and MBB traffic models are sufficient to evaluate all the use cases identified by SA1.

SA1 use cases involves all types of transmissions i.e. unicast, broadcast or multicast.

- Mixed scenario considering multiple use cases are not considered as a baseline for decisions of technical solutions.

Coexistence of different traffic types is to be considered.

# References

- 3GPP TR 36.885, “Study on LTE-based V2X services”

- 3GPP TR 22.886, “Study on enhancement of 3GPP support for 5G V2X services”