3GPP TR 37.885 V0.2.0 (2018-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR;

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<LTE, NR, Vehicular communication>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Introduction	6

5	Regulation for ITS operation in frequency band above 6 GHz	7

5.1	International recommendation	7

5.2	European regulation	7

5.3	Korean regulation	8

5.4	Chinese regulation	8

6	Evaluation methodology	8

6.1	System level simulation assumptions	8

6.1.1	Evaluation scenarios	8

6.1.2	UE drop and mobility modeling	10

6.1.3	BS and RSU deployment	10

6.1.4	Antenna model	10

6.1.5	Traffic model	11

6.1.6	Performance metric	11

6.2	Channel model	12

6.2.1	Pathloss model	12

6.2.2	Shadowing model	12

6.2.3	Fast fading model	12

6.3	Link level simulation assumptions	13

6.4	Additional assumptions to evaluate vehicle positioning	13

Annex A:	Road configuration for urban grid and highway	14

Annex B:	Change history	16

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document captures the findings of the study item, "Study on evaluation methodology of new V2X use cases for LTE and NR" [2]. The purpose of this TR is to document the evaluation methodology to be used in evaluating technical solutions to support the full set of 5G V2X use cases as identified in [3] and the full set of 5G RAN requirements in [4].

This document addresses completion of the evaluation methodology in [4] and [5] to compare the performance of different technical options for the new 5G V2X use cases.

This document captures identification of the regulatory requirements and design considerations of potential operation of direct communications between vehicles in spectrum allocated to ITS beyond 6GHz in different regions.

This document is a 'living' document, i.e. it is permanently updated and presented to TSG-RAN meetings.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-170837: "New SI proposal: Study on evaluation methodology of new V2X use cases for LTE and NR".

[3]	3GPP TR 22.886: "Study on enhancement of 3GPP Support for 5G V2X Services".

[4]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

[5]	3GPP TR 38.802: "Study on New Radio Access Technology; Physical Layer Aspects".

[6] 	ITU-R M.1452-2 (05/2012), “Millimetre wave vehicular collision avoidance radars and radiocommunication systems for intelligent transport system applications”

[7] 	ECC/DEC/(09)01 The harmonised use of the 63-64 GHz frequency band for Intelligent Transport Systems (ITS)

[8] 	ETSI TR 102 400 v1.2.1(2006-07), "Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices (SRD); Intelligent Transport Systems (ITS); Road Traffic and Transport Telematics (RTTT); Technical characteristics for communications equipment in the frequency band from 63 GHz to 64 GHz; System Reference Document".

[9] 	ETSI EN 302 686 V1.1.1(2011-02), ” Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 63 GHz to 64 GHz frequency band; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive”

[10] 	ERMTG37(17)026012, “ITS in 60GHz proposal ”, FBConsulting, ETSI ERM-TG37#26

[11] 	IEEE 802.18-17/0097, ITS in 60GHz - An updated proposal discussed in ETSI TC BRAN and TC ITS, Friedbert Berens, Markus Mueck

[12] 	Republic of Korea table of frequency allocations, https://spectrum.or.kr:5017/download.php?dnfile=%EC%A3%BC%ED%8C%8C%EC%88%98%EB%B6%84%EB%B0%B0%EB%8F%84%ED%91%9C%282015%EB%85%84+1%EC%9B%94%29.pdf&file=/www/spectrum_or_kr/webapp/../upload_dir/banner/175666235554bdd5483c499.pdf

[13]	3GPP TR 36.885: "Study on LTE-based V2X services".

[14]	3GPP TR 36.873: "Study on 3D channel model for LTE".

[15]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

B2R	Base station to road side unit

P2B	Pedestrian to base station

RSU	Road side unit

V2B	Vehicle to base station

V2V	Vehicle to vehicle

4	Introduction

To expand the 3GPP platform to the automotive industry, the initial standard on support of V2V services was completed in September 2016. Further enhancements that focusing on additional V2X operation scenarios leveraging the cellular infrastructure, also for inclusion in Release 14, is targeting completion in March 2017 as 3GPP V2X phase 1.

Currently, SA1 is working on enhancement of 3GPP support for V2X services in FS_eV2X. SA1 has identified 25 use cases for advanced V2X services and they are categorized into four use case groups: vehicles platooning, extended sensors, advanced driving and remote driving. The detailed description of each use case group is provided as below.

1)	Vehicles Platoonning enables the vehicles to dynamically form a platoon travelling together. All the vehicles in the platoon obtain information from the leading vehicle to manage this platoon. These information allow the vehicles to drive closer than normal in a coordinated manner, going to the same direction and travelling together.

2)	Extended Sensors enables the exchange of raw or processed data gathered through local sensors or live video images among vehicles, road site units, devices of pedestrian and V2X application servers. The vehicles can increase the perception of their environemnt beyond of what their own sensors can detect and have a more broad and holistic view of the local situation. High data rate is one of the key characteristics.

3)	Advanced Driving enables semi-automated or full-automated driving. Each vehicle and/or RSU shares its own perception data obtained from its local sensors with vehicles in proximity and that allows vehicles to synchronize and coordinate their trajectories or manoeuvres. Each vehicle shares its driving intention with vehicles in proximity too.

4)	Remote Driving enables a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive by themselves or remote vehicles located in dangerous environments. For a case where variation is limited and routes are predictable, such as public transportation, driving based on cloud computing can be used. High reliability and low latency are the main requirements.

The consolidated requirements for each use case group are captured in TR 22.886. Based on the input from FS_eV2X, SA1 will generate a set of normative requirements for Release 15. Inline with these requirements, TSG RAN has been defining a set of corresponding 5G RAN requirements within 3GPP TR38.913.

In order to study technical solutions for the new V2X use cases, new evaluation methodology needs to be defined.

5	Regulation for ITS operation in frequency band above 6 GHz

5.1	International recommendation

ITU-R recommends technical and operational characteristics for millimeter wave radiocommunication systems for ITS applications for V2V and V2I in the frequency band 57.0-66.0 GHz [Annex 2 in 6]. Channel arrangements will be specified by regions or countries separately. ITU-R identifies the transmission parameter requirements for ITS with three types of system bandwidth as follows:

Table 5.1-1 Transmission parameter requirements in [6].

|  | System A | System B | System C |
| --- | --- | --- | --- |
| System bandwidth | 63-64 GHz | 59-66 GHz | 57-64 GHz |
| E.I.R.P | 40 dBm |  |  |
| Transmit power |  | 10mW | 10mW |
| Antenna Gain | 23 dBi or less | 47 dBi or less | 17 dBi or less |

5.2	European regulation

CEPT states that 63-64 GHz frequency bands are allowed for ITS on harmonized use basis and the maximum radiated power should be limited to 40 dBm E.I.R.P [7]. ETSI also indicates that this frequency band can be used for ITS providing traffic safety and traffic efficiency applications (including V2V and V2I) all over Europe, and the following transmission parameters were considered in the analysis in [8, 9]:

- E.I.R.P

  - 40 dBm (E.I.R.P. max mean power), 43 dBm (E.I.R.P. max peak power)

- Transmit power

  - 27 dBm (max peak conducted power)

- Unwanted emission level

  - Less than -30 dBm

- Antenna gain

  - RSU : 23 dBi, Vehicle : 21 dBi (for V2V), 14 dBi (for V2I)

The definition of RF output power “the RF output power is the mean equivalent isotropic radiated power (e.i.r.p.) for the equipment during a transmission burst. The mean e.i.r.p. refers to the highest power level of the transmitter power control range during the transmission cycle if the transmitter power control is implemented” in [9] may need to be clarified, e.g., the notion of “transmission burst.”

It is noted that there is ongoing discussion in ETSI to adjust ITS spectrum at 63-64 GHz because this spectrum overlaps with two RLAN channels [10, 11]. Based on further progress, ITS frequency allocation above 6 GHz in Europe may be updated, e.g., shift the center frequency of ITS band to 62.64 or 64.80 GHz and enlarge its bandwidth up to 2.16 GHz.

5.3	Korean regulation

In Korean regulations, frequency band 57.0-66.0 GHz is currently assigned for the purpose of communications for earth exploration satellite(Passive), fixed and Inter-satellite [12], but no specific use cases are determined yet and the spectrum is being underutilized [Footnote K176C in 12].

5.4	Chinese regulation

There is no Chinese regulation for using ITS application in above 6GHz.

6	Evaluation methodology

The evaluation methodology in this document is used as a baseline for evaluating technical solutions and can be modified later as necessary.

6.1	System level simulation assumptions

6.1.1	Evaluation scenarios

For both below and above 6 GHz, the road configuration for urban grid and highway in [4] is used and the details are provided in Annex A.

Parameters regarding evaluation scenarios below 6 GHz are given in the following table:

Table 6.1.1-1: Evaluation scenarios below 6 GHz

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Carrier frequency | Macro to/from vehicle/pedestrian UE : 4 GHz Between vehicle/pedestrian UE: 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink | Macro to/from vehicle/pedestrian UE : 2 GHz or 4GHz Between vehicle/pedestrian UE: 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink |
| Aggregated system bandwidth | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) |
| Simulation bandwidth | 20 or 40 MHz (DL+UL) FFS: SL | 20 or 40 MHz (DL+UL) FFS: SL |
| BS Tx power | Macro BS: 49dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm BS-type-RSU: 24dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm for RSU is not precluded | Macro BS: 49dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm BS-type-RSU: 24dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm for RSU is not precluded |
| UE Tx power | Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm is not precluded | Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm is not precluded |
| BS receiver noise figure | 5dB | 5dB |
| UE receiver noise figure | 9 dB |  |

Note #1: Macro-BS parameters may also be used for BS-type RSU

Note #2: Aggregated sidelink bandwidth of 100 MHz at 6GHz is not available in the current frequency allocations for ITS and its future availability is subject to the progress in the potential additional ITS spectrum allocation.

Parameters regarding evaluation scenarios above 6 GHz are given in the following table:

Table 6.1.1-2: Evaluation scenarios above 6 GHz

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Carrier frequency | Macro to/from vehicle/pedestrian UE : 30 GHz Between vehicle/pedestrian UE: 63 GHz BS-type-RSU to/from vehicle/pedestrian UE : 30 GHz UE-type-RSU to/from vehicle/pedestrian UE: 63 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink | Macro to/from vehicle/pedestrian UE : 30 GHz Between vehicle/pedestrian UE: 63 GHz BS-type-RSU to/from vehicle/pedestrian UE : 30 GHz UE-type-RSU to/from vehicle/pedestrian UE: 63 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink |
| Aggregated system bandwidth | FFS | FFS |
| Simulation bandwidth | FFS | FFS |
| BS Tx power | FFS | FFS |
| UE Tx power | FFS | FFS |
| BS receiver noise figure | 7 dB | 7 dB |
| UE receiver noise figure | 13 dB (baseline), 10 dB (optional) |  |

6.1.2	UE drop and mobility modeling

Three vehicle types are defined as follows:

- Type 1 (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters

- Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters

- Type 3 (truck): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters

- FFS how to drop different vehicle types

- The difference of the vehicle type does not change the channel model potentially except the following aspects:

  - Pathloss equation where the antenna height is set according to the vehicle type

  - Loss caused by vehicle blockage (details to be discussed in the vehicle blockage modeling)

  - Radiation pattern

Vehicles are dropped according to the following process:

- The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {1 meter, an exponential random variable with the average of the speed * x sec}.

  - FFS for x sec.

- All the vehicles in the same lane have the same speed.

Clustered vehicle UE dropping is defined as follows:

- A cluster consists of a number vehicle UEs located in the same lane and having the same direction/speed. Two closest UEs belonging to the same cluster are separated with a fixed distance and no other UEs can be located between them.

- The distance between a platoon and a vehicle not belonging to the platoon follows the statistics of the distance between two vehicles not belonging to any platoon.

- Only Type 3 vehicles form a cluster.

- Clustered UE dropping is used only in the highway scenario.

The following UE dropping options are supported for the highway scenario:

- Option A

  - Vehicle type distribution: 100% vehicle type 2.

  - Clustered dropping is not used.

  - Vehicle speed is [140 and/or 70] km/h in all the lanes.

- Option B

  - Vehicle type distribution: [20]% vehicle type 1, [60]% vehicle type 2, [20]% vehicle type 3.

  - Clustered dropping is not used.

  - Vehicle speed in each lane is as follows:

    - Speed in Lane 1: 80km/h

    - Speed in Lane 2: 100km/h

    - Speed in Lane 3: 140km/h

    - Speed in Lane 4: 40km/h

    - Speed in Lane 5: 30km/h

    - Speed in Lane 6: 20km/h

- Option C

  - Vehicle type distribution: 0% vehicle type 1, [67]% vehicle type 2, [33]% vehicle type 3.

  - Clustered dropping is used. Each cluster consists of [6] Type 3 vehicles with a gap of [2] meters.

    - FFS how to drop multiple clusters

  - Vehicle speed is [140] km/h in all the lanes.

The following UE dropping options are supported for the urban grid scenario:

- Option A

  - Vehicle type distribution: 100% vehicle type 2.

  - Clustered dropping is not used.

  - Vehicle speed is [60 and/or 15] km/h in all the lanes.

  - In the intersection, a UE goes straight, turns left, turns right with the probability of 0.5, 0.25, 0.25, respectively.

- Option B

  - Vehicle type distribution: [20]% vehicle type 1, [60]% vehicle type 2, [20]% vehicles type 3.

  - Clustered dropping is not used.

  - Vehicle speed in each lane is as follows:

    - In the East-West direction:

      - Speed in Lane 1: 60km/h

      - Speed in Lane 2: 50km/h

      - Speed in Lane 3: 25km/h

      - Speed in Lane 4: 15km/h

    - In the North-South direction:

      - 0 km/h in all the lanes.

  - FFS how to handle the vehicle dropping and direction change at the intersection.

  - FFS whether to consider a reduced layout (e.g., covering a single intersection)

6.1.3	BS and RSU deployment

Parameters regarding BS and RSU deployment below 6 GHz are given in the following table:

Table 6.1.3-1: BS and RSU deployment below 6 GHz

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Layout | Option 1: Macro only (with the road configuration in Figure 6.1.9-1 in [4]) Note: Out of coverage can be evaluated assuming eNB to be disabled. | Option 1: Macro only (straight line eNB placement with Road configuration in [13]) Note: Out of coverage can be evaluated assuming eNB to be disabled. |
| Inter-BS distance | Inter Macro: 500m | Inter Macro: 1732m, 500m (optional) |
| RSU | FFS | FFS |

6.1.4	Antenna model

Parameters regarding antenna model below 6 GHz are given in the following table:

Table 6.1.4-1: Antenna model below 6 GHz

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| BS antenna height | Macro BS: 25m BS-type-RSU: 5m | Macro BS: 35m for ISD 1732m 25m for ISD 500m BS-type-RSU: 5m |
| BS antenna element gain + connector loss | Macro BS: 8dBi BS-type-RSU: 8dBi | Macro BS: 8dBi BS-type-RSU: 8dBi |
| BS antenna configurations | Number of BS antenna elements across all panels: - Macro BS: Up to 256 TX/RX antenna elements - BS-type-RSU: Up to 8 TX/RX antenna elements BS antenna element gain pattern: - Macro BS: Follow the modelling of [14] - BS-type RSU: Follow the modelling of micro BS in [14] | Number of BS antenna elements across all panels: - Macro BS: Up to 256 TX/RX antenna elements - BS-type-RSU: Up to 8 TX/RX antenna elements BS antenna element gain pattern - Macro BS: Follow the modelling of [14] - BS-type RSU: Follow the modelling of micro BS in [14] |
| UE antenna height | Vehicle/pedestrian UE: FFS UE-type-RSU: 5 m | Vehicle/pedestrian UE: FFS UE-type-RSU: 5 m |
| UE antenna gain | Vehicle UE: FFS Pedestrian UE: 0dBi UE-type RSU: 3dBi | Vehicle UE: FFS Pedestrian UE: 0dBi UE-type RSU: 3dBi |

Note #1: Macro-BS parameters may also be used for BS-type RSU

Note #2: The values for UE antenna may be revised after discussions on antenna placement, etc., if any.

Editor’s note: Some parameters in the above table may overlap with those in the other table in this subsection, They will be deleted later.

Macro BS antenna element pattern and array configuration are given in the following tables:

Table 6.1.4-2: Antenna element pattern for Macro BS

|  | For 4 GHz | For 30 GHz |
| --- | --- | --- |
| Antenna element gain vertical pattern |  |  |
| Antenna element gain horizontal pattern |  |  |
| Pattern combining method for 3D |  |  |
| Max direct. gain of the antenna element | 8 dBi |  |

Editor’s note: All the parameters in the above table are from [15] as per the agreement “gNB antenna element pattern and array configuration follows [15].” FFS which BS configuration is used in which cases.

Table 6.1.4-3: Antenna array configuration for Macro BS

|  | For 4 GHz | For 30 GHz |
| --- | --- | --- |
| TXRU mapping | FFS | FFS |
| Number of antenna elements across all panels | Up to 256 TX/RX antenna elements | FFS |
| Antenna array configuration (M, N, P, Mg, Ng) | FFS | FFS |
| Antenna array spacing (dH,dV,dH,g,dV,g) | FFS | FFS |
| Antenna tilt, deg | FFS | FFS |

Editor’s note: No parameters are found in [15] for the above table, so all the parameters are FFS expect the number of antenna elements across all panels below 6 GHz which is already agreed and captured in Table 6.1.4-1.

UE-type RSU antenna element pattern and array configuration are given in the following tables:

Table 6.1.4-4: Antenna element pattern for UE-Type RSU

|  | For 6 GHz | For 63 GHz |
| --- | --- | --- |
| Antenna element gain vertical pattern | FFS | FFS |
| Antenna element gain horizontal pattern | FFS | FFS |
| Pattern combining method for 3D | FFS | FFS |
| Max direct. gain of the antenna element | FFS | FFS |

Table 6.1.4-5: Antenna array configuration for UE-Type RSU

|  | For 6 GHz | For 63 GHz |
| --- | --- | --- |
| TXRU mapping | FFS | FFS |
| Number of antenna elements across all panels | FFS | FFS |
| Antenna array configuration (M, N, P, Mg, Ng) | FFS | FFS |
| Antenna array spacing (dH,dV,dH,g,dV,g) | FFS | FFS |
| Antenna tilt, deg | FFS | FFS |

Pedestrian UE antenna element pattern and array configuration are given in the following tables:

Table 6.1.4-6: Antenna element pattern for pedestrian UE

|  | Pedestrian UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz (according to the UE antenna radiation pattern parameters specified in Table A.2.1-8 of TR 38.802) |
| Antenna element gain vertical pattern | Omni-directional |  |
| Antenna element gain horizontal pattern |  |  |
| Pattern combining method for 3D |  |  |
| Max direct. gain of the antenna element | 0 dBi | 5 dBi |

Table 6.1.4-7: Antenna array configuration for pedestrian UE

|  | Pedestrian UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | FFS TR 38.802, Table A.1.6-1 | (2, 4, 2, 1, 2) Panel bearing angle: Ω0,1=Ω0,0+180° TR 38.802, Table A.1.6-2 |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.5)λ TR 38.802, Table A.1.6-1 | (dH, dV) = (0.5, 0.5)λ; (dH,g, dV,g) = (0, 0)λ TR 38.802, Table A.1.6-2 |
| Antenna tilt, deg | 0 | 90 |

Note: Antenna pattern can be different in different carrier frequencies.

Vehicle UE antenna element pattern and array configuration are given in the following tables:

Table 6.1.4-8: Antenna element pattern for vehicle UE

|  | Vehicle UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| Antenna element gain vertical pattern | FFS | FFS |
| Antenna element gain horizontal pattern | FFS | FFS |
| Pattern combining method for 3D | FFS | FFS |
| Max direct. gain of the antenna element | FFS | FFS |

Table 6.1.4-9: Antenna array configuration for vehicle UE

|  | Vehicle UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| TXRU mapping | FFS | FFS |
| Number of antenna elements across all panels | FFS | FFS |
| Antenna array configuration (M, N, P, Mg, Ng) | FFS | FFS |
| Antenna array spacing (dH,dV,dH,g,dV,g) | FFS | FFS |
| Antenna tilt, deg | FFS | FFS |

For both below and above 6 GHz, an option for “collocated antenna case” is supported. Note that this can be revised based on input from other organizations.

6.1.5	Traffic model

The following options are supported for the traffic model:

- Periodic traffic

  - Packets are generated periodically with a fixed inter-packet arrival time.

    - FFS the inter-packet arrival time

  - FFS on which option(s) is(are) supported for the message size.

    - Message size varies in time in a deterministic manner.

    - Message size varies in time in a random manner.

- Aperiodic traffic

  - Packets are generated with a random inter-packet arrival time.

    - Working assumption: Inter-packet arrival time = a non-negative constant value + a random variable following an exponential distribution

  - Message size varies in time in a random manner.

FFS message size details (e.g., how to implement randomness in message size, not precluding the possibility of defining multiple options)

Further discussion till RAN1#93 whether both options have equal priority or one of them has a higher priority.

6.1.6	Performance metric

Packet reception ratio (PRR) has two types and is defined as follows:

- PRR type 1: For one Tx packet, the PRR is calculated by X/Y, where Y is the number of UE/vehicles that located in the range (a, b) from the TX, and X is the number of UE/vehicles with successful reception among Y. CDF of PRR and the following average PRR are used in evaluation

  - CDF of PRR with a = 0, b = baseline of 320 meters for highway and 150 meters for urban. Optionally, b = 50 meters for urban with 15 km/h vehicle speed.

  - Average PRR, calculated as (X1+X2+X3….+Xn)/(Y1+Y2+Y3…+Yn) where n denotes the number of generated messages in simulation. with a = i*20 meters, b = (i+1)*20 meters for i=0, 1, …, 25

- PRR type 2: For one Tx packet, the PRR is calculated by S/Z, where Z is the number of UEs in the intended set of receivers, and S is the number of UE with successful reception among Z.

  - Unicast is the special case where Z includes a single UE, where the PRR is average of packets of the unicast link

PRR type 1 is included as a performance metric at least for the broadcast-type use cases.

PRR type 2 is supported and can be used as performance metric in scenarios such as multicast/unicast or to see the performance of links in a certain condition (e.g., links blocked by a building). When used, the company needs to clarify how the intended set of receivers is determined and what the motivation is.

Packet Inter-Reception (PIR) is defined as follows and used as an additional metric for persistent collision at least for the use cases requiring reliability higher than that of LTE V2X:

- PIR is the time elapsed between two successive successful receptions of two different packets transmitted from node A to node B for the same application.

  - FFS how to collect results of PIR

6.2	Channel model

The V2V sidelink channel is modeled according to the following three states:

- LOS:

  - A V2V link is in LOS state if the two vehicles are in the same street and the LOS path is not blocked by vehicles.

- NLOS: LOS path blocked by buildings

  - A V2V link is in NLOS state if the two vehicles are in different streets.

- NLOSv: LOS path blocked by vehicles

  - A V2V link is in NLOSv state if the two vehicles are in the same street and the LOS path is blocked by vehicles.

A link between two vehicles is considered blocked by vehicles (i.e., in NLOSv state)

- Option 1: if a line connecting the antennas of the two vehicles in the same street intersects any vehicle (including either of the two vehicles) in the 3-dimensional space.

- Option 2: with a probability

  - Distribution is FFS. Note that vehicle width and height will not affect this option once the probability is defined.

Vehicle UE location is updated every [100] ms. The link state (LOS, NLOS, or NLOSv) is updated for each link during the SLS runtime. FFS whether the state is updated by Option 1 or Option 2 w.r.t the link between two vehicles is considered blocked by vehicles (i.e., in NLOSv state). At each state, each link uses pathloss, shadowing, and fast fading parameters corresponding to the state as described in the subsequent subsections.

- FFS how to model smooth transition between different states

6.2.1	Pathloss model

Pathloss model for V2V links is given in the following table:

Table 6.2.1-1: Pathloss for V2V links

| LOS/NLOS/NLOSv | Pathloss [dB] |
| --- | --- |
| LOS, NLOSv | For Freeway case, PL = 32.4 + 20log10(d) + 20log10(fc) (fc is in GHz and d is in meters) For Urban case, PL= 38.77 + 16.7log10(d) + 18.2log10(fc) (fc is in GHz and d is in meters) |
| NLOS | FFS including Option 1: where and Option 2: NLOS pathloss equation in R1-1803671 Other options are not precluded. |

When a V2V link is in NLOSv, additional vehicle blockage loss is added. FFS details (e.g., how to determine value of additional loss, whether the additional loss is a function of the number and size of blocking vehicles). The additional loss is a random variable and the distribution is FFS.

Pathloss in V2B, P2B, B2R link is given as follows:

- LOS propagation type is used for V2B and B2R links in the highway scenario.

- LOS/NLOS propagation types are used for V2B, P2B and B2R links in the urban scenario and spatial consistency is maintained following procedure in Subclause 7.6.3.3 in [15].

  - Derive propagation type based on probability formula

Table 6.2.1-2: Pathloss for V2B, P2B, B2R links

|  | Below 6 GHz |  | Above 6 GHz |  |
| --- | --- | --- | --- | --- |
|  | LOS | NLOS | LOS | NLOS |
| B2V B2P B2R | Urban: TR 38.901 UMa LOS Freeway: TR 38.901 RMa LOS | Urban: TR 38.901 UMa NLOS Freeway: N/A | Urban: TR 38.901 UMa LOS Freeway: FFS | Urban: TR 38.901 UMa NLOS Freeway: N/A |

For above 6 GHz, oxygen absorption is modelled by introducing additional loss which is derived based on [15].

6.2.2	Shadowing model

6.2.3	Fast fading model

For above 6GHz, dual mobility should be modelled. FFS details (e.g., how to handle impact of moving scatters).

For sidelink in the urban and highway scenarios, the fast fading parameters of “UMi-Street Canyon in TR 38.901” is the starting point and to be modified considering sidelink characteristics. FFS fading parameters in the urban and highway scenarios.

Table 6.2.3-1: Fast fading parameters for V2V link in urban scenario

| V2V link |  | LOS | NLOS | NLOSv |
| --- | --- | --- | --- | --- |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | [-0.24 log10(1+ fc) - 7.14] | [-0.24 log10(1+ fc) - 6.83] | FFS |
|  | lgDS | [0.38] | [0.16 log10(1+ fc) + 0.28] | FFS |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | [-0.05 log10(1+ fc) + 1.21] | [-0.23 log10(1+ fc) + 1.53] | FFS |
|  | lgASD | [0.41] | [0.11 log10(1+ fc) + 0.33] | FFS |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | [-0.08 log10(1+ fc) + 1.73] | [-0.08 log10(1+ fc) + 1.81] | FFS |
|  | lgASA | [0.014 log10(1+ fc) + 0.28] | [0.05 log10(1+ fc) + 0.3] | FFS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | [-0.1 log10(1+ fc) + 0.73] | [-0.04 log10(1+ fc) + 0.92] | FFS |
|  | lgZSA | [-0.04 log10(1+ fc) + 0.34] | [-0.07 log10(1+ fc) + 0.41] | FFS |
| Shadow fading (SF) [dB] | SF | [4] | [7.82] | FFS |
| K-factor (K) [dB] | K | [9] | N/A | FFS |
|  | K | [5] | N/A | FFS |
| Cross-Correlations | ASD vs DS | [0.5] | [0] | FFS |
|  | ASA vs DS | [0.8] | [0.4] | FFS |
|  | ASA vs SF | [-0.4] | [-0.4] | FFS |
|  | ASD vs SF | [-0.5] | [0] | FFS |
|  | DS vs SF | [-0.4] | [-0.7] | FFS |
|  | ASD vs ASA | [0.4] | [0] | FFS |
|  | ASD vs  | [-0.2] | N/A | FFS |
|  | ASA vs  | [-0.3] | N/A | FFS |
|  | DS vs  | [-0.7] | N/A | FFS |
|  | SF vs  | [0.5] | N/A | FFS |
| Cross-Correlations | ZSD vs SF | [0] | [0] | FFS |
|  | ZSA vs SF | [0] | [0] | FFS |
|  | ZSD vs K | [0] | N/A | FFS |
|  | ZSA vs K | [0] | N/A | FFS |
|  | ZSD vs DS | [0] | [-0.5] | FFS |
|  | ZSA vs DS | [0.2] | [0] | FFS |
|  | ZSD vs ASD | [0.5] | [0.5] | FFS |
|  | ZSA vs ASD | [0.3] | [0.5] | FFS |
|  | ZSD vs ASA | [0] | [0] | FFS |
|  | ZSA vs ASA | [0] | [0.2] | FFS |
|  | ZSD vs ZSA | [0] | [0] | FFS |
| Delay scaling parameter r |  | [3] | [2.1] | FFS |
| XPR [dB] | XPR | [9] | [8.0] | FFS |
|  | XPR | [3] | [3] | FFS |
| Number of clusters |  | [12] | [19] | FFS |
| Number of rays per cluster |  | [20] | [20] | FFS |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | lgZSD | [max[-0.21, -14.8(d2D/1000) + 0.01¦hUT-hBS¦ + 0.83]] | [max[-0.5, -3.1(d2D/1000) + 0.01 max(hUT-hBS,0) +0.2]] | FFS |
|  | lgZSD | [0.35] | [0.35] | FFS |
| ZOD offset | µoffset,ZOD | [0] | [-10^{-1.5log10(max(10, d2D))+3.3}] | FFS |

Table 6.2.3-2: Fast fading parameters for V2V link in highway scenario

| V2V link |  | LOS | NLOS | NLOSv |
| --- | --- | --- | --- | --- |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | [-0.24 log10(1+ fc) - 7.14] | [-0.24 log10(1+ fc) - 6.83] | FFS |
|  | lgDS | [0.38] | [0.16 log10(1+ fc) + 0.28] | FFS |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | [-0.05 log10(1+ fc) + 1.21] | [-0.23 log10(1+ fc) + 1.53] | FFS |
|  | lgASD | [0.41] | [0.11 log10(1+ fc) + 0.33] | FFS |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | [-0.08 log10(1+ fc) + 1.73] | [-0.08 log10(1+ fc) + 1.81] | FFS |
|  | lgASA | [0.014 log10(1+ fc) + 0.28] | [0.05 log10(1+ fc) + 0.3] | FFS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | [-0.1 log10(1+ fc) + 0.73] | [-0.04 log10(1+ fc) + 0.92] | FFS |
|  | lgZSA | [-0.04 log10(1+ fc) + 0.34] | [-0.07 log10(1+ fc) + 0.41] | FFS |
| Shadow fading (SF) [dB] | SF | [4] | [7.82] | FFS |
| K-factor (K) [dB] | K | [9] | N/A | FFS |
|  | K | [5] | N/A | FFS |
| Cross-Correlations | ASD vs DS | [0.5] | [0] | FFS |
|  | ASA vs DS | [0.8] | [0.4] | FFS |
|  | ASA vs SF | [-0.4] | [-0.4] | FFS |
|  | ASD vs SF | [-0.5] | [0] | FFS |
|  | DS vs SF | [-0.4] | [-0.7] | FFS |
|  | ASD vs ASA | [0.4] | [0] | FFS |
|  | ASD vs  | [-0.2] | N/A | FFS |
|  | ASA vs  | [-0.3] | N/A | FFS |
|  | DS vs  | [-0.7] | N/A | FFS |
|  | SF vs  | [0.5] | N/A | FFS |
| Cross-Correlations | ZSD vs SF | [0] | [0] | FFS |
|  | ZSA vs SF | [0] | [0] | FFS |
|  | ZSD vs K | [0] | N/A | FFS |
|  | ZSA vs K | [0] | N/A | FFS |
|  | ZSD vs DS | [0] | [-0.5] | FFS |
|  | ZSA vs DS | [0.2] | [0] | FFS |
|  | ZSD vs ASD | [0.5] | [0.5] | FFS |
|  | ZSA vs ASD | [0.3] | [0.5] | FFS |
|  | ZSD vs ASA | [0] | [0] | FFS |
|  | ZSA vs ASA | [0] | [0.2] | FFS |
|  | ZSD vs ZSA | [0] | [0] | FFS |
| Delay scaling parameter r |  | [3] | [2.1] | FFS |
| XPR [dB] | XPR | [9] | [8.0] | FFS |
|  | XPR | [3] | [3] | FFS |
| Number of clusters |  | [12] | [19] | FFS |
| Number of rays per cluster |  | [20] | [20] | FFS |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | lgZSD | [max[-0.21, -14.8(d2D/1000) + 0.01¦hUT-hBS¦ + 0.83]] | [max[-0.5, -3.1(d2D/1000) + 0.01 max(hUT-hBS,0) +0.2]] | FFS |
|  | lgZSD | [0.35] | [0.35] | FFS |
| ZOD offset | µoffset,ZOD | [0] | [-10^{-1.5log10(max(10, d2D))+3.3}] | FFS |

Fast fading parameters for V2B, P2B, B2R links are in the following table:

Table 6.2.3-3: Fast fading parameters for V2B, P2B, B2R

|  | B2V/B2P/B2R |  |
| --- | --- | --- |
|  | LOS | NLOS |
| Freeway | RMa LOS in [15] | N/A |
| Urban | UMa LOS [15] | UMa NLOS [15] |

For sidelink, Rel-14 dual mobility is modified such that a random Doppler shift is added to each reflected path.

- FFS details of the additional random Doppler shift

6.3	Link level simulation assumptions

The assumption for system level simulation is used for link level simulation if available.

The parameters related to solutions need to be clarified by each company. At least the following parameters are the list needs to be clarified.

- Carrier frequency

- Channel model (e.g. fast fading model)

- PHY packet size

- Channel codes (for control and data channels)

- Modulation and code rates (for control and data channels)

- Signal waveform (for control and data channels)

- Subcarrier Spacing

- CP length

- Frequency synchronization error

- Time synchronization error

- Channel estimation (e.g. DMRS pattern and symbol location)

- Number of retransmission and combining (if applied)

- Number of antennas (at UE and BS)

- Transmission diversity scheme (if applied)

- UE receiver algorithm

- [AGC settling time and guard period]

- [EVM (at TX and RX)]

6.4	Additional assumptions to evaluate vehicle positioning

The simulation assumptions for “vehicle positioning” reuse those defined in Subclauses 6.1, 6.2, and 6.3.

Performance metrics are defined as follows:

- Absolute and relative UE poisoning error in meter

Annex A:	Road configuration for urban grid and highway

Parameters regarding the road configuration for urban grid and highway are given in the following table:

Table A-1: Road configuration for urban grid and highway

| Parameter | Urban case | Highway case |
| --- | --- | --- |
| Number of lanes | 2 in each direction (4 lanes in total in each street) | 3 in each direction (6 lanes in total in the highway) |
| Lane width | 3.5 m | 4 m |
| Road grid size by the distance between intersections | 433 m * 250 m. NOTE1 | N/A |
| Simulation area size | Minimum 1299 m * 750 m NOTE2 | Highway length >= 2000 m. Wrap around should be applied to the simulation area. |

NOTE1: 3 m is reserved for sidewalk per direction (i.e., no vehicle or building in this reserved space).

NOTE2: This value is tentative and could be modified after SA1’further input.

Figure A-1 and A-2 show illustrative diagrams of urban grid and highway, respectively.

Figure A-1: Road configuration for urban grid

Figure A-2: Road configuration for highway scenario

Annex B:	Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-05 | RAN1#83 | R1-1806178 |  |  |  | Inclusion of RAN1#92bis agreements | 0.2.0 |
| 2018-04 | RAN1#92bis | R1-1804589 |  |  |  | Inclusion of RAN1#92 agreements and the text in RP-172401. | 0.1.0 |
| 2018-02 | RAN1#92 | R1-1803309 |  |  |  | Skeleton TR | 0.0.1 |