3GPP TSG RAN WG1 Meeting #93	R1-1806231

Busan, Korea, May 21st – 25th, 2018

Source:	Ericsson

Title:	Benefits of resource specific DMRS mapping

Agenda:	7.1.2.3.8

Document for:	Discussion and Decision

# On resource-specific DMRS mapping

The resource mapping of DM-RS is captured in [TS 38.211] as follows

(UL)        (DL)

where rpj(2n+k') refer to the DM-RS sequence after applying OCC to DM-RS port pj. Figure 1 illustrates the mapping of the DM-RS in frequency-domain for single-symbol DM-RS. From this figure we observe that the DM-RS sequence in one CDM group is copied into the other CDM group(s). Such mapping will have CM/PAPR issues when DM-RS ports associated with same OCC are spatially multiplexed in a way that ports are sharing same PA’s. Thus, this issue occurs when a precoding matrix has multiple non-zero row elements,

bk,l(TXn)=wn,0ak,l(p0,μ)+wn,1ak,l(p1,μ)+…+wn,L-1ak,l(pL-1,μ),

and where at least two ports associate with the same OCC. For example, this PAPR issue occurs when scheduling two layers using the ports {0,2} and then applying precoding with mapping of the layers to same TX antennas, wn,i≠0, i=0,1, as well as when scheduling more than 2 layers with ports mapped to same TX antennas.

To illustrate this PAPR issue, we determine the CCDFs of the power-to-average power ratios when mapping 2, 3, 4 and 6 layers to a common set of transmit antennas. Figure 2 depicts the CCDFs for an allocation of 50 RBs and with power samples from 100 OFDM symbols. We observe that the current DM-RS mapping can significantly increase the PAPR with respect to the CP-OFDM waveform (waveform with Rayleigh distributed amplitude), e.g. 6 layers with mapping type 2 (ports: 0-5) degrades the PAPR with nearly 4dB at the 99.99% percentile.

This observed increased PAPR can be avoided by considering resource-specific DM-RS sequence mapping, in which DM-RS sequences across CDM groups are uncorrelated or weakly correlated. Thus, each CDM group has its own DM-RS sequence as illustrated in Figure 3. These sequences, rim, i=0,1,2, could refer to either a subsampled long Gold sequence (rim=r(k)) or to a CDM group specific Gold sequence. It can be noticed that the correlation properties of the subsampled Gold sequence will not be affected [1].

Figure 4 shows the corresponding CCDFs of the power-to-average ratio with resource-specific DM-RS mapping. Evidently, with this mapping the PAPR will be similar to CP-OFDM.

Observation 1: By introducing resource-specific mapping of DM-RS, the observed PAPR issue with current resource mapping of DM-RS can be avoided.

Figure 1. Single-symbol DM-RS mapping to CDM groups in TS 38.211.

Figure 2. PAPR with current DM-RS mapping.

Figure 3. Resource-specific DM-RS mapping.

Figure 4. PAPR with resource-specific DM-RS mapping.

# Conclusion

In this contribution, we evaluated the power-to-average power ratios when mapping DM-RS ports to a common set of transmit antennas. We observed that the current resource mapping of DM-RS to CDM groups will degrade the PAPR. We then observed that this PAPR issue can be avoided by introducing resource-specific mapping of DM-RS.

# References

[1]	R1-1806232, “On correlation of subsampled Gold sequences”, Ericsson