3GPP TSG-RAN WG1 Meeting #93	Tdoc R1-1806241

Busan, South Korea, 21-25 May, 2018

Agenda Item:	7.4.1

Source:	Ericsson

Title:	Signature design for NoMA

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss signature design for WSMA as a NoMA scheme and address some issues related to flexibility of the scheme for varying number of users.

# 2	Discussion

In NOMA, each UE spreads its QAM information symbols using an N-length spreading sequence {sk}. Let K denote the number of simultaneously active UEs. For a single antenna BS, the received signal vector y∈CN, where N is the number of REs spanned by the signature vectors and carry the same QAM information symbols, at the eNB can be written as

y=k=1Ksk⊙hkbk+w,

where hk is the channel vector between UEk and the gNB, bk is the QAM symbol of UEk and the operator ⊙ stands for the pointwise multiplication/product of two vectors. For multiple RX antennas, we can form the received signal corresponding to a single QAM symbol per UE, simply by concatenating the N-length received vector y from each RX antenna. From a system performance point-of-view, it is optimal to jointly choose the transmit strategies for all UEs and then employ a joint MUD detector. In some NoMA schemes, the QAM symbols are spread using sequences that are designed to have certain desired correlation properties. The differences between various schemes lie in how the sequences {sk} are constructed.

In Welch bound equality (WBE) based spreading, the design metric, for the signature vectors is the total squared cross-correlation Tc≜i,j|siHsj|2. The lower bound on the total squared cross-correlation of any set of K vectors of length N, is K2/N≤Tc. The WBE sequences are designed to meet the bound on the total squared cross-correlations of the vector set with equality  BWelch≜K2/N. We call such sequences Welch bound equality spread multiple access (WSMA).

Figure 1 shows a simplified block diagram of WSMA transmitter. As it can be seen the spreading operation is done on the QAM modulated symbols.

Channel  encoder

Modulator

Spreading

Resource mapping

IFFT

info

bits

Figure 1: Simplified block diagram of WSMA transmitter.

It has been shown that WSMA sequences are capacity-optimal ‎[1], hence they are excellent candidates for achieving high spectral efficiency (eMBB) and high UE capacity (mMTC).

There are known methods to iteratively construct such codes for arbitrary N, K. Further details are provided in section 2.1.

## 2.1	Iterative Sequence generation

For the construction of the WBE SS, well known Interference Avoidance (IA) techniques exist [4]. A nice property of the IA methods is that the SS can be obtained iteratively in a sequential and distributed manner. It is guaranteed that the iterations converge, since there exists a fixed-point for S that is optimum. Though verifying if the obtained optimum is local or global is not easy. By optimum it is meant that the entire matrix S converges as an ensemble (as against each CW convergence), with the considered performance indicator (PI) reaching the required tolerance. Hence the converged WBE CWs are not unique. At convergence, all the mentioned PIs are optimized. This makes the selection of the WBE SS for NOMA attractive.

For a user k, let bk be the transmitted symbol that modulates a unit norm SS vector sk. The Additive White Gaussian Noise (AWGN) signal model may be given as y=Sb+z, where z is the zero-mean AWGN vector with a covariance matrix I, i.e., an Identity matrix. the overall SS matrix with a CW in each of its columns is S, the transmit symbol vector is b.  The transmit power of each user is set to unity, so the power control problem is not addressed here. A unit norm temporal receive filter fk, such as a Matched Filter (MF) or a linear Minimum Mean Squared Error (MMSE) filter, may be employed by the receiver to obtain an estimate bk for the transmitted symbol bk. The post processed SINR γk of a user k is given as

γk=fkHsk2j=1,j≠kKfkHsj2+fkHz2=fkH(skskH)fkfkH(j=1,j≠kKsjsjH+I)fkfk=sk1traceSST2-1+νk,

where trace∙ is the trace operator, νk is the noise component in the SINR γk. The trace∙ term in the denominator is the TSC, which also contains the desired unit signal power. So an additional unity term arises in the denominator. If the post processed noise is white, i.e., the noise power of each νk is the same, then the TSC can directly be used as a PI.

From the center part of the SINR equation, let Rk=(j=1,j≠kKsjsjH+I), which is the correlation matrix of the interference plus noise. It can be identified that minimizing the denominator (or equivalently maximizing γk) is a well known Rayleigh-Quotient problem. From this, the Eigen vector corresponding to the minimum Eigen value of Rk may be considered as CW for UE k, if it is assumed that fk is matched to sk.

The fixed-point iterations start from the users choosing a random CW. In a given sequential user order, say k=1,2,…K, each user updates its SS sk by solving the Eigen value problem while other SS, sj, j≠k, are kept fixed, i.e. After user k, the next user updates it’s CW in the same way by assuming the other CWs to be fixed. The iterations progress up to the final user in the order, such that in each iteration there are K updates, one for each CW in S. After the final update in the given iteration, the first user in the order restarts the updates. This repeats until convergence.

Again, from the center part of the SINR equation, the solution to fk can also be identified as the well known Generalized Eigen Value Problem (GEVP), i.e., finding a common Eigen value for the matrix pair (I,Rk). The solution to which is the linear MMSE vector given as Rk-1skskHRk-2sk1/2, in its normalized form. Sequential iterations as mentioned before can be used, except that instead of solving the Eigen value problem, the normalized linear MMSE expression is used during updates. For this SINR maximization problem (or TSC minimization), the obtained solution to S from both the MMSE IA iterations and the Eigen vector IA iterations is the same fixed-point.

A Kronecker product based approach may be employed to obtained (or construct) higher dimensional WBE SS, i.e., higher N values, from lower dimensional WBE SS.

## 2.2	Equiangular subset of WBE sequences

If the WB on the performance indicator TSC is satisfied with equality, then the set of obtained SS vectors are optimum in the overall MSE sense (and also in the capacity sense). These are the WBE sequences. TSC captures the total permissible interference for each of the K active users in an overloaded system (K>N). Any change to any of SS vectors will change the TSC and hence every user’s SINR also changes. The SS design while optimizing the overall MSE can be viewed as a two-stage procedure, 1) construction and 2) selection. This separation into two will be evident next.

In general, K may vary in the NW. Each time a user drops out or a new UE is admitted, the existing set of WBE SS are no longer valid for that given instance. Since, a change in K will change the TSC and hence the SINR at each user. This implies reconstruction of the WBE SS by obtaining the new correlation values (which correspond to the multiple access interference (MAI)) among the SS vectors for the new set of active users. So, it is reasonable to consider another design criterion, i.e., constraint the WBE SS vectors, such that the SS vector of each user is at the same angle (equiangular or same correlation) from every other user’s SS vector and select an appropriate PI that captures this constraint. With such a requirement, the TSC and hence the SINR may be easily evaluated. The increase or decrease in the MAI at each user is already known, since the correlation value is known at the construction stage and the SS vectors have a unit norm. This is the Interference Invariance property of the SS. For a given N and K, the SS vectors are constructed only once. Large K may be assumed in the initial construction, so that any user that joins can only select (or obtain) its SS vector from the already evaluated SS vectors. Two-stage problem at each instance is then reduced to a single-stage selection problem.

To address this WBE SS with equiangular property, a PI known as the worst-case coherence of the signature matrix, given as the  μS=maxi≠j|siHsj|, is suitable. A lower bound on μS is once again identified by the WB and is given as K-NN(K-1)≤ μ2(S). If this bound is met with equality, then WBE SS with equiangular property is obtained. These newly obtained SS vectors are a subset of the WBE sequences obtained while minimizing the TSC. To meet the lower bound with equality, the optimization problem must minimize μS, the outcome of which is a SS for which the correlation between any two vectors is the same. Such equiangular WBE SS is known as Grassmannian SS or Equiangular Tight Frames (ETF) [2].  It must be emphasized that the existence of an ETF is not guaranteed for any random combination of N and K.

WBE sequences with equiangular property are chosen for NOMA, especially when the number of UEs is varying.

For Proposal 1, the chosen performance indicator while evaluating the ETFs, is the worst-case matrix coherence μS instead of the TSC. From the hardware point of view where the number of Radio Frequency (RF) chains in mostly limited to one at the users, it is preferable to choose a subset of ETFs called Harmonic ETFs. These SS vectors are relatively easy to generate. In addition to having a unit vector norm, each element in each vector will also have a unit magnitude. Such a SS is constructed from the complex roots of unity.  Figure 2 shows various subsets of WB sequences that were outlined in this subsection.

Figure 2:  Various subsets of Welch Bound sequences are shown. The more inner the subset is, the more constrained it is.

## 2.3	An extension to the WBE sequences in multidimension

### 2.3.1	Multiple symbol transmission

When the UEs transmit more than one symbol, say r (with r≤N) symbols per user, then the users will have more than one SS vector for itself, resulting in an (N x r) SS matrix. This SS matrix will then multiplex the symbols at each UE before spreading them onto the N time slots. Since the concept of ETFs is already outlined before, an extension to this is outlined here in the context of multiple symbols.

For multi symbol transmission, the function of the SS matrix at each UE may be that of multiplexing the r symbols. A simple design criterion could be to implement orthogonality at each UE while multiplexing the r symbols and permitting a controlled interference across the UEs as before. This problem is well understood in the context of subspace packing, where K subspaces, each of which is r dimension of CN , need to be packed together [3]. For a user  k, the SS matrix Sk could contain the r basis of its subspace to satisfy the orthogonality. By introducing the controlled interference among the users, the subspaces are allowed to overlap such that the principal angle between any two subspaces is the same. It is analogous to the equiangular criterion when r=1. When Kr≤N, there can always be orthogonal columns across all K users such that the user subspaces are non-overlapping.

Such a packing problem also provides the Upper Bounds (UB) on the admissible number of UEs under some design criteria (like equidistant or equiangular). For equiangular subspaces criteria it is given as K≤ (N2-r2+1) [3].

WBE sequences with multiplexing are chosen for NOMA, especially when the UEs are in different dimensional subspaces and wish to transmit multiple symbols

### 2.3.2	Multiple Antennas

Since most of the systems are equipped with multiple antennas, it is important to understand the temporal SS design criteria w.r.t. the spatial dimension. Let the receiver have Nrx  antennas while each user is equipped with Ntx antennas, but each user transmits only one symbol.

The received (and sampled) temporal signal vector (in a row form) at a receive antenna m from all the K users is

ymT=k=1KskThk,m,:Ttkpk bk+zmT ,

where for a user k, the spatial channel from all its Ntx transmit antennas to the receive antenna m is hk,m,:T, the unit norm spatial BF vector is tk, the transmit power is pk (it is assumed to be unity here), and the transmitted symbol is bk. For a spatially flat (Nrx x Ntx) MIMO channel Hk of user k at a given time instance, hk,m,:T denotes all the elements in its row m. The temporally uncorrelated white noise for all the users at the receive antenna m is zmT.  Each UE must now adapt (by reselecting a SS) to optimize its Space Time (ST) block code matrix (under the assumption that the spatial transmit/receive filters are fixed) to minimize the overall MSE. An observation from the received symbol ymT is that with the inclusion of the spatial domain, the required correlation to satisfy the WBE by the temporal SS vectors is perturbed at each UE. This is because the overall effective ST channel of each user is changed. Also, the overall MSE is affected by the transmit strategy of each user and receive strategies across both the spatial and temporal domains.

WBE sequence design in the temporal domain that consider jointly the spatial transmitter and/or spatial receiver strategy improves performance for NOMA, when there are multiple antennas are involved

A transmitter and/or receiver strategy here is applicable to both spatial and temporal domains.

# Conclusion

In this contribution we presented a NoMA signature design based on Welch bound equality and made the following observations and proposals.

- WBE sequences with equiangular property are chosen for NOMA, especially when the number of UEs is varying.

- WBE sequences with multiplexing are chosen for NOMA, especially when the UEs are in different dimensional subspaces and wish to transmit multiple symbols

- Observation 1 	WBE sequence design in the temporal domain that consider jointly the spatial transmitter and/or spatial receiver strategy improves performance for NOMA, when there are multiple antennas are involved.

# References

S. Ulukus and R. Yates, “Iterative construction of optimum signature sequence sets in synchronous CDMA systems,” IEEE Trans. Inf. Theory, vol. 47, no. 5, pp. 1989 - 1998, July 2001

J.A. Tropp, Complex equiangular tight frames, in: Proceedings of SPIE 2005 (Wavelets XI), San Diego, August 2005

R. Calderbank, A. Thompson and Yao Xie, On block coherence of frames, Applied and Computational Harmonic Analysis, Vol.38 (2015), 50-71.

D. C. Popescu, O. Popescu, and C. Rose, “Interference Avoidance for Multiaccess Vector Channels,” in Proceedings of the International Symposium on Information Theory, p. 499, IEEE, July 2002.

R1-1805006, “Evaluation methodology and metrics for NoMA”, Ericsson