3GPP TSG RAN WG1 Meeting #93		R1-1806271
Busan, Korea, May 21st – 25th, 2018

Source:	CATT

Title:	Further details on NR RACH format

Agenda Item:	7.1.1.4.1

Document for:	Discussion and Decision

# Introduction

In RAN1#92bis meeting, the following agreements were made on NR RACH format [1],

Agreements:

- Update the WA (remains as WA) on FR1 configuration Table 6.3.3.2-3 in 38.211 section 6.3.3.2, with the following update for configuration index 166 for format B4:

Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a RACH slot |  |  | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 166 | B4 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 3,4,8,9 | 0 | 2 |  | 1 | 12 |  |

Agreements:

- Replace the previous working assumption (remains WA) for FR2 RACH configuration table, with the following table.

  - The ones in yellow background are not final

  - Note: RACH configuration indexing will be up to editor to update.

=============== Text proposal for 38.211, section 6.3.3 =============================

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| PRACH Config. Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a RACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 1 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 3 | 2 |
| 2 | A1 | 1 | 0 | 9,19,29,39 | 0 | 2 | 6 | 2 |
| 3 | A1 | 1 | 0 | 24,29,34,39 | 7 | 1 | 3 | 2 |
| 4 | A1 | 1 | 0 | 17,19,37,39 | 0 | 1 | 6 | 2 |
| 5 | A1 | 1 | 0 | 19,39 | 7 | 1 | 3 | 2 |
| 6 | A1 | 1 | 0 | 9,19,29,39 | 7 | 2 | 3 | 2 |
| 7 | A1 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 3 | 2 |
| 8 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 9 | A1 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 3 | 2 |
| 10 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 11 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 3 | 2 |
| 12 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 6 | 2 |
| 13 | A1 | 1 | 0 | 3,5,7 | 0 | 1 | 6 | 2 |
| 14 | A1 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 3 | 2 |
| 15 | A1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 16 | A1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 17 | A1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 18 | A1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 19 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 20 | A1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 21 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 22 | A1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 23 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 6 | 2 |
| 24 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 25 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 26 | A1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 3 | 2 |
| 27 | A1 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 6 | 2 |
| 28 | A1 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 6 | 2 |
| 0 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 1 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 5 | 1 | 2 | 4 |
| 2 | A2 | 1 | 0 | 9,19,29,39 | 0 | 2 | 3 | 4 |
| 3 | A2 | 1 | 0 | 24,29,34,39 | 5 | 1 | 2 | 4 |
| 4 | A2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 3 | 4 |
| 5 | A2 | 1 | 0 | 19,39 | 5 | 1 | 2 | 4 |
| 6 | A2 | 1 | 0 | 9,19,29,39 | 5 | 2 | 2 | 4 |
| 7 | A2 | 1 | 0 | 0,1,2,…,39 | 5 | 1 | 2 | 4 |
| 8 | A2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 9 | A2 | 1 | 0 | 23,27,31,35,39 | 5 | 1 | 2 | 4 |
| 10 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 11 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 5 | 1 | 2 | 4 |
| 12 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 3 | 4 |
| 13 | A2 | 1 | 0 | 3,5,7 | 0 | 1 | 3 | 4 |
| 14 | A2 | 1 | 0 | 3,5,7,9,11,13 | 5 | 1 | 2 | 4 |
| 15 | A2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 16 | A2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 17 | A2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 18 | A2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 19 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 20 | A2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 21 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 22 | A2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 23 | A2 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 3 | 4 |
| 24 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 25 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 26 | A2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 5 | 2 | 2 | 4 |
| 27 | A2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 3 | 4 |
| 28 | A2 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 3 | 4 |
| 0 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 1 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 1 | 6 |
| 2 | A3 | 1 | 0 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| 3 | A3 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 | 6 |
| 4 | A3 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 |
| 5 | A3 | 1 | 0 | 19,39 | 7 | 1 | 1 | 6 |
| 6 | A3 | 1 | 0 | 9,19,29,39 | 7 | 2 | 1 | 6 |
| 7 | A3 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 | 6 |
| 8 | A3 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 9 | A3 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 1 | 6 |
| 10 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 11 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 |
| 12 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 |
| 13 | A3 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 |
| 14 | A3 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 1 | 6 |
| 15 | A3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 16 | A3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 17 | A3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 18 | A3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 19 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 20 | A3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 21 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 22 | A3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 23 | A3 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 2 | 6 |
| 24 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 25 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 26 | A3 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 1 | 6 |
| 27 | A3 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| CMCC | A3 | 1 | 0 | 7,15,23,31,39 or 9,11,13 | 2 | 1 | 2 | 6 |
| 0 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 1 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 2 | B1 | 1 | 0 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 3 | B1 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 | 2 |
| 4 | B1 | 1 | 0 | 17,19,37,39 | 2 | 1 | 6 | 2 |
| 5 | B1 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 6 | B1 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 |
| 7 | B1 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 | 2 |
| 8 | B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 9 | B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 10 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 11 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 12 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 | 2 |
| 13 | B1 | 1 | 0 | 3,5,7 | 2 | 1 | 6 | 2 |
| 14 | B1 | 1 | 0 | 3,5,7,9,11,13 | 8 | 1 | 3 | 2 |
| 15 | B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 16 | B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 17 | B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 18 | B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 19 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 20 | B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 21 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 22 | B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 23 | B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 | 2 |
| 24 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 25 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 26 | B1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 | 2 | 3 | 2 |
| 27 | B1 | 8 | 1,2 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 0 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 1 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 or 2 | 1 | 12 |
| 2 | B4 | 1 | 0 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 3 | B4 | 1 | 0 | 24,29,34,39 | 2 | 1 | 1 | 12 |
| 4 | B4 | 1 | 0 | 17,19,37,39 | 0 | 1 | 1 | 12 |
| 5 | B4 | 1 | 0 | 19,39 | 2 | 1 or 2 | 1 | 12 |
| 6 | B4 | 1 | 0 | 9,19,29,39 | 2 | 2 | 1 | 12 |
| 7 | B4 | 1 | 0 | 0,1,2,…,39 | 2 | 1 | 1 | 12 |
| 8 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 9 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 1 or 2 | 1 | 12 |
| 10 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 11 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 or 2 | 1 | 12 |
| 12 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 1 | 12 |
| 13 | B4 | 1 | 0 | 3,5,7 | 0 | 1 | 1 | 12 |
| 14 | B4 | 1 | 0 | 3,5,7,9,11,13 | 2 | 1 | 1 | 12 |
| 15 | B4 | 16 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| FO | B4 | 16 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 17 | B4 | 8 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 18 | B4 | 8 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 19 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 20 | B4 | 4 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 21 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 22 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 23 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 or 2 | 1 | 12 |
| 24 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 25 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 26 | B4 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 2 | 2 | 1 | 12 |
| 27 | B4 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 28 | B4 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 1 | 12 |
| 0 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 1 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 2 | C0 | 1 | 0 | 9,19,29,39 | 0 | 2 | 7 | 2 |
| 3 | C0 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 | 2 |
| 4 | C0 | 1 | 0 | 17,19,37,39 | 0 | 1 | 7 | 2 |
| 5 | C0 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 6 | C0 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 |
| 7 | C0 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 | 2 |
| 8 | C0 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 9 | C0 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 10 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 11 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 12 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 7 | 2 |
| 13 | C0 | 1 | 0 | 3,5,7 | 0 | 1 | 7 | 2 |
| 14 | C0 | 1 | 0 | 3,5,7,9,11,13 | 8 | 1 | 3 | 2 |
| 15 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 or 2 | 7 | 2 |
| EF | C0 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 17 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 or 2 | 7 | 2 |
| 18 | C0 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 19 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 20 | C0 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 21 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 22 | C0 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 23 | C0 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 7 | 2 |
| 24 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 25 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 26 | C0 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 | 2 | 3 | 2 |
| 27 | C0 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 7 | 2 |
| 28 | C0 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 7 | 2 |
| 0 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 1 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 or 2 | 1 | 6 |
| 2 | C2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 2 | 6 |
| 3 | C2 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 | 6 |
| 4 | C2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 |
| 5 | C2 | 1 | 0 | 19,39 | 2 | 1 | 2 | 6 |
| 6 | C2 | 1 | 0 | 9,19,29,39 | 7 | 2 | 1 | 6 |
| 7 | C2 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 | 6 |
| JF | C2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 9 | C2 | 1 | 0 | 23,27,31,35,39 | 7 | 1 or 2 | 1 | 6 |
| 10 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 11 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 or 2 | 1 | 6 |
| 12 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 |
| 13 | C2 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 |
| 14 | C2 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 1 | 6 |
| 15 | C2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 16 | C2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 17 | C2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 18 | C2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 19 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 20 | C2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 21 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 22 | C2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 23 | C2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 or 2 | 2 | 6 |
| 24 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 25 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 26 | C2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 1 | 6 |
| 27 | C2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| 28 | C2 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 2 | 6 |
| 0 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| HK | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 2 | A1/B1 | 1 | 0 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 3 | A1/B1 | 1 | 0 | 9,19,29,39 | 8 | 1 | 3 | 2 |
| 4 | A1/B1 | 1 | 0 | 17,19,37,39 | 2 | 1 | 6 | 2 |
| 5 | A1/B1 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 6 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 7 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 8 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 9 | A1/B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 | 2 |
| 10 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 | 2 |
| 11 | A1/B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 12 | A1/B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 13 | A1/B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 14 | A1/B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 15 | A1/B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 16 | A1/B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 17 | A1/B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 0 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 1 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 6 | 1 | 2 | 4 |
| 2 | A2/B2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 3 | 4 |
| 3 | A2/B2 | 1 | 0 | 9,19,29,39 | 6 | 1 | 2 | 4 |
| 4 | A2/B2 | 1 | 0 | 17,19,37,39 | 2 | 1 | 3 | 4 |
| 5 | A2/B2 | 1 | 0 | 19,39 | 6 | 1 | 2 | 4 |
| 6 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 7 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 6 | 1 | 2 | 4 |
| 8 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 9 | A2/B2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 3 | 4 |
| 10 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 3 | 4 |
| 11 | A2/B2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 12 | A2/B2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 13 | A2/B2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 14 | A2/B2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 15 | A2/B2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 16 | A2/B2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 17 | A2/B2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 0 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 1 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 | 6 |
| 2 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 | 2 | 2 | 6 |
| 3 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 | 1 | 2 | 6 |
| 4 | A3/B3 | 1 | 0 | 17,19,37,39 | 2 | 1 | 2 | 6 |
| 5 | A3/B3 | 1 | 0 | 19,39 | 2 | 1 | 2 | 6 |
| 6 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 7 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 2 | 6 |
| 8 | A3/B3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 9 | A3/B3 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 2 | 6 |
| 10 | A3/B3 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 2 | 6 |
| 11 | A3/B3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 12 | A3/B3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 13 | A3/B3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 14 | A3/B3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 15 | A3/B3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 16 | A3/B3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 17 | A3/B3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |

=============== End Text proposal for 38.211, section 6.3.3 =============================

Agreements:

- To adopt the following TP to 6.3.3.1 of 38.211

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter PRACHRootSequenceIndex. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic: the logical index 0 is consecutive to 837 when  and is consecutive to 137 when . The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 and 6.3.3.1-4.

Agreements:

- For both FR1 and FR2, the previous working assumption is replaced with the following working assumption

  - Note: The value with yellow background is not final

- For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is more than at least N symbols after DL part and/or the last symbol of an SSB.

  - N = 0 for Msg1 SCS = 1.25kHz, 5kHz

  - N = 2 for Msg1 SCS = 15kHz

  - N = 2 for Msg1 SCS = 30kHz

  - N = 2 for Msg1 SCS = 60kHz

  - N = 2 or 3 for Msg1 SCS = 120kHz

  - Note for format B4, N = 0 irrespectively of Msg1 SCS

- UE is not expected to receive DL signals during any valid RACH occasion

- The SSB to RO association pattern period shall repeat at most every 160 ms

  - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid.

- Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

- FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx

In this paper, we continue the discussion on the remaining issues on the PRACH format.

# Discussion

## Timing advance in PRACH baseband signal generation

In Section 5.3.2 of TS 38.211 for PRACH baseband signal generation, it is defined that “a timing advance value NTA=0 shall be assumed”

“The starting position  of the PRACH preamble in a subframe (for ) or in a 60 kHz slot (for ) is given by

where

-	the subframe or 60 kHz slot is assumed to start at  ;

-	a timing advance value NTA=0 shall be assumed;

….

However, the timing advance value NTA  is actually not used in the generation of the PRACH OFDM baseband signal. Furthermore, timing advance is more related to the uplink signals transmission instead of uplink signals generation. Thus, it seems the requirement for the timing advance NTA should be defined in another section or specification, where the PRACH signal transmission or the timing advance requirements are defined. In addition, for a UE already in connected state, the uplink transmission timing of the UE should be already adjusted by TA commands from the network. Thus, it would be better to continue using it, at least for the case when the PRACH preamble is sent to the current serving cell, instead of resetting it to zero.

Observation 1: The timing advance value NTA  is not used in the generation of the OFDM baseband signals of PRACH. Thus, Section 5.3.2 of TS 38.211 might not be the best place for placing the timing advance requirement, which might be placed under a section for PRACH transmission or TA requirement.

Proposal 1: For a UE in RRC_CONNECTED, when a UE transmits the PRACH preamble to the current serving cell, the UE should use the previous received TA adjustments for the PRACH transmission instead of setting the N_TA = 0.

## PRACH Format for FR2

In RAN1#92bis, the working assumption for FR2 RACH configuration table (Table 6.3.3.2-4 in TS 38.211) was updated. The following entries are not finalized in the table:

| PRACH Config. Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | A1 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 6 | 2 |
| 22 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 6 | 2 |
| 37 | A2 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 3 | 4 |
| 47 | A2 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 3 | 4 |
| 75 | A3 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 2 | 6 |
| 78 | A3 | 1 | 0 | 7,15,23,31,39 or 9,11,13 | 2 | 1 | 2 | 6 |
| 104 | B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 | 2 |
| 123 | B4 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 1 | 12 |
| 127 | B4 | 1 | 0 | 19,39 | 2 | [1 or 2] | 1 | 12 |
| 133 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | [1 or 2] | 1 | 12 |
| 135 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | [1 or 2] | 1 | 12 |
| 138 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | [1 or 2] | 1 | 12 |
| 141 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | [1 or 2] | 1 | 12 |
| 144 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | [1 or 2] | 7 | 2 |
| 146 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | [1 or 2] | 7 | 2 |
| 152 | C0 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 7 | 2 |
| 163 | C0 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 7 | 2 |
| 181 | C2 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 2 | 6 |
| 191 | C2 | 1 | 0 | 7,15,23,31,39 | 2 | [1 or 2] | 2 | 6 |
| 193 | C2 | 1 | 0 | 23,27,31,35,39 | 7 | [1 or 2] | 1 | 6 |
| 195 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | [1 or 2] | 1 | 6 |
| 198 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | [1 or 2] | 1 | 6 |

As shown in the following Figure 1, including slot numbers 7 and 15 would be help when the semi-static TDD UL-DL configuration of, for example, alternative DL and UL, or 3 DLs and 1 UL configurations. Thus, we think it would be fine to include the slots numbers 7 and 15 in above table.

Figure 1: Examples of using slots 7 and 15 for supporting semi-static TDD UL-DL configuration patterns.

For the undecided number of PRACH slots within a 60 kHz slot is [1 or 2] for the Forma4 B4 and C2, we don’t have very strong view either way. Having the number of PRACH slots = 1 may be preferable based on the consideration of the reduction of the RO resource usage.

# Conclusion

In this contribution, we discussed the remaining issues in PRACH format design. The following proposals were made based on the discussion:

Observation 1: The timing advance value NTA  is not used in the generation of the OFDM baseband signals of PRACH. Thus, Section 5.3.2 of TS 38.211 might not be the best place for placing the timing advance requirement, which might be placed under a section for PRACH transmission or TA requirement.

Proposal 1: For a UE in RRC_CONNECTED, when a UE transmits the PRACH preamble to the current serving cell, the UE should use the previous received TA adjustments for the PRACH transmission instead of setting the N_TA = 0.

# References

  - Draft Report of 3GPP TSG RAN WG1 #92bis v0.2.0, ETSI

  - R1-1805738, Summary of Remaining details on PRACH formats, Convida Wireless

  - R1-1803732, Further details on NR RACH format, CATT