3GPP TSG RAN WG1 Meeting #93		R1-1806341

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.1.2.3.2

Source:	Lenovo, Motorola Mobility

Title:	Remaining issues on CSI-RS

Document for:	Discussion

- Introduction

In RAN1#92bis meeting, the agreement regarding CSI-RS is as following:

Agreement

- The UE expects that all the CSI resources of a resource set are configured with the same starting RB and number of RBs and the same CDM-type.

  - FFS: Other parameters

In this contribution, we mainly address the FFS issues.

- Discussion

2.1 Background on CSI-RS configuration

The RRC parameters for CSI-RS resource is captured in IE NZP-CSI-RS-Resource as following:

The content of IE CSI-RS-ResourceMapping is as following:

The content of IE CSI-ResourcePeriodicityAndOffset is as following:

Regarding the usage of a CSI-RS resource set, the corresponding RRC parameter is captured in IE NZP-CSI-RS-ResourceSet as following:

It can be concluded that the purpose of CSI-RS resource set is for beam management or time/frequency tracking(TRS).

2.2 Parameter restriction for beam management

For a CSI-RS resource set used for beam management, there are four parameters without clear agreement on the restriction: density, powerControlOffset, PowerControlOffsetSS, frquencyDomainAllocation.

Regarding the density, if the CSI-RS is configured with 1/2 port, the available density for beam management can be 0.5 or 1. As the beam management CSI-RS is to compare the power of different beams, which is reflected by different NZP CSI-RS resources, it is fair to keep the same density for the CSI-RS resources within the CSI-RS resource set, as the average among different CSI-RS REs will impact the measured RSRP. So here is our proposal:

Proposal 1: The density should be set the same for all the CSI-RS resources within a CSI-RS resource set for beam management.

Regarding powerControlOffset and PowerControlOffsetSS, they are used to indicate the power difference between CSI-RS and PDSCH, and CSI-RS and SS, respectively. Different power offsets for different beams can facilitate biased beam selection, which is useful for load balancing among different beams. So we prefer that there is no restriction for these two parameters, and it is up to gNB implementation to set the suitable power for each beam.

Proposal 2: PowerControloffset and PowerControlOffsetSS can be different for different CSI-RS resources within a CSI-RS resource set for beam management.

Regarding the frequencyDomainAllocation, it indicates the occupied subcarriers for a CSI-RS resource. There will be other signals/channels multiplexed with CSI-RS in a same symbol, and it is difficult to guarantee the same subcarriers among different symbols. So we prefer to give gNB more scheduling freedom for subcarrier allocation.

Proposal 3: FrequencyDomainAllocaiton can be different for different CSI-RS resources within a CSI-RS resource set for beam management.

2.3 Parameter restriction for TRS

For a CSI-RS resource set for TRS, there are already clear agreement in the specification that FrequencyDomainAllocation, PowerOffset, PowerOffsetSS should be set the same for all CSI-RS resources within a CSI-RS resource set.

- Conclusion

In this contribution, we discussed the CSI-RS remaining issues, and our proposals are as following:

Proposal 1: The density should be set the same for all the CSI-RS resources within a CSI-RS resource set for beam management.

Proposal 2: Poweroffset and PowerOffsetSS can be different for different CSI-RS resources within a CSI-RS resource set for beam management.

Proposal 3: FrequencyDomainAllocaiton can be different for different CSI-RS resources within a CSI-RS resource set for beam management.

- References

3GPP RAN1#92bis chairman notes

3GPP TS 38.331 v.f.1.0, “NR Radio Resource Control (RRC) protocol specification”

R1-1805557, “Remaining issues on CSI-RS”, Huawei, HiSilicon, 3GPP RAN1#92bis

3GPP TS 38.214 v.f.1.0, “NR; Physical layer procedures for data”