3GPP TSG-RAN WG1 Meeting #93	 R1-1806435

Busan, Korea, 21st – 25th May, 2018	(Resubmission of R1-1805230)

Agenda Item:	7.8.2

Source:	Ericsson

Title:	IMT2020 self evaluations: On eMBB area traffic capacity with NR

Document for:	Discussion

# Introduction

It has been agreed within 3GPP that according to the New Study Item on Self Evaluation towards IMT-2020 Submission [1] self evaluation results should be provided against technical performance requirements for extreme mobile broadband (eMBB) as defined in [2], including area traffic capacity.

The key minimum technical performance requirements defined by the ITU-R are intended for assuring that IMT-2020 technologies are able to fulfil the objectives of IMT-2020 and to set a specific level of performance that each of the proposed radio interface technologies needs to achieve in order to be considered by ITU-R for IMT-2020.

This contribution presents the preliminary results on area traffic capacity for eMBB and shows that the corresponding IMT-2020 requirements can be met.

# Requirements on area traffic capacity

Area traffic capacity is defined the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e., the number of bits contained in the service data units delivered to layer 3, over a certain period of time.

This can be derived for one frequency band and one transmission reception point (TRxP) layer, based on the achievable average spectral efficiency, network deployment (e.g., TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation

Carea=ρ W SEavg.

In case multiple bands are aggregated, the area traffic capacity will be summed over the bands.

The conditions for evaluation including supportable bandwidth are described in [3]. The requirement is defined for the purpose of evaluation in the Indoor Hotspot (InH) eMBB test environment, where the target value for the area traffic capacity in downlink is 10 Mbit/s/m2.

Table 1 Scenario configurations.

| 5G Indoor Hotspot |  |
| --- | --- |
| Traffic model | Full buffer, 10 UE/cell |
| Channel model | 5G Indoor Office, 20m ISD, AP height 3 m |
| UE speed | 0.5 km/h |
| Carrier frequency | 30 GHz |
| Number of cells | 12 |
| UE Antenna | 1x4 cross poles, 0.5λ separation, random orientation in xy-plane |
| BS Antenna | 4x32 cross poles, 0.5λ in vertical, 0.5λ in horizontal |
| UE antenna gain | 5 dBi |

# System performance

The evaluations herein are based on the preliminary simulation results of spectral efficiency for the modified InH test environment. The simulation details are summarized in Table 1, and the details of the corresponding NR-like 5G prototype are discussed in the companion submission [4].

For the given 8-TRxP InH scenario, the TRxP density is calculated as ρ=0.005 TRxP/m2. The obtained spectral efficiency in the downlink has harmonic mean of SEavg=9.12 bit/s/Hz. The corresponding area traffic capacity is presented in Table 2 below.

Table 2 System performance in the 5G InH scenario

| Bandwidth, W | Area traffic capacity, Carea [Mbit/s/m2] |
| --- | --- |
| 100 MHz | 4.5 |
| 219 MHz | 10 |
| 1 GHz | 45 |

Based on the above results, the following observation can be made.

To achieve the IMT-2020 requirements in terms of area traffic capacity for the specified test environment bandwidth must be larger than 219 MHz .

# Conclusion

In this contribution, we have presented the preliminary results of self-evaluation of the IMT-2020 requirements, as well as provided our preliminary input to the Study Item on Self Evaluation towards IMT-2020 Submission. It has been shown that the IMT-2020 requirements can be fulfilled in terms of area traffic capacity for the corresponding test environment specified by ITU-R.

The following observations have been made:

Observation 1	To achieve the IMT-2020 requirements in terms of area traffic capacity for the specified test environment bandwidth must be larger than 219 MHz .

# References

- RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission,” RAN#75, Ericsson et al.

Draft new Report ITU-R M.[IMT-2020.TECH PERF REQ] - Minimum requirements related to technical performance for IMT-2020 radio interface(s).

Draft new Report ITU-R M.[IMT-2020.EVAL] - Guidelines for evaluation of radio interface technologies for IMT-2020.

R1-1806433, “IMT2020 self evaluation: On eMBB spectral efficiency with NR,” RAN#93, Ericsson.