3GPP TSG RAN WG1 Meeting #93		R1-1806469

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.4.1

Source:	NEC

Title:	Discussion on NoMA Schemes for NR

Document for:	Discussion

# Introduction

In RAN meeting #75, a new SID on Non-Orthogonal Multiple Access (NoMA) for NR was approved [1]. The objective of the NoMA SI is to further progress on the NoMA performance evaluation focusing on uplink, and provide recommendation about the key design features to be specified later.

Up to RAN1#86, 15 candidate MA schemes have been proposed and it was agreed:

- Capture the following observations in the TR

  - All proposed non-orthogonal MA schemes for UL transmission share the following common features:

    - At the transmitter side: using MA signature(s)

    - At the receiver side: allowing multi-user detector

  - All proposed non-orthogonal MA schemes for UL transmission on a high level follow the following basic diagram. Note that the basic diagram is not intended to capture all the details or to be a complete diagram.

NoMA schemes can be used for both grant-based and grant-free transmission. Several benefits of NoMA have been discussed in the literature and a number of contributions submitted to 3GPP by several companies which motivate its application in diverse NR use cases and deployment scenarios including eMBB, mMTC and URLLC as summarized in Table 1.

Table 1 Motivation and expected benefits of NoMA

| Usage Scenario | Motivation | Expected benefits of NoMA |
| --- | --- | --- |
| mMTC | - Massive connections - Infrequent small traffic - High power efficiency (of device and network) | - Higher connection density with high overloading - Robust and high capacity grant-free transmission |
| URLLC | - High reliability - Low latency - Multiplexing with eMBB traffic | - Higher data transmission reliability through diversity gain achieved by spreading and coding - Higher detection reliability and robustness to collision by carefully designing the MA signature - Signalling delay reduction by enabling grant-free access - Ability to multiplex mixed traffic types |
| eMBB | - High spectrum efficiency - High user density - Uniform user experience | - Higher capacity by non-orthogonal user multiplexing - Robustness to fading and interference with code-domain design - Robust link adaptation with relaxed channel state information (CSI) accuracy - Support for both grant-free and grant based access |

In this contribution, we review general categories into which existing NoMA schemes can be divided and discuss potential benefits of NoMA in different scenarios and identify suitability of different schemes in these scenarios.

# NoMA Schemes

Different NoMA schemes can be divided into following categories with respect to multiple access signature at the transmitter:

- Code-based NoMA

In code-based NoMA schemes [2]-[12], different users are assigned different codewords or spreading sequences and multiplexed over the same time-frequency resources.

- Interleave-based NoMA

Interleave-based NoMA schemes [12]-[14] employ user specific bit- and/or symbol-level interleavers for user separation.

- Power domain NoMA

Power domain multiplexing [15] exploits SINR difference among users either due to natural near-far effect or by non-uniform power allocation at the transmitter according to users’ channel conditions to obtain maximum gain in system performance.

In the following, we discuss the features of different schemes, belonging to the aforementioned categories, in terms of transmit side multiple access (MA) signature, receiver side multi-user detector (MUD), their pros and cons and associated challenges to help understand their suitability for different 5G scenarios.

Irrespective of the category, standardization impact of UE signature (codes/interleavers/mapping patterns) design must be identified.

Observation 1: Standardization impact of UE signature (code/interleaver/mapping pattern) design for NoMA schemes must be considered when evaluating NoMA for use in NR.

Moreover, although different NoMA schemes have shown support for higher overloading, it is observed that support for massive connectivity is not fully investigated. Hence, we make following observation:

Observation 2: Evaluation of NoMA for mMTC must consider envisaged support for massive connection density.

## Code-based NoMA

The NoMA schemes in this category can be further divided into sub-categories based on the spreading signature design features such as sparsity, type (sequence/codeword), length and assignment (pseudo-random/fixed). Table 2 shows summary of different code-based NoMA schemes. The pros and cons of these schemes and their suitability for different scenarios are governed by: i) spreading signature design features, and ii) MUD receiver.

As can be seen from Table 2, signature design to support massive connectivity is a challenging task for all schemes. Signature design also affects choice of receiver and its complexity. Generally, design of user signature with long sequence is desirable since it tends to exhibit low cross-correlation among contending user’s signals and increases multiplexing capacity. However, a long user signature introduces high decoding complexity and latency. Moreover, long user signatures may not be suitable for transmission with small bandwidth allocation. Therefore, NoMA techniques which can support high user overloading with short length signatures (e.g. LSSA) must be investigated particularly for mMTC and URLLC.

Observation 3: Short length user signature design helps reduce decoding complexity and latency.

Observation 4: Trade-off among signature length, overloading factor, decoder complexity and latency must be investigated considering flexible bandwidth allocation.

Decoding complexity and latency also depends on the MUD algorithm at the receiver. Successive interference cancellation (SIC) receiver is considered the simplest whose complexity increases linearly with the number of users/data layers to be decoded. SIC receiver is generally suitable for schemes employing long or non-sparse spreading/scrambling sequences (e.g. MUSA, NOCA, LCRS, RSMA LSSA, GOCA) or with unequal diversity order for different users (e.g. PDMA) to reduce receiver complexity. However, it’s performance degrades when there is negligible SNR difference or channel fading is correlated for the superposed users. Moreover, pipeline of processing may be changed to perform SIC operation. SIC receiver also suffers from error propagation problem which makes its adoption a challenge particularly when initial transmission is required with larger target BLER and when large number of users are superposed. Therefore, although low-complexity SIC helps reduce receiver complexity, its performance must be evaluated considering practical deployment conditions (including operating SNR, channel fading, user location, connection density etc.) and BLER and latency target for each usage scenario.

Observation 5: Feasibility of SIC receiver must be evaluated considering practical deployment conditions (including operating SNR, channel fading, user location etc.) and performance targets (BLER, connection density and latency) for each usage scenario.

Advanced receivers using message parsing algorithm (MPA) can greatly improve detection performance that is particularly important for URLLC at the cost of increased complexity. However, sparse signature design (e.g. in SCMA, PDMA and LDS-SVE) which reduces multi-user interference (MUI) can help reduce MPA receiver complexity. Hence, while it is challenging to adopt such schemes in mMTC due to challenging codebook/signature design for massive connectivity, sparse signature design with MPA can be investigated as a solution for high reliability grant-free transmission in URLLC and higher capacity in grant-free/grant-based transmission in eMBB.

Observation 6: Sparse signature design with MPA is a potential solution for high reliability grant-free transmission in URLLC and higher capacity in grant-free/grant-based transmission in eMBB.

Besides performances, backward compatibility of the receiver architecture must also be considered for NoMA evaluation.

Observation 7: Backward compatibility of the receiver architecture must be considered for NoMA evaluation.

Table 2 Summary of Code-based NoMA

| Scheme | Transmit & Receive features | Pros and Cons | Challenges |
| --- | --- | --- | --- |
| SCMA | - Coded bits are directly mapped to a codeword from a built codebook according to a multi-dimensional constellation - Sparse symbol to resource element (RE) mapping pattern is used - MPA or MPA with SIC is used at the receiver | - Shaping gain due to multi-dimensional constellation - Near optimal performance using MPA detector - Favorable for high reliability transmission - High receiver complexity - PAPR performance is similar to OFDMA - Code design is complicated and varies with e.g. different number of users and/or modulation order | - Support for massive connectivity due to challenging codebook design - Achieving latency target in URLLC |
| PDMA | - Sparse pattern for symbol-to-RE mapping - Unequal diversity order for different users/codes - Belief propagation (BP) algorithm at the receiver - Advanced receiver combining MPA and SIC decoding techniques can also be used | - Flexible in terms of receiver complexity - Can alleviate error propagation problem of SIC - PAPR may increase depending on a diversity pattern - Variable multi-user interference due to unequal diversity orders for different users | - Code matrix design to reach good trade-off among overload factor, diversity order and detection complexity |
| LDS-SVE | - LDS spreading with signature vector extension - MPA receiver | - Performance enhancement by exploiting diversity gain - More robustness against inter-user interference/collision - May save reference signal overhead - High receiver complexity - Increase in PAPR | - Design of LDS code and signature vector extension method |
| MUSA | - Random complex spreading codes - Users’ data are used for channel estimation - Code level SIC receiver | - Low complexity receiver - Capability to support massive connections - Improved system efficiency since no pilots or preambles are employed for channel estimation - Users must be synchronized for optimal decoding performance - Pipeline of processing may be changed in order to perform SIC operation | - Design of spreading code and its impact on the complexity of SIC - Handling of error propagation problem, in particular when initial transmission is required with larger target BLER than 10% or when large number of users are superposed |
| NOCA | - LTE defined low correlation sequences for spreading - SIC receiver | - Low complexity receiver - Can support high overloading and facilitate inter-cell coordination for interference mitigation | - Handling of error propagation problem in SIC receiver |
| NCMA | - Grassmannian spreading sequence based non-orthogonal code cover (NCC) allocated to each UE - Parallel interference cancellation (PIC) at the receiver | - Low complexity MUD - Capability to support massive connectivity or contention based MA | - Spreading code design to handle mutual interference in mMTC |
| LCRS | - Combines channel coding with spreading via low rate codes to maximize the coding gain - UE specific channel interleaver - SIC receiver | - Low complexity receiver using SIC | - Spreading code design - Design of channel interleaver for multi-UE signal separation at the receiver in mMTC |
| RSMA | - Uses combination of low rate channel codes and scrambling codes (and optionally different interleavers) with good correlation properties - SIC based receiver | - Potential to support grant free transmission of small data - At low spectral efficiency (SE), RSMA is a good trade-off between performance, scalability and complexity - Single carrier (SC) RSMA supports asynchronous transmission - Link budget gain with SC RSMA - Self-interference caused by multipath fading channel in case wide bandwidth is utilized for resource allocation in SC based RSMA - Advanced receiver needed for frequency selective channel - Needs synchronous multiplexing for OFDM-based RSMA | - Defining SC based new waveform for asynchronous transmission - Scrambling sequence and interleaver design for massive connectivity |
| LSSA | - Bit/symbol level signature multiplexing - SIC/PIC receiver | - Linear complexity receiver - Can support asynchronous UL transmission multiplexing - Potential to support massive connectivity | - Signature design to support envisaged massive connectivity in mMTC |
| GOCA | - Orthogonal sequences in a group and non-orthogonal sequences for different groups - SIC receiver | - Significant MUI reduction by orthogonal sequences in the same group - Can support high overloading due to better MUI suppression | - Handling of error propagation problem in SIC receiver - Orthogonal sequence design for massive connectivity |

## Interleave-based NoMA

NoMA schemes in this category include repetition division multiple access (RDMA) [12], interleave division multiple access (IDMA) [13] and interleave-grid multiple access (IGMA) [14]. These techniques are summarized in the following table.

Table 3 Summary of Interleave-based NoMA

| Scheme | Transmit & Receive features | Pros and Cons | Challenges |
| --- | --- | --- | --- |
| IGMA | - Flexibility to choose bit level interleavers and/or grid mapping pattern to separate users - Symbol level interleaver - Elementary signal estimator (ESE) at the receiver - Maximum a posteriori (MAP) and message parsing algorithm (MPA) detectors are also applicable | - Scalability to support different connection densities - More robustness to frequency selective fading and inter-cell interference due to symbol-level interleaver as compared to resource mapping using code-matrices/ codebooks - Low complexity MUD using ESE - Flexibility to improve detection performance using MPA/MAP | - Design of interleavers and grid mapping patterns to support massive connectivity |
| IDMA | - low rate coding and user-specific bit-level interleavers to separate UEs - ESE at the receiver - Iterative detections between ESE and channel decoder | - Strong robustness against asynchronicity and tolerance upon users’ overloading - Simple and effective receiver - Exploits frequency diversity due to wider frequency allocation size - Suffers less from ICI from adjacent users - High detection and decoding complexity due to iterative detections between ESE and channel decoder | - Avoidance of interleaver pattern collision - Impact of non-ideal channel estimation on iterative detection |
| RDMA | - Cyclic-shift repetition to separate different users’ signals - SIC receiver | - Simple transmitter because no scrambler or random interleave is required - Good trade-off between receiver complexity and performance | - Absolute value of position in repetition pattern should be as small as possible to utilize diversity better |

Similar to challenging code book design, interleaver pattern design and collision avoidance is a challenging task and the evaluation for mMTC must consider envisaged support for massive connection density.

## Power domain NoMA

Power domain NOMA does not rely on the knowledge of instantaneous CSI of frequency-selective fading. Thus, a robust performance gain in wide area deployments can be expected irrespective of UE mobility or CSI feedback latency. On the other hand, this scheme relies on channel difference for user differentiation and, therefore, may not be suitable in small cells wherein location distribution of users is concentrated. In grant-based transmission, scheduling flexibility is limited using power domain NoMA, especially with increased number of users, due to constraints on channel condition difference the paired users shall have to achieve good detection and demodulation performance. This scheme is also considered unsuitable for small packet transmission [2] that is a typical characteristic of mMTC. It is also not feasible for use in scenarios with channel and interference uncertainties, e.g., in grant-free UL transmission.

However, NOMA with user grouping i.e. NOMA combined with multi-level average received powers and/or signature grouping can provide performance enhancement [16].

At the receiver, low complexity receiver based on SIC is used for multi-user detection (MUD). Considering the requirement to support massive connectivity in mMTC scenario, error propagation problem may be a difficult challenge to overcome in SIC receiver based power domain NoMA.

Observation 8: Power domain NoMA can provide robust performance gain in wide area deployment.

Observation 9:  Power domain NoMA may not be suitable for grant-free UL transmission, small packet transmission with high reliability requirement, in mMTC, and when location distribution of users is concentrated.

# Conclusion

From the discussion, we have the following observations.

Observation 1: Standardization impact of UE signature (code/interleaver/mapping pattern) design for NoMA schemes must be considered when evaluating NoMA for use in NR.

Observation 2: Evaluation of NoMA for mMTC must consider envisaged support for massive connection density.

Code-domain NoMA:

Observation 3: Short length user signature design helps reduce decoding complexity and latency.

Observation 4: Trade-off among signature length, overloading factor, decoder complexity and latency must be investigated considering flexible bandwidth allocation.

Observation 5: Feasibility of SIC receiver must be evaluated considering practical deployment conditions (including operating SNR, channel fading, user location etc.) and performance targets (BLER, connection density and latency) for each usage scenario.

Observation 6: Sparse signature design with MPA is a potential solution for high reliability grant-free transmission in URLLC and higher capacity in grant-free/grant-based transmission in eMBB.

Observation 7: Backward compatibility of the receiver architecture must be considered for NoMA evaluation.

Power-domain NoMA:

Observation 8: Power domain NoMA can provide robust performance gain in wide area deployment.

Observation 9:  Power domain NoMA may not be suitable for grant-free UL transmission, small packet transmission with high reliability requirement, in mMTC, and when location distribution of users is concentrated.

# References

RP-170829, “New SID on Non-Orthogonal Multiple Access (NoMA) for NR”, RAN#75, Dubrovnik, Croatia, March 6 - 9, 2017.

R1-162153, Huawei, HiSilicon, “Overview of non-orthogonal multiple access for 5G,” Apr. 2016.

R1-162202, Qualcomm Inc., “Candidate NR Multiple Access Schemes,” Apr. 2016.

R1-162226, ZTE, “Discussion on multiple access for new radio interface,” Apr. 2016.

R1-162385, Intel Corporation, “Multiple access schemes for new radio interface,” Apr. 2016.

R1-162517, LG electronics, “Considerations on DL/UL multiple access for NR,” Apr. 2016.

R1-164329, Fujitsu, “Initial LLS results for UL non-orthogonal multiple access,” May 2016.

R1-163510, Qualcomm Inc., “Candidate NR Multiple Access Schemes,” Apr. 2016.

R1-164869, ETRI, “Low code rate and signature based multiple access scheme for New Radio,” May 2016.

R1-165019, Nokia, Alcatel-Lucent Shanghai Bell, “Non-orthogonal multiple access for New Radio,” May 2016.

R1-163383, CATT, “Candidate Solution for New Multiple Access,” Apr. 2016

R1-167535, Media Tek Inc., “New uplink non-orthogonal multiple access schemes for NR,” Aug. 2016.

R1-165021, Nokia, “Performance of Interleave Division Multiple Access (IDMA) in Combination with OFDM Family Waveforms,” May 2016.

R1-163992, Samsung, “Non-orthogonal Multiple access candidate for NR,” May 2016.

R1-163111, NTT DOCOMO, “Initial views and evaluation results on non-orthogonal multiple access for NR uplink,” Apr. 2016.

R1-1800686, NTT DOCOMO, “NOMA scheme with user grouping,” Jan. 2018.