3GPP TSG RAN WG1 Meeting #93																R1-1806486

Busan, Korea, May 21st – 25th, 2018

Source:	Intel Corporation

Title:	UE Capabilities for LTE Rel.15 V2X Communication

Agenda item:	6.2.5.5

Document for:	Discussion and Decision

# Introduction

The LTE Rel.15 work item on V2X communication is approaching its successful completion. In order to finalize the work, UE capability needs to be discussed, which is the main scope of this contribution. Our views on other remaining opens are provided in our companion contributions [1]-[5].

# Summary of Introduced L1 Features

From Layer-1 perspective, the following new UE features were introduced in the LTE Rel.15:

- PSSCH demodulation enhancements

  - Rate matching over the last symbol of sidelink subframe

  - TBS scaling

  - New MCS/TBS table entries – still pending RAN1 decision on specific entries

  - Support of 64QAM

- Enhanced synchronization for sidelink CA

  - Single synchronization reference for TX/RX across aggregated sidelink CCs

  - Aligned sync source priority rules, DFN and SLSS synchronization resources across aggregated sidelink CCs

  - Sidelink synchronization carrier (re)-selection

  - SLSS transmission across aggregated CCs

    - SLSS transmission on single synchronization carrier

    - SLSS transmission across multiple synchronization carriers (Set-B sync CCs)

- Mode-3/Mode-4 pool sharing

  - SPS period indication in SCI by Mode-3 UEs

  - [Mode-3 UE sidelink measurements and reporting - still open for discussion]

- Resource selection latency reduction

  - Configurable minimum resource selection window T2min = [10], 20 and its (pre)-configuration based on PPPP

  - [SPS = 10ms –  proposed / agreed in RAN2 and subject to further discussion in RAN1]

- Sidelink carrier aggregation for transmission/reception

  - Sidelink transmit power allocation across aggregated CCs

  - Enhanced resource selection across aggregated CCs in case of limited TX capabilities (exclusion of resources)

- Transparent transmit diversity scheme

  - Up to RAN4 to conclude on how/whether it is defined/signaled

Considering that new UE features were introduced, in the next sections we further discuss UE sidelink categories and decoding capabilities and general UE capabilities for the LTE Rel.15 V2X communication.

# UE Sidelink Categories

In the LTE Rel.14 specification, the following sidelink categories are described (see Table 1), where SL-C Category 2 corresponds to sidelink V2X communication introduced in the LTE Rel.14.

Table 1: Reception and transmission of physical parameter values set by ue-CategorySL-C

| UE SL-C Category | Maximum number of SL-SCH transport block bits received within a TTI | Maximum number of bits of a SL-SCH transport block received within a TTI | Maximum number of SL-SCH transport block bits transmitted within a TTI | Maximum number of bits of a SL-SCH transport block transmitted within a TTI | Maximum number of supported layers for spatial multiplexing in SL-C |
| --- | --- | --- | --- | --- | --- |
| SL-C Category 1 | 25456 | 25456 | 25456 | 25456 | 1 |
| SL-C Category 2 | 31704 | 31704 | 31704 | 31704 | 1 |

Sidelink Transmission and Reception Capabilities

As it can be seen from Table 1, the common sidelink category defines UE transmission and reception capabilities for sidelink communication. SL-C Category 2 defined for LTE V2X was introduced under assumption of single carrier operation. Sidelink carrier aggregation was introduced in Rel.15 and additional band combinations were defined by RAN4 (e.g. 10MHz+10MHz, 20MHz+10MHz, 10MHz+10MHz+10MHz). Therefore, additional UE categories covering CA cases may need to be defined. Furthermore it is assumed that UE might support different aggregated bandwidth for transmission and reception. So, it is obvious that based on UE implementation SL-C category from transmission and reception perspective may be significantly different. Therefore, we propose to separately define UE SL-C Category for TX and RX.

Proposal 1

- Separately define UE SL-C Category for transmission (UE SL-C-TX) and reception (UE SL-C-RX).

In Rel.15, the support of 64-QAM modulation for sidelink communication was introduced. The support of 64QAM transmission and reception is considered to be an optional UE capability. Given that maximum modulation order supported by UE determines the maximum number of bits of a SL-SCH transport block transmitted within a TTI, we suggest to define different categories depending on whether 64QAM is supported by UE from TX/RX perspective. The proposal on maximum number of bits of a SL-SCH transport block within a TTI is provided in Table 2 where in case of 64 QAM we use MCS index 30 / TBS index 29 and apply TBS scaling factor 0.8 assuming 96 PRBs are allocated for PSSCH transmission in case of 20MHz spectrum. Other entries are determined by proportional scaling and finding the nearest TBS.

Table 2: Transmission of physical parameter values set by ue-CategorySL-C-TX

| UE SL-C-TX Category | Maximum number of SL-SCH transport block bits transmitted within a TTI | Maximum number of bits of a SL-SCH transport block transmitted within a TTI | Maximum number of supported layers for spatial multiplexing in SL-C | Comments |
| --- | --- | --- | --- | --- |
| SL-C-TX Category 1 | 25456 | 25456 | 1 |  |
| SL-C-TX Category 2 | 31704 | 31704 | 1 | 20MHz aggregated BW 16QAM |
| SL-C-TX Category 3 | 32856+16416 | 32856 | 1 | 30MHz aggregated BW 16QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |
| SL-C-TX Category 4 | 57336 | 57336 | 1 | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-TX Category 5 | 57336+28336 | 57336 | 1 | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

Soft-channel bits

In the LTE Rel.14, the amount of soft channel bits were derived assuming uncoded 16-QAM transmission over 16 parallel RX processes 737280 = 4 (16QAM) ·12 (REs) · 96 (PRBs) · 10 (Symbols) · 16 (Processes). For the LTE R15, similar approach can be used assuming 64QAM but using 9 symbols given that last symbol of subframe is rate-matched. In this case amount of soft channel bits in 20MHz carrier is equal to 6 (64QAM) ·12 (REs) · 96 (PRBs) · 9 (Symb) · 16 (Processes) = 995328. In case of 30MHz aggregated bandwidth, the amount of soft bits will be equal to 1492992. The calculated values on number of soft channel bits are summarized in Table 3 jointly with RX reception parameters.

Table 3: Reception of physical parameter values set by ue-CategorySL-C-RX

| UE SL-C-RX Category | Maximum number of SL-SCH transport block bits received within a TTI | Maximum number of bits of a SL-SCH transport block received within a TTI | Total number of soft channel bits | Comments |
| --- | --- | --- | --- | --- |
| SL-C-RX Category 1 | 25456 | 25456 |  |  |
| SL-C-RX Category 2 | 31704 | 31704 | 737280 | 20MHz aggregated BW 16QAM |
| SL-C-RX Category 3 | 32856+16416 | 32856 | 995328 | 30MHz aggregated BW 16QAM |
| SL-C-RX Category 4 | 57336 | 57336 | 995328 | 20MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz and 20MHz) |
| SL-C-RX Category 5 | 57336+28336 | 57336 | 1492992 | 30MHz aggregated BW 64QAM (Applicable for 10MHz+10MHz+10MHz and 10MHz+20MHz) |

Proposal 2

- Adopt UE SL-C-TX and SL-C-RX categories defined in Table 2 and Table 3 respectively.

# UE Sidelink Processing Capabilities

In LTE R14, a UE is not expected to attempt to decode more than 20 PSCCHs and 136 RBs in a subframe depending on UE capability (v2x-HighReception-r14). By default UE is not expected to attempt to decode more than 10 PSCCHs and 100 RBs in a subframe depending on UE capability. These UE processing capabilities were defined assuming 20MHz carrier. In case of sidelink CA and considering that minimum carrier bandwidth is 10 MHz the UE capabilities can be scaled according to the aggregated sidelink RX bandwidth. Table 4 represents approach based on scaling of the V2X sidelink processing capabilities based on the RX BW aggregated by the UE. If this approach is agreed then the same processing capability (v2x-HighReception or default) will be applied to all CCs and overall UE processing capabilities will depend only on the RX aggregated bandwidth.

Table 4: V2X Sidelink Processing Capabilities – Aggregated RX BW Approach

| Aggregated RX BW, MHz | V2X High Reception Capability | V2X Default Reception Capability |
| --- | --- | --- |
| 20 (Rel.14) | 20 PSCCHs and 136 RBs | 10 PSCCHs and 100 RBs |
| 30 (Rel.15) | 30 PSCCHs and 204 RBs | 15 PSCCHs and 150 RBs |

The alternative and more flexible approach is to define V2X sidelink processing capabilities for 10MHz and 20MHz CCs and signal whether UE supports high reception capability per CC per band per band combination. In this case signalling of high reception (v2x-HighReception) capability can be signalled per CC. For example, if UE indicates support of 10 and 20 MHz CA, it can indicate whether high reception capability is supported only for CC of 10MHz, 20MHz or both.

Table 5: V2X Sidelink Processing Capabilities – Indication per CC

| Aggregated RX BW, MHz | V2X High Reception Capability | V2X Default Reception Capability |
| --- | --- | --- |
| 10 (Rel.15) | 10 PSCCHs and 68 RBs | 5 PSCCHs and 50 RBs |
| 20 (Rel.14) | 20 PSCCHs and 136 RBs | 10 PSCCHs and 100 RBs |

Proposal 3

- Specify V2X sidelink processing (UE decoding) capabilities for 10MHz bandwidth as shown in Table 5.

- Define signalling of v2x-HighReception capability per CC per band per band combination.

# UE Sidelink Capabilities

In this section, we finally discuss UE capabilities as we view them according to the L1 features introduced in LTE Rel.15. Our views are summarized in Table 6.

Table 6: UE capabilities for LTE R15 Sidelink V2X communication

| Feature Group | Components | Prerequisite Feature Groups | Need for eNB to know | Responsible WG | Recommendation |
| --- | --- | --- | --- | --- | --- |
| 1-PSSCH enhancements | 1-Rate Matching 2-TBS Scaling |  | Yes | RAN1 | Optional. Per UE capability signalling. |
| 2-64QAM | 1-Support of 64-QAM and three new MCS / TBS table entries | 1 | Yes | RAN1 | Optional No dedicated UE capability signalling. Part of UE category. |
| 3-Sidelink Single Carrier SLSS TX | 1-Sync Carrier Selection 2-SLSS Transmission at selected sync carrier | Support of R14 SLSS TX/RX | Yes | RAN1 | Optional Per UE capability signalling. |
| 4-Sidelink Multi-Carrier SLSS TX | 1-Sync Carrier Selection 2-Multi-Carrier SLSS Transmission | 3 | Yes | RAN1 | Optional Per UE capability signalling. |
| 5-Sidelink CA RX | 1-Reception across aggregated sidelink CCs | 1 |  | RAN1 | Optional (Signalling of CA RX band combinations – existing signalling may be sufficient) Note: RAN2 to discuss whether packet duplication or packet reception from aggregated CCs is separate capability) |
| 6-Sidelink CA TX | 1-Enhanced resource selection 2-Transmission across multiple CCs | 1, 5 | Yes | RAN1 | Optional (Signalling of CA TX band combinations – existing signalling may be sufficient) Note: RAN2 to discuss whether packet duplication or packet transmission across CCs is separate capability |
| 7-Latency of Resource Selection | 1-Configurable resource selection window duration {10, 20} and dependence on PPPP [2-Support of 10ms SPS by Mode-3 UEs – TBD in RAN1] | R14 Mode-4 Support w/ Full Sensing | Yes | RAN1 | Optional Per UE capability signalling. |
| 8-Pool sharing | 1-Indication of SPS period by Mode-3 UEs [2-Mode-3 UE sidelink measurements and reporting, if it is finally agreed – TBD in RAN1/RAN2] | R14 Mode-3 Support | Yes | RAN1 | Optional. Per UE capability signalling. |
| 9 – PSSCH and PSCCH Transparent Transmit Diversity | 1 – SD-CDD for PSSCH and PSCCH Transmit Diversity |  | No | RAN4 | Optional without UE capability signalling |

Proposal 4

- Further discuss and agree on UE capabilities for R15 sidelink V2X communication as proposed in Table 6.

# Summary

In this contribution, we have provided our initial views on UE capabilities for LTE R15 V2X communication. In summary, we suggest further discuss and agree on proposals made in this contribution.

# References

- R1-1806481, “Remaining physical layer aspects of sidelink carrier aggregation for mode-4 LTE V2V communication”, Intel Corporation, Busan, Korea, May 2018.

- R1-1806482, “Remaining sidelink synchronization details for LTE V2V carrier aggregation”, Intel Corporation, Busan, Korea, May 2018.

- R1-1806483, “Remaining details of demodulation enhancements for LTE V2V sidelink communication”, Intel Corporation, Busan, Korea, May 2018.

- R1-1806484, “On sidelink resource pool sharing for eNB-controlled and UE-autonomous V2V transmission modes”, Intel Corporation, Busan, Korea, May 2018.

- R1-1806485, “Remaining details of reduced resource selection time for LTE V2V sidelink communication”, Intel Corporation, Busan, Korea, May 2018.