3GPP TSG RAN WG1 Meeting #93	R1-1806582

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	6.2.5.1.2

Source: 	LG Electronics

Title:	Remaining issues on synchronization for sidelink CA

Document for:	Discussion and Decision

Introduction

In RAN1 #92bis, the following agreements which are relevant to synchronization for sidelink CA were made:

Agreement

- For UEs operating with CA

  - RAN1 assumes a UE may be configured a non-synchronization carrier by defining the location of the SLSS resources and by configuring the UE to not transmit SLSS on that carrier.

    - Rel. 14 RRC signalling is not sufficient.

    - Include an RRC parameter to introduce such mechanism.

      - A Rel.15 UE using the carrier without CA does not apply this parameter.

      - It is up to RAN2 to design the signalling to support this feature

Agreement

The working assumption from RAN1#92 is confirmed

- The UE is configured one of the following options based on UE capability:

  - SLSS is transmitted (based on Rel-14 procedure) on selected sync carrier from Set-B

  - SLSS is transmitted on all carriers from Set-B

- Each option is an independent UE capability

- On top of this, Release-14 configuration applies to each carrier individually

Agreement

- For the case of limited TX capabilities, for UE SLSS transmission, it is up to UE implementation on which synchronization carrier(s) from Set B UE transmits SLSS

  - The above applies for the case when SLSS is transmitted on all carriers from Set-B

Agreement

- PSBCH content other than bandwidth, TDD configuration, reserved bits are generated following the Rel. 14 procedure following the selected synchronization reference.

  - Note if there is an issue with reserved bits, it will be addressed in RAN1#93

- SLSS ID is derived from the selected synchronization source.

Agreement

- When synchronization is lost, synchronization carrier reselection is up to UE implementation.

In this contribution, we discuss remaining issues on synchronization for sidelink CA.

Discussion

In RAN1 #92bis, it was agreed that UE is configured one of the following options based on UE capability

  - SLSS is transmitted (based on Rel-14 procedure) on selected sync carrier from Set-B

  - SLSS is transmitted on all carriers from Set-B

On the first option, when a UE selects GNSS or eNB as synchronization reference, the meaning of “selected” sync carrier is unclear. It is desirable that the UEs selected GNSS as the sync reference transmit SLSS/PSBCH on the same carrier as possible from the viewpoint of expanding the coverage of the GNSS or eNB timing. On the other hand, when the carrier transmitting the SLSS / PSBCH is different from the carrier transmitting the PSCCH / PSSCH, unnecessary RF retuning time occurs.

Based on the discussion, we consider the following alternatives,

Alt. 1) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured (i.e. SLSS is transmitted on selected sync carrier from Set-B, UE transmits SLSS/PSBCH in the lowest index carrier among Set-B.

Alt. 2) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured, SLSS/PSBCH transmitted carrier is (pre)configured.

Alt. 3) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured, the UE transmits SLSS/PSBCH on one of selected carrier(s) for PSCCH/PSSCH transmission.

Note that all UEs may not transmit SLSS/PSBCH on same carrier in alt. 1 but carriers transmitting SLSS/PSBCH are not expected to be fully distributed and alt. 3 is to minimize unnecessary RF retuning time.

Observation 1: When a UE selects GNSS or eNB as synchronization reference and option 1 (SLSS is transmitted (based on Rel-14 procedure) on selected sync carrier from Set-B) is configured, the meaning of “selected” sync carrier needs to be clarified.

Proposal 1: One of following alternatives is supported,

Alt. 1) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured (i.e. SLSS is transmitted on selected sync carrier from Set-B, UE transmits SLSS/PSBCH in the lowest index carrier among Set-B.

Alt. 2) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured, SLSS/PSBCH transmitted carrier is (pre)configured.

Alt. 3) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured, the UE transmits SLSS/PSBCH on one of selected carrier(s) for PSCCH/PSSCH transmission.

Conclusion

This contribution discussed on synchronization for sidelink CA. The discussions can be summarized as follows:

Observation 1: When a UE selects GNSS or eNB as synchronization reference and option 1 is configured, the meaning of “selected” sync carrier needs to be clarified.

Proposal 1: One of following alternatives is supported,

Alt. 1) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured (i.e. SLSS is transmitted on selected sync carrier from Set-B, UE transmits SLSS/PSBCH in the lowest index carrier among Set-B.

Alt. 2) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured, SLSS/PSBCH transmitted carrier is (pre)configured.

Alt. 3) When a UE selects GNSS or eNB as synchronization reference and option 1 is configured, the UE transmits SLSS/PSBCH on one of selected carrier(s) for PSCCH/PSSCH transmission.