3GPP TSG RAN WG1 Meeting #93					R1-1806608

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.1.1.5.2

Source: 	LG Electronics

Title: 	Remaining details on remaining issues for RLM in NR

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss about remaining issues of RLM in order to finalize RLM operation in NR Rel-15 system.

- Discussion on remaining issues on RLM

  - Relation with beam failure recovery

- BFR failure

Basically, it was suggested that the need of higher-layer indication depends on the configuration of RLM-RS resources and candidate beam RS resources assuming that only CFRA procedure is used for BFR procedure [3]. Under the condition of candidate beam RS resource set not including RLM-RS resource set, UE could not find proper serving beam regardless of IS indication in RLM procedure. However, RAN2 determined that CBRA using all SS/PBCH blocks configured by serving cell is adopted as a part of BFR process. According to the decision, status mismatch between RLM and BFR (IS in RLM / No detection of candidate beam for BFR) does not occurs, and indication to higher-layer for BFR failures could be replaced with indication of RACH failure already defined in NR.

- BFR success

When channel quality of serving cell becomes worse, it is usual that beam failure in beam management and OOS in RLM occur simultaneously. In this case, if the periodicity of RLM-RS is very long, RLF might be declared due to late indication of IS regardless of success of BFR. However, such a long periodicity of RLM-RS that the RLF is declared due to late indication is undesirable even for RLM operation without any consideration of BFR. In other words, the problem might not be a serious problems, and it can be solved by network optimization. In addition to the above discussion, RAN2 made the following agreement for aperiodic indication related to RLM [1].

- No aperiodic indication of a successful beam recovery will be reported to RRC.

- BFR failure will result in a RACH failure reported to RRC and will trigger RRC to perform either re-establishment or SCG failure. This is already the behaviour according to the current MAC and RRC specs (nothing extra to specify)

Based on the discussion, it is proposed that higher layer indication of BFR status is not introduced in Rel.-15.

Proposal 1. Higher layer indication of BFR status is not introduced in Rel.-15

  - Fast reconfiguration of RLM-RS resources

It was agreed that the maximum number of configured RLM-RS (which corresponds to 2, 4 and 8 according to frequency band) could be configured to be less than the number of transmission beam used in gNodeB. With the limited number of configured RLM-RS resources, RLM-RS resources might be re-configured whenever serving beam is updated if the number of configured RLM-RSs (candidate beams) is less than the number of total transmission beams. It might occur large signalling overhead and we need to discuss how to reduce the signalling overhead for RLM-RS reconfiguration [2].

For this problem, a company suggested that it could be solved by MAC CE activation of RLM-RS resources, similarly to TCI state activation in beam management procedure [3]. Conceptually, the proposal might be effective in reducing RRC overhead. Based on the proposal, after changing serving beam, gNodeB chooses the BM-RS, BFD-RS and RLM-RS resource set and transmits MAC CEs for activation of RS resource set, where the RLM-RS (and BFD-RS) resource set is determined by gNodeB based on the beam index of serving beam. It means that the RLM-RS resource set could be semi-statically configured by gNodeB based on beam deployment within a cell. Therefore, if the RLM-RS (and BFD-RS) resource set is defined per TCI state, RLM-RS resource set could be configured (or activated) just by activation of PDCCH TCI state without any additional MAC-CE. In the configuration method, if TCI state is well-configured, RLM-RS resource might be indicated using TCI state index because each TCI state includes the information for SS/PBCH block index or CSI-RS resource.

In addition, according to RAN2 agreement, if the RLM-RS (and BFD-RS) resource set is not included in TCI state of PDCCH, (spatially) QCLed RS defined in TCI state is used as default RLM-RS (and BFD-RS) resource.

Proposal 2. RLM-RS (and BFD-RS) resource set is configured per TCI-state, and RLM-RS resource set is automatically configured (or activated) by activation of PDCCH TCI state.

- Conclusion

In this contribution, we discuss about RLM operation and provide the following proposals.

Proposal 1. Higher layer indication of BFR status is not introduced in Rel.-15

Proposal 2. RLM-RS (and BFD-RS) resource set is defined in TCI-state, and RLM-RS resource set is automatically configured (or activated) by activation of PDCCH TCI state.

- Reference

RAN2 #101bis Chairman’s notes

R1-1800357, “Remaining details on remaining issues for RLM in NR”, LGE, RAN1-AH#1801, Vancouver, Canada

R1-1719900, “Discussion on Radio Link Monitoring in NR”, LGE, RAN1 #91, Reno, USA

R1-1805223, “Remaining issues for radio link monitoring”, Ericsson, RAN1 #92bis, Sanya, China