3GPP TSG-RAN WG1 #93	R1-1807367

May 21 – May 25, 2018

Busan, Korea

Agenda item:	7.1.3.3.6

Source: 	Qualcomm Incorporated

Title: 	Considerations on differentiating eMBB and URLLC

Document for:	Discussion/Decision

# Introduction

When a UE is able to provide the eMBB and URLLC services, the network needs to configure different transmission parameters to the services via downlink control information (DCI).  The transmission parameters include CQI/MCS, time-domain resource allocation, power control, rank, etc. The UE needs a method to differentiate eMBB DCIs and URLLC DCIs. One approach is by defining compact DCI for URLLC, and a UE blindly decodes different DCI payload sizes to know which service the DCI is intended to. Yet, it was agreed in the RAN1#92b meeting [1] that there is no consensus to support compact DCI for URLLC in Release 15:

As a result, the payload size of URLLC fallback/non-fallback DCIs follow the same eMBB configurations and rules, and cannot be used for differentiating eMBB and URLLC. Next, we present other approaches to distinguish eMBB and URLLC services.

# Differentiating eMBB and URLLC services

## Different C-RNTIs

A new C-RNTI can be RRC-configured for the URLLC service and is referred to as CU-RNTI in the rest of this contribution. CU-RNTI is applied in a similar way to CS-RNTI. This will have specification impact as RAN2 needs to review MAC layer procedures to adopt the new CU-RNTI.

## Different CRC masking

Another method to distinguish between eMBB and URLLC DCIs is to modify the CRC masking procedure for URLLC DCIs. Currently, the 16-bit RNTI is masked on the last 16 bits of the 24-bit DCI CRC to indicate the intendent recipient of the PDCCH transmission. As an example proposal for service differentiation, a bit pattern of 5 bits is prepended to C-RNTI as follows:

For eMBB, the CRC mask is (000 + 00000 + 16-bit C-RNTI), which is unchanged from the current eMBB DCI masking procedure.

For URLLC, the CRC mask is (000 + 11111 + 16-bit C-RNTI)

The same C-RNTI is used is scramble coded bits of eMBB and URLLC data, and the CRC masks are used to decide which service a DCI is intended to.

Proposal 1: Consider using a new CU-RNTI for URLLC or CRC masks to distinguish eMBB and URLLC services in NR Release 15.

# Conclusion

Proposal 1: Consider using a new CU-RNTI for URLLC or CRC masks to distinguish eMBB and URLLC services in NR Release 15.

# References

- 3GPP RAN1 #92b, “Chairman's Notes RAN1 92bis final,“ Sanya, China, April, 2018.