3GPP TSG RAN WG1 Meeting 93                          	          R1-1807371

Busan, Korea, May 21st– 25th, 2018

Agenda item:	7.1.5

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on coexistence

Document for:	Discussion/Decision

# Introduction

This contribution discusses some aspects of LTE-NR coexistence.

# Discussion

At RAN1 #92, an LS [1] was received from RAN2 informing RAN1 that RAN2 had decided not to differentiate SUO case 2 (TDM Case 2) from dual UL operation.

This makes TDM Case 2 unavailable for single Tx capable UEs, therefore these UEs can be only configured with TDM Case 1.

As it was pointed out in [2], Case 1 suffers from at least 2.2dB LTE coverage loss compared to Case 2 when either the LTE traffic or NR traffic, or both, are sporadic. In order to mitigate this loss, it is beneficial to make TDM Case 2 available for single Tx capable UEs. In order to achieve this, we make the following proposals:

Proposal 1

Send and LS to RAN2 to ask for a new EN-DC configuration parameter SUO-case2 to be introduced, together with applicable UE capability indication, similar to SUO-case1.

Proposal 2:

In 38.213, adopt the following text proposal:

## 7.6	Dual connectivity

### 7.6.1	EN-DC

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by higher layer parameter P-LTE and a maximum power  for transmissions on the SCG by higher layer parameter P-NR. The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power in frequency range 1 for the SCG as described Subclauses 7.1 through 7.5 using  as the maximum transmission power for .

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for frequency range 1, the UE determines a transmission power on the SCG as follows.

-	If the UE is configured with reference TDD configuration for EUTRA (by higher layer parameter SUO-case1 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between EUTRA and NR, the UE is not expected to transmit in a slot on the SCG when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between EUTRA and NR and

-	if the UE is not configured for operation with shortened TTI and processing time on the MCG [13, TS 36.213], and

-	if the UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG, and

-	if  in a portion of slot  of the SCG,

the UE reduces transmission power in the portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG, respectively.-

-	If the UE does not indicate a capability for dynamic power sharing between EUTRA and NR, the UE is expected to be configured with reference TDD configuration for EUTRA (by higher layer parameter SUO-case1 in [13, TS 36.213]).

-	If the UE is configured with higher layer parameter SUO-case2, the UE is not expected to transmit on the MCG and SCG simultaneously.

# Conclusion

Proposal 1

Send and LS to RAN2 to ask for a new EN-DC configuration parameter SUO-case2 to be introduced, together with applicable UE capability indication, similar to SUO-case1.

Proposal 2:

In 38.213, adopt the following text proposal

## 7.6	Dual connectivity

### 7.6.1	EN-DC

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by higher layer parameter P-LTE and a maximum power  for transmissions on the SCG by higher layer parameter P-NR. The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power in frequency range 1 for the SCG as described Subclauses 7.1 through 7.5 using  as the maximum transmission power for .

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for frequency range 1, the UE determines a transmission power on the SCG as follows.

-	If the UE is configured with reference TDD configuration for EUTRA (by higher layer parameter SUO-case1 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between EUTRA and NR, the UE is not expected to transmit in a slot on the SCG when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between EUTRA and NR and

-	if the UE is not configured for operation with shortened TTI and processing time on the MCG [13, TS 36.213], and

-	if the UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG, and

-	if  in a portion of slot  of the SCG,

the UE reduces transmission power in the portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG, respectively.-

-	If the UE does not indicate a capability for dynamic power sharing between EUTRA and NR, the UE is expected to be configured with reference TDD configuration for EUTRA (by higher layer parameter SUO-case1 in [13, TS 36.213]).

-	If the UE is configured with higher layer parameter SUO-case2, the UE is not expected to transmit on the MCG and SCG simultaneously.

# Reference

- R1-1701652, “LS on supporting power sharing”, RAN2, Huawei

- R1-1719194, “WF on UL TDM in single Tx EN-DC”, Qualcomm, Vodafone