3GPP TSG RAN WG1 Meeting #93		R1-1807428

Busan, Korea, May 21st – 25th, 2018

Source:	Sierra Wireless

Title:	efeMTC Key Issue Summary for PUSCH Sub-PRB Allocation

Agenda Item:	6.2.6.4

Document for:	Discussion and Decision

# Introduction

The following tdoc provides a summary of the key open issues and proposals based on the tdoc’s submitted to this agenda item for RAN1#93. The issues are listed approximately in order of priority where agreements that may affect RAN2 and RAN4 work have been prioritized.

Note: Previous agreements are listed in appendix I.

# Key Issues and Proposals

## DMRS Pattern for 2 of 3 SC pi/2 BPSK

Issue: DMRS pattern has not yet been agreed.

Given this agreement in RAN#92bis:

Each DMRS symbol is length-2 BPSK with DFT-S-OFDM.

The block diagram of the above agreement would look like this figure:

2:1

De-multiplexer

BPSK Modulation

DFT

Spread

Input Data Stream

Sub-carrier

Modulate

Add CP

Two DMRS

BSPK Sequences

R1(n), R2(n)

Pi/2

Rotation

Where the sequences R1(n) and R2(n) need to be defined.

Proposal #1: The DMRS pi/2 BPSK sequences R1(n) and R2(n) are calculated as:

R1(n) =    B(n)

R2(n) =    B(n) (-1) (n+Physical_Cell_ID)

B(n) is the same as the NB-IOT single tone DRMS sequence

Supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Sony, Qualcomm

Proposal #2: For PUSCH with 2 out of 3 allocated subcarriers, length-16 DMRS sequence defined in NB-IoT for single-tone transmission is reused.

Two length-16 DMRS sequences are mapped to 2 of 3 subcarriers for PUSCH, where each sequence is mapped to one subcarrier across 16 DMRS symbols within in one RU.

Supported by: Intel

Recommendation: Proposal #1.

## TDD RU sizes

- Issue: No RU sizes have been decided for TDD.

- The following RU sizes have been agreed for FDD:

6 sub-carriers QPSK option = 2 SF

3 sub-carriers QPSK option = 4 SF

2 of 3 sub-carriers pi/2 BPSK option = 8 SF

The following down selection was made in RAN#92bis:

Agreement

For sub-PRB transmissions in TDD, down-select between the following at RAN1#93

- Alt. 1: In TDD, the RU length depends on TDD UL/DL configuration.

- Alt. 2: In TDD, the same RU lengths supported in FDD mode are re-used.

Alt 1 supported by: none

Alt2 supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Intel, Sony, Qualcomm

Recommendation: Alt#2

For sub-PRB transmissions in TDD, the same RU lengths supported in FDD mode are re-used.

# DCI design Proposals

## CE Mode A

### Proposal #1 – 3 bits added – 7 companies

- A new 1-bit field to indicate “Sub-PRB Allocation” is added to DCI.

When “Sub-PRB Allocation” field=0 this means the grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 5 | - Legacy |
| - MCS index | - 4 | - Legacy |
| - Spare padding | - 2 | - New |
| - Total Bits | - 11 |  |

When “Sub-PRB Allocation” field=1 this means the grant is for a sub-PRB allocations, with the following fields:

| Field Name | Bits | Description |
| --- | --- | --- |
| Allocation of PRB within NB and SC within PRB | 6 | 6 PRBs within a NB and 10 values for the number of SC and their location within a PRB => 60 possible allocations within a NB. |
| MCS index | 3 | 8 values for MCS index |
| Number of RUs | 2 | 3 values for number of RUs (1,2,4) |
| Total Bits | 11 |  |

Note: If CSI and SRS field are re-purposed for sub-PRB allocations, this proposal goes to 1 bit added.

Supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Intel, Samsung, Sony

### Proposal #2 – 2 bits added – 5 companies

Add new 2 bits “# of RUs” field to indicate # of RUs and legacy resource allocation according to the following table:

Table 1: RU indication

| 2-bit states | Interpretation |
| --- | --- |
| 00 | Legacy PRB level RA |
| 01 | RU number=1, Sub-RB RA |
| 10 | RU number=2, Sub-RB RA |
| 11 | RU number=4, Sub-RB RA |

When “# of RU” field=0 this means the grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 5 | - Legacy |
| - MCS index | - 4 | - Legacy |
| - Total Bits | - 9 |  |

Otherwise sub-PRB allocation with the following fields:

| Field Name | Bits | Description |
| --- | --- | --- |
| Allocation of PRB within NB and SC within PRB | 6 | 6 PRBs within a NB and 10 values for the number of SC and their location within a PRB => 60 possible allocations within a NB. |
| MCS index | 3 | 8 values for MCS index |
| Total Bits | 9 |  |

Supported by: Huawei, HiSilicon, Sierra Wireless, Sharp, Sony

### Proposal #3 – 3 bits added – 1 company

‘Resource block assignment’ field is extended from 5 bits to 8 bits. ‘MCS’ field keeps the same size with new TBS mapping.

| Field Name | Bits | Description |
| --- | --- | --- |
| Resource block assignment within narrowband | 8 | 21 values for Full PRB level allocation; Sub-PRB 10 values for #of SC and location 6 values for PRB within narrowband 3 values for # of RUs (1,2,4); Joint coded = 10 × 6 × 3 + 21 = 201 |
| MCS | 4 | 11 values for QPSK or pi/2 BPSK according to TBS mapping table |

Supported by: ZTE

### Proposal #4 – 1 bits added – 1 company

- A new 1-bit field to indicate “Sub-PRB Allocation” is added to DCI.

When “Sub-PRB Allocation” field=0 this means the grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 5 | - Legacy |
| - MCS index | - 4 | - Legacy |
| - CSI Request | - 1 | - Legacy |
| - Total Bits | - 10 |  |

When “Sub-PRB Allocation” field=1 this means the grant is for a sub-PRB allocations, with the following fields:

| Field Name | Bits | Description |
| --- | --- | --- |
| Allocation of PRB within NB and SC within PRB | 6 | 6 PRBs within a NB and 10 values for the number of SC and their location within a PRB => 60 possible allocations within a NB. |
| MCS/TBS index # of RUs | 4 | MCS index and # of RU joint coded (see TBS proposal #3) |
| Total Bits | 10 |  |

Supported by: Samsung

### Proposal #5 – 3 bits added – 1 company

- A new 1-bit field to indicate “Sub-PRB Allocation” is added to DCI.

When “Sub-PRB Allocation” field=0 this means the grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 5 | - Legacy |
| - # of Repeats | - 2 | - Legacy |
| - spare | - 2 | - Legacy |
| - Total Bits | - 9 |  |

When “Sub-PRB Allocation” field=1 this means the grant is for a sub-PRB allocations, with the following fields:

| Field Name | Bits | Description |
| --- | --- | --- |
| PRB assignment and # of SC Location within PRB, #RUs and # repetition level | 9 | 6 PRB locations The #locations within PRB is (4, 4, 2) for the 2 of the 3 subcarriers pi/2 BPSK, 3-subcarrier and 6-subcarriers The (#RU, #repetition level) combination is (6, 9, 12) for the 2 of 3-subcarrers pi/2 BPSK, 3-subcarrier and 6-subcarriers Joint coded = (4 x 6 + 4 x 9 + 2 x 12) x 6 = total 504 states, 8 spare states |
| Total Bits | 9 |  |

Supported by: Qualcomm

### Recommendation and Summary

Proposal #1 [7 companies] and #2 [5 companies] have the majority of support

Main Issue: Number of MCS

Option 1 For Sub-PRB, # of MCS for CE Mode A = 8 values

Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Intel, Huawei, HiSilicon, Samsung, Sony, LGE, Sharp,

Option 2: For Sub-PRB, # of MCS for CE Mode A = legacy

Qualcomm, ZTE

DCI Design:

Proposal #1 and 2 designs are very similar – technically both work. Suggest some short online discussion then to agree to the one with majority support.

Proposal #1 – Add 3 bits – Single bit to determine full vs sub-PRB

Note: Optionally can save 1 bit by not have CSI field applicable to sub-PRB

- Supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Intel, Samsung, Sony

Proposal #2 – – Add 2 bits - saves 1 bit by joint coding #RUs and Full-PRB

Note: Proposal #2 can’t save another bit by not have CSI field applicable to sub-PRB.

Supported by: Huawei, HiSilicon, Sierra Wireless, Sharp, [ZTE, Softbank], Sony

## CE Mode B

### Proposal #1 – 2 bits added – 7 companies

The PRB location within the narrowband is configured by RRC.

- A new 1-bit field to indicate “Sub-PRB Allocation” is added to DCI.

When “Sub-PRB Allocation” field=0 this means grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 3 | - Legacy |
| - MCS index | - 4 | - Legacy |
| - Spare | - 1 | - New |
| - Total Bits | - 8 |  |

When “Sub-PRB Allocation” field=1 this means grant is for a sub-PRB allocations, with the following fields:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 0 | - Configured by RRC |
| - Allocation of SC within PRB | - 4 | - 10 values for the number of SC and their location within a PRB - Six spare states |
| - MCS index | - 3 | - 8 values for MCS index |
| - Number of RUs | - 1 | - 2 or 4 |
| - Total Bits | - 8 |  |

Supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Sony, [Intel- 3-bit allocation of SC within PRB saving 1 bit]

### Proposal #2 – 2 bits added – 4 companies

| - Field Name | - Bits | - Description |  |
| --- | --- | --- | --- |
|  |  | - RB | - Sub-RB |
| - PRB Assignment - Location of SC | - 5 | - 8 (legacy) + 1 (PRB#4, PRB#5 in narrowband) allocations Full PRB within narrowband | - 10 values for the number of SC and their location within PRB location #1 - 10 values for the number of SC and their location within PRB location #2 |
| - MCS index - # of RUs | - 4 | - 4 bits MCS - 0 bits # of RUs | - 3 bits: 8 values for MCS index - 1bit: 2 or 4 RUs |
| - Total Bits | - 9 |  |  |

- Advantage: Two possible PRB locations within the narrowband is configured by RRC.

Supported by: Huawei, HiSilicon, Sierra Wireless, Sharp

### Proposal #3 – 2 bits added – 1 company

| Field Name | Bits | Description |
| --- | --- | --- |
| - PRB Assignment - #of SC and location # of RUs | 5 or 6 | 8 values for legacy Full PRB level allocation Sub-PRB: 10 values for #of SC and location 1 or 2 values for PRBs within narrowband 2 values for # of RUs (2,4) Joint coded = 10 × 1 × 2 + 8 = 28 or 10 × 2 × 2 + 8 = 48 |
| Modulation and coding scheme | 4 | 11 values for QPSK or pi/2 BPSK according |

Supported by: ZTE

- Note: 2 or 3 bits are added to DL grant Format 6-1B as well with this proposal.

### Proposal #4 – 2 bits added – 1 company

The PRB location within the narrowband is configured by RRC.

- A new 1-bit field to indicate “Sub-PRB Allocation” is added to DCI.

When “Sub-PRB Allocation” field=0 this means grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 3 | - Legacy |
| - MCS index | - 4 | - Legacy |
| - Spare | - 1 | - New |
| - Total Bits | - 8 |  |

When “Sub-PRB Allocation” field=1 this means grant is for a sub-PRB allocations, with the following fields:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 0 | - Configured by RRC |
| - Allocation of SC within PRB | - 4 | - 10 values for the number of SC and their location within a PRB - Six spare states |
| - MCS index - # of RUs | - 4 | - 16 values for MCS index and # of RUs - (see TBS proposal #4) |
| - Total Bits | - 8 |  |

Supported by: Samsung

### Proposal #5 - 2 bits added – 1 company

The PRB location within the narrowband is configured by RRC.

- A new 1-bit field to indicate “Sub-PRB Allocation” is added to DCI.

When “Sub-PRB Allocation” field=0 this means grant is for a Full-PRB allocations, with the following legacy fields unchanged:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 3 | - Legacy |
| - MCS index | - 4 | - Legacy |
| - Spare | - 1 | - New |
| - Total Bits | - 8 |  |

When “Sub-PRB Allocation” field=1 this means grant is for a sub-PRB allocations, with the following fields:

| - Field Name | - Bits | - Description |
| --- | --- | --- |
| - PRB Assignment | - 0 | - Configured by RRC |
| - Allocation of SC within PRB - MCS Index | - 7 | 10 values for the number of SC and location within PRB 12 values for MCS same as legacy normal PRB allocation - Joint coded = 10 x 12 = total 120 states, 8 spare states |
| - # of RUs | - 1 | - 2 values for #RUs (2 or 4) |
| - Total Bits | - 8 |  |

Supported by: Qualcomm

Note: Supports 12 MCS values are supported by joint coding MCS with the 10 values for the number of SC and location within PRB.

### Recommendation and summary

Proposal #1 [7 companies] and #2 [4 companies] have the majority of support

DCI Design:

Proposal #1 - Summary

What: adds flag to separate full and sub-PRB allocations

Advantage: No joint coding – no linkage between Full PRB and sub-prb processing

Disadvantage: Only support 1 PRB location for sub-PRB

Supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Sony, Intel,Sony

Proposal #2 – Summary

What: Joint encodes allocation of legacy and sub-PRB allocations together

Advantage: Allows DCI scheduling of 2 PRB locations for sub-PRB (i.e. more scheduling flexibility)

Disadvantage: Joint coding – it will require legacy full-prb to process DCI in a new way.

Supported by: Huawei, HiSilicon, Sierra Wireless, Sharp ,[ZTE, Softbank], Sony

## Sub-PRB Repetitions

Issue: How the maximum sub-PRB transmission duration is signaled and how the Repetition Number field is interpreted for a sub-PRB allocation has not yet been decided.

How the maximum sub-PRB transmission duration is signaled:

For full-PRB allocations, to control the amount of coverage the cell can provide, the maximum repetition is signaled in SI.  The TS 36.331 states:

pusch-maxNumRepetitionCEmodeA-r13   ENUMERATED {

r8, r16, r32 }                OPTIONAL,   -- Need OR

pusch-maxNumRepetitionCEmodeB-r13   ENUMERATED {

r192, r256, r384, r512, r768, r1024,

r1536, r2048}                 OPTIONAL,   -- Need OR

A similar mechanism is needed for sub-PRB allocation. Since the coverage provided by repeats for Full-PRB is similar to “SF of transmission” for sub-PRB, having these two items linked together is practical method to save RRC signalling. This method also insures that the maximum Sub-PRB transmission times of 32Sf and 2048SF for CE Mode A and B are not exceeded. The proposal below captures this concept:

Proposal #1.1: For Sub-PRB allocations, for CE mode A and CE mode B, the maximum number of valid subframes of transmission is given by ‘pusch-maxNumRepetitionCEmodeA’ and ‘pusch-maxNumRepetitionCEmodeB’ respectively.

Supported by: Sierra Wireless, [ZTE], Samsung, Sharp, Ericsson, HiSilicon, Huawei

Alternatively, we could add new RRC signalling.

Proposal #1.2: For CE Mode B, the maximum number of repetitions for sub-PRB allocation is configured by RRC. For CE Mode A, the maximum number of repetitions for sub-PRB allocation is based on # of RUs .

Supported by: Qualcomm

How the Repetition Number field is interpreted for a sub-PRB allocation:

For a full-PRB allocation, the DCI Repetition number field takes values nX and based on the maxNumRepetition looks up values from the table below from TS 36.213:

Table 8-2b: PUSCH repetition levels (DCI Format 6-0A)

| Higher layer parameter 'pusch-maxNumRepetitionCEmodeA' |  |
| --- | --- |
| Not configured | {1,2,4,8} |
| 16 | {1,4,8,16} |
| 32 | {1,4,16,32 } |

Table 8-2c: PUSCH repetition levels (DCI Format 6-0B)

| Higher layer parameter 'pusch-maxNumRepetitionCEmodeB' |  |
| --- | --- |
| Not configured | {4,8,16,32,64,128,256,512} |
| 192 | {1,4,8,16,32,64,128,192} |
| 256 | {4,8,16,32,64,128,192,256} |
| 384 | {4,16,32,64,128,192,256,384} |
| 512 | {4,16,64,128,192,256,384,512} |
| 768 | {8,32,128,192,256,384,512,768} |
| 1024 | {4,8,16,64,128,256,512,1024} |
| 1536 | {4,16,64,256,512,768,1024,1536} |
| 2048 | {4,16,64,128,256,512,1024,2048} |

It is proposed to re-use the above tables but now calculate the number of sub-PRB repetitions via an equation. The following proposal captures this idea:

Proposal #2: For sub-PRB allocation, the number of repetitions is calculated as:

# of Sub-PRB Repetitions = Ceil(“DCI Repetition Number Field”/(RU_length*# of RUs))

Supported by: Sierra Wireless, [ZTE], Samsung, Sharp, Ericsson, Huawei, HiSilicon

Recommendation: Proposal #1.1 and Proposal #2

## General DCI Optimizations

These topics are more for information and will only be discussed if a full proposal is not agreeable.

### Introduce Sub-PRB vs Full-PRB Allocation Field

Introducing a new field to differentiate a sub-PRB vs full-PRB allocation which allows some of the full-PRB fields to be re-purposed thus saving DCI bits.

Proposal #1: Introduce 1 bit to differentiate sub-PRB allocation and full-PRB allocation for both CE Mode A and CE Mode B.

Supported by: Samsung, Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Intel, Qualcomm

### Using bits from CSI Request

Samsung suggested CSI request field can be re-used with a sub-PRB allocation is specified.

Proposal: CSI request is not supported for the sub-PRB allocation in CE Mode A.

### Using bits from SRS Request

Qualcomm suggest that SRS may not be needed for Sub-PRB allocations. A concern is that the subPRB allocation is a narrowband technique while the minimum SRS bandwidth is 4 PRBs.

Proposal: SRS request is not supported for the sub-PRB allocation in CE Mode A.

### Using bits from Repetition Field

Since several of the repeats are invalid for larger # of RUs especially in CE Mode A, some of those states can be re-used as suggested by Qualcomm.

### Using bits from HARQ ID field in CE Mode A

As indicated by Huawei in [2], For CE Mode A, if Ack timing is maintained, less HARQ processes can be supported for a sub-PRB allocation so the HARQ ID field for CE Mode A can be reduced, and bits re-purposed when a sub-PRB is allocated.

Proposal:	In CE Mode A,

Support a maximum of 4 HARQ processes for 6 subcarriers

Support a maximum of 2 HARQ processes 3 and 2 of 3 subcarriers

Shorten HARQ ID field by 1 bit for sub-PRB allocation.

# TBS Proposals

## CE Mode A

### Proposal #1 – 5 companies

| MCS Index | # PRBs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 |  |  | 1 RU |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 |  |  | 2 RUs |  |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 |  |  | 4 RUs |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 |  |  |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 |  |  |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 |  |  |  |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 |  |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 |  |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 10321000 |  |  |  |

In CE mode A, the PRB #2, PRB #3, and PRB #6 take the role of 1RU, 2RUs, and 4RUs respectively. The Transport Blocks Sizes associated 1RU, 2RUs, and 4RUs are {32, 56, 72, 104, 120, 144, 176, 224}, {144, 176, 208, 224, 256, 328, 392, 456}, and {328, 408, 504, 600, 712, 808, 936, 1000} respectively.

Supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Sony

### Proposal #2 – 1 company

|  | 1 | 2 | 4 |
| --- | --- | --- | --- |
| 0 | 32 | 56 | 152 |
| 1 | 40 | 88 | 208 |
| 2 | 56 | 144 | 256 |
| 3 | 72 | 176 | 328 |
| 4 | 88 | 208 | 392 |
| 5 | 104 | 224 | 456 |
| 6 | 120 | 256 | 504 |
| 7 | 136 | 328 | 616 |
| 8 | 144 | 392 | 680 |
| 9 | 176 | 456 | 872 |
| 10 | 208 | 504 | 1000/936 |

Supported by: ZTE

### Proposal #3 – 1 company

Table.3 MCS/RU/TBS table for 3/6 subcarriers QPSK

| MCS/TBS index | Modulation | TBS | NRU |
| --- | --- | --- | --- |
| 0 | QPSK | 56 | 1 |
| 1 | QPSK | 88 |  |
| 2 | QPSK | 144 |  |
| 3 | QPSK | 176 |  |
| 4 | QPSK | 208 |  |
| 5 | QPSK | 224 |  |
| 6 | QPSK | 256 | 2 |
| 7 | QPSK | 328 |  |
| 8 | QPSK | 392 |  |
| 9 | QPSK | 456 |  |
| 10 | QPSK | 504 |  |
| 11 | QPSK | 600 | 4 |
| 12 | QPSK | 712 |  |
| 13 | QPSK | 808 |  |
| 14 | QPSK | 936 |  |
| 15 | QPSK | 1000 |  |

Table.4 MCS/RU/TBS table for 2 of 3 subcarriers pi/2 BPSK

| MCS/TBS index | Modulation | TBS | NRU |
| --- | --- | --- | --- |
| 0 | pi/2-BPSK | 56 | 1 |
| 1 | pi/2-BPSK | 88 |  |
| 2 | pi/2-BPSK | 144 |  |
| 3 | pi/2-BPSK | 176 | 2 |
| 4 | pi/2-BPSK | 208 |  |
| 5 | pi/2-BPSK | 224 |  |
| 6 | pi/2-BPSK | 256 |  |
| 7 | pi/2-BPSK | 328 |  |
| 8 | pi/2-BPSK | 392 | 4 |
| 9 | pi/2-BPSK | 456 |  |
| 10 | pi/2-BPSK | 504 |  |
| 11 | pi/2-BPSK | 600 |  |
| 12 | pi/2-BPSK | 712 |  |
| 13 | pi/2-BPSK | 808 |  |
| 14 | pi/2-BPSK | 936 |  |
| 15 | pi/2-BPSK | 1000 |  |

Supported by: Samsung

### Proposal #4 – 1 company

| MCS index | Modulation scheme | TBS index | CE mode A |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | PRB number RU number |  |  |
|  |  |  | 1 | 2 | 4 |
| 0 | QPSK | 0 | 16 | 32 | 88 |
| 1 | QPSK | 2 | 32 | 72 | 176 |
| 2 | QPSK | 3 | 40 | 104 | 208 |
| 3 | QPSK | 4 | 56 | 120 | 256 |
| 4 | QPSK | 5 | 72 | 144 | 328 |
| 5 | QPSK | 6 | 88 | 176 | 392 |
| 6 | QPSK | 7 | 104 | 224 | 472 |
| 7 | QPSK | 8 | 120 | 256 | 536 |
| 8 | QPSK | 10 | 144 | 328 | 680 |
| 9 | QPSK | 12 | 176 | 376 | 776 |
| 10 | QPSK | 13 | 224 | 488 | 1000 |
| 11 | pi/2 BPSK | 0 | 16 | 32 | 88 |
| 12 | pi/2 BPSK | 3 | 40 | 104 | 208 |
| 13 | pi/2 BPSK | 6 | 88 | 176 | 392 |
| 14 | pi/2 BPSK | 9 | 136 | 296 | 616 |
| 15 | pi/2 BPSK | 10 | 144 | 328 | 680 |

Supporting by: NTT Docomo

### Proposal #5 – 1 company

Proposed TBS values are from columns of the legacy TBS table.

Supported by: Qualcomm

## CE Mode B

### Proposal #1 – 5 companies

| MCS Index | # PRBs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 |  |  |  |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 |  |  | 2 RUs |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 |  |  | 4 RUs |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 |  |  |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 |  |  |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 |  |  |  |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 |  |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 |  |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 |  |  |  |

In CE Mode B, the PRB #1, and PRB #2 take the role of 2RUs, and 4RUs respectively. The Transport Blocks Sizes associated 2RUs, and 4RUs are {56, 88, 144, 176, 208, 224, 256, 328}, and {256, 328, 408, 504, 600, 712, 808, 936} respectively.

Supported by: Ericsson, Sierra Wireless, Sony, Huawei, HiSilicon

### Proposal #2  – 2 companies

| ITBS | NRU |  |
| --- | --- | --- |
|  | 2 | 4 |
| 0 | 144 | 256 |
| 1 | 176 | 328 |
| 2 | 208 | 408 |
| 3 | 224 | 504 |
| 4 | 256 | 600 |
| 5 | 328 | 712 |
| 6 | 392 | 808 |
| 7 | 456 | 936 |

Supported by:  Nokia, Nokia Shanghai Bell

### Proposal #3  – 1 company

|  | 2 | 4 |
| --- | --- | --- |
| 0 | 56 | 152 |
| 1 | 88 | 208 |
| 2 | 144 | 256 |
| 3 | 176 | 328 |
| 4 | 208 | 392 |
| 5 | 224 | 456 |
| 6 | 256 | 504 |
| 7 | 328 | 616 |
| 8 | 392 | 680 |
| 9 | 456 | 872 |
| 10 | 504 | 1000/936 |

Supported by: ZTE

### Proposal #4  – 1 company

Table.3 MCS/RU/TBS table for 3/6 subcarriers QPSK

| MCS/TBS index | TBS | NRU |
| --- | --- | --- |
| 0 | 56 | 2 |
| 1 | 88 |  |
| 2 | 144 |  |
| 3 | 176 |  |
| 4 | 208 |  |
| 5 | 224 |  |
| 6 | 256 | 2 |
| 7 | 328 |  |
| 8 | 392 |  |
| 9 | 456 |  |
| 10 | 504 |  |
| 11 | 600 | 4 |
| 12 | 712 |  |
| 13 | 808 |  |
| 14 | 936 |  |
| 15 | 1000 |  |

Table.4 MCS/RU/TBS table for 2 of 3 subcarriers pi/2 BPSK

| MCS/TBS index | TBS | NRU |
| --- | --- | --- |
| 0 | 56 | 2 |
| 1 | 88 |  |
| 2 | 144 |  |
| 3 | 176 | 2 |
| 4 | 208 |  |
| 5 | 224 |  |
| 6 | 256 |  |
| 7 | 328 |  |
| 8 | 392 | 4 |
| 9 | 456 |  |
| 10 | 504 |  |
| 11 | 600 |  |
| 12 | 712 |  |
| 13 | 808 |  |
| 14 | 936 |  |
| 15 | 1000 |  |

Supported by: Samsung

### Proposal #5  – 1 company

| MCS index | Modulation scheme | TBS index | CE mode B |  |
| --- | --- | --- | --- | --- |
|  |  |  | RU number |  |
|  |  |  | 2 | 4 |
| 0 | QPSK | 0 | 56 | 152 |
| 1 | QPSK | 1 | 88 | 208 |
| 2 | QPSK | 2 | 144 | 256 |
| 3 | QPSK | 3 | 176 | 328 |
| 4 | QPSK | 4 | 208 | 408 |
| 5 | QPSK | 5 | 224 | 504 |
| 6 | QPSK | 6 | 256 | 600 |
| 7 | QPSK | 7 | 328 | 712 |
| 8 | QPSK | 8 | 392 | 808 |
| 9 | QPSK | 9 | 456 | 936 |
| 10 | QPSK | 10 | 504 | 1032 |
| 11 | pi/2 BPSK | 0 | 56 | 152 |
| 12 | pi/2 BPSK | 1 | 88 | 208 |
| 13 | pi/2 BPSK | 3 | 176 | 328 |
| 14 | pi/2 BPSK | 4 | 208 | 408 |
| 15 | pi/2 BPSK | 6 | 256 | 600 |

Supporting by: NTT Docomo

### Proposal #6  – 1 company

Proposed TBS values are from columns of the legacy TBS table.

Supported by: Qualcomm

## Common CE Mode A and B table

|  |  | NPRB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  |
|  | 0 | 16 | 32 | 56 | 88 | 120 | 152 |  |  |  |
|  | 1 | 24 | 56 | 88 | 144 | 176 | 208 |  |  |  |
|  | 2 | 32 | 72 | 144 | 176 | 208 | 256 | Option |  |  |
|  | 3 | 40 | 104 | 176 | 208 | 256 | 328 | ->408 |  |  |
|  | 4 | 56 | 120 | 208 | 256 | 328 | 408 | ->456 |  |  |
|  | 5 | 72 | 144 | 224 | 328 | 424 | 504 |  |  |  |
|  | 6 | 328 | 176 | 256 | 392 | 504 | 600 |  |  | 1 RU |
|  | 7 | 104 | 224 | 328 | 472 | 584 | 712 |  |  | 2 RUs |
|  | 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  | 4 RUs |
|  | 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |  |
|  | 10 | 144 | 328 | 504 | 680 | 872 | 10321000 |  |  |  |

Supported by: Sierra Wireless, Qualcomm, Samsung,

## Recommendation

Try to agree to a single table for CE Mode A and B.

# Lower priority issues and new enhancements:

The following issues are of lower priority as they are seen as maintenance issues or see as new optional enhancements:

## Selection of SC for 2 of 3 SC pi/2 BPSK

Issue: Although there have been agreements made WRT to this issue already in RAN1#92:

- The location of the 2-of-3 subcarriers within the 3-subcarrier allocation is cell-specific

- Use the Physical Cell ID along with a Modulo 2 operation to determine which 2 of 3 subcarriers will be used

There are still some remaining details.

Proposal #1:  Support changing 2-of-3 subcarrier location within 3 subcarriers periodically.

Supported by: Huawei, HiSilicon

Proposal #2:  2-of-3 subcarrier location is fixed in time where a Modulo 2 output of:

“0” means that the two subcarriers with the lowest indexes

“1” means that the two subcarriers with the highest

Supported by: Ericsson, Sierra Wireless, Qualcomm

Recommend proposal: Offline discussion

## RV Assignment

Issue: How the RV is assigned and changed has not been explicitly agreed. There were no tdocs related to this open issue, so the following proposal is based on offline discussion:

Proposal #1:  Support changing 2-of-3 subcarrier location within 3 subcarriers periodically.

- For sub-PRB allocations,

  - RV changes per repetition which occurs every RU_size*number_RUs subframes.

  - RV for the first repetition is the same as legacy (i.e. for CE mode A the first RV is indicated in DCI and subsequent RVs continue from there following the RV legacy order 0, 2, 3, 1, for CE mode B, the RV order is always 0, 2, 3, 1).

Supported by: Sharp, Sierra Wireless,

Recommend proposal: Proposal#1

## Power Control

Issue: Some companies feel that power control in CE Mode A for sub-PRB is still unclear given the following agreement:

Agreement

For sub-PRB allocation, the maximum transmission power is used in CE Mode B and for power control in CE Mode A, MPUSCH,c(i) ={1/6, 1/4, 1/2} for 2-subcarrier, 3-subcarrier and 6-subcarrrier allocation, respectively and for PRB level allocation, there are no changes to transmit power control.

Proposal #1: For sub-PRB transmissions in CE mode A, closed-loop power control is supported.

- Supported by:  Samsung, Sierra Wireless

Recommend proposal: Proposal #1

## Sub-PRB applicable to Cat.M2

Issue: It has not yet been decided if category M2 (5MHz BL/CE UEs) can support sub-PRB transmissions.

Proposal #1: The sub-PRB allocation is supported only for UEs configured with max PUSCH channel BW of 1.4 MHz.

- Supported by:  Intel, Sierra Wireless, Sharp

Recommend proposal: Proposal #1

## Cyclic Repetition

Issue: It has not yet been decided if cyclic repetition will be supported for eMTC Sub-PRB transmissions for CE Mode B.

Proposal #1: Cyclic repetition is not supported for sub-PRB transmissions over PUSCH in CE Mode B.

Supported by: Ericsson, Sierra Wireless, Nokia, Nokia Shanghai Bell, Sharp, Qualcomm

Proposal #2: In CE Mode B, cyclic repetition is supported for sub-PRB transmissions when “subframes of transmission” >= RU_size*4*4*2.

The number of repeated subframes, MidenticalPUSCH, is calculated as:

MidenticalPUSCH=1NabsPUSCH<2×RUsize×4×422×RUsize×4×4≤NabsPUSCH<4×RUsize×4×44 for FDD, 5 for TDDotherwise

- Repeated subframes align with Nacc (i.e. same as legacy Full-PRB eMTC subframe repetition)

Note: RU_size*4*4*2 => RU_size*Max_RVs*Max_RUs*Min_Repeats

Supported by: Huawei, HiSilicon, Intel, Sony, Sierra Wireless

Recommend proposal: offline discussion

## UCI and PUSCH collision resolution

Issue: For eFeMTC, piggybacking on PUSCH with sub-PRB allocation is not supported but which channel should be dropped has not been agreed.

Proposal #1: The hierarchical dropping rules as defined in the standard for handling collisions between a UCI and PUSCH transmissions are kept and followed.

- Supported by:  Ericsson

Proposal #2:  Which UL channel is dropped in case of collisions should take into account the relative priority of the data or UCI on the channel, the number of subframes of collision, and the number of repetitions.

- Supported by:  Huawei, HiSilicon

Proposal #3: UE drops PUSCH transmission in subframes that collide with PUCCH.

Supported by: Nokia, NSB, Intel, Samsung

Recommend Proposal: offline discussion

# Tdocs

- R1-1805852 PUSCH sub-PRB allocation for MTC Ericsson

- R1-1805981 On Sub-RB resource allocation for MTC PUSCH Huawei, HiSilicon

- R1-1806000 Remaining Issues for PUSCH Sub-PRB Allocation Sierra Wireless, S.A.

- R1-1806175 Design of PUSCH Sub-PRB Allocation Nokia, Nokia Shanghai Bell

- R1-1806188 Remaining issues on PUSCH sub-PRB allocation for MTC ZTE

- R1-1806490 Design of sub-PRB PUSCH for efeMTC Intel Corporation

- R1-1806561 Remaining issues in Sub-PRB PUSCH transmission in efeMTC Sony

- R1-1806685 Discussion on sub-PRB allocation for eFeMTC Samsung

- R1-1806937 Remaining issues of PUSCH sub-PRB allocation Sharp

- R1-1807043 Views on PUSCH sub-PRB allocation for MTC NTT DOCOMO, INC.

- R1-1807105 Remaining issues for sub-PRB allocation Qualcomm Incorporated

# Appendix I: Previous Agreements

The following has been agreed:

Agreement in RAN1#90Bis: - Sub-PRB shall be supported at least in CE Mode B - Working assumption: Sub-PRB shall be supported in CE Mode A. - RAN1 will prioritize optimization of Sub-PRB for CE Mode B over optimization of Sub-PRB for CE Mode A. - For Sub-PRB, the maximum total number of (valid) subframes of transmission is: - 32 subframes for CE Mode A - 2048 subframes for CE Mode B - FFS: Supported transport block sizes and numbers of repetitions (for each supported CE Mode) - Sub-PRB rate matching is performed across a resource unit (RU) spanning multiple subframes - The RU length depends on number of subcarriers in the Sub-PRB allocation - FFS: RE mapping - FFS: whether more than one RU is allocated per transport block - For Sub-PRB, increasing DMRS shall not be supported - For Sub-PRB allocation in connected mode, - The Sub-PRB feature is configured/enabled by RRC signaling - The Sub-PRB resource allocation shall be signaled by DCI - FFS: Support of Sub-PRB allocation in Msg3 Agreement in RAN1#90Bis: - When the Sub-PRB feature is configured/enabled in connected mode in CE mode B, - DCI format 6-0B shall support both sub-PRB allocation and allocation of at least 1 PRB. - Sub-PRB allocation shall support a maximum TBS of at least [504] bits. Agreement in RAN1#91 Confirm the above WA and choose option 1 with the following changes: - For Sub-PRB allocation, only the following are supported: - 6 subcarriers with SC-FDMA QPSK modulation, at least for CE Mode A - FFS: CE Mode B - 3 subcarriers with SC-FDMA QPSK modulation - 3 subcarriers with SC-FDMA Pi/2 BPSK modulation - The Pi/2 rotation is performed across SC-FDMA symbols - Use only 2 adjacent subcarriers out of the 3 allocated subcarriers with DFT-spread of length 2 - FFS: which 2 subcarriers out of the 3 allocated subcarriers are used - Working assumption: The 2 used subcarriers shall be fixed per cell in specification - FFS: semi-statically configured - FFS: Frequency hopping case Agreement in RAN1#91 - Confirm WA: Sub-PRB shall be supported in CE Mode A. - RAN1 will prioritize optimization of Sub-PRB for CE Mode B over optimization of Sub-PRB for CE Mode A. Agreement in RAN1#91 - When the Sub-PRB feature is used, - Mapping one TB to 1 RU shall be supported at least for CE Mode A - Mapping one TB to a maximum of [FFS:2 or 4] resource units (RUs) shall be supported - Sub-PRB allocation shall support a maximum TBS of 1000 bits for CE Mode A and 936 bits for CE Mode B - Maximum TBS within a single RU is FFS Agreement in RAN1#91 - UCI Piggybacking on PUSCH with sub-PRB allocation is not supported. - FFS: which UL channel is dropped Agreements in RAN1#92 Agreement - For a 2 of 3 sub-carrier pi/2 BPSK transmission, the SC-FDMA symbols over which the DMRS is mapped shall provide a PAPR that is roughly the same as the PAPR of the SC-FDMA symbols not carrying DMRS. - Cross-correlation properties for the DM-RS need to be considered. - Note that Alt. 1 and Alt. 2 may or may not meet the above requirement - Alt. 1: Each DMRS symbol is length-2 BPSK with DFT-S-OFDM - Alt. 1: is the same modulation as data - Alt. 2: Each DMRS symbol is mapped to three sub-carriers - Other alternative(s) are not precluded Agreement - The DMRS sequence and DMRS RE mapping are the same as the NB-IOT DMRS design for 3 and 6 sub-carrier QPSK transmissions. - The above is not applied to DMRS for 2 of 3 sub-carrier pi/2 BPSK transmission Agreement - For all sub-PRB transmissions, the DMRS is transmitted in the fourth SC-FDMA symbol of the slot Agreement - For 2 of 3 subcarriers pi/2 BPSK option, average the phase advancement across the cyclic prefix between the two tones and use this averaged phase advancement for pi/2 phase rotation - This can be revisited if RAN4 identifies any issues Agreement - For the 2 Sub-Carrier Pi/2 BSPK modulation, the pi/2 rotation algorithm is the same as NB-IOT pi/2 BPSK where the pi/2 rotation is not a function of the sub-carrier index. Agreement - For 6 sub-carriers QPSK allocations, 2 non-overlapping allocations per PRB, i.e., {0, 1, 2, 3, 4, 5} and {6, 7, 8, 9, 10, 11}, will be defined - For 3 sub-carrier QPSK allocations and 2 of 3 sub-carrier pi/2 BSPK allocations, 4 non-overlapping allocations per PRB, i.e., {0, 1, 2}, {3, 4, 5}, {6, 7, 8} and{9, 10, 11}, will be defined - FFS: Which of the above sub-PRB allocations are granted by DCI Agreement - The location of the 2-of-3 subcarriers within the 3-subcarrier allocation is cell-specific - Use the Physical Cell ID along with a Modulo 2 operation to determine which 2 of 3 subcarriers will be used - This can be revisited if RAN4 identifies any issues Agreement: Confirm the WA below • The 2 used subcarriers shall be fixed per cell in specification Agreement - RU Size of 6 sub-carriers QPSK option = 2 ms - RU Size of 3 sub-carriers QPSK option = 4 ms Agreement 4 RVs will be used for sub-PRB transmission Agreement The RE mapping of sub-PRB within an RU over PUSCH is performed first within a sub-frame then across sub-frames Agreement - Within a subframe, RE mapping is the same as legacy PUSCH. Agreement - Frequency hopping of PRB between 2 narrowbands for sub-PRB allocation is the same as legacy. Sub-carrier locations in the PRB are the same within both narrowbands Agreement 6 subcarriers with SC-FDMA QPSK modulation is supported in CE mode B Agreement - The candidates for FDD are Proposal #1, #2 and #4 for CE mode A and Proposal#1, #2, and #3 for CE mode B and down-select the candidates at RAN1#92bis - Proposal #1:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 8 ms - Proposal #2:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 6 ms - Proposal #3:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 16 ms - Proposal #4:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 4 ms FFS whether the above can be reused for TDD or not Agreement - A maximum of 4 resource units (RUs) is supported in CE Mode A Agreements in RAN1#92bis Agreement Each DMRS symbol is length-2 BPSK with DFT-S-OFDM. Agreement - For CE Mode B, only 2 and 4 RUs can be allocated. - For CE Mode A, 1, 2 and 4 RUs can be allocated. Agreement - For FDD, RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 8 ms for CE Mode A and CE Mode B Agreement For sub-PRB transmissions in TDD, down-select between the following at RAN1#93 - Alt. 1: In TDD, the RU length depends on TDD UL/DL configuration. - Alt. 2: In TDD, the same RU lengths supported in FDD mode are re-used. Agreement For sub-PRB allocation, the maximum transmission power is used in CE Mode B and for power control in CE Mode A, MPUSCH,c(i) ={1/6, 1/4, 1/2} for 2-subcarrier, 3-subcarrier and 6-subcarrrier allocation, respectively and for PRB level allocation, there are no changes to transmit power control. Agreement Cyclic repetition is not supported for sub-PRB allocation for CE Mode A - FFS: For CE Mode B Agreement The Sub-PRB DCI design for Mode A and B should not increase the DCI size by more than 3 bits Agreement - For sub-PRB allocation in CE Mode B, the PRB location(s) within the [system bandwidth or narrowband] is configured by RRC. Agreement - For sub-PRB allocation in CE Mode A, there is no RRC configuration needed to indicate the PRB location in the system bandwidth Sub-PRB location within PRB # of sub-carriers # of RUs MCS Index Agreement - The Sub-PRB DCI design will not limit scheduling options for Full-PRB legacy allocations for CE Mode A and B