TSG-RAN WG1 #93	R1-1807622

Busan, Korea, May 21 – 25, 2018

Source:	Ericsson

Title:	Summary of 7.1.3.31 (resource allocation)

Agenda Item:	7.1.3.3.1

Document for:	Discussion and Decision

# Introduction

In the following, some questions relevant to 7.1.3.3.1 is provided based on the views expressed in the contributions (listed in the appendix) and the agreements and working assumptions from previous meetings.

# Summary

## Time-domain allocation

At RAN1#92bis, default time-domain allocation tables for PUSCH and SIB1 delivery on PDSCH were agreed. There were also agreements which tables to use when scheduling in USS as follows:

Agreements:

For time-domain signaling for PUSCH scheduled by DCI in USS

- if dedicated RRC signaling has provided a table

  - Use this table for PUSCH transmission scheduled by DCI in USS

- else if RMSI signaling has provided a table

  - Use this table for PUSCH transmission scheduled by DCI in USS

- else

  - Use the default PUSCH table

For time-domain signaling for PDSCH scheduled by DCI in USS

- if dedicated RRC signaling has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in USS

- else if RMSI signaling has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in USS

- else

  - Use pattern 1 from the agreed default table

However, which tables to use for scheduling in the CSS for different RNTIs was not completely resolved last meeting. Several companies mentioned that tables for the fallback formats with C(S)-RNTI (at least in CSS) need to use the RMSI-configured table or the default table for multiplexing pattern 1 (table A). There were also two companies proposing to also allow to use a table signaled through dedicated RRC signaling.

One company proposed to define two sets of PDSCH default tables, one being used for SIB1-carrying PDSCH (RMSI, the tables agreed last meeting), and one table used for non-SIB1-carrying PDSCH. This could reduce the need for RRC configured tables and decrease the configuration overhead.

One company proposed to allow up to 64 entries (instead of the maximum 16 entries) for time-domain allocation with DCI format 0-1 in order to facilitate better analog beam forming.

Proposal: (the text in red may need discussion)

For C-RNTI and CS-RNTI in CSS:

- if dedicated RRC signaling has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in CSS

  - Note: this is the same table used in USSif SIB1 has provided a table for PDSCH

- else if  SIB1 has provided a table

  - Use this table for PDSCH transmission scheduled by DCI in CSS

- else

  - Use the default PDSCH table A (mux pattern 1)

For C-RNTI and CS-RNTI in CSS:

- if dedicated RRC signaling has provided a table

  - Use this table for PUSCH transmission scheduled by DCI in CSS

  - Note: this is the same table used in USSif SIB1 has provided a table for PDSCH

- else if SIB1 has provided a table for PUSCH

  - Use this table for PUSCH transmission scheduled by DCI in CSS

- else

  - Use the default PUSCH table

## Default tables for time-domain allocation

The content of the default tables were agreed at RAN1#92bis. Some companies proposed to add more entries to tables B and C (multiplexing pattern 2 and 3) where there are unused entries available.

Note: In the proposal below, the table content was common across the two companies making detailed proposals except row 14 in table C where there were some differences.

Proposal: Update the tables as shown below.

Table 5.1.2.1.1-3: Default PDSCH time domain resource allocation B

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2,3 | Type B | 0 | 2 | 2 |
| 2 | 2,3 | Type B | 0 | 4 | 2 |
| 3 | 2,3 | Type B | 0 | 6 | 2 |
| 4 | 2,3 | Type B | 0 | 8 | 2 |
| 5 | 2,3 | Type B | 0 | 10 | 2 |
| 6 | 2,3 | Type B | 1 | 2 | 2 |
| 7 | 2,3 | Type B | 1 | 4 | 2 |
| 8 | 2,3 | Type B | 0 | 2 | 4 |
| 9 | 2,3 | Type B | 0 | 4 | 4 |
| 10 | 2,3 | Type B | 0 | 6 | 4 |
| 11 | 2,3 | Type B | 0 | 8 | 4 |
| 12 | 2,3 | Type B | 0 | 10 | 4 |
| 13 | 2,3 | Type B | 0 | 2 | 7 |
| 14 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |

Table 5.1.2.1.1-4: Default PDSCH time domain resource allocation C

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2,3 | Type B | 0 | 2 | 2 |
| 12 | 2,3 | Type B | 0 | 4 | 2 |
| 23 | 2,3 | Type B | 0 | 6 | 2 |
| 34 | 2,3 | Type B | 0 | 8 | 2 |
| 45 | 2,3 | Type B | 0 | 10 | 2 |
| 6 | Reserved |  |  |  |  |
| 7 | Reserved |  |  |  |  |
| 8 | 2,3 | Type B | 0 | 2 | 4 |
| 9 | 2,3 | Type B | 0 | 4 | 4 |
| 10 | 2,3 | Type B | 0 | 6 | 4 |
| 11 | 2,3 | Type B | 0 | 8 | 4 |
| 12 | 2,3 | Type B | 0 | 10 | 4 |
| 13 | 2,3 | Type B | 0 | 2 | 7 |
| 14 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |

## Resource allocation tables for ECP

The default PDSCH and PUSCH resource allocations tables were defined assuming NCP. One company proposed to add ECP tables to be used as default tables for PDSCH and PUSCH once SIB1 is received.

Proposal: Define ECP default tables from the NCP tables as shown below.

Table 2. Default PDSCH time-domain RA table for ECP

| Row index | DL-DMRS-typeA-pos | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Type A | 0 | 2 | 12=>6 |
|  | 3 | Type A | 0 | 3 | 11=>5 |
| 2 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 3 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 4 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 5 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 6 | 2 | Type B | 0 | 9=>6 | 4 |
|  | 3 | Type B | 0 | 10=>8 | 4=>2 |
| 7 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 8 | 2,3 | Type B | 0 | 5 | 7=>6 |
| 9 | 2,3 | Type B | 0 | 5 | 2 |
| 10 | 2,3 | Type B | 0 | 9 | 2 |
| 11 | 2,3 | Type B | 0 | 12=>10 | 2 |
| 12 | 2,3 | Type A | 0 | 1 | 13=>11 |
| 13 | 2,3 | Type A | 0 | 1 | 6 |
| 14 | 2,3 | Type A | 0 | 2 | 4 |
| 15 | 2,3 | Type B | 0 | 4 | 7=>6 |
| 16 | 2,3 | Type B | 0 | 8 | 4 |

Table 3. Default PUSCH time domain RA table with ECP

| Row index | PUSCH mapping type | K2' | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14=>8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10=>4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14=>8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type B | j+2 | 0 | 14=>8 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6=>4 |
| 15 | Type A | j+3 | 0 | 14=>8 |
| 16 | Type A | j+3 | 0 | 10 |

## Slot aggregation

Slot aggregation has been agreed at a previous meeting. For semi-static UL/DL configuration, several companies proposed to only count slots in the “same direction” as the scheduling DCI (i.e. for PUSCH repetition, only slots semi-statically configured as UL or flexible are counted, for PDSCH only slots semi-statically configured as DL or flexible are counted). The fact that the semi-static direction is configured on symbol level and not slot level need to be accounted for in the final specification text. One company also wanted to take BWP switching time into account although it can be discussed if slot repetition is used in combination with BWP switching.

Regarding the redundancy version to use, several companies proposed to use the “standard” RV order 0, 2, 3, 1. The RV field in the DCI scheduling the transmission indicates the RV of the first transmission, the subsequent slots cycle through the RVs in order starting from the one indicated in the DCI.

Proposal:

- When UL/DL allocation is semi-statically configured, and slot aggregation is used

  - For PDSCH, only slots configured with a semi-static allocation that allow for the same allocation as used for the PDSCH in the first of the aggregated slots are counted

  - For PUSCH, only slots configured with a semi-static allocation that allow for the same allocation as used for the PUSCH in the first of the aggregated slots are counted

- For slot aggregation,

  - the RV field in the DCI scheduling the transmission indicates the RV of the first transmission,

  - slots cycle through the RVs in order 0, 2, 3, 1 starting from the RV indicated in the DCI

## Timing relation between PDCCH and PDSCH

At previous meetings, it was agreed that PDSCH can start from the beginning of a slot assuming that the PDCCH is located somewhere in the first three OFDM symbols of a slot. The reason was to be able to reuse unused CORESETs for data. For the processing time discussion, the following assumption has been made: “First symbol of PDSCH assumed to be no earlier than the first symbol of PDCCH scheduling this PDSCH”. However, it seems that these agreements have not been captured in the RAN1 specifications. Furthermore, if the PDSCH can start a few symbols before the PDCCH for mapping type A, it seems reasonable to allow this also for mapping type B as there is no difference from a buffering perspective.

Proposal: Capture the previous RAN1 agreements in 38.214.

## Paging scheduling

For paging, two companies pointed out the need for cross-slot scheduling fairly far into the future. This to support beam sweeping, e.g. with many beams with PDCCH in the first sweep and the PDSCH in the second. Since there are several spare bits in the DCI using P-RNTI, one possibility is to increase the size of the cross-slot timing indication to 7 bits.

Proposal: Increase the slot offset field in the DCI with P-RNTI to 7 bits.

## Frequency-domain resource allocation for msg3

At RAN1#92bis, the following agreement was made:

Working assumption:

interpret RIV in RAR for initial msg3 according to the same method as in LTE

One company pointed out that for PUSCH allocations of less than 5 OFDM symbols, the resource allocations possible to signal would be insufficient for the maximum payload of 20 bytes agreed in RAN2.

Proposal: Discuss if modifications to the RIV interpretation for RAR are needed.

## Transport block size determination

One company proposed to set Xoh=0 for all broadcast RNTIs in order to ensure that all UEs have the same understanding of the TB size regardless of their Xoh configuration.

One company proposed to refine the N^PRB_DMRS for broadcast RNTIs to get a more accurate calculation of available REs.

One company had a proposal to simplify the TB size equations (without affecting the technical behavior).

Proposal: For TB size determination, assume Xoh=0 for all broadcast RNTIs.

## Frequency hopping and non-contiguous frequency allocations

One company pointed out that PUSCH frequency hopping may result in non-contiguous frequency allocations due to “wrap around”. Rel 15 UEs does not support non-contiguous allocations.

Proposal:  Clarify in the frequency hopping description that non-contiguous allocations due to wrap around are not supported.

## VRB-to-PRB interleaving

Two companies raised the issue on interleaved VRB-to-PRB mapping for SIB transmission. Note that this issues was raised under 7.1.3.1.4 as well. Before reception of SIB1, the CRB grid is not known. Hence, if interleaved VRB-to-PRB mapping is to be supported for SIB1, the resource grid needs to be determined, e.g. following the initial DL BWP. Offline, it was pointed out that interleaved VRB-to-PRB mapping for system information might be complicated to use in practice, especially if multiple BWS are used from the NW perspective.

Proposal: Discuss whether to support interleaved VRB-to-PRB mapping for SIB1 transmission

## Rate matching around SSB

One company proposed to include a bitmap in the DCI scheduling SIB1 to indicate presence/absence of other SSBs.

Proposal: Discuss further.

## Peak rate definition

One company proposed to allow the UE to skip deciding downlink transmissions resulting in a data rate higher than the peak rate computed by the previsouly agreed peak rate formula. However, the need for this limitation is not clear as the peak rate formula can be seen more for informative purpose.

Proposal: No spec change.

## URLLC

Enhanced URLLC support require CQI for BLER of 1E-5 and a MCS table with low code rates for reliability. Support for this should be a UE capability.

# Appendix: Proposals in the summarized contributions

## R1-1805893 (Huawei)

- Proposal 1: Support VRB-to-PRB interleaving for SIB transmission.

- Proposal 2: A UE shall assume that the precoding of the potential co-scheduled UE(s) in other DM-RS port of the same CDM group is same in continuously 2 resource blocks (CRB=2K and 2K+1, K is an integer).

- Proposal 3: The RMSI-configured table or the agreed default table can be used for PDSCH/PUSCH scheduled by DCI in CSS.

- Proposal 4: For fallback DCI in USS, the resource allocation table should be a RMSI-configured table or default table and should not be updated by the RRC signaling.

- Proposal 5: The default PDSCH time domain RA table for ECP is adjusted as below:

| Row index | DL-DMRS-typeA-pos | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Type A | 0 | 2 | 12=>6 |
|  | 3 | Type A | 0 | 3 | 11=>5 |
| 2 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 3 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 4 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 5 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 6 | 2 | Type B | 0 | 9=>6 | 4 |
|  | 3 | Type B | 0 | 10=>8 | 4=>2 |
| 7 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 8 | 2,3 | Type B | 0 | 5 | 7=>6 |
| 9 | 2,3 | Type B | 0 | 5 | 2 |
| 10 | 2,3 | Type B | 0 | 9 | 2 |
| 11 | 2,3 | Type B | 0 | 12=>10 | 2 |
| 12 | 2,3 | Type A | 0 | 1 | 13=>11 |
| 13 | 2,3 | Type A | 0 | 1 | 6 |
| 14 | 2,3 | Type A | 0 | 2 | 4 |
| 15 | 2,3 | Type B | 0 | 4 | 7=>6 |
| 16 | 2,3 | Type B | 0 | 8 | 4 |

- Proposal 6: The default PUSCH time domain RA table for ECP is adjusted as below:

| Row index | PUSCH mapping type | K2' | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14=>8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10=>4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14=>8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type B | j+2 | 0 | 14=>8 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6=>4 |
| 15 | Type A | j+3 | 0 | 14=>8 |
| 16 | Type A | j+3 | 0 | 10 |

- Proposal 7: If semi-static DL/UL assignment configuration of a slot has direction confliction with scheduled PDSCH/PUSCH assigned symbols, the slot is not counted as an aggregated slot.

- Proposal 8: For DCI based retransmission of slot aggregation, the same aggregation factor is reused, which should be specified in the specification.

- Proposal 9: For DCI based retransmission of slot aggregation, a field in DCI to indicate a RV sequence from several RV sequence candidates should be supported.

- Proposal 10: The number of REs for DMRS per PRB should be clarified for PDSCH/PUSCH scheduled by PDCCH with CRC scrambled by different RNTIs or DCI format 1_0/0_0 in TS38.214.

## R1-1805900 (Huawei)

- Proposal 1: The support of ultra-reliability shall not compromise resource efficiency at the cost of excessive repetitions.

- Proposal 2: The following resource efficient schemes shall be supported for URLLC

  - Low latency CSI report

  - Dynamic indication of the aggregation factor and/or aggregation pattern

- Proposal 3: NR considers using multiple TRPs with the following techniques to enhance PDCCH and/or PDSCH reliability of URLLC services,

  - Coordinated or independent scheduling/transmission

  - Dynamic resource muting

## R1-1806067 (vivo)

- Proposal 1: Resource block bundle for VRB to PRB mapping for PDSCH carrying RMSI is defined in the PRB grid of initial BWP, i.e., resource block bundles are partitioned from the lowest-numbered common resource block in the CORESET configured by the PBCH.

- Proposal 2: When a valid DCI with SI-RNTI is detected in either CSS type 0 or CSS type 0a, the 1 bit VRB-to-PRB mapping apply.

## R1-1806134 (ZTE)

- Proposal 1: For UL time-domain resource allocation, the 7-bit index can be directly interpreted as SLIV value.

- Proposal 2: For DL time-domain resource allocation, four separate tables 2.1-2 to 2.1-5 for the mapping between 6-bit index and SLIV value should be considered.

- Proposal 3: For slot-aggregation, the following two criteria should be support to choose the slots used for slot-aggregation.

- 1. The first criterion: For UL slot-aggregation, only the “U” and “X” slot can be chosen as the transmission slot; for DL slot-aggregation, only the “D” and “X” slot can be chosen as the transmission slot. The SLIV value of the first aggregation slot corresponding to the combination of start symbol and length can be used in the following chosen slots.

- 2. The second criteria: the slot must have enough frequency resource to support the traffic transmitted in the first aggregation slot.

- Proposal 4: The slots in the transition time of BWP switching should not be counted in the slot-aggregation.

- Proposal 5: The text in 38.214 should be changed as:

- Proposal 6: The RB mapping between two BWPs should be considered for BWP switching in slot-aggregation.

- Proposal 7: NR should support discontinuous time domain RA for RMSI PDSCH with consideration of potential collision between RMSI PDSCH and SS/PBCH block for pattern 1.

- Proposal 8: UE behavior in RRC_IDLE: If any two PDSCH among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell,the decoding prioritization of the PDSCH needs to be followed by RA-RNTI/TC-RNTI PDSCH > P-RNTI PDSCH > SI-RNTI PDSCH.

- Proposal 9: For Scheduling/HARQ session, from MAC overhead, scheduling flexibility and BLER performance perspective, it is unnecessary to add missing VoIP packet sizes of 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712 into current TBS table.

- Proposal 10: For Scheduling/HARQ session, we propose that it is unnecessary to discuss the issue on VoIP support of current TBS determination procedure before RAN2 send RAN1 a LS of the result of NR VoIP sizes.

## R1-1806299 (CATT)

- Proposal 1: For PDSCH scheduled by DCI in Type0A-PDCCH, Type1-PDCCH, Type2-PDCCH and Type3-PDCCH common search space,

  - if RMSI signaling has provided a table

    - Use this table for PDSCH transmission scheduled by DCI

  - else

    - Use default table A

- Proposal 2: For PUSCH scheduled by MAC RAR and DCI in Type1-PDCCH and Type3-PDCCH common search space,

  - if RMSI signaling has provided a table

    - Use this table for PUSCH transmission scheduled by DCI

  - else

    - Use the default PUSCH table

- Proposal 3: Additional slot delay defined in Table 6.1.2.1.1-4 in 38.214 is applied to MSG3 scheduled by MAC RAR only.

- Proposal 4: The text proposal in section 2.2 is adopted.

## R1-1806367 (CMCC)

- Proposal 1: Time domain resource allocation combinations for RMSI scheduling that proposed in Table 1 & 2 towards CORESET & SS/PBCH block multiplexing pattern 2 & 3 need to be included. The other values can be further discussed to fully exploit the 4 bits DCI for resource allocation.

Table 1 Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 2

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type A | 0 | 2 | 2 |
| 1 | Type B | 0 | 4 | 2 |
| 2 | Type B | 0 | 6 | 2 |
| 3 | Type B | 0 | 8 | 2 |
| 4 | Type B | 0 | 10 | 2 |
| 5 | Type A | 1 | 2 | 2 |
| 6 | Type B | 1 | 4 | 2 |
| 7 | Type A | 0 | 2 | 4 |
| 8 | Type A | 1 | 0 | 4 |
| 9 | Type A | 1 | 2 | 4 |
| 10 | Type B | 0 | 4 | 4 |
| 11 | Type B | 1 | 4 | 4 |
| 12 | Type B | 0 | 6 | 4 |
| 13 | Type B | 0 | 8 | 4 |
| 14 | Type B | 0 | 10 | 4 |

Table 2: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 3

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type B | 0 | 4 | 2 |
| 1 | Type B | 0 | 6 | 2 |
| 2 | Type B | 0 | 8 | 2 |
| 3 | Type B | 0 | 10 | 2 |
| 4 | Type B | 0 | 4 | 4 |
| 5 | Type B | 0 | 6 | 4 |
| 6 | Type B | 0 | 8 | 4 |
| 7 | Type B | 0 | 10 | 4 |
| 8 | Type B | 0 | 4 | 7 |
| 9 | Type B | 0 | 6 | 7 |

## R1-1806625 (LG)

- Proposal 1: For default PDSCH time domain resource allocation B, add following rows:

| Row index | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 8 | Type B | 0 | 4 | 4 |
| 9 | Type B | 0 | 6 | 4 |
| 10 | Type B | 0 | 8 | 4 |
| 11 | Type B | 1 | 2 | 4 |

- Proposal 2: For default PDSCH time domain resource allocation C, add following rows:

| Row index | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 5 | Type A | 0 | 0 | 6 |
| 6 | Type A | 0 | 2 | 6 |
| 7 | Type B | 0 | 8 | 4 |
| 8 | Type B | 0 | 10 | 4 |
| 5’ | Type B | 0 | 2 | 4 |
| 6’ | Type B | 0 | 4 | 4 |
| 7’ | Type B | 0 | 6 | 4 |
| 8’ | Type B | 0 | 8 | 4 |

- Proposal 3: For DCI in at least Type0/0A/1/2 CSS, a set of time-domain resource allocation for PDSCH or PUSCH will not be changed by RRC configuration.

- Proposal 4: Further consider different options for applying multiple time-domain RA table when RMSI also indicates time-domain RA table.

## R1-1806737 (Samsung)

- Proposal 1. A UE is allowed to skip decoding when the actual instantaneous data rate is larger than the max data rate.

- Proposal 2. LTE Data rate in EN-DC is defined by using “Maximum number of DL-SCH transport block bits received within a TTI” in UE categories.

## R1-1806815 (Mediatek)

- Proposal 1:  Use the typical order of the RV sequence 0->2->3->1 for slot aggregation.

- Proposal 2:  Add the following clarification in the resource allocation specification:

  - The UE is not expected to be scheduled with PDSCH where the first symbol of PDSCH is earlier than the first symbol of the PDCCH scheduling this PDSCH.

- Proposal 3: Adopt the following TP for further clarification of the relative position of PDCCH and PDSCH

- Proposal 4: UE is not expected to receive out-of-order scheduling grants in Release 15.

- Proposal 5: Simplify the TBS determination equation in step 4 when  R>1/4 and  3824<N_info^'≤8424 as follows:

  - TBS= N_info^'

- Proposal 6: Adopt the following TP for further simplification of the TBS determination procedure.

- Proposal 7: NR supports the cross-Nslots scheduling to enable the separated beam-sweeping for Paging PDCCH and PDSCH

- Proposal 8: The starting time for Paging PDSCH transmission is determined by K0+N, where N can be the number of slots or ms configured in RMSI.

## R1-1806867 (Oppo)

- Proposal 1: For DL N-slot scheduling, the same symbol allocation is applied across consecutive slots in which corresponding symbols are DL or flexible.

- Proposal 2: For multi-slot PDSCH/PUSCH transmission, the RV changes across the slots counted according to 0, 2, 3, 1. The starting position in the cycle is 0.

## R1-1807041 (Ericsson)

- Proposal 1	Add two features to the UE feature list for supporting URLLC:

  - Support CQI report for BLER=1e-5.

  - Scheduling PDSCH and PUSCH with the MCS table of high reliability

## R1-1807066 (DOCOMO)

- Increase the max number of bits for PUSCH time-domain RA field in DCI format 0_1 from 4 to 6.

  - Corresponding RAN1 spec need to be changed.

  - Corresponding RAN2 parameters need to be changed.

- Send a LS to RAN2 to update the parameter.

- Adopt the following text proposals.

  - Check further whether there are any other fields that need to be updated.

## R1-1807255 (Ericsson)

- Proposal 1	Use some of DCI reserved bit field in P-RNTI DCI to indicate the slot gap. UE derives the PDSCH slot for paging using DCI slot gap field and time domain resource allocation index.

- Proposal 2	Using 7 bits for slot gap in DCI field when paging message is scheduled.

- Proposal 3	With the 7 bits representing value from 0 to 127. Code points of value 0-96 map directly to the offset values 0-96 slots (i.e. offsets in relation to the PDCCH transmission). The remaining code points, i.e. 97-127 is used when the PDSCH beam sweep extend/slide beyond the next PO. Code points 97-127 map to offset values 1-31 measured from the end of the next PO (i.e. from the end of the next paging PDCCH beam sweep).

- Proposal 4	For multiplexing type 1, configuration with PDCCH using only 1 symbol should be added with S = 1, L = {10, 11, 12} that can be used to achieve high downlink throughput.

- Proposal 5	Define separate RMSI default table and non-RMSI default tables. If they turn out to be 100% overlapping, indicate clearly in spec which part of table should be considered valid for RMSI. Non-RMSI default table works as default table for all other PDSCH transmissions if not override by RRC signaling.

- Proposal 6	P-RNTI, SI-RNTI (except SIB1), RA-RNTI, TC-RNTI use pdsch-configCommon/pusch-configCommon provided table, if defined in RRC, otherwise using default PDSCH/PUSCH table.

- Proposal 7	Clarify which table/list should be applied for C-RNTI, CS-RNTI, if that should conditioned on where the PDCCH is received.

- Proposal 8	C-RNTI and CS-RNTI always use the dedicated table defined in pdsch-config/pusch-config when configured.

- Proposal 9	For number of PUSCH symbols larger than or equal to 5, the frequency domain RA field should be interpreted with RIV and MSB padding (as the way treated in LTE).

- Proposal 10	For number of PUSCH symbols smaller than 5, and BWP larger than 90 RBs, choose one of the two methods below to interpret the frequency allocation.

  - Option 1: 12 bits are used to indicate start RB range in {0,1,2,…, 89} and RB length range in {1,2, …, 90}

  - Option 2: 12 bits are used to indicate start RB range in {0, 4, 8, 12, …, 272} and RB length range in {1,2, …, 59}

- Proposal 11	If FH is enabled, RB granularity scales with hopping bits. RB granularity is 2 for 1 hopping bit, and 4 for 2 hopping bits.

- Proposal 12	For TBS determination of broadcast PDSCH (P-RNTI, RA-RNTI and SI-RNTI), xOverhead-PDSCH is set to 0, according to [1].

- Proposal 13	Adopt TP in [1] to 38.214 for MCS/TBS determination for PUSCH scheduled by RAR UL grant.

- Proposal 14	The LTE data rate is derived by the following formula  LTE Data rate in EN-DC (in Mbps) = 10-3c=1Cj=1JTBSc,j, wherein C is the number of aggregated LTE component carriers in EN-DC band combination J is the maximum number of transport blocks configured for the given carrier, i.e. either 1 or 2 TBS is the total maximum number of DL-SCH transport block bits within a 1ms TTI for c-th CC, as derived from TS36.213 based on the UE supported maximum MIMO layers for the c-th carrier, and based on the modulation order and number of PRBs based on the bandwidth of the c-th carrier. A draft LS is provided in [2]

## R1-1807280 (Nokia)

- Proposal 1: The default PDSCH time domain resource allocation B is extended as shown in the table 5.1.2.1.1-3 above

- Proposal 2: The default PDSCH time domain resource allocation B is used for OSI, random access response and Paging when the SS/PBCH multiplexing patter 2 is used in the cell.

- Proposal 3: The default PDSCH time domain resource allocation C row index numbering is changed to match the numbering of the allocation B, or alternatively the two tables are merged into a single table.

- Proposal 4: The default PDSCH time domain resource allocation C is extended as shown in the table 5.1.2.1.1-4 above

- Proposal 5: The default PDSCH time domain resource allocation C is used for OSI, random access response and Paging when the SS/PBCH multiplexing patter 3 is used in the cell.

- Proposal 6: Resource allocation for all common search space scheduled PDSCH transmissions other than SIB1 will always follow the common TD-RA configuration provided in SIB1, if present. Even in the presence of dedicated TD-RA configuration.

- Proposal 7: If the common TD-RA configuration is not present in SIB1, the type0a, type1 and type2 common search space scheduled PDSCH follow the same PDSCH allocation table as SIB1

- Proposal 8: If the common TD-RA configuration is not present in SIB1, the type3 common search space scheduled PDSCH follow the PDSCH allocation table A.

- Proposal 9: Resource allocation for all common search space scheduled PUSCH transmissions will always follow the common TD-RA configuration provided in SIB1, if present. Even in the presence of dedicated TD-RA configuration. The modification to table 6.1.2.1.1-1 as proposed below is adopted.

## R1-1807363 (Qualcomm)

- Proposal 1: If frequency hopping is enabled, UE is not expected to transmit PUCCH or PUSCH with non-contiguous RB allocation in either the first or the second frequency hop.

- Proposal 2: The PDCCH Format carrying RMSI PDSCH grant shall support time-domain indication K_0 equal or greater than half-Frame duration.

- Proposal 3: NR shall use the following table for RMSI PDSCH SLIV indication. Furthermore, some bits in Format 1_0 such as TPC command [2], PUCCH resource indicator [2] and PDSCH-to-HARQ feedback timing indicator [3] can be repurposed for RMSI PDSCH SLIV.

- Proposal 4: NR shall adopt the Text Proposal in Appendix for RMSI SLIV and rate-matching indication

- Proposal 5:

  - For time-domain signaling for PUSCH scheduled by DCI scrambled by C-RNTI (or CS-RNTI) in CSS

    - if dedicated RRC signaling has provided a table

      - Use this table for PUSCH transmission scheduled by DCI in CSS

      - Note: this is the same table used in USS

    - else if RMSI signaling has provided a table

      - Use this table for PUSCH transmission scheduled by DCI in CSS

    - else

      - Use the default PUSCH table

  - else for other DCI in CSS

    - if RMSI signaling has provided a table

      - Use this table for PUSCH transmission scheduled by DCI in CSS

    - else

      - Use pattern 1 from agreed default table

- Proposal 6:

  - For time-domain signaling for PDSCH scheduled by DCI scrambled by C-RNTI (or CS-RNTI) in CSS

    - if dedicated RRC signaling has provided a table

      - Use this table for PDSCH transmission scheduled by DCI in CSS

      - Note: this is the same table used in USS

    - else if RMSI signaling has provided a table

      - Use this table for PDSCH transmission scheduled by DCI in CSS

    - else

      - Use pattern 1 from the agreed default table

  - else for other DCI in CSS

    - if RMSI signaling has provided a table

      - Use this table for PDSCH transmission scheduled by DCI in CSS

    - else

      - Use pattern 1 from the agreed default table

- Proposal 7: In the case when slot-aggregation is configured, for grant-based UL and DL transmissions the RVID field in DCI is used to indicate the starting RV within the default sequence {0,2,3,1}.

- Proposal 8: The causality of PDCCH relative to PDSCH according to previous agreements should be captured in specification with the following text proposal below.