3GPP TSG RAN WG1 Meeting #93		R1-1807881

Busan, Korea, May 21st – 25th, 2018

Title:	[draft] LS on BFR search space configuration

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	MediaTek [RAN 1]

To:	TSG RAN WG2

Cc:	-

Contact Person:

#### Name: Chia-Hao Yu

E-mail Address: chia-hao.yu@mediatek.com

1. Overall Description:

RAN1 discussed the following description for recoverySearchSpaceId in current ASN.1 definition for BeamFailureRecoveryConfig.

BeamFailureRecoveryConfig field descriptions

candidateBeamRSList A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters

candidateBeamThreshold L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free Random Access to recover from beam failure. The signalled threshold is applied directly for SSB; a threshold for CSI-RS is determined by linearly scaling singalled value based on Pc_ss corresponding to the CSI-RS resource. (see FFS_Specification, FFS_Section)

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 36.321. The mask is valid for all SSB resources

rach-ConfigBFR Configuration of contention free random access occasions for BFR

recoverySearchSpaceId Search space to use for BFR RAR. If the field is absent, the UE uses the initial Serach Space (SearchSpaceId = 0).

ssb-perRACH-Occasion Number of SSBs per RACH occasion (L1 parameter 'SSB-per-rach-occasion')

After discussion, RAN1 reached the following agreement:

Agreement:

- (Working assumption) If either CORESET associated with SearchSpace-BFR or SearchSpace-BFR is not configured, UE assumes contention-free PRACH BFR is not performed.

  - In this case, SearchSpace-0 or CORESET-0 is not used for monitoring contention-free PRACH BFR response.

- If either CORESET associated with SearchSpace-BFR or SearchSpace-BFR is not configured, UE can still perform contention-based PRACH BFR based on configuration.

2. Actions:

RAN1 kindly asks RAN2 to take into account the above RAN1 agreements in their design.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 #94		2018.08.20 – 2018.08.24		Gothenburg, Sweden

TSG-RAN WG1 #94Bis		2018.10.08 – 2018.10.12		Chengdu, China