3GPP TSG RAN WG1 Meeting #94	R1-1808090

Gothenburg, Sweden, August 20-24, 2018

Agenda Item:	7.2.3.2

Source:	Huawei, HiSilicon

Title:	System performance evaluation in multi-hop topology IAB network

Document for:	Discussion and decision

Introduction

Coverage holes due to NLoS or blockage is one of the biggest challenges for commercial deployment of mmWave. Many field tests show that macro only deployment in a typical urban environment may not provide acceptable system performance even with very high EIRP. IAB, as a promising technology for mmWave deployment, is expected to provide a low cost coverage enhancement and capacity improvement solution.

In this paper, we present some preliminary system evaluations to demonstrate the benefits of introducing IAB for mmWave deployment in terms of coverage, system capacity. Furthermore, dynamic/flexible TDM with SDM resource allocation between backhaul and access, cross-link interference and backhaul link high order modulation are also investigated.

System performance evaluation summary

Evaluation assumption and parameters

The evaluation methodology and assumption are based on the agreement in [1], and the detailed parameters applied are listed in the appendix. Specifically, in the evaluation, multi-hop topology is assumed with 3 IAB nodes deployed randomly within each sector.  Max. RSRP criteria together with max. 4 hop number and 2 IAB node connectivity degree constraint [2] is used to build the topology among IAB nodes. It should be noted that the absolute system performance largely depends on the topology, frame structure and algorithm etc. applied. In this paper, we mainly study the relative performance with the same topology, frame structure and algorithm assumptions as much as possible.

The frame structure in the evaluation is illustrated in Figure 1, and thee resource allocation schemes between access and backhaul (static TDM/dynamic TDM/dynamic TDM with flexible SDM) are evaluated.

- Static TDM slot allocation

With static TDM scheme, a predefined fixed TDM slot allocation for backhaul link is applied, and the access link is not allowed to be scheduled in backhaul slot even if the slot is not fully occupied.

- Dynamic TDM slot allocation

The TDM slot number and location for backhaul link can be flexibly configured according to the backhaul transmission capacity requirement, and also for each specific configuration, the backhaul slot can be used for access link by dynamic scheduling, if the backhaul transmission is not scheduled in that slot.

- Dynamic TDM slot allocation with flexible SDM

In this scheme, the SDM scheme between backhaul and access links is applied on top of the dynamically allocated backhaul transmission slot, according to the scheduling decision at IAB node.

(a) Static TDM scheme

(b) Dynamic TDM scheme

(c) Dynamic TDM + SDM scheme

Figure 1: Resource allocation schemes in the evaluation

Coverage performance

Figure 2 shows the UE geometry distribution with 30 UEs per sector. The UE DL geometry is significantly improved as expected if IAB nodes are introduced, since UEs get closer to network. UEs attached to IAB nodes have better geometry compared to those attached to donor. The SINR improves further with more IAB nodes deployment.

Figure 2: UE geometry performance of IAB

In Figure 3(a), it can be observed that without IAB node, the outage ratio with typical 80% indoor/20% outdoor UE distribution is rather high for 500m ISD, and downlink above 30% and uplink above 40% UEs suffer outage (no throughput). The outage ratio is significantly reduced with the introduction of more IAB nodes. Actually the multiplexing scheme between backhaul and access link has negligible impact on outage ratio since outage is mainly dependent on UE’s SINR, instead of how to multiplex access and backhaul.

If UE distribution is changed to 100% outdoor, the outage improves even without IAB nodes as shown in Figure 3(b), due to fewer coverage holes. Therefore, it is necessary to take UE distribution to account when discussing IAB scenarios. To be specific, if 100% of UEs are dropped outdoor, the ISD should be enlarged further to create more coverage holes.

Observation 1: Introducing IAB nodes can significantly improve coverage performance for mmWave.

- 80% indoor/20% outdoor, ISD=500m

- 100% outdoor, ISD=500m

Figure 3: Outage performance with static TDM scheme

System capacity with dynamic/flexible resource allocation

In LTE relay, backhaul link is multiplexed with access in semi-static TDM manner. The time slot allocation pattern for backhaul link is predefined according to the MBSFN subframe location and the access link is not allowed to occupy the backhaul slot even though the backhaul slot is not utilized. While the TDM in NR IAB is much more flexible, without the specific MBSFN subframe location constraint. In addition, the multi-beam operation in NR mmWave creates an opportunity to deploy IAB with flexible SDM between access and backhaul links. As a result, system capacity and transmission delay will be improved.

In the evaluation, dynamic TDM slot allocation between backhaul and access link with and without SDM based multiplexing are evaluated, in multi-hop scenario. As a comparison, the static TDM based slot allocation scheme is also presented.

Table 1 illustrates system capacity evaluation comparison for full buffer traffic with various resource allocation schemes discussed above, considering different number of IAB node deployed per sector. It can be observed that:

- Better system capacity can be achieved with the introduction of IAB nodes.

- Compared to static TDM, dynamic TDM scheme brings clear performance enhancement due to the better resource utilization.

- SDM further improves system capacity by utilizing the beams unavailable to backhaul links to serve UEs, and the growing benefits of SDM can be observed if more RNs are deployed

Table 1:  System capacity comparison

| IAB node number | Capacity performance gain, baseline: w/o IAB nodes |  |  |
| --- | --- | --- | --- |
|  | Static TDM | Dynamic TDM | Dynamic TDM+ SDM |
| 1 IAB node/sector | 7.23% | 17.23% | 25.15% |
| 3 IAB node/sector | 10.23% | 28.31% | 45.08% |

Observation 2: Compared to static TDM, dynamic TDM brings significant system capacity gain due to better resource utilization, and dynamic TDM+SDM further improves the system capacity by utilizing the beams unavailable to backhaul link. More benefits of SDM can be observed if more IAB nodes are deployed.

Backhaul link high order modulation

In Table 2, the system performance comparison with various modulation order on backhaul link is presented. Only relative gain is given with the no-IAB node deployment as the baseline. Obviously, system capacity has obvious improvement with higher modulation. In addition, dynamic TDM with SDM benefits more from the increased backhaul capacity.

The evaluation shows that the end-to-end system capacity is highly dependent on the backhaul link capacity. If backhaul capacity is constrained, congestion will happen in the intermediate IAB nodes and data cannot reach the destination timely. For dynamic TDM with SDM, the access link transmission opportunity is increased since the access links can share the same backhaul slots in different beam directions, therefore more gain can be reaped if backhaul link allows higher transmission capacity.

It is necessary to support high modulation backhaul transmission.

Table 2: System capacity comparison with 1024QAM/256QAM/64QAM backhaul

| Maximum MCS for backhaul link | IAB node number | System capacity performance gain, baseline: w/o IAB nodes |  |  |
| --- | --- | --- | --- | --- |
|  |  | Static TDM | Dynamic TDM | Dynamic SDM |
| 64 QAM | 1 IAB node/sector | 7.23% | 17.23% | 25.15% |
|  | 3 IAB node/sector | 10.23% | 28.31% | 45.08% |
| 256 QAM | 1 IAB node/sector | 15.08% | 28.08% | 39.00% |
|  | 3 IAB node/sector | 20.62% | 46.54% | 70.23% |
| 1024 QAM | 1 IAB node/sector | 21.46% | 38.00% | 52.15% |
|  | 3 IAB node/sector | 30.46% | 61.85% | 93.69% |

Observation 3: High order modulation on backhaul link (e.g., 256QAM, 1024QAM) can significantly enhance the system capacity and higher system performance can be obtained with dynamic resource allocation.

Cross-link interference performance

In this section, we study the downlink performance and focus on the impact of CLI to IAB node’s MT (i.e., DL backhaul link), which is generated by other IAB node’s DU as shown in Figure 4(a) and (b). In order to understand the impact of different interfering source better, the CLI is separated into two cases according to the link types, corresponding to the interference illustration in Figure 4.

- Case 1:  CLI from interfering IAB node’s backhaul transmission to interfered IAB node’s MT.

- Case 2:  CLI from interfering IAB node’s access transmission to interfered IAB node’s MT.

For simplicity, SDM is not evaluated to avoid introducing inter-UE interference.

- Case 1:  CLI from DL backhaul to DL backhaul            (b) Case2: CLI from DL access to DL backhaul

Figure4:  CLI from DU to MT

Figure 5 shows the downlink receive post-SINR and the downlink system capacity, with static TDM scheme between access and backhaul link applied. The CLI of case 1 and case 2 are eliminated separately so that their impact on performance can be observed individually. The system capacity with all CLI taken into consideration is used as the baseline.

The following observations could be made

- The impact of inter-IAB node CLI is non-negligible. By eliminating CLI completely, the post-SINR at the receiver can be improved around 5dB.

- The CLI level from interfering access transmission is higher than the interference from the interfering backhaul transmission. 2.67% and 9% system performance improvement can be achieved if the CLI of case 1 and case 2 is eliminated separately.

- Although the post-SINR can be improved around 3dB by eliminating the case 1 CLI, the system throughput gain is only 9%. In order words, the significant post-SINR improvement does not bring obvious system performance enhancement..

- Post-SINR                                                  (b)  Downlink capacity

Figure 5: Downlink performance with static TDM resource allocation

Figure 6: RU on access and backhaul link with static TDM resource allocation

We further analyze the RU on access and backhaul link. From the evaluation results in Figure 7, it can be found that the RU for the 2nd and 3rd hop IAB node is rather low (42%, and 28%), meaning that congestion happens in the 1st hop IAB node and the downlink data cannot reach the UE, thus leads to even lower RU on access links. Therefore, congestion in the intermediate IAB nodes becomes the bottleneck of system performance.

With dynamic TDM resource allocation, the time slot for backhaul link can dynamically be allocated according to the backhaul link transmission requirement. Therefore, the backhaul link congestion can be alleviated to some extent. Figure 8 gives the RU and system capacity evaluation results with dynamic TDM resource allocation. As expected, the RU and system downlink capacity are greatly improved although the congestion still exists.

It should be noted that by alleviating the congestion using dynamic TDM slot allocation, the impact of CLI also becomes more significant. In Figure 8, it can be observed that the system capacity gain is increased from 9% to 17% by eliminating case 2-2 CLI. This implies that CLI become more pronounced when congestion at the IAB nodes is alleviated.

(a) Downlink capacity                                                      (b) RU on access and backhaul link

Figure 7: System performance with dynamic TDM resource allocation

Based on the above analysis, the following observations can be made,

Observation 4: The inter-IAB node CLI impact on system performance is noticeable.

Observation 5: In multi-hop topology, congestion easily happens in the intermediate IAB nodes and significantly degrades system performance.

Observation 6: When congestion becomes the bottleneck, CLI mitigation does not bring significant system capacity improvement.

Observation 7: With dynamic resource allocation between the backhaul and access link, congestion at the intermediate IAB nodes can be alleviated.

Observation 8: The impact of CLI among IAB nodes becomes more pronounced when congestion at the IAB nodes is alleviated.

Conclusions

In this contribution, we further evaluate the system performance in multi-hop IAB deployment in terms of coverage, system capacity and inter-IAB node interference for mmWave band, by system evaluation. We make the following observations:

Observation 1: Introducing IAB nodes can significantly improve coverage performance for mmWave.

Observation 2: Compared to static TDM, dynamic TDM brings significant system capacity gain due to better resource utilization, and dynamic TDM+SDM further improves the system capacity by utilizing the beams unavailable to backhaul link. More benefits of SDM can be observed if more IAB nodes are deployed.

Observation 3: High order modulation on backhaul link (e.g., 256QAM, 1024QAM) can significantly enhance the system capacity and higher system performance can be obtained with dynamic resource allocation.

Observation 4: The inter-IAB node CLI impact on system performance is noticeable.

Observation 5: In multi-hop topology, congestion easily happens in the intermediate IAB nodes and significantly degrades system performance.

Observation 6: When congestion becomes the bottleneck, CLI mitigation does not bring significant system capacity improvement.

Observation 7: With dynamic resource allocation between the backhaul and access link, congestion at the intermediate IAB nodes can be alleviated.

Observation 8: The impact of CLI among IAB nodes becomes more pronounced when congestion at the IAB nodes is alleviated.

- References

RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #93.

R1-1805924, IAB system evaluation methodology and preliminary results, Huawei, RAN1#93, Busan, May 2018.

TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, Dec.2017.

- Appendix

Table A-1: Evaluation scenarios for IAB

| Attribution | Assumption |
| --- | --- |
| Network Layout | Hexagonal cellular network (ISD = 500m) with multi-hop relaying |
| Number of TRPs | 19 macro TRPs and 57*Nr rTRPs where Nr is the number of rTRPs per sector. The value of Nr is {3}. |
| Deployment of RN | Random |
| UE distribution | Uniform random deployment |
| Node selection for UE | Max RSRP |
| Topology building method | Maximum RSRP, together with the following constraints. - Maximum 4 hops - Maximum 2 node degree for each IAB node[2] |
| Carrier Frequency | In-band backhaul: 30GHz backhaul and access |
| Large-scale channel parameters | - Macro-to-UE: 5GCM UMa - Micro-to-UE: UMi-Street canyon - Macro-to-Macro: 5GCM UMa (hUE =25m) - Macro-to-Micro: 5GCM UMa (hUE =10m) - Micro-to-Micro: UMi-Street canyon (hUE =10m) - UE-to-UE: UMi-Street canyon (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 of TR38.802 |
| Fast fading parameters | - Macro-to-UE: 5GCM UMa - Micro-to-UE: UMi-Street canyon - Macro to macro: 5GCM UMa O-to-O (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro to micro: 5GCM UMa O-to-O; ASA and ZSA statistics updated to be the same as ASD and ZSD for UMi-Street canyon; ZoD offset = 0 - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - UE to UE: UMi-Street canyon; ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. |
| UE number per sector | 30 (80% indoor/ 20% outdoor, or 100% outdoor) |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| Slot length | 0.125ms with 14 symbols |
| TDD UL/DL configuration | D:U = 3:2 for baseline(without RN) |
| MIMO | MU-MIMO |
| Scheduling | PF |
| HARQ | CC |
| MCS | Up to 1024QAM for backhaul link |
| Receiver | MMSE-IRC |
| Traffic model | Full buffer |
| Codebook for analog beamforming | DFT-based, no oversampling |
| Penetration loss | 50% high loss, 50% low loss |
| Metric | Full buffer: Area traffic capacity |

Table A-2: Antenna configuration for IAB

| Attribution | Assumption |
| --- | --- |
| gNB height | 25 m |
| RN height | 10 m |
| UE height | 3D distributing [3] |
| TRP Tx power | 3 dBm |
| RN Tx power | 33 dBm |
| UE Tx power | 23dBm |
| TRP antenna configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ |
| RN antenna configuration for each side/sector | (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ |
| UE antenna configuration | (M,N,P,Mg,Ng) = (2,4,2,1,2) , (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (0, 0)λ, Θmg,ng=90, Ω0,0 uniformly distributed in [0, 360] degrees, Ω0,1=Ω0,0+180 |
| RN antenna pattern | See Table A.2.1-6 of TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 of TR 38.802 |