3GPP TSG RAN WG1 Meeting #94	R1-1808128

Gothenburg, Sweden, August 20-24, 2018

Agenda Item:	6.1.5.1.1

Source:	Huawei, HiSilicon, Ericsson, Qualcomm, LG, Samsung, ZTE

Title:	Corrections on NWUS

Document for:	Discussion and decision

# Introduction

In NB-IoT Rel-15, the agreements related to narrowband wake up signal (NWUS) are summarized in [1]. A number of agreements remain to be captured between RAN1 and RAN2 specifications. This paper presents text proposals for TS 36.211 and TS 36.213 which have been converged among the sourcing companies.

# Discussion and TPs

The following agreements have not been captured or precisely captured in RAN1 specifications yet:

RAN1#90 agreements: - For idle mode, - The power saving signal applicable to a UE is sent on the same paging carrier as the associated subsequent physical channel(s) RAN1#90bis agreements: - The UE is configured with a transmission duration of WUS by higher layers RAN1#92 agreements: - WUS actual transmission duration can be shorter than the configured maximum duration of WUS. RAN1#92bis agreements: - Confirm the following working assumption - WUS transmission relative to associated PO of subgroup of UEs is aligned to the start of the configured maximum duration of WUS. - Note: the above applies to at least the case where the gap is large enough for scheduling UE - Note: the above does not imply that subgroup of UEs is introduced and that subgroup is TDM - The gap between the end of configured maximum WUS duration and the first associated PO is equal or larger than the minimum value which is implicitly or explicitly configured and is an absolute number of subframes - There are at least 10 valid subframes between the end of configured maximum WUS duration and the first associated PO. RAN1#93 agreements: - Define candidate values for actual WUS duration for UE blind detection. - The UE is not required to monitor the actual WUS duration other than power of 2 values from 1ms to the maximum WUS duration. - Higher-layer configured WUS_offset is the non-zero gap from the end of the configured maximum WUS duration to the associated PO, as a number of absolute subframes. - WUS is dropped in subframes that carry SIB1-NB and other SIs. - WUS is postponed in subframes that are not NB-IoT DL subframes and does not carry SIB1-NB. - Note: “Postpone” means the corresponding subframes are not counted as configured maximum WUS transmission duration and actual WUS transmission duration. - Note: This does not imply that the minimum gap between the end of actual WUS duration and the first associated PO is reduced

There are also RAN1#93 agreements relating to the operation of the gap between the end of the maximum NWUS duration and the (first) associated PO, relating to configurations timeOffset-DRX, timeOffset-eDRX-Short, timeOffset-eDRX-Long, and the UE signaling of wakeUpSignalMinGap-eDRX. These will be captured by RAN2 in TS 36.304, resulting in the subframe g0 in the text proposals in this paper.

For the equivalent changes in MWUS, see [2]. The 213 specification editor is entrusted to handle the section numbering, and to note the common addition in 36.213 section 2.

Proposal 1: Adopt the following text proposals for TS 36.211 and TS 36.213:

# TP for NWUS in TS 36.211

---------------------------------------------- Start of Text Proposal --------------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- #### 10.2.6B Narrowband wake up signal (NWUS) #### 10.2.6B.1 Sequence generation The NWUS sequence w(m) in subframe x=0, 1, …, M-1 is defined by wm=θnf,nsm'∙e-jπunn+1131 m=0, 1,…, 131 m'=m+132x n=m mod 132 θnf,nsm'=1, if cnf,ns2m'=0 and cnf,ns2m'+1=0-1, if cnf,ns2m'=0 and cnf,ns2m'+1=1j, if cnf,ns2m'=1 and cnf,ns2m'+1=0-j, if cnf,ns2m'=1 and cnf,ns2m'+1=1 u=NIDNcellmod 126+3 where M is the actual duration of NWUS 1≤M≤MNWUSmaxMNWUSmaxas defined in [4]. The scrambling sequence cnf,nsi, i=0, 1, …, 2∙132M-1 is given by clause 7.2, and shall be initialized at the start of the NWUS with cinit_WUS=(NIDNcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell where nf_start_PO is the first frame of the first PO to which the NWUS is associated, and ns_start_PO is the first slot of the first PO to which the NWUS is associated. #### 10.2.6B.2 Mapping to resource elements The same antenna port shall be used for all symbols of the NWUS within a subframe. The UE shall not assume that the NWUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. If only one NRS port is configured by the eNB, the UE may assume the transmission of all NWUS subframes is using the same antenna port; otherwise, the UE can assume the transmission of NWUS in at least one subframe is using the same antenna port. The NWUS sequence is mapped to the set of subframes in the actual NWUS duration as defined in [4], where in a subframe #4 in which SystemInformationBlockType1-NB is transmitted or a subframe in which an SI message is transmitted, the subframe is counted in the NWUS mapping but not used for transmission of NWUS. On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating inband-SamePCI, inband-DifferentPCI, guardband or standalone or on an NB-IoT carrier for which DL-CarrierConfigCommon-NB is present, the NWUS sequence wm shall be mapped to resource elements (k,l) in sequence, starting with w(0) in increasing order of first the index k=0,1,…,NscRB-1, over the 12 assigned subcarriers and then the index l=3, 4, …, 2NsymbDL-1 in each subframe in which NWUS is transmitted. -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

# TP for NWUS in TS 36.213

---------------------------------------------- Start of Text Proposal --------------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- # 2 References The following documents contain provisions which, through reference in this text, constitute provisions of the present document. <Unchanged parts omitted> [13] 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode". <Unchanged parts omitted> ## 16.X UE procedure for receiving narrowband wake up signal A NB-IoT UE using NWUS can assume the actual duration of NWUS, starting in subframe w0, is one of the values in the set listed in Table 16.X-1 corresponding to the maximum duration of NWUS, LNWUS_max, configured by higher layers. The maximum duration of NWUS starts in subframe w0 and ends in subframe (g0-1), where g0 is defined by [13] and w0 is the latest subframe such that there is a total of LNWUSmax NB-IoT DL subframes and subframes #4 carrying SystemInformationBlockType1-NB in the maximum duration. The UE may assume that NWUS and its associated NB-IoT paging occasion subframes are on the same NB-IoT carrier. Table 16.X-1: Actual NWUS durations in NB-IoT DL subframes or subframes containing SystemInformationBlockType1-NB. LNWUS_max; Actual NWUS durations set. 1; {1}. 2; {1, 2}. 4; {1, 2, 4}. 8; {1, 2, 4, 8}. 16; {1, 2, 4, 8, 16}. 32; {1, 2, 4, 8, 16, 32}. 64; {1, 2, 4, 8, 16, 32, 64}. 128; {1, 2, 4, 8, 16, 32, 64, 128}. 256; {1, 2, 4, 8, 16, 32, 64, 128, 256}. 512; {1, 2, 4, 8, 16, 32, 64, 128, 256, 512}. 1024; {1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024}. A NB-IoT UE using NWUS can assume there are at least 10 NB-IoT DL subframes between the end of the maximum duration of NWUS and the first associated NB-IoT paging occasion subframe. -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

# Conclusion

The TPs in Section 2 above are proposed for completing the specification of NWUS in TS 36.211 and TS 36.213.

# References

R1-1807590, “RAN1 agreements for Rel-15 Further NB-IoT enhancements”, Ericsson, RAN1#93, Busan, Korea, May 2018.

R1-1808126, “Correction on MWUS”, Huawei, HiSilicon, Ericsson, Qualcomm, LG, Samsung, ZTE, RAN1#94, Gothenburg, Sweden, August 2018.