3GPP TSG-RAN WG1#94	R1-1808308

Gothenburg, Sweden, August 20-24, 2018

Agenda item:		7.2.6.1

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	HARQ Processing Timeline Enhancements for NR URLLC

Document for:		Discussion and Decision

# 1	Introduction

In RAN#80, the following was agreed [1]:

URLLC L1 improvements (RAN1) for further improved reliability/latency and for other requirements related to the use cases identified,

- PDCCH enhancements. Study focus on Compact DCI, PDCCH repetition, increased PDCCH monitoring capability.

- UCI enhancements. Study focus on Enhanced HARQ feedback methods (increased number of HARQ transmission possibilities within a slot), CSI feedback enhancements.

- PUSCH Enhancements. Study focus on mini-slot level hopping & retransmission/repetition enhancements.

- Enhancements to scheduling/HARQ/CSI processing timeline (UE and gNB), (for existing TTI durations).

In this Tdoc, we discuss potential improvements on the HARQ processing timeline.

# 2	Motivation for HARQ RTT reduction

HARQ has successfully proven to enhance the reliability and spectral efficiency in previous standards. Incorporating feedback from the receiver achieves high reliability at a good spectral efficiency, which is impossible to achieve for very low BLERs with a one shot transmission. However, the main drawback of HARQ is the latency introduced by the so-called HARQ RTT, which poses a challenge for its applicability to URLLC.

In NR, the HARQ timeline is designed in a very flexible manner, allowing feedback bundling in the PUCCH. Although this concept incorporates many advantages, the feedback latency is significantly increased depending on the time instant the dedicated PUCCH is scheduled. This possesses a severe limitation for achieving a low HARQ RTT. Even for UL transmissions, the PUSCH resource for the next retransmission has to be scheduled in a PDCCH occasion, which might not be available in the current timeslot when the feedback is ready at the gNB.

Even if the parameters mentioned in the previous section are further reduced in NR, the HARQ RTT is composed of propagation delay, processing times, and TTI durations. Note, the propagation delay cannot be decreased since it is a function of the physical distance between a transmitter and a receiver. Mainly the processing time at the UE can be reduced by performing a low-complexity prediction of decodability of the received CBs. Further reduction can be achieved by performing the prediction based on a portion of the CBs such that feedback can be provided significantly earlier. Feedback could be provided directly after full reception of the TTI or even earlier, if the following constraint can be met

TTTI≥TS+Tproc+TA

Where, TTTI is the TTI duration, TS is the time of the symbols used for evaluation, Tproc is the processing time and TA is the timing advance.

# 3	Fast HARQ for URLLC

The flexible structure of NR suits perfectly the needs of eMBB with high data rates and the support of multi-user scheduling. However in case of URLLC, this flexibility might increase the latency. Hence, the HARQ RTT would benefit from a semi-static HARQ processing timeline for URLLC, which is not signaled for each URLLC transmission seperately.

The gNB could assign dedicated control resources (PUCCH, PDCCH) in DL and UL for each TB for providing the HARQ feedback. This would allow to issue feedback as fast as possible and thus minimize the HARQ RTT, as depicted in Figure 1.

Figure 1: Semi-static feedback timeline.

Proposal 1: Study semi-static feedback timeline for NR URLLC in Rel. 16.

To reduce the HARQ delay for URLLC applications, feedback can be even provided at an earlier time instant. Earlier feedback is obtained based on low-complexity evaluations of the received Log-Likelihood Ratios (LLRs), referred to as Early HARQ. This can be either done on the whole codeword or on portions of the CBs which are received in the first OFDM symbols. This approach is designated as “Aggressive Early” HARQ.

Figure 2: Comparison of different HARQ feedback approaches.

Figure 2 shows the comparison of HARQ feedback delays. The regular HARQ delay consists of the processing times at the UE and eNB and propagation delays. By employing an early HARQ scheme, e.g. as also proposed in [2], the processing time can be reduced as shown in Figure 1. A further reduction of the processing time can be achieved by providing “Aggressive Early” HARQ feedback based on a portion of the CB.

Proposal 2: Study ‘Early HARQ’ and ‘Aggressive Early HARQ’ feedback mechanisms for HARQ RTT reduction in NR URLLC.

# 4	Conclusion

In this Tdoc, we discussed potential improvements on the HARQ processing timeline to enable NR URLLC and propose the following:

Proposal 1: Study semi-static feedback timeline for NR URLLC in Rel. 16.

Proposal 2: Study ‘Early HARQ’ and ‘Aggressive Early HARQ’ feedback mechanisms for HARQ RTT reduction in NR URLLC.

# References

[1] RP-181477, “SID on Physical Layer Enhancements for NR URLLC.”

[2] G. Berardinelli et al., “Enabling early HARQ feedback in 5G networks,” in IEEE Vehicular Technology Conference (VTC’16), May 2016.

[3] T. Fehrenbach et. Al., ‘‘URLLC Services in 5G, Low Latency Enhancements for LTE’’, accepted for publication in IEEE Vehicular Technology Conference (VTC’18), August 2018.