3GPP TSG RAN WG1 Meeting #94	R1-1808451

Gothenburg, Sweden, August 20 – 24, 2018

Agenda item:		7.2.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	IMT-2020 self-evaluation: UP latency analysis for static TDD with UE processing capability 2 (URLLC)

Document for:		Discussion and Decision

# Introduction

The user plane (UP) latency analysis for FDD and dynamic TDD is presented in the companion contributions [1] and [2] for UE processing capabilities 1 and 2, respectively. The UP latency analysis for static TDD is also presented in the companion contribution [3] for UE processing capability 1. In this contribution, we present the UP latency evaluation results for static TDD with UE processing capability 2.

ITU-R reports M.2410 [4] and M.2412 define the user plane latency requirement and evaluation guidelines in section 4.7.1 and 7.2.6 respectively, as follows:

### 4.7.1	User plane latency [5]

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the eMBB and URLLC usage scenarios.

The minimum requirements for user plane latency are:

–	4 ms for eMBB

–	1 ms for URLLC

assuming unloaded conditions (i.e. a single user) for small IP packets (e.g. 0 byte payload + IP header), for both downlink and uplink.

### 7.2.6	User plane latency calculation [5]

The proponent should provide the elements and their values in the calculation of the user plane latency, for both UL and DL. TABLE 3 provides an example of the elements in the calculation of the user plane latency.

TABLE 3

Example of user plane latency analysis template

| Step | Description | Value |
| --- | --- | --- |
| 1 | UE processing delay |  |
| 2 | Frame alignment |  |
| 3 | TTI for data packet transmission |  |
| 4 | HARQ retransmission |  |
| 5 | BS processing delay |  |
|  | Total one way user plane latency |  |

# Evaluation methodology

The evaluation method in this contribution is in accordance with the one used for the companion contributions in [1], [2] and [3]. We assume that the DL processing latency of the BS is the same as UL processing latency requirement of the UE and vice versa. The initial UE (UL) or BS (DL) processing delay and the final BS (UL) or UE (DL) processing delay are considered as N2/2 and N1/2, respectively. For DL, the UE processing time for PDSCH follows [6], where the processing time considers the UE capabilities, the mapping types of PDSCH packages, and the slot scheduling setup.

The static slot scheduling pattern is DDDXU, where D, X and U stand for downlink, flexible and uplink slots, respectively. The actual channel timing has been considered for frame alignment and control alignment for uplink feedback transmissions. The packet could arrive at any timing of any OFDM symbol with an assumption that the average time duration from packet arrival and the starting of the next OFDM symbol is a half of symbol duration. Hence, the initial UE/BS processing delay starts from the beginning of the OFDM symbols in a slot. Considering the 5 slots scheduling pattern DDDXU, there are 14x5 = 70 scenarios of channel timing calculation for each UE capability, slot scheduling and subcarrier spacing. The UP latency with 1 full retransmission for the 70 cases are shown in Appendix A, while channel timing calculations are illustrated in Appendix B to show the difference between the highest and lowest latency scenarios.

Frequency multiplexing between PDCCH and PDSCH is considered for DL. PDCCH and PUCCH could be transmitted at any symbol in a slot. For granted uplink transmission, UE should send the scheduling request and wait for the scheduling message from BS. PUSCH could then be transmitted at any symbol in a slot as long as the TTI still match with the remaining symbols in that slot. For grant-free uplink transmission, UE could transmit PUSCH without sending the scheduling request. The starting of PUSCH, however, should follow a periodicity of length TTI.

# UP latency – UE processing capability 2 (URLLC)

UE processing capability 2 could be considered as the UE capability for URLLC with the following processing requirement.

Table 1: PDSCH processing time for PDSCH processing capability 2 [6]

| Subcarrier spacing | PDSCH decoding time (front loaded DMRS) |  |
| --- | --- | --- |
|  | Symbols | ms |
| 15 kHz | 3 | 0.21 |
| 30 kHz | 4.5 | 0.16 |
| 60 kHz | 9 | 0.16 |

Table 2: PUSCH preparation time for PUSCH timing capability 2 [6]

| Subcarrier spacing | PUSCH preparation time |  |
| --- | --- | --- |
|  | Symbols | ms |
| 15 kHz | 5 | 0.36 |
| 30 kHz | 5.5 | 0.19 |
| 60 kHz | 11 | 0.19 |

The following tables show the one-way latency of no retransmission, 1 retransmission, and 10% HARQ retransmission in milliseconds for different slot scheduling and subcarrier spacings. For each subcarrier spacing, the scenarios that result in the highest and lowest 1 retransmission latency are reported together with the average latency. The scenarios are different at the frame and control alignments. As the highest and lowest latency schemes are selected based on 1 retransmission latency, they might or might not be the highest and lowest latency schemes for no retransmission and 10% HARQ retransmission latency.

## 3.1	One-way latency with full slot scheduling

### Downlink latency

Table 3: Downlink UP latency, full 14-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  | 30 kHz |  |  | 60 kHz |  |  |
|  |  | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low |
| 1 | Avg. symbol aglignment | 0,036 |  |  | 0,018 |  |  | 0,009 |  |  |
| 2 | BS processing delay | 0,179 |  |  | 0,098 |  |  | 0,098 |  |  |
| 3 | Frame alignment | 1,964 | 0,700 | 0,036 | 0,973 | 0,341 | 0,009 | 0,491 | 0,175 | 0,009 |
| 4 | TTI for data packet transmission | 1,000 |  |  | 0,500 |  |  | 0,250 |  |  |
| 5 | a) UE processing delay | 0,214 |  |  | 0,161 |  |  | 0,161 |  |  |
|  | b) Alignment to control opportunity | 1,786 | 0,871 | 0,000 | 0,839 | 0,411 | 0,018 | 0,339 | 0,154 | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) BS processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
|  | e) Frame alignment | 1,571 | 1,286 | 0,357 | 0,768 | 0,596 | 0,089 | 0,286 | 0,221 | 0,125 |
|  | f) TTI for data packet transmission | 1,000 |  |  | 0,500 |  |  | 0,250 |  |  |
| 6 | UE processing delay | 0,107 |  |  | 0,080 |  |  | 0,080 |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 3,286 | 2,021 | 1,357 | 1,670 | 1,038 | 0,705 | 0,929 | 0,613 | 0,446 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 8,286 | 5,821 | 3,357 | 4,170 | 2,938 | 1,705 | 2,179 | 1,613 | 1,196 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 3,786 | 2,401 | 1,557 | 1,920 | 1,228 | 0,805 | 1,054 | 0,713 | 0,521 |

### Granted uplink latency

Table 4: Granted uplink UP latency, full 14-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  | 30 kHz |  |  | 60 kHz |  |  |
|  |  | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  | 0,018 |  |  | 0,009 |  |  |
| 2 | a) UE processing delay | 0,179 |  |  | 0,098 |  |  | 0,098 |  |  |
|  | b) Alignment to SR | 3,036 | 0,957 | 0,036 | 1,509 | 0,470 | 0,009 | 0,759 | 0,239 | 0,009 |
|  | c) Scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) BS delay + frame alignment | 1,000 |  |  | 0,500 |  |  | 0,250 |  |  |
|  | e) Scheduling grant | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | f) UE processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
| 3 | Frame alignment | 3,500 | 3,114 | 1,571 | 1,732 | 1,539 | 0,768 | 0,768 | 0,671 | 0,286 |
| 4 | TTI for data packet transmission | 1,000 |  |  | 0,500 |  |  | 0,250 |  |  |
| 5 | a) BS processing delay | 0,214 |  |  | 0,161 |  |  | 0,161 |  |  |
|  | b) Alignment to control opportunity | 0,786 | 0,786 | 0,786 | 0,339 | 0,339 | 0,339 | 0,089 | 0,089 | 0,089 |
|  | c) Transmission of the scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) UE processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
|  | e) Frame alignment | 2,571 | 2,571 | 2,571 | 1,268 | 1,268 | 1,268 | 0,536 | 0,536 | 0,536 |
|  | f) TTI for data packet transmission | 1,000 |  |  | 0,500 |  |  | 0,250 |  |  |
| 6 | BS processing delay | 0,107 |  |  | 0,080 |  |  | 0,080 |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 9,357 | 6,893 | 4,429 | 4,705 | 3,473 | 2,241 | 2,446 | 1,830 | 1,214 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 14,357 | 11,893 | 9,429 | 7,205 | 5,973 | 4,741 | 3,696 | 3,080 | 2,464 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 9,857 | 7,393 | 4,929 | 4,955 | 3,723 | 2,491 | 2,571 | 1,955 | 1,339 |

### Grant-free uplink latency

Table 5: Uplink UP latency, grant-free, full 14-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  | 60 kHz |  |  |  |
|  |  | High | Avg. |  | Low | High |  | Avg. | Low |  | High | Avg. |  | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  |  | 0,018 |  |  |  |  | 0,009 |  |  |  |
| 2 | UE processing delay | 0,179 |  |  |  | 0,098 |  |  |  |  | 0,098 |  |  |  |
| 3 | Frame alignment | 3,964 | 1,700 |  | 0,036 | 1,973 |  | 0,841 | 0,009 |  | 0,991 | 0,425 |  | 0,009 |
| 4 | TTI for data packet transmission | 1,000 |  |  |  | 0,500 |  |  |  |  | 0,250 |  |  |  |
| 5 | a) BS processing delay | 0,214 |  |  |  |  |  |  |  | 0,161 |  |  | 0,161 |  |
|  | b) Alignment to control opportunity | 0,786 | 0,629 |  | 0,000 | 0,339 |  | 0,275 | 0,018 |  | 0,089 | 0,071 |  | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  |  | 0,036 |  |  |  |  | 0,018 |  |  |  |
|  | d) UE processing delay | 0,357 |  |  |  |  |  |  |  | 0,196 |  |  | 0,196 |  |
|  | e) Frame alignment | 2,571 | 2,529 |  | 2,357 | 1,268 |  | 1,232 | 1,089 |  | 0,536 | 0,504 |  | 0,375 |
|  | f) TTI for data packet transmission | 1,000 |  |  |  | 0,500 |  |  |  |  | 0,250 |  |  |  |
| 6 | BS processing delay | 0,107 |  |  |  | 0,080 |  |  |  |  | 0,080 |  |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 5,286 | 3,021 |  | 1,357 | 2,670 |  | 1,538 | 0,705 |  | 1,429 | 0,863 |  | 0,446 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 10,286 | 7,821 |  | 5,357 | 5,170 |  | 3,938 | 2,705 |  | 2,679 | 2,063 |  | 1,446 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 5,786 | 3,501 |  | 1,757 | 2,920 |  | 1,778 | 0,905 |  | 1,554 | 0,983 |  | 0,546 |

## 3.2	One-way latency with shortened data allocation

### Downlink latency

Table 6: Downlink UP latency, 7-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  | 30 kHz |  |  | 60 kHz |  |  |
|  |  | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  | 0,018 |  |  | 0,009 |  |  |
| 2 | BS processing delay | 0,179 |  |  | 0,098 |  |  | 0,098 |  |  |
| 3 | Frame alignment | 1,464 | 0,314 | 0,036 | 0,723 | 0,148 | 0,009 | 0,366 | 0,079 | 0,009 |
| 4 | TTI for data packet transmission | 0,500 |  |  | 0,250 |  |  | 0,125 |  |  |
| 5 | a) UE processing delay | 0,286 |  |  | 0,196 |  |  | 0,179 |  |  |
|  | b) Alignment to control opportunity | 2,214 | 1,096 | 0,000 | 1,054 | 0,514 | 0,018 | 0,446 | 0,204 | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) BS processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
|  | e) Frame alignment | 0,000 | 0,187 | 0,000 | 0,018 | 0,114 | 0,018 | 0,286 | 0,212 | 0,000 |
|  | f) TTI for data packet transmission | 0,500 |  |  | 0,250 |  |  | 0,125 |  |  |
| 6 | UE processing delay | 0,143 |  |  | 0,098 |  |  | 0,089 |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 2,321 | 1,171 | 0,893 | 1,188 | 0,613 | 0,473 | 0,688 | 0,400 | 0,330 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 5,750 | 3,668 | 2,107 | 2,938 | 1,920 | 1,188 | 1,938 | 1,334 | 0,848 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 2,664 | 1,421 | 1,014 | 1,363 | 0,743 | 0,545 | 0,813 | 0,493 | 0,382 |

Table 7: Downlink UP latency, 4-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  | 30 kHz |  |  | 60 kHz |  |  |
|  |  | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  | 0,018 |  |  | 0,009 |  |  |
| 2 | BS processing delay | 0,179 |  |  | 0,098 |  |  | 0,098 |  |  |
| 3 | Frame alignment | 1,250 | 0,210 | 0,036 | 0,616 | 0,096 | 0,009 | 0,313 | 0,053 | 0,009 |
| 4 | TTI for data packet transmission | 0,286 |  |  | 0,143 |  |  | 0,071 |  |  |
| 5 | a) UE processing delay | 0,286 |  |  | 0,196 |  |  | 0,179 |  |  |
|  | b) Alignment to control opportunity | 2,429 | 1,185 | 0,000 | 1,161 | 0,559 | 0,018 | 0,500 | 0,222 | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) BS processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
|  | e) Frame alignment | 0,000 | 0,144 | 0,000 | 0,018 | 0,093 | 0,018 | 0,286 | 0,217 | 0,000 |
|  | f) TTI for data packet transmission | 0,286 |  |  | 0,143 |  |  | 0,071 |  |  |
| 6 | UE processing delay | 0,143 |  |  | 0,098 |  |  | 0,089 |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 1,893 | 0,853 | 0,679 | 0,973 | 0,453 | 0,366 | 0,580 | 0,320 | 0,277 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 5,321 | 3,182 | 1,679 | 2,723 | 1,676 | 0,973 | 1,830 | 1,223 | 0,741 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 2,236 | 1,086 | 0,779 | 1,148 | 0,576 | 0,427 | 0,705 | 0,411 | 0,323 |

Table 8: Downlink UP latency, 2-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  | 30 kHz |  |  | 60 kHz |  |  |
|  |  | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  | 0,018 |  |  | 0,009 |  |  |
| 2 | BS processing delay | 0,179 |  |  | 0,098 |  |  | 0,098 |  |  |
| 3 | Frame alignment | 1,107 | 0,161 | 0,036 | 0,545 | 0,072 | 0,009 | 0,277 | 0,040 | 0,009 |
| 4 | TTI for data packet transmission | 0,143 |  |  | 0,071 |  |  | 0,036 |  |  |
| 5 | a) UE processing delay | 0,286 |  |  | 0,196 |  |  | 0,179 |  |  |
|  | b) Alignment to control opportunity | 2,571 | 1,229 | 0,000 | 1,232 | 0,581 | 0,018 | 0,536 | 0,233 | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) BS processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
|  | e) Frame alignment | 0,000 | 0,116 | 0,000 | 0,018 | 0,079 | 0,018 | 0,000 | 0,030 | 0,000 |
|  | f) TTI for data packet transmission | 0,143 |  |  | 0,071 |  |  | 0,036 |  |  |
| 6 | UE processing delay | 0,143 |  |  | 0,098 |  |  | 0,089 |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 1,607 | 0,661 | 0,536 | 0,830 | 0,357 | 0,295 | 0,509 | 0,272 | 0,241 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 5,036 | 2,863 | 1,393 | 2,580 | 1,517 | 0,830 | 1,473 | 0,964 | 0,670 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 1,950 | 0,881 | 0,621 | 1,005 | 0,473 | 0,348 | 0,605 | 0,342 | 0,284 |

### Granted uplink latency

Table 9: Granted uplink UP latency, 7-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  | 30 kHz |  |  | 60 kHz |  |  |
|  |  | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  | 0,018 |  |  | 0,009 |  |  |
| 2 | a) UE processing delay | 0,179 |  |  | 0,098 |  |  | 0,098 |  |  |
|  | b) Alignment to SR | 3,036 | 0,957 | 0,036 | 1,509 | 0,470 | 0,009 | 0,009 | 0,239 | 0,009 |
|  | c) Scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) BS delay + frame alignment | 0,500 |  |  | 0,250 |  |  | 0,125 |  |  |
|  | e) Scheduling grant | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | f) UE processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
| 3 | Frame alignment | 0,000 | 0,786 | 2,071 | 0,018 | 0,425 | 1,018 | 0,857 | 0,235 | 0,000 |
| 4 | TTI for data packet transmission | 0,500 |  |  | 0,250 |  |  | 0,125 |  |  |
| 5 | a) BS processing delay | 0,214 |  |  | 0,161 |  |  | 0,161 |  |  |
|  | b) Alignment to control opportunity | 0,286 | 0,182 | 0,000 | 0,054 | 0,040 | 0,018 | 0,214 | 0,080 | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) UE processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
|  | e) Frame alignment | 2,571 | 1,831 | 0,000 | 1,268 | 0,885 | 0,018 | 0,536 | 0,446 | 0,375 |
|  | f) TTI for data packet transmission | 0,500 |  |  | 0,250 |  |  | 0,125 |  |  |
| 6 | BS processing delay | 0,107 |  |  | 0,080 |  |  | 0,080 |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 4,857 | 3,564 | 3,929 | 2,491 | 1,859 | 1,991 | 1,536 | 1,144 | 0,679 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 8,857 | 6,719 | 5,071 | 4,455 | 3,427 | 2,670 | 2,786 | 2,170 | 1,554 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 5,257 | 3,880 | 4,043 | 2,688 | 2,016 | 2,059 | 1,661 | 1,247 | 0,766 |

Table 10: Granted uplink UP latency, 2-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  | 30 kHz |  |  | 60 kHz |  |  |
|  |  | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  | 0,018 |  |  | 0,009 |  |  |
| 2 | a) UE processing delay | 0,179 |  |  | 0,098 |  |  | 0,098 |  |  |
|  | b) Alignment to SR | 3,036 | 0,957 | 0,036 | 1,509 | 0,470 | 0,009 | 0,759 | 0,239 | 0,009 |
|  | c) Scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) BS delay + frame alignment | 0,143 |  |  | 0,071 |  |  | 0,036 |  |  |
|  | e) Scheduling grant | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | f) UE processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
| 3 | Frame alignment | 0,000 | 0,394 | 0,000 | 0,018 | 0,232 | 0,018 | 0,000 | 0,145 | 0,000 |
| 4 | TTI for data packet transmission | 0,143 |  |  | 0,071 |  |  | 0,036 |  |  |
| 5 | a) BS processing delay | 0,214 |  |  | 0,161 |  |  | 0,161 |  |  |
|  | b) Alignment to control opportunity | 1,000 | 0,706 | 0,000 | 0,411 | 0,287 | 0,018 | 0,036 | 0,022 | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  | 0,036 |  |  | 0,018 |  |  |
|  | d) UE processing delay | 0,357 |  |  | 0,196 |  |  | 0,196 |  |  |
|  | e) Frame alignment | 2,571 | 2,198 | 2,357 | 1,268 | 1,064 | 1,089 | 0,536 | 0,402 | 0,375 |
|  | f) TTI for data packet transmission | 0,143 |  |  | 0,071 |  |  | 0,036 |  |  |
| 6 | BS processing delay | 0,107 |  |  | 0,080 |  |  | 0,080 |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 4,143 | 2,458 | 1,143 | 2,134 | 1,308 | 0,634 | 1,250 | 0,875 | 0,500 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 8,500 | 6,148 | 4,286 | 4,277 | 3,123 | 2,205 | 2,232 | 1,710 | 1,286 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 4,579 | 2,827 | 1,457 | 2,348 | 1,490 | 0,791 | 1,348 | 0,959 | 0,579 |

### Grant-free uplink latency

Table 11: Grant-free uplink UP latency, 7-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  | 60 kHz |  |  |  |
|  |  | High | Avg. |  | Low | High |  | Avg. | Low |  | High | Avg. |  | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  |  | 0,018 |  |  |  |  | 0,009 |  |  |  |
| 2 | UE processing delay | 0,179 |  |  |  | 0,098 |  |  |  |  | 0,098 |  |  |  |
| 3 | Frame alignment | 0,464 | 1,300 |  | 0,036 | 0,223 |  | 0,641 | 0,009 |  | 0,866 | 0,325 |  | 0,009 |
| 4 | TTI for data packet transmission | 0,500 |  |  |  | 0,250 |  |  |  |  | 0,125 |  |  |  |
| 5 | a) BS processing delay | 0,214 |  |  |  |  |  |  |  | 0,161 |  |  | 0,161 |  |
|  | b) Alignment to control opportunity | 0,786 | 0,107 |  | 0,000 | 0,339 |  | 0,057 | 0,018 |  | 0,214 | 0,159 |  | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  |  | 0,036 |  |  |  |  | 0,018 |  |  |  |
|  | d) UE processing delay | 0,357 |  |  |  |  |  |  |  | 0,196 |  |  | 0,196 |  |
|  | e) Frame alignment | 2,571 | 1,000 |  | 0,357 | 1,268 |  | 0,425 | 0,089 |  | 0,536 | 0,516 |  | 0,375 |
|  | f) TTI for data packet transmission | 0,500 |  |  |  | 0,250 |  |  |  |  | 0,125 |  |  |  |
| 6 | BS processing delay | 0,107 |  |  |  | 0,080 |  |  |  |  | 0,080 |  |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 1,286 | 2,121 |  | 0,857 | 0,670 |  | 1,088 | 0,455 |  | 1,179 | 0,638 |  | 0,321 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 5,786 | 4,371 |  | 2,357 | 2,920 |  | 2,213 | 1,205 |  | 2,429 | 1,813 |  | 1,196 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 1,736 | 2,346 |  | 1,007 | 0,895 |  | 1,200 | 0,530 |  | 1,304 | 0,755 |  | 0,409 |

Table 12: Grant-free uplink UP latency, 2-symbol slot scheduling

| Step | Description | Value [ms] |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  |  |  | 30 kHz |  |  |  |  | 60 kHz |  |  |  |
|  |  | High | Avg. |  | Low | High |  | Avg. | Low |  | High | Avg. |  | Low |
| 1 | Avg. symbol alignment | 0,036 |  |  |  | 0,018 |  |  |  |  | 0,009 |  |  |  |
| 2 | UE processing delay | 0,179 |  |  |  | 0,098 |  |  |  |  | 0,098 |  |  |  |
| 3 | Frame alignment | 0,107 | 1,014 |  | 0,036 | 0,045 |  | 0,498 | 0,009 |  | 0,027 | 0,254 |  | 0,009 |
| 4 | TTI for data packet transmission | 0,143 |  |  |  | 0,071 |  |  |  |  | 0,036 |  |  |  |
| 5 | a) BS processing delay | 0,214 |  |  |  |  |  |  |  | 0,161 |  |  | 0,161 |  |
|  | b) Alignment to control opportunity | 0,929 | 0,100 |  | 0,000 | 0,482 |  | 0,068 | 0,018 |  | 0,232 | 0,025 |  | 0,000 |
|  | c) Transmission of the scheduling request | 0,071 |  |  |  | 0,036 |  |  |  |  | 0,018 |  |  |  |
|  | d) UE processing delay | 0,357 |  |  |  |  |  |  |  | 0,196 |  |  | 0,196 |  |
|  | e) Frame alignment | 2,571 | 0,706 |  | 0,071 | 1,268 |  | 0,366 | 0,018 |  | 0,536 | 0,183 |  | 0,018 |
|  | f) TTI for data packet transmission | 0,143 |  |  |  | 0,071 |  |  |  |  | 0,036 |  |  |  |
| 6 | BS processing delay | 0,107 |  |  |  | 0,080 |  |  |  |  | 0,080 |  |  |  |
| Total one way user plane latency without retransmission (1+2+3+4+6) |  | 0,571 | 1,479 |  | 0,500 | 0,313 |  | 0,766 | 0,277 |  | 0,250 | 0,477 |  | 0,232 |
| Total one way user plane latency with 1 retransmission (1+2+3+4+5+6) |  | 4,857 | 3,070 |  | 1,357 | 2,527 |  | 1,664 | 0,777 |  | 1,429 | 1,095 |  | 0,661 |
| Total one way user plane latency with 10% HARQ retransmission (1+2+3+4+ 0.1x5+6) |  | 1,000 | 1,638 |  | 0,586 | 0,534 |  | 0,856 | 0,327 |  | 0,368 | 0,539 |  | 0,275 |

# 4	Conclusions

This contribution considered a 5-slot long static TDD configuration of DDDXU, which naturally severely limits the worst case latency.

The analysis in this contribution indicates that the 3GPP specifications can meet the URLLC latency requirement of 1 ms for the analysed sattic TDD configuration, but not for the worst case packet with 1 HARQ retransmission. For downlink and grant-free uplink, the requirement can be met on average with subcarrier spacings 30 and 60 kHz using even full 14-symbol slot scheduling and taking 10% HARQ retransmission into account. With shorter data allocation durations, the 1 ms latency requirement can be met on average for subcarrier spacings 30 and 60 kHz and taking 10% HARQ retransmission into account.

# Appendix A

For the sake of brevity, only 2-symbols and 7-symbols slot scheduling schemes are presented for downlink and uplink grant-free schemes. Let us denote by n ∈{0, 1, 2, …, 69} index of the first symbol after packet arrival in a static TDD pattern. Thus, n=symb#+14(slot#-1), where slot# ∈{1,2,3,4,5} is the corresponding slot number of the scheduling pattern DDDXU, symb# ∈ 0,1,2,…, 13 is the symbol number in a slot.

a) 2-symbol slot scheduling									b) 7-symbol slot scheduling

Figure 1. Downlink UP latency vs 70 possible cases.

a) 2-symbol slot scheduling									b) 7-symbol slot scheduling

Figure 2. Grant-free UP latency vs 70 possible cases.

# Appendix B

This appendix illustrates the channel timing scenarios for the highest and lowest latency cases. 2-symbols slot scheduling with SCS = 30 kHz is considered for illustration.

Figure 3. Channel timing scenario for DL, 2-symbols slot scheduling, SCS = 30 kHz, BS delay starts at n=52.

Figure 4. Channel timing scenario for DL, 2-symbols slot scheduling, SCS = 30 kHz, BS delay starts at n=36.

Figure 5. Channel timing scenario for UL grant-free, 2-symbols slot scheduling, SCS = 30 kHz, UE delay starts at n=46.

Figure 6. Channel timing scenario for UL grant-free, 2-symbols slot scheduling, SCS = 30 kHz, UE delay starts at n=45.

# References

- R1-1808448, “IMT-2020 self-evaluation: UP latency analysis for FDD and dynamic TDD with UE processing capability 1 (eMBB)”, Nokia, Nokia Shanghai Bell.

- R1-1808449, “IMT-2020 self-evaluation: UP latency analysis for FDD and dynamic TDD with UE processing capability 2 (URLLC)”, Nokia, Nokia Shanghai Bell.

- R1-1808450, “IMT-2020 self-evaluation: UP latency analysis for static TDD with UE processing capability 1 (eMBB)”, Nokia, Nokia Shanghai Bell.

- Report M.2410,” Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, 
ITU-R, 11/2017

- Report M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020), ITU-R, 10/2017

- 3GPP TS 38.214, “NR; Physical layer procedures for data”, V15.1.0