3GPP TSG RAN WG1 Meeting #94	R1-1808452

Gothenburg, Sweden, August 20 – 24, 2018

Agenda item:		7.2.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	IMT-2020 Self Evaluation: Control plane latency

Document for:		Discussion and Decision

# 1	Introduction

ITU-R Report M.2410 [1] defines the control plane latency requirements as follows.

We follow the evaluation guidelines provided in [2] and discuss the control plane latency of the NR TDD and FDD systems to see that IMT-2020 requirements can be satisfied with NR.

# 2	Calculations of NR control plane latency

In NR, the transition time required to change from RRC INACTIVE mode to RRC CONNECTED mode can be considered as the control plane latency. Compared to previous releases, NR Release 15 has certain improvements in RACH procedure, support higher sub-carrier spacing and fast turn-around with different slot structures, lower latencies in the coding schemes, and many other.  Figure 1 shows the transition from INACTIVE to CONNECTED mode together with the latency contributions from the steps associated with the transition procedure.

gNB

UE

- RACH preamble

- Delay for RACH Scheduling Period

- Processing delay in gNB

- RA response

- Processing delay in UE

- RRC connection resume request

- Processing delay in gNB

- RRC connection resume

- RRC connection resume complete

- Processing delay in UE

Figure 1: C-plane activation procedure

## 2.1	C-plane latency for TDD scenario

Table 2 provides C-plane timing analysis of the flow depicted in Figure 1. The assumptions are given in the notes of the table.

Table 1: C-plane latency analysis for TDD

| Component | Description | CP Latency for UL data transfer |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period | 0 |
| 2 | Transmission of RACH Preamble (format B3) | 6 symbols |
| 3 | Preamble detection and processing in gNB (the value is function of SCS) | f(SCS) |
| 4 | Transmission of RA response | 1 TTI |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) (the value is function of SCS) | g(SCS) |
| 6 | Transmission of RRC Connection Resume Request | 1 TTI |
| 7 | Processing delay in eNB (L2 and RRC) | 3 ms |
| 8 | Transmission of RRC Connection Resume | 1 TTI |
| 9 | Processing delay in the UE of RRC Connection Resume including grant reception | 3 ms |
| 10 | Transmission of RRC Connection Resume complete and UP data | 0 |
|  | Total delay [ms] |  |
| Notes - Step 2, the preamble format B3 is selected for the analysis as it is mainly fitting the normal coverage scenarios. More details on preamble formats are given in Annex 1. - Step 3, the duration for preamble detection and processing in gNB depends on the SCS. The values are, fscs= 4 slots scs=15 kHz7 slots scs=30 kHz8 slots scs=60 kHz12 slots scs=120 kHz - Step 5, UE processing delay depends on the SCS, and given by the formula NT,1+NT,2+0.5 ms. Details are given in Annex II. gscs= 2.14 ms scs=15 kHz1.39 ms scs=30 kHz1.29 ms scs=60 kHz1.03 ms scs=120 kHz - UL/DL slot alignment is not captured in the table as it depends on the SCS and TTI duration. |  |  |

Latencies are next calculated for two cases of TDD configurations by also considering SCS equal to 15 kHz, 30 kHz, 60 kHz, and 120 kHz.

- Case 1: 50/50 traffic scenario in DL and UL. (DL-UL)

- Case 2: 80/20 traffic scenario, DL heavy. (DL-DL-DL-DL-UL)

We also consider two cases of TTI durations, 7 and 14 symbols, in the latency analysis in Table 2.

We consider that any UL slots can be used for PRACH preamble transmission and start from there. When the UE or gNB processing is completed in the middle of a TTI duration, we round-up the UL/DL transmissions to next UL/DL slot if the number of symbols is not sufficient to schedule a transmission.

Table 2: C-plane latency calculation for TDD

| SCS | Case 1 (DL-UL) |  | Case 2 (DL-DL-DL-DL-UL) |  |
| --- | --- | --- | --- | --- |
|  | TTI = 7 | TTI = 14 | TTI = 7 | TTI = 14 |
| 15 kHz | 24 TTI = 12 ms | 15 TTI = 15 ms | 29 TTI = 14.5 ms | 18 TTI = 18 ms |
| 30 kHz | 42 TTI = 10.5 ms | 24 TTI = 12 ms | 41 TTI = 10.25 ms | 23 TTI = 11.5 ms |
| 60 kHz | 72 TTI = 9 ms | 38 TTI = 9.5 ms | 76 TTI = 9.5 ms | 41 TTI = 10.25 ms |
| 120 kHz | 130 TTI = 8.125 ms | 68 TTI = 8.5 ms | 133 TTI = 8.3125 ms | 71 TTI = 8.875 ms |

Observation 1: NR TDD scenario can meet the IMT-2020 C-plane latency requirements.

## 2.2	C-plane latency for FDD scenario

Here, we provide the latency analysis for NR FDD scenario with the same processing time assumptions used in the NR TDD scenario.

Table 3: C-plane latency analysis for FDD

| Component | Description | CP Latency for UL data transfer |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period | 0 |
| 2 | Transmission of RACH Preamble (format B3) | 6 symbols |
| 3 | Preamble detection and processing in gNB (the value is function of SCS) | f(SCS) |
| 4 | Transmission of RA response | 1 TTI |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) (the value is function of SCS) | g(SCS) |
| 6 | Transmission of RRC Connection Resume Request | 1 TTI |
| 7 | Processing delay in eNB (L2 and RRC) | 3 ms |
| 8 | Transmission of RRC Connection Resume | 1 TTI |
| 9 | Processing delay in the UE of RRC Connection Resume including grant reception | 3 ms |
| 10 | Transmission of RRC Connection Resume complete and UP data | 0 |
|  | Total delay [ms] |  |

Latencies are next calculated considering slot durations of SCS equal to 15 kHz, 30 kHz, 60 kHz, and 120 kHz. There is no timing alignment for UL/DL slots, and we calculate the latencies as in Table 4.

Table 4: C-plane latency calculation for FDD

| SCS | Latency [ms] |  |
| --- | --- | --- |
|  | TTI = 7 | TTI = 14 |
| 15 kHz | 12.0686 | 13.5686 |
| 30 kHz | 10.1043 | 10.8543 |
| 60 kHz | 8.7721 | 9.1471 |
| 120 kHz | 8.0211 | 8.2086 |

Observation 2: NR FDD scenario can meet the IMT-2020 CP latency requirements.

# 3	Conclusions

In this contribution, we provide control plane latency calculations for NR release 15, where we have following observations.

Observation 1: NR TDD scenario can meet the IMT-2020 CP latency requirements.

Observation 2: NR FDD scenario can meet the IMT-2020 CP latency requirements.

# References

- Report M.2410,” Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, 
ITU-R, 11/2017

- Report M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020), ITU-R, 10/2017

# Annex I:

Time durations for different preamble formats are given as follows.

Table 5: Time duration for PRACH preamble formats for  and .

| Format |  |  |  |  | NGPRA | Use Case | Time Duration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz |  |  | 2975 k | LTE refarming | 1 ms |
| 1 | 839 | 1.25 kHz |  |  | 21904 k | Large cell, 100 km | 3 ms |
| 2 | 839 | 1.25 kHz |  |  | 4528 k | Coverage enhancement | 3.5 ms |
| 3 | 839 | 5 kHz |  |  | 2976 k | High speed case | 1 ms |

Table 6: Time duration for Preamble formats for  and  where .

| Format |  |  |  |  | NGPRA | Use Case | Time duration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 139 |  |  |  | 0 | Small cell | 2 symbols |
| A2 | 139 |  |  |  | 0 | Normal cell | 4 symbols |
| A3 | 139 |  |  |  | 0 | Normal cell | 6 symbols |
| B1 | 139 |  |  |  | 72 k∙2-μ | Small cell | 2 symbols |
| B2 | 139 |  |  |  | 216 k∙2-μ | Normal cell | 4 symbols |
| B3 | 139 |  |  |  | 360 k∙2-μ | Normal cell | 6 symbols |
| B4 | 139 |  |  |  | 792 k∙2-μ | Normal cell | 12 symbols |
| C0 | 139 |  |  |  | 1096 k∙2-μ | Normal cell | 2 symbols |
| C2 | 139 |  |  |  | 2916 k∙2-μ | Normal cell | 6 symbols |

# Annex II:

The parameters to determine the UE processing delay is given by the Tables 7 and 8.

Table 7: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  | Time duration NT,1 for “Additional PDSCH DM-RS configured” (NOTE: the length of one OFDM symbol is assumed to be the average value over one slot, i.e., LOS=slot length / 14) |
| --- | --- | --- | --- |
|  | No additional PDSCH DM-RS configured | Additional PDSCH DM-RS configured |  |
| 0 | 8 | 13 | 0.93ms |
| 1 | 10 | 13 | 0.46ms |
| 2 | 17 | 20 | 0.38ms |
| 3 | 20 | 24 | 0.21ms |

Table 8: PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] | Time duration NT,2 (NOTE: the length of one OFDM symbol is assumed to be the average value over one slot, i.e., LOS=slot length / 14) |
| --- | --- | --- |
| 0 | 10 | 0.71ms |
| 1 | 12 | 0.43ms |
| 2 | 23 | 0.41ms |
| 3 | 36 | 0.32ms |