3GPP TSG RAN WG1 Meeting #94 	R1-1808595

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR |  | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the partial PUSCH mode field for FeLAA in 36.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_unlic-Core |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current version of 36.212 specifies that the NDI is 1 bit for TM1 and 2 bits for TM2, which means the eNB has to perform blind decoding on AUL-UCI with two payload sizes when TM1 UE and TM2 UE are allocated with the same DMRS sequence. Details can refer to R1-1808585. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the AUL-UCI payload to align the NDI field bitwidth for both TM1 and TM2 to be 2 bits. For TM1, only the first bit is valid, and the second bit is reserved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eNB needs to perform blind decoding for AUL-UCI with two different payload sizes, leading to more complexity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.6A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

## 5.2.2.6A	Channel coding of AUL-UCI

< Unchanged parts are omitted >

Table 5.2.2.6A-1 shows the fields and the corresponding bit widths for the AUL-UCI that is transmitted on the AUL PUSCH configured for 1 transport block or 2 transport blocks.

Table 5.2.2.6A-1: Fields for AUL-UCI

| Field | Bit width |
| --- | --- |
| AUL C-RNTI | 16 |
| HARQ process number | 4 |
| Redundancy version | 2 |
| New data indicator | 2 |
| PUSCH starting symbol | 1 |
| PUSCH ending symbol | 1 |
| Channel Occupancy Time (COT) sharing indication | 1 |
| NOTE: For New data indicator field, the MSB bit corresponds to the first transport block, and the LSB bit corresponds to the second transport block if any. |  |

< Unchanged parts are omitted >