3GPP TSG RAN WG1 Meeting #94		                                                                R1-1808677

Gothenburg, Sweden, August 20th – 24th, 2018

Source: 	Intel Corporation

Title:	NOMA transmission scheme

Agenda item:		7.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#93, potential NOMA transmission schemes were listed as shown below.

- Detailed transmission schemes particularly MA signature design per scheme will be captured in TR. Performance and complexity comparisons and observation/conclusion should at least be made scheme-wise. - Transmitter side data processing for NOMA can be based on one or more of the following aspects - UE -specific bit-level scrambling - UE -specific bit-level interleaving - UE -specific symbol-level spreading - Can be with NR legacy modulation or modified modulation - UE -specific symbol-level scrambling - UE -specific symbol-level interleaving, with symbol-level zero padding - UE -specific power assignment - UE-specific sparse RE mapping - Cell-specific MA signature - Multi-branch/MA signature transmission (irrespective of rank) per UE

In this contribution, we discuss some details of the data pressing aspects listed in RAN1#93 and provide the details of Low Code Rate Spreading (LCRS) which is utilizing UE-specific bit-level scrambling.

# Transmitter side data processing

As shown in section 1, we have listed up potential transmitter side data processing schemes for NOMA in RAN1 #93. This section discusses the detailed aspects of each scheme and provide pros and cons for accurate comparison between different NOMA transmission schemes.

| Tx side processing | Genera properties |  |
| --- | --- | --- |
|  | Pros | Cons |
| UE-specific bit-level scrambling | Bit level scrambling is randomizing the interference from other users, which enables UE differentiation in the receiver side |  |
|  | - Negligible spec impact for supporting NOMA - sufficient number of MA signatures can be supported (almost infinite number can be supported) | N.A. |
| UE-specific bit-level interleaving | Bit level scrambling is randomizing the interference from other users, which enables UE differentiation in the receiver side |  |
|  | - Large number of MA signatures can be supported | - Additional spec impact is required for defining interleaver |
| UE-specific symbol-level spreading | Symbol-level spreading is randomizing the interference by well-defined spreading sequences |  |
|  | - It has been observed from some contributions that well-defined spreading sequence provides good correlation properties which may enable the receiver to differentiate signals from different UEs. | - Additional spec impact - Number of MA signatures are limited (depends on which sequences are used) - Full coding gain may not be obtained for high spectral efficiency transmission scenario since |
| UE-specific symbol-level scrambling | Not much difference from UE-specific bit-level scrambling from the performance perspective |  |
|  | - Potential PAPR decrease | - Additional spec impact is required compared to UE-specific bit-level scrambling - Bigger memory size is required compared to bit-level scrambling |
| UE-specific symbol-level interleaving, with symbol-level zero padding | Not much difference from UE-specific symbol-level interleaving from the performance perspective |  |
|  | N.A. | - Bigger memory size is required compared to bit-level interleaving |
| UE-specific power assignment | Different power is used for different users in order to facilitate interference cancellation in the receiver side |  |
|  | - IC performance can be improved in the receiver side | - Additional configuration is required for differentiating Tx power between users |
| UE-specific sparse RE mapping | Sparse mapping can be done by ether RE mapping or designing spreading sequences. It is preferable to have sparsity (if needed) by defining the sparse spreading sequence not by modifying sparse RE mapping from the spec impact perspective |  |
|  | - Sparse spreading can suppress the inter-user interference | - Significant spec impact is required |
| Cell-specific MA signature | Cell-specific MA signature is not for differentiating UEs sharing the same resource but potentially for other purpose, e.g., PAPR reduction. No big motivation is observed for cell-specific MA signature. |  |
|  | N.A. | N.A. |
| Multi-branch /MA signature transmission per UE | Multi-layer can be used for increasing per-UE spectral efficiency while keeping sufficient code rate as well as enough spreading factor |  |
|  | - High per-UE SE can be supported | - overloading factor is decreased: less number of users can be supported inside the same NOMA resource - significant spec impact is required |

Observation 1

- NOMA can be supported by applying UE-specific bit-level scrambling with negligible spec impact

- Unless sufficient gain is observed by applying other Tx side processing schemes over UE-specific bit-level scrambling, we should not introduce any additional schemes for the Tx side processing

# Low Code Rate Spreading (LCRS)

## Basic Transmission processing

It is well known that simultaneous transmissions from two users on a common resource along with the use of the SIC receiver is a channel capacity achieving transmission strategy [2]. Since non-orthogonal multi-user multiplexing can accommodate more number of UEs simultaneously, it potentially leads to user and system throughput gains. Furthermore, non-orthogonal multi-user multiplexing schemes efficiently allow grant-free operation, which may be beneficial for supporting large number of UEs requesting intermittent transmissions of small data packets. On the other hand, the receiver complexity for a certain non-orthogonal multi-user multiplexing scheme may increase significantly with the number of simultaneously transmitting UEs, and error propagation in the SIC receiver may limit the throughput gains.

Some non-orthogonal multi-user multiplexing schemes exploit power- and/or code-domain multiplexing. In addition, time- and/or frequency-domain spreading is essential to make effective SINRs of de-spread symbols high enough for successful demodulation and decoding. Thus, the simple spreading schemes for synchronous non-orthogonal multi-user multiplexing may be desirable, which can maintain the common waveform (e.g. OFDM) irrespective of multiple access schemes and can still take advantage of grant-free non-orthogonal transmission for a certain scenario such as small data transmission.

For having the spreading effects in order to support non-orthogonal multi-user multiplexing, there can be multiple ways. One simple alternative is to utilize actual spreading sequence for signal spreading on top of the encoded bits or modulated symbols and the other alternative is to reduce the code rate by utilizing rate matching block so as to effectively spread the encoded bits. Both schemes can be potential candidate methods for NOMA Tx scheme but we believe the second alternative, Low Code Rate Spreading (LCRS), is more promising from the following perspectives:

- Support of flexible code rate,

- Support of (comparatively) full coding gain

- Support of NOMA without introducing new transmission schemes,

The illustration of the LCRS transmission processing is given in Figure 1.

Figure 1. Transmission process of LRCS

LCRS transmission scheme is basically the same as the current NR UL transmission. As shown in Figure 1, it utilizes same coding procedure as PUSCH including channel encoding, rate matching, bit-level scrambling and then modulator. As can be seen in the Figure, either OFDMA or SC-FDMA could be used in the last part of the transmission processing. The difference from the normal PUSCH is that the physical resources mapped to the generated signals can be shared by multiple users. The bit-level scrambling part performs as the user separation by randomizing the signals from other users and then making multi-user interference as random as possible. Bit level scrambling function defined in 38.211 [3] can be one example as it is defined in a UE-specific manner.  The only thing to be taken into account is to make sure the code rate is sufficiently low for achieving the maximum coding gain in order to overcome the inter-user interference.

Observation 2

- Low Code Rate Spreading (LCRS) is a promising candidate for NOMA from the perspectives of

  - Support of flexible code rate,

  - Support of (comparatively) full coding gain

  - Support of NOMA without introducing new transmission schemes,

## Potential specification impacts

In the previous section, it was mentioned that LCRS is just reusing the currently defined NR PUSCH coding chain. Therefore, if we just consider the specification impact from the transmission signal processing schemes, there would be no additional block or function to be introduced or defined at least in 38.212 [4].

However, depending on the maximum possible number of users to be multiplexed in the same resources, DMRS design may need to be improved. The performance of NOMA scheme depends substantially on the advanced receiver structure and the performance of the advanced receiver depends a lot on whether the channel estimation is accurate or not. Therefore, DMRS has to support multiplexing of multiple users and multiple DMRS’s with sufficient separation from each other for the desirable channel estimation performance even though the data parts are not separated.

Current NR DMRS can support only up to 12 users. (Up to 12 orthogonal ports are supported) If there is a need to increase the number of simultaneous users, certain improvement is required. One note is that this DMRS design improvement is needed regardless of what kind of NOMA transmission scheme is introduced.

Observation 3

- No (or negligible) specification impact is needed for the LCRS transmission scheme

- Possible improvement could be needed for PUSCH DMRS design regardless of NOMA transmission schemes

# Conclusions

In this contribution, we introduced our proposed NOMA transmission scheme, LCRS with some initial evaluation results. Based on the discussions presented, we summarize our views through the following observations. Based on observations, we also propose to consider LCRS as a NOMA transmission scheme.

Observation 1

- NOMA can be supported by applying UE-specific bit-level scrambling with negligible spec impact

- Unless sufficient gain is observed by applying other Tx side processing schemes over UE-specific bit-level scrambling, we should not introduce any additional schemes for the Tx side processing

Observation 2

- Low Code Rate Spreading (LCRS) is a promising candidate for NOMA from the perspectives of

  - Support of flexible code rate,

  - Support of (comparatively) full coding gain

  - Support of NOMA without introducing new transmission schemes,

Observation 3

- No (or negligible) specification impact is needed for the LCRS transmission scheme

- Possible improvement could be needed for PUSCH DMRS design regardless of NOMA transmission schemes

Proposal 1

- LCRS is the starting point for the NOMA transmission scheme

- Other NOMA transmission scheme can be considered only when noticeable gain is observed over LCRS

# References

- RP-171043, “Revision of Study on 5G Non-orthogonal Multiple Access”, ZTE, West Palm Beach, USA, June, 2017

- T. Cover and J. Thomas, “Elements of Information Theory,” Wiley, 2006.

- 3GPP TS 38.211 V15.0.0, “Physical channels and modulation (Release 15)”

- 3GPP TS 38.212 V15.0.0, “Multiplexing and channel coding (Release 15)”

- R1-1805632, “Receiver structure for NOMA”, Intel Corporation

- R1-162226, “Discussion on multiple access for new radio interface”, ZTE

- R1-162155, “Sparse Code Multiple Access (SCMA) for 5G Radio Transmission”, Huawei