3GPP TSG-RAN WG1 #94	R1-1809016

20th – 24th August 2018

Gothenburg, Sweden

Agenda Item:	6.1.5.3.3

Source:	Qualcomm Incorporated

Title:	Maintenance for Common Aspects of TDD NB-IoT

Document for:	Discussion and Decision

# 1	Introduction

In RAN1#93, the following agreements were made for data scrambling for TDD NB-IoT.

RAN1#93 agreements:

- The Rel-14 FDD interference randomization based on RE level rotation is applied to downlink channels for both anchor and non-anchor carrier in TDD NB-IoT.

According to the agreement, the RE-level non-linear data scrambling is used for all the downlink channels on both anchor and non-anchor carrier in TDD NB-IoT, which is different from FDD. However, the current specification for TDD NB-IoT assumes the support of the RE-level data scrambling is based on UE capability as following FDD. In such case, the RE-level data scrambling cannot be used for broadcast channels on the anchor carrier, which is not aligned with the agreement. Therefore, we propose to have the mandatory support of the RE-level data scrambling for TDD NB-IoT. The text proposal for TS 36.211 can be found in Section 2.

# 2	Text Proposal

#### 10.2.3.4	Mapping to resource elements

NPDSCH can be mapped to one or more than one subframes, , as given by clause 16.4.1.3 of 3GPP TS 36.213 [4], each of which shall be transmitted MrepNPDSCH times.

For frame structure type 1 and for each of the antenna ports used for transmission of the physical channel, the block of complex-valued symbols  shall be mapped to resource elements  which meet all of the following criteria in the current subframe:

-	the subframe is not used for transmission of NPBCH, NPSS, or NSSS, and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.4.1.4 of 3GPP TS 36.213 [4].

The mapping of  in sequence starting with  to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. For NPDSCH not carrying BCCH, after mapping to a subframe, the subframe shall be repeated for  minMrepNPDSCH,4-1 additional subframes, before continuing the mapping of  to the following subframe.

For frame structure type 2, and for each of the antenna ports used for transmission of the physical channel,  and MrepNPDSCH=1, the block of complex-valued symbols  shall be mapped to resource elements  which meet all of the following criteria in the current subframe:

-	the subframe is not used for transmission of NPBCH, NPSS, or NSSS, and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	they belong to DwPTS if the subframe is a special subframe, and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.4.1.4 of 3GPP TS 36.213 [4].

The mapping of  in sequence starting with  to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe.

For frame structure type 2, and for each of the antenna ports used for transmission of the physical channel,  and MrepNPDSCH>1, the block of complex-valued symbols  shall be mapped to resource elements  which meet all of the following criteria in the current subframe:

-	the subframe is not used for transmission of NPBCH, NPSS, or NSSS, and

-	they are assumed by the UE not to be used for NRS when l= NsymbDL-2, NsymbDL-1, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.4.1.4 of 3GPP TS 36.213 [4].

The mapping of  in sequence starting with  to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. For NPDSCH not carrying BCCH, after mapping to a subframe, the subframe shall be repeated for minMrepNPDSCH,4-1 additional subframes, before continuing the mapping of  to the following subframe. The resource elements in a special subframe that are not part of DwPTS are counted but not used in the mapping. When l= NsymbDL-5, NsymbDL-4, the resource elements in a special subframe assumed by the UE for NRSs are counted but not used in the mapping.

For frame structure type 1, for NPDSCH associated with C-RNTI when interferenceRandomisationConfig is configured, or NPDSCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or NPDSCH associated with G-RNTI or SC-RNTI or for frame structure type 2, for NPDSCH not carrying the BCCH, define as the block of complex-valued symbols mapped to slot number and radio frame number . Each complex-valued symbol  shall be multiplied with before its transmission, with

where the scrambling sequence is given by clause 7.2, and shall be initialized at the start of each subframe with .

### <Unchanged parts are omitted>

#### 10.2.5.5	Mapping to resource elements

For frame structure type 1, the block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the NCCE(s) assigned for the NPDCCH transmission, and

-	they are not used for transmission of NPBCH, NPSS, or NSSS , and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.6.1 of 3GPP TS 36.213 [4].

For frame structure type 2, when the NPDCCH is transmitted in one subframe, the block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the NCCE(s) assigned for the NPDCCH transmission, and

-	they are not used for transmission of NPBCH, NPSS, or NSSS, and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.6.1 of 3GPP TS 36.213 [4].

The mapping to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. Denote  as the complex-valued symbols that are mapped to resource elements meeting the criteria above, with the insertion of <NIL> elements in the locations of resource elements which are not part of the NCCE(s) assigned for the NPDCCH transmission.

For frame structure type 2, when the NPDCCH is transmitted in more than one subframe, the block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the NCCE(s) assigned for the NPDCCH transmission, and

-	they are not used for transmission of NPBCH, NPSS, or NSSS , and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.6.1 of 3GPP TS 36.213 [4].

The mapping to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. Denote  as the complex-valued symbols that are mapped to resource elements meeting the criteria above. The resource elements in a special subframe that are not part of DwPTS are counted but not used in the mapping.

For frame structure type 1, for NPDCCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or NPDCCH associated with G-RNTI or SC-RNTI, or for NPDCCH associated with C-RNTI when interferenceRandomisationConfig is used according to [11], or for frame structure type 2, each complex-valued symbol , shall be multiplied with ,where

where the scrambling sequence  is given by clause 7.2, and shall be initialized at the start of each subframe with .

### <Unchanged parts are omitted>

# References

TS 36.211, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, version 15.2.0