3GPP TSG RAN WG1 Meeting #94			R1-1809169

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	NTT DOCOMO, INC.

Title:	Preliminary evaluation results of mobility for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

Mobility evaluation is used to exam the maximum mobile station speed at which a defined QoS can be achieved. ITU-R defined a procedure for mobility evaluation as follow [1].

The evaluator shall perform the following steps in order to evaluate the mobility requirement.

- Step 1: 	Run uplink system-level simulations, identical to those for average spectral efficiency, and 5th percentile user spectral efficiency except for speeds taken from Report ITU-R M.2410 [2], using link-level simulations and a link-to-system interface appropriate for these speed values, for the set of selected test environment(s) associated with the candidate RITs/SRITs and collect overall statistics for uplink SINR values, and construct CDF over these values for each test environment.

- Step 2:	Use the CDF for the test environment(s) to save the respective 50th-percentile SINR value.

- Step 3: 	Run new uplink link-level simulations for the selected test environment(s) for either NLOS or LOS channel conditions using the associated speeds in Table 4 of Report ITU-R M.2410 [2], as input parameters, to obtain link data rate and residual packet error ratio as a function of SINR. The link-level simulation shall use air interface configuration(s) supported by the proposal and take into account retransmission, channel estimation and phase noise impact.

- Step 4: 	Compare the uplink spectral efficiency values (link data rate normalized by channel bandwidth) obtained from Step 3 using the associated SINR value obtained from Step 2 for selected test environments, with the corresponding threshold values in the Table 4 of Report ITU-R M.2410 [2].

- Step 5: 	The proposal fulfils the mobility requirement if the spectral efficiency value is larger than or equal to the corresponding threshold value and if also the residual decoded packet error ratio is less than 1%, for all selected test environments. For the selected test environment, it is sufficient if one of the spectral efficiency values (using either NLOS or LOS channel conditions) fulfils the threshold.

At the RAN1#92bis meeting, the pre-processing SINR calculation method is agreed in [3] to simplify the evaluation complexity of link level simulation. Based on the agreed SINR calculation method, we present both the CDF of pre-processing SINR and data rate of each test envirnments.

- ITU Requirements

ITU-R provides the requirement of data rate on uplink in following table [2], which assumes the user is moving at the maximum speed in that mobility class in each of the test environments. Meanwhile, the BLER should be less than 1%. This requirement is defined for the purpose of evaluation in the eMBB usage scenario.

TABLE 1

Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (Bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 1.5 | 10 |
| Dense Urban – eMBB | 1.12 | 30 |
| Rural – eMBB | 0.8 | 120 |
|  | 0.45 | 500 |

- System level evaluation of UL pre-processing SINR

In this section, we provide the evaluation results of UL pre-processing SINR on Indoor Hotspot – eMBB, Dense Urban–eMBB and Rural – eMBB.

In Figure 1, the CDF curve for UL pre-processing SINR on Dense Urban-eMBB is provided. In the evaluation, the power control parameters of Alpha = 0.6 and P0 = -60dBm are used. The antenna tilt for Dense Urban-eMBB Config. A is 108 degree. The evaluation assumption is the same as that used for evaluations on sepectrum efficiency and could be found in companion contributions [4].

Figure 1  CDF curves for UL pre-processing SINR on Dense Urban-eMBB

In Figure 2, the CDF curves for UL pre-processing SINR on Indoor Hotspot-eMBB (12 TRxP)  are provided. The evaluation assumption is the same as that used for evaluations on sepectrum efficiency and could be found in companion contributions [5].

Figure 2  CDF curves for UL pre-processing SINR on Indoor Hotspot -eMBB

In Figure 3, the CDF curves for UL pre-processing SINR on Rural-eMBB  are provided. In the evaluation, the power control parameter of Alpha = 0.6, P0 = -60dBm is used. The antenna tilt of 99/99/96 degree is used in Rural-eMBB Config. A/B/C. The evaluation assumption is the same as that used for evaluations on sepectrum efficiency and could be found in companion contributions [6].

Figure 3 CDF curves for UL pre-processing SINR on Rural -eMBB

The 50% percentile SINR of the CDF of different scenarios are shown in the following table.

TABLE 2

| Scenario | SINR (dB) |
| --- | --- |
| Dense Urban_ConfigA | 4.818 |
| Dense Urban_ConfigB | 4.495 |
| Indoor Hotspot– eMBB ConfigA | 2.816 |
| Indoor Hotspot– eMBB ConfigB | 21.17 |
| Indoor Hotspot– eMBB ConfigC | 26.84 |
| Rural – eMBB ConfigA | 8.643 |
| Rural – eMBB ConfigB | 4.894 |
| Rural – eMBB ConfigC | 3.558 |

- Link level evaluation results of data rate

Based on the UL pre-processing SINR provided in TABLE 2, link-level evaluations are performed in accordance with the evaluation assumption presented in TABLE 3 and some initial evaluation results are shown in TABLE 4.

TABLE 3

The additional link-level evaluation assumption

| Assumption | Value |
| --- | --- |
| Subcarrier spacing | 15kHz(700MHz), 30kHz(4GHz), 60kHz(30GHz),120kHz(70GHz) |
| Channel model | TDL-A for 10 km/h, TDL-C for other UE speed |
| Waveform | CP-OFDM |
| Antenna configuration | 1 TX antenna, 4 RX antenna |
| Transmission rank | 1 |
| Channel coding scheme | LDPC |
| HARQ | No HARQ |
| Link adaptation | Yes |
| Channel estimation | MMSE |
| DMRS | 2 symbol DMRS with configuration type 1 |
| Additional overhead | 1 symbol |

TABLE 4

Link-level evaluation results

| Scenario | Config. | Mobility (km/h) | Target SE (bit/s/Hz) | BLER (%) | SE (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| Indoor Hotspot (12 TRxP) | B | 10 | 1.5 | 0.75 | 2.84 |
|  | C |  |  | 0.36 | 2.22 |
| Dense Urban | A | 30 | 1.12 | 0.74 | 1.574 |
|  | B |  |  | 0.08 | 1.2434 |
| Rural | A | 120 | 0.8 | 0.84 | 1.7505 |
|  |  | 500 | 0.45 | 0.93 | 0.856 |
|  | B | 120 | 0.8 | 0.77 | 1.0761 |

Comparing the above evaluation results with the requirements shown in TABLE 1, we can conclude that the required normalized traffic channel link data rates can be reached in all evaluated environments.

- Summary

In this contribution, we provide the preliminary results of UL pre-processing SINR and normalized traffc data rate on Indoor Hotspot – eMBB, Dense Urban–eMBB and Rural – eMBB. Based on the obtained 50% UL pre-processing SINR, the achieved traffic data rate can fulfill the IMT-2020 mobility requirements in all required test environments.

References

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, June 2017.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei, April 2018.

- R1-1809167, “Updated evaluation results of dense urban-eMBB for IMT-2020 self-evaluation”, NTT DOCOMO, August 2018.

- R1-1809166 “Updated evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation”, NTT DOCOMO, August 2018.

- R1-1809168, “Updated evaluation results of rural-eMBB for IMT-2020 self-evaluation”, NTT DOCOMO, August 2018.