3GPP TSG-RAN WG1 Meeting #94	Tdoc R1-1809210

Gothenburg, Sweden, 20th – 24th August, 2018

Agenda Item:	7.2.2.4.5

Source:	Ericsson

Title:	Indoor coexistence evaluation results for NR-U and WiFi

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#92 [1], the following agreements regarding to NR-U coexistence with other networks have been reached

Agreement:

- Coexistence with other networks (e.g. WiFi, LAA LTE, NR-U)

  - When coexistence with WiFi is evaluated, only consider deployed WiFi systems (e.g. 11ac for 5 GHz)

    - Fairness criterion for coexistence with 11ax can be further discussed at plenary level

    - The coexistence evaluation applies to 5GHz band (11ac) and 60GHz (11ad)

    - From SID: NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier

  - For sub-7 GHz bands, coexistence simulations will be performed using technology neutral assumptions (eg. channel access mechanism) at an arbitrary carrier frequency in 5GHz band for application to bands other than 5GHz which may become available subject to regulations

    - Note: The study assumes regulation will provide the framework concerning the protection for the technologies not using unlicensed access in those bands

In this contribution, we provide the coexistence evaluation results for NR-U and WiFi.

# 2	Coexistence Evaluation

## 2.1	Coexistence evaluation assumptions

Similar to LTE-based LAA, coexistence evaluation methodology is as follow:

- WiFi and NR-U coexistence

  - For each UE and eNB/AP drop

    - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

    - Step 2: Wi-Fi is replaced with NR-U for the group of gNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the NR-U network are evaluated and recorded.

  - Performance metrics for the Wi-Fi operator common to the two steps are compared.

- NR-U and NR-U coexistence

  - Performance metrics for two NR-U operators coexisting in a given evaluation scenario are evaluated and recorded.

  - Performance metrics for the two NR-U operators are compared.

In our evaluation, we consider two operators named Operator A and Operator B. In the Wi-Fi and NR-U coexistence scenario, in the first step Operator A and B both use Wi-Fi. In the second step, operator A and its corresponding UEs are replaced by an NR-U operator and NR-U UEs while operator B and its UEs remain unchanged. In the NR-U and NR-U coexistence scenario both operators that coexist with each other are NR-U networks.

The simulation assumptions are based on the agreed coexistence assumptions in RAN1, in which some main parameters are defined in Table 3 in the Appendix. Our preferences on the assumptions that remained optional or need clarifications when results are provided below.

In all the indoor coexistence evaluations, the transmit power of the base station is assumed to be 20 dBm. The average received power CDFs are given in the appendix, which show that there are more than 10% of the serving links have received power below -72dBm as in RAN1 agreements. We also assume the mixed traffic scenarios in which the traffic ratio between DL and UL is 50:50. Channel access mechanisms follow the same principles defined for LAA for both DL and UL. The used LBT parameters are listed in the appendix.

NR-U uses self-scheduling for UL transmissions, i.e. the grant is sent on the same unlicensed carrier as the data transmission. Additionally, no licensed carrier is being considered and the Processing delays are modelled according to UE Capability #1.

For Wi-Fi networks, we evaluate with 802.11ac system assumptions. Finally, Table 1 and Table 2 capture our assumptions for Wi-Fi and NR-U systems, respectively.

Table 1 Additional WiFi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna configuration |  | 4Tx2Rx in DL, Cross-polarized |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500B MSDU + 14 B header |
| MCOT |  | 6 ms |
| MAC | Coordination | DCF |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
|  | Contention window | Per DCF |
| CCA-CS |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| Rate control |  | Minstrel algorithm |
| OFDM symbol length |  | 4 micro second |
| Contention Window {min, max} |  | DL: {15,63} UL: {15,1023} |

Table 2 Additional NR-U system evaluations assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 4Tx2Rx in DL, Cross-polarized. |
| Transmission schemes | QPSK/16QAM/64QAM/256QAM |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -72 dBm |
| Cyclic Prefix | Normal |
| Subcarrier spacing | 30KHz |
| MCOT | 6 ms |
| UE processing capability | Capability #1 |
| Contention Window {min, max} | DL: {15,63} UL: {15,1023} |

## 2.2	Coexistence evaluation results

Based on the simulation assumption that specified in Table 1 and Table2 we set up three evaluation scenarios to study coexistence of Wi-Fi network with Wi-Fi, Wi-Fi with NR-U and finally NR-U with NR-U to see coexistence of two NR-U operators.

Figure 1: (a) DL performance of Wi-Fi coexisting with Wi-Fi/NR-U. (b) DL performance of NR-U coexisting with Wi-Fi/NR-U.

(a)

(b)

(a)

(b)

Figure 2: (a) UL performance of Wi-Fi coexisting with Wi-Fi/NR-U. (b) UL performance of NR-U coexisting with Wi-Fi/NR-U.

The system performance results in figure 1a and 2a show that not only does NR-U with DL and UL traffic coexists in a friendly manner with Wi-Fi but also boosts Wi-Fi performance as compared to the case where two Wi-Fi networks coexist with each other. Some of the major elements which make NR-U a good neighbor to Wi-Fi in the unlicensed band where both technologies have to share the medium are listed below:

- The NR-U uplink traffic is scheduled by the NR-U gNBs, which reduces the number of contending nodes at any given time. In the Wi-Fi network, all UEs with non-empty UL buffer contend to access the medium and by that introduce intra-cell collisions and higher inter-cell collisions.

- NR-U, as compared to the evaluated 802.11ac Wi-Fi, has more robust interference mitigation, error correction and retransmission schemes which result in greater efficiency in serving the traffic and reducing resource utilization thus providing more opportunities for other systems to access the medium.

- NR-U uses the same sensing threshold irrespective of transmissions from NR-U or Wi-Fi nodes as opposed to Wi-Fi which uses a higher sensing threshold for NR-U as compared to Wi-Fi.

NR-U can improve the performance of a coexisting Wi-Fi operator.

NR-U networks on the unlicensed band can coexist well with Wi-Fi networks.

Additionally, the results in Figure 1b and Figure 2b demonstrate that when NR-U coexists with another NR-U network, it can achieve better performance than coexisting with Wi-Fi.

NR-U networks on the unlicensed band can coexist well with each other.

# Conclusion

In the previous sections we made the following observation and proposals for indoor coexistence evaluation for NR-U and WiFi:

- NR-U can improve the performance of a coexisting Wi-Fi operator.

- NR-U networks on the unlicensed band can coexist well with Wi-Fi networks.

NR-U networks on the unlicensed band can coexist well with each other.

# References

Chairmain’s notes, RAN1#92, Athens, Greece.

# Appendix

Table 3 Simulation parameters for indoor sub-7GHz scenario

| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB |
| SCS | 30kHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 20dBm (subtract 3dBm from 23dBm to meet EIRP limit with BF) |
| UE/STA Tx Power | 18dBm |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| UE/STA dropping per network | All UEs should be randomly dropped and be within coverage of the gNB/AP in the unlicensed band. Example of a dropping method to achieve this with N=5 UEs/STAs per gNB /AP: - Randomly drop a large enough number of UEs over the whole 120mx80m building. - Each UE/STA select the best gNB /AP (according to RSSI) as its serving gNB /AP. The number of dropping UEs in the first step should be large enough such that at least 5 UEs/STAs are associated to each BS/AP. - Each gNB AP randomly selects 5 UEs/STAs from its associated UEs/STAs. |

Figure 5: CDF of Average Rx power for DL(left) and UL(right) transmissions.