3GPP TSG RAN WG1 Meeting #94			                                                                        R1-1809435

Gothenburg, Sweden, August 20st – 24th, 2018

Agenda item:	7.2.1.2

Source: 	Qualcomm Incorporated

Title: 	Receivers for NOMA

Document for:	Discussion/Decision

# Introduction

In RAN1 #92b, the following agreements have been made [1]:

- Adopt Figure 1 as the general block diagram of multi-user receiver for UL data transmissions. Besides,

- The algorithms for the detector block (for data) can be e.g. MMSE, MF, ESE, MAP, MPA, EPA.

- The interference cancellation can be hard, soft, or hybrid, and can be implemented in serial, parallel, or hybrid.

- The interference cancellation block may or may not be used.

- The input to interference cancellation may come directly from the detector for some cases

Figure 1. A high-level block diagram of multi-user receiver

In RAN1-93, the following agreements have been made [2]:

- In performing performance evaluation, companies should provide analysis of receiver complexity. Particularly (with details FFS):

  - Detector complexity

  - Decoding complexity

  - Interference cancellation complexity, if any

  - Number of iteration(s), if any

  - Other receiver optimization, if any

  - Complexity for the preamble/DMRS detection

  - Memory requirements

  - Latency

  - FFS which simulation cases to be selected for evaluation

In this contribution, we provide a general framework for NOMA UL receiver, with a focus on linear multi-user detector schemes. MF and ESE based NOMA receivers can be applied to both CP-OFDM and DFT-s-OFDM waveforms, and synchronized and asynchronized transmissions. This facilitates scalable operation and receiver complexity reduction. As a way forward, we propose the study and standardization of NOMA UL transmission should consider the best trade-off among the design objectives of BLER performance, scalability, flexibility, PAPR and complexity.

# Linear Receivers for Linear-Spreading-based NOMA schemes

Figure 2. General Diagram for Linear NOMA Receiver for Linear Spreading NOMA Schemes

As shown in Figure 1, even though the details of the NOMA receivers for different NOMA transmission schemes can vary, the NOMA receiver schemes share the same high-level structure. Figure 2 shows a detailed description of the general NOMA receiver diagram.

The NOMA receiver consists of three parts. The first part is multi-user detector where the superposed received signal is jointly processed across the UEs to derive the LLR for each UE. The example of multi-user detectors includes LMMSE/ESE/MF/MPA depending on NOMA transmission schemes. The second part of channel decoder which receives LLR from multi-user detector and decode the transmitted codeword. The output from the channel decoder can be decoded codeword in the case of successful decoding. It can be also intermediate LLR for each bit refined through the message passing decoding. Third part is the iteration between multi-user detectors and LDPC decoders. They can exchange both soft-LLR information and hard decision information. When they exchange soft-LLR, soft interference cancellation at multi-user detector is feasible. We will call this as turbo iteration or soft interference cancellation. When they only exchange hard-decision, we will call this as hard interference cancellation. In turbo iteration, multi-user detector and channel decoder exchanges LLR information. Interference cancellation can be performed both parallelly and successively.

## Linear Receivers for Linear-Spreading-based NOMA schemes

Since outer iteration and LDPC decoder are straightforward and not in the scope of this contribution, we will focus on multi-user detector in the rest of the section. Especially, we will focus on ESE (elementary signal estimator) and LMMSE (linear minimum square error estimator).

Without loss of generality, we can focus on single symbol processing. Suppose that there are J many users and K many resources (spreading factor). The received signal at resource k can be written as

yk=j=1Jhkj xkj+nk,

where hkj is the channel coefficient corresponding to resource k from user j, xkj is the transmitted signal by user j on resource k, and nk~CN(0,N0).

For linear spreading codes, each user is assigned a spreading code sequence. Let  ckj be the k’th coefficient of the spreading code for user j. Suppose that all users share the same modulation alphabet M=w1,w2,⋯,wM. Then, xkj=ckjsj for k=1,2,…, K where sj∈ M is the transmitted symbol by user j. Now, we can write the received vector

y=y1⋮yK=H s+n,

where s=[s1,…,sJ]T, n=[n1,…,nJ]T, and H= hkjk,j is the KxJ matrix with entries hkj=hkj ckj. Let hj denote the j’th column of matrix H.

Multi-user detectors estimate the LLR for s based on y.

### MF/ESE Multi-User Detector

Figure 3. Diagram for ESE Multi-User Detector

MF (matched filter) based multi-user detector is well known, and ESE can be thought as a generalization of MF which can accommodate soft-interference cancellation. Therefore, in this section, we will focus on ESE multi-user detector and leave MF as a degenerated case.

ESE multi-user detector first compress the received signals to scalar values for each UE by MF. The output of the matched filter can be written as

y=y1⋮yJ=HHy.

To benefit from soft information computed at channel decoder, we can apply elementary signal estimator to y which approximates signal and interference as Gaussian random variables. More explicitly, for user j,

yj=hj2sj+ i≠jhjHhi si+hjHn εj,

where εj is residual interference plus noise. Then, εj is approximated as a Gaussian random variable which can be described by mean and variance. We can see that the mean and variance of εj is given as follows:

mεj=i≠jhjHhi m(si),

varεj=i≠jhjHhi2var(si)+hj2N0,

and  m(si) and  var(si) are the a priori mean and variance for the symbol transmitted from user i, which can be computed using a priori bit LLRs.

Now both sj and εj are approximated as Gaussian random variables. LMMSE estimation can be used to estimate sj from yj. From the estimation, LLR for each bit can be derived from conventional marginalization.

Here, we can notice that ESE multi-user detector without MF can be also used without symbol spreading. For random symbol interleaver cases, ESE multi-user detector can be applicable with an assumption of  p(y  si=s=k=1K p(yk  si=s. ESE multi-user detector is also applicable for bit level interleaving cases.

It can be also shown that the computation complexity of ESE multi-user detector scales as O(KJ2) for J UEs and spreading factor of K.

| MU Detector | Computation Complexity |
| --- | --- |
| ESE | O(KJ2) |

### LMMSE Multi-User Detector

Figure 4. Diagram for LMMSE Multi-User Detector

Figure 4 shows general diagram for LMMS multi-user detector. Compared to ESE multi-user detector, the only difference is ESE block is replaced by LMMSE estimator. Unlike ESE multi-user detector, LMMSE estimator considers the received signal as a vector and apply LMMSE estimation matrix for transmitted signal estimation for each UE.

Let mprior and  Vprior be the prior mean and variance of each UE’s transmitted signal derived from LLR. Receiver applies the following LMMSE filter to the received vector. The output of the LMMSE filter is the mean and variance vectors, and can be computed as

m=m1⋮mJ=mprior+ Vprior HH (N0 I + H VpriorHH)-1 (y-Hmprior),

V= v1⋮vJ,   where    vi=viprior-viprior2*(hj)H N0 I + H VpriorHH-1 hj.

As mentioned, mprior is the a priori mean vector and Vprioris the diagonal matrix with diagonal entries are a priori variance values for the corresponding transmitted symbols. A priori mean and variance values can be computed using the bit LLRs computed at the channel decoder.

Based on the LMMSE output, the receiver generates extrinsic bit LLR values for channel decoder by marginalization.

It can be also shown that the computation complexity of LMMSE multi-user detector scales as O(K3+K2J +KJ2) for J UEs and spreading factor of K.

| MU Detector | Computation Complexity |
| --- | --- |
| LMMSE | O(K3+K2J +KJ2) |

We propose to consider at least NOMA receivers described above should be considered in the study of NOMA schemes.

## Consideration for DFT-s-OFDM

For cell edge and power-limited UEs, PAPR and MCL can be a crucial factor in evaluation of NOMA schemes’ performances. To match with NB-IoT and eMBB coverage, DFT-s-OFDM waveform has to adopted as a baseline especially in mMTC case. Therefore, NOMA transmitters and receivers with DFT-s-OFDM waveform also should be considered.

The main issue with DFT-s-OFDM waveform is that DFT-s-OFDM can introduce different multi-path channels for each UEs.

MF multi-user detector is readily applicable in time domain for RSMA with DFT-s-OFDM waveform, which is known as Rake receiver. Likewise, ESE multi-user detector can also be easily applicable for DFT-s-OFDM waveform.

However, for LMMSE receivers which relies on chip level alignment and synchronization, it is unclear how to define modulated symbol boundary with different multi-path channels for NOMA UEs in DFT-s-OFDM waveform cases. Therefore, we propose the following

Proposal 1: Companies should provide NOMA transmitter and receiver schemes for both CP-OFDM and DFT-s-OFDM waveforms.

## Consideration for Asynchronous Transmission

For mMTC use case, managing timing advance and keeping synchronization across the NOMA UEs requires large power consumption. Therefore, the ability to operate asynchronously without any TA is an important feature for NOMA schemes.

For long scrambling sequence based RSMA with DFT-s-OFDM waveform, as mentioned above, the MF/ESE based NOMA receivers are still applicable for NOMA multi-user receiver even for asynchronous case, which are known as Rake receivers.

However, like DFT-s-OFDM case, the LMMSE based NOMA receivers which relies on chip level alignment and synchronization cannot be directly applicable for asynchronous transmission case. Therefore, we propose to provide details of NOMA schemes for asynchronous transmission.

Proposal 2: Companies should provide NOMA transmitter and receiver schemes for both synchronous and asynchronous transmission.

# Receiver Complexity and LLS Simulation Results

Together with BLER and PAPR performances, transmitter and receiver complexity should be also considered given that some of the receiver proposals are based on complicated non-linear message passing algorithms.

Complexity of decoding (per outer iteration) one symbol transmitted from all users is given in Table 1, where K is the number of resources, J is the number of users, M is the modulation alphabet size, N is the number of internal iterations on the factor graph for MPA algorithm, and J is the number of users transmitting on the same resource.

| NOMA Receiver | Computational Complexity (per Outer Iteration) |
| --- | --- |
| LMMSE | O(K3+K2J +KJ2) |
| ESE | O(KJ2) |
| MPA | O(NKJ2MJ) |

Table 1: Computational Complexity comparison for different NOMA receivers

As seen from this table, the computational complexity of MPA algorithm scales exponentially in  J. Therefore, when large number of UEs are colliding on the same resources, the MPA computational complexity can be much larger than those of LMMSE and ESE which scales polynomially on the number of UEs and spreading factor.

The entire receiver complexity can be quantified based on the multi-user detection/demodulation (MUD) and interference cancellation (IC) algorithms. The entire complexity of receiver (excluding DMRS processing) can be represented by:

(MUD & LDPC decoding complexity per outer iteration) × (# of outer iteration for IC)

Therefore, quantification of the receiver complexity can be split into two steps:

- Firstly, quantify multi-user detector/demodulator (MUD) complexity;

- Secondly, account for LDPC decoding complexity and the number of outer iterations between MUD and LDPC decoders.

Since LDPC decoding complexity is already extensively studied in NR Rel-15 [6], receiver complexity analysis for NOMA schemes can focus on the multi-user detector/demodulator complexity analysis.

Figure 5: BLER performance of NOMA schemes with Grant-based and Synchronized Transmission.

Figure 6: Computational Complexity of Multi-User Detectors

Figure 5 shows link level BLER performances for different NOMA schemes with grant-based and synchronized transmission. As we can observe in the figure, there is no significant difference in BLER performance across the several of the different NOMA proposals, though in this particular case we can see some benefit from RSMA. Figure 6 shows the computational complexity for different types of multi-user detectors for different spreading factors when the overloading factor is fixed as 150%. We can notice that the message passing based non-linear algorithms suffers from large computational complexity scaling as the spreading factor and the number of UEs grows.

Observation 1: Linear multi-user detectors (LMMSE/ESE/MF) have much lower complexity compared to non-linear multi-user detectors (MPA/MAP).

Therefore, we propose the followings:

Proposal 3: Transmitter/Receiver complexity and memory requirement should be considered in evaluating NOMA schemes’ performances. Receiver complexity can be measured by counting the number of add/sub/mul/div required for each multi-user detector.

Proposal 4: Companies should provide receiver and transmitter computational complexity and memory requirement analysis for the proposed NOMA schemes.

# Summary

In this contribution, we discussed receivers for NOMA and made following observations.

Observation 1: Linear multi-user detectors (LMMSE/ESE/MF) have much lower complexity compared to non-linear multi-user detectors (MPA/MAP).

Based on the above observations, we propose the following for receivers for NOMA schemes.

Proposal 1: Companies should provide NOMA transmitter and receiver schemes for both CP-OFDM and DFT-s-OFDM waveforms.

Proposal 2: Companies should provide NOMA transmitter and receiver schemes for both synchronous and asynchronous transmission.

Proposal 3: Transmitter/Receiver complexity and memory requirement should be considered in evaluating NOMA schemes’ performances. Receiver complexity can be measured by counting the number of add/sub/mul/div required for each multi-user detector.

Proposal 4: Companies should provide receiver and transmitter computational complexity and memory requirement analysis for the proposed NOMA schemes.

# References

- Chairman’s Notes, 3GPP TSG RAN1 #92b, Sanya, China.

- Chairman Notes, 3GPP TSG RAN1 #93, Busan, Korea.

- RP-181403, “Study on Non-orthogonal Multiple Access for NR,” La Jolla, USA.

- R1-1803412, “Link Level Simulation Assumptions and Evaluation Metrics for NR NOMA,” ZTE.

- Email discussions on Tx/Rx Clarifications for NOMA, Coordinated by ZTE.

- R1-164704, “Channel coding evaluation assumptions - performance and complexity”, Qualcomm Inc.