3GPP TSG RAN1#94	R1-1809495

# Gothenburg, Sweden

# 20 - 24 August, 2018

# Agenda item:     	7.2.7.2
Source: 	Motorola Mobility, Lenovo

# Title:	IMT-2020  Downlink Spectral Efficiency Results

# Document for: 	Discussion

# Introduction

This contribution provides evaluation results for FR1 eMBB downlink spectral efficiency results.

# Test environments and evaluation configurations

Report ITU-R M.2412 defines five test environments, each reflecting a geographic environment and usage scenario [1].  Of these five environments, the three associated with the eMBB usage scenario were simulated. These environments are summarized in Table 1.

Table 1: Test environments and evaluation configurations simulated

| Test environment | Evaluation configuration | Carrier frequency | ISD | Remark |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB | Config. A | 4 GHz | 20 m |  |
| Dense Urban – eMBB | Config. A | 4 GHz | 200 m | Macro layer only |
| Rural - eMBB | Config. A | 700 MHz | 1732 m |  |
|  | Config. B | 4 GHz | 1732 m |  |
|  | Config. C (LMLC) | 700 MHz | 6000 m |  |

# Average and 5th Percentile Spectral Efficiency Requirements

ITU-R M.2410 defines two spectral efficiency (SE) requirements for IMT-2020.  The first, defined in Sec. 4.5, is the average spectral efficiency, defined as:

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e., the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bps/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Mathematically, the average SE can be expressed

,

where Ri (T) is the number of correctly received bits through time T by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs.  The channel bandwidth is denoted by W.

The second spectral efficiency requirement, 5th percentile spectral efficiency, is defined in Section 4.4:

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth has the same definition as given in the definition of average spectral efficiency. The normalized user throughput of user i is defined as

,

with Ri (Ti) denoting the number of correctly received bits of user i with active session time Ti and channel bandwidth W.

## Minimum Spectral Efficiency Requirements

The minimum requirement for downlink 5th percentile user SE and downlink average SE are given below: [2]

Table 2: Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) |
| --- | --- |
| Indoor Hotspot – eMBB | 9 |
| Dense Urban – eMBB (Macro layer) | 7.8 |
| Rural - eMBB | 3.3 |

Table 3: 5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) |
| --- | --- |
| Indoor Hotspot – eMBB | 0.3 |
| Dense Urban – eMBB (Macro layer) | 0.225 |
| Rural - eMBB | 0.12 |

# Evaluation Results

Based on RAN1 agreements in [3] and channel model email reflector discussion summarized in [4], simulations for the test environments and configurations of Table 1 were performed and the average and 5th percentile SE statistics calculated.  The results are shown in Table 4. The results for Rural eMBB 4 GHz will be add in a revision to this contribution. The simulation assumption are given in Table 5  ̶ Table 8.  It can be observed that both average and cell edge spectral efficiency exceed the IMT-2020 requirements for all three environments.

# Observation 1: NR FDD in FR1 has the potential to fulfill DL spectral efficiency requirements in Indoor hotspot, Dense Urban, and Rural – eMBB Configurations A and C.

Table 4: FR1 FDD Simulation Results

| Environment | Performance Metric | Evaluation Result | IMT-2020 Requirement |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB Configuration A (12 TRP) | Average SE | 9.18 | 9.0 |
|  | 5th Percentile SE | 0.40 | 0.3 |
| Dense Urban –eMBB Configuration A | Average SE | 10.1 | 7.8 |
|  | 5th Percentile SE | 0.45 | 0.225 |
| Rural – eMBB 700 MHz Configuration A | Average SE | 4.00 | 3.3 |
|  | 5th Percentile SE | 0.14 | 0.12 |
| Rural – eMBB 4 GHz Configuration B | Average SE | 9.64 | 3.3 |
|  | 5th Percentile SE | 0.36 | 0.12 |
| Rural – eMBB LMLC Configuration C | Average SE | 4.40 | 3.3 |
|  | 5th Percentile SE | 0.21 | 0.12 |

# Conclusions

# This contribution presented simulated evaluation results for NR FDD downlink spectral efficiency in frequency range 1. Based on the evaluation, the following observation can be made:

# Observation 1: NR FDD in FR1 has the potential to fulfill DL spectral efficiency requirements in Indoor hotspot, Dense Urban, and Rural – eMBB Configuration A.

# References

“Guidelines for evaluation of radio interface technologies for IMT-2020,” Report ITU-R M.2412-0, October 2017.

Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017

R1-1803386, “Summary of offline discussion for IMT-2020 self-evaluation”, Huawei, March 2018.

RT-170019, “Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”, Huawei

# Appendix

Table 5: FDD Indoor Hotspot Simulation Parameters

| Technical configuration Parameters |  |
| --- | --- |
| Modulation | Up to 256 QAM |
| Coding on PDSCH | LDPC |
| Numerology | 15 kHz SCS, 14 OFDM symbol slot |
| Guard band ratio on simulation bandwidth | 6.40% |
| Simulation bandwidth | 10 MHz |
| Transmission scheme | closed SU/MU-MIMO adaptation |
| DL CSI measurement | Non-precoded CSI-RS based |
| DL codebook | Type II codebook; 4beam, wb+sb, 8psk |
| MU dimension | Up to 12 layers |
| SU dimension | Up 2 layers (due to Type II feedback) |
| CSI feedback | PMI, CQI: every 5 slot; RI: every 5 slot; Subband based |
| Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
| ACK/NACK delay | The next available UL slot |
| Re-transmission delay | The next available DL slot after receiving NACK |
| Antenna configuration at TRxP | (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ |
| Antenna configuration at UE | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) (dH,dV) = (0.5, 0.5)λ |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Non-ideal |
| System configuration parameters |  |
| TRxP number per site | 1 |
| Mechanic tilt | 180° in GCS (pointing to the ground) |
| Electronic tilt | 90° in LCS |
| Handover margin (dB) | 1 |
| Other Parameters |  |
| Channel Model | Channel Model A |
| TRxP total transmit power | 21 dBm |

Table 6: FDD Dense Urban Simulation Parameters

| Technical configuration Parameters |  |
| --- | --- |
| Modulation | Up to 256 QAM |
| Coding on PDSCH | LDPC |
| Numerology | 15KHz 14 OFDM symbol slot |
| Guard band ratio on simulation bandwidth | 6.4% |
| Simulation bandwidth | 10 MHz |
| Transmission scheme | Closed SU/MU-MIMO adaptation |
| DL CSI measurement | Non-precoded CSI-RS based |
| DL codebook | Type II codebook; 4beam, wb+sb, 8psk |
| PRBs bundling per SB | 4 PRBs |
| MU dimension | Up to 12 layers |
| SU dimension | Up 2 layers (due to Type II feedback) |
| Codeword (CW)-to-layer mapping | For 1~4 layers, CW1; For 5 layers or more, two CWs |
| CSI feedback | PMI, CQI: every 5 slot; RI: every 5 slot; Subband based |
| Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
| ACK/NACK delay | The next available UL slot |
| Re-transmission delay | The next available DL slot after receiving NACK |
| Antenna configuration at TRxP | (8,8,2,1,1;2,8) (dH, dV)=(0.5, 0.8)λ |
| Antenna configuration at UE | (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Non-ideal |
| System configuration parameters |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 105 degrees |
| Handover margin (dB) | 1 |
| Other Parameters |  |
| Channel Model | Channel Model A |
| TRxP total transmit power | 21 dBm |

Table 7: FDD Rural Simulation Parameters

|  | Rural - eMBB 700 MHz | Rural - eMBB 4 GHz | Rural - eMBB LMLC |
| --- | --- | --- | --- |
| Technical configuration Parameters |  |  |  |
| Modulation | Up to 256 QAM | Up to 256 QAM | Up to 256 QAM |
| Coding on PDSCH | LDPC | LDPC | LDPC |
| Numerology | 15 kHz/14 OFDM symbol | 15 kHz/14 OFDM symbol | 15 kHz/14 OFDM symbol |
| Guard band ratio on simulation bandwidth | 6.4% | 6.4% | 6.4% |
| Simulation bandwidth | 10 MHz | 10 MHz | 10 MHz |
| Transmission scheme | Closed SU/MU-MIMO adaptation | Closed SU/MU-MIMO adaptation | Closed SU/MU-MIMO adaptation |
| DL CSI measurement | Non-precoded CSI-RS based | Non-precoded CSI-RS based | Non-precoded CSI-RS based |
| DL codebook | Type II codebook; 4beam, wb+sb, 8psk | Type II codebook; 4beam, wb+sb, 8psk | Type II codebook; 4beam, wb+sb, 8psk |
| MU dimension | Up to 8 layers | Up to12 layers | Up to 8 layers |
| SU dimension | Up to 2 layers | Up to 2 layers | Up to 4 layers |
| CSI feedback | CQI: every 5 slot; RI: every 5 slot, CRI: every 5 slot Subband based | CQI: every 5 slot; RI: every 5 slot, CRI: every 5 slot Subband based | PMI, CQI: every 5 slot; RI: every 5 slot; Subband based |
| Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
| ACK/NACK delay | The next available UL slot | The next available UL slot | The next available UL slot |
| Re-transmission delay | The next available DL slot after receiving NACK | The next available DL slot after receiving NACK | The next available DL slot after receiving NACK |
| Antenna configuration at TRxP | (4,8,2, 1, 1; 4, 1), (dH, dV)=(0.5, 0.8)λ | (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) (dH, dV)=(0.5, 0.8)λ | (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) (dH, dV)=(0.5, 0.8)λ |
| Antenna configuration at UE | (M,N,P,Mg,Ng; Mp,Np) = (1,1,2,1,1; 1,1) | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Scheduling | PF | PF | PF |
| Receiver | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Channel estimation | Non-ideal | Non-ideal | Non-ideal |
| System configuration parameters |  |  |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 100° in LCS | 100° in LCS | 96° in LCS |
| Handover margin | 1 dB | 1 dB | 1 dB |
| Other parameters |  |  |  |
| Channel Model | Channel Model A | Channel Model A | Channel Model A |
| TRxP transmit power | 46 dBm | 46 dBm | 46 dBm |

Table 8: Overhead Assumptions

| PDCCH | DMRS | SSB | CSI-RS (for CSI) | CSI-RS (for IM) | TRS |
| --- | --- | --- | --- | --- | --- |
| 2 complete symbols | 24 RE/PRB/slot | 1 SSB/ 20 ms | 32 ports for 32 TX 8 ports for 8 TX 5 slot periodicity | 4 RE/PRB/10 ms | 8 RE/PRB/20ms |