3GPP TSG RAN WG1 #94	        		             R1-1809707

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.1.3.2

Source: 	Samsung

Title: 		Specification Update on UL Control Incorporating Proposals in RAN1#94

Document for:	Discussion

Introduction

This contribution provides updates to Subclauses 9 and 9.2 of TS 38.213 v 2.0.0 in relation to UL control signalling to incorporate some of the proposals provided in contributions for RAN1#94.

The updates aim to facilitate/expedite discussions in RAN1#94 and are limited to editorial aspects such as capturing missing agreements from past RAN1 meetings, editorial corrections, and text simplifications.

# 9	UE procedure for reporting control information

If a UE is configured with a SCG, the UE shall apply the procedures described in this subclause for both MCG and SCG.

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group

-	When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the primary PUCCH group respectively.

-	When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

If a UE would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and the PUSCH and PUCCH transmissions fulfill the conditions in Subclause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the PUSCH transmission and does not transmit the PUCCH.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information and/or periodic/semi-persistent CSI reports in a PUCCH that overlaps with the PUSCH, the UE multiplexes the HARQ-ACK information and/or periodic/semi-persistent CSI reports in the PUSCH.

If a UE transmits multiple PUSCHs in a slot on respective serving cells that include first PUSCHs that are scheduled by DCI format(s) 0_0 or DCI format(s) 0_1 and second PUSCHs configured by respective higher layer parameters ConfiguredGrantConfig, and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in Subclause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs.

If a UE transmits multiple PUSCHs in a slot on respective serving cells and the UE would multiplex UCI in one of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in Subclause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in Subclause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

A HARQ-ACK information bit value of 0 represents a negative acknowledgement (NACK) while a HARQ-ACK information bit value of 1 represents a positive acknowledgement (ACK).

## 9.2	UCI reporting in physical uplink control channel

UCI types reported in a PUCCH include HARQ-ACK information, SR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in Subclause 9.1.

A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot of  symbols as defined in [4, TS 38.211]. When the UE transmits two PUCCHs in a slot, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2.

### 9.2.1	PUCCH Resource Sets

If a UE does not have dedicated PUCCH resource configuration, provided by higher layer parameter PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by higher layer parameter pucch-ResourceCommon in SystemInformationBlockType1 through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial active UL BWP of  PRBs provided by SystemInformationBlockType1. The PUCCH resource set includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset , and a cyclic shift index set for a PUCCH transmission. The UE transmits a PUCCH using frequency hopping. The UE transmits the PUCCH using the same spatial domain transmission filter as for the Msg3 PUSCH transmission.

The UE does not expect to generate more than one HARQ-ACK information bit.

If the UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a DCI format 1_0 or DCI format 1_1, the UE determines a PUCCH resource with index ,  , as , where  is a number of CCEs in a control resource set of a PDCCH reception with DCI format 1_0 or DCI format 1_1, as described in Subclause 10.1,  is the index of a first CCE for the PDCCH reception, and  is a value of the PUCCH resource indicator field in the DCI format 1_0 or DCI format 1_1.

If

-	the UE determines the PRB index of the PUCCH transmission in the first hop as  and the PRB index of the PUCCH transmission in the second hop as , where  is the total number of cyclic shift indexes in the cyclic shift index set

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

If

-	the UE determines the PRB index of the PUCCH transmission in the first hop as  and the PRB index of the PUCCH transmission in the second hop as

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

If a UE has dedicated PUCCH resource configuration, the UE is provided by higher layers with one or more PUCCH resources.

A PUCCH resource includes the following parameters:

-	a PUCCH resource index provided by higher layer parameter pucch-ResourceId

-	an index of the first PRB prior to frequency hopping or for no frequency hopping by higher layer parameter startingPRB

-	an index of the first PRB after frequency hopping by higher layer parameter secondHopPRB;

-	an indication for intra-slot frequency hopping by higher layer parameter intraSlotFrequencyHopping

-	a configuration for a PUCCH format, from PUCCH format 0 through PUCCH format 4, provided by higher layer parameter format

If the higher layer parameter format indicates PUCCH-format0, the PUCCH format configured for a PUCCH resource is PUCCH format 0, where the PUCCH resource also includes an index for an initial cyclic shift provided by higher layer parameter initialCyclicShift, a number of symbols for a PUCCH transmission provided by higher layer parameter nrofSymbols, a first symbol for the PUCCH transmission provided by higher layer parameter startingSymbolIndex.

If the higher layer parameter format indicates PUCCH-format1, the PUCCH format configured for a PUCCH resource is PUCCH format 1, where the PUCCH resource also includes an index for an initial cyclic shift provided by higher layer parameter initialCyclicShift, a number of symbols for a PUCCH transmission provided by higher layer parameter nrofSymbols, a first symbol for the PUCCH transmission provided by higher layer parameter startingSymbolIndex, and an index for an orthogonal cover code by higher layer parameter timeDomainOCC.

If the higher layer parameter format indicates PUCCH-format2 or PUCCH-format3, the PUCCH format configured for a PUCCH resource is PUCCH format 2 or PUCCH format 3, respectively, where the PUCCH resource also includes a number of PRBs provided by higher layer parameter nrofPRBs, a number of symbols for a PUCCH transmission provided by higher layer parameter nrofSymbols, and a first symbol for the PUCCH transmission provided by higher layer parameter startingSymbolIndex.

If the higher layer parameter format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by higher layer parameter nrofSymbols, a length for an orthogonal cover code by higher layer parameter occ-Length, an index for an orthogonal cover code by higher layer parameter occ-Index, and a first symbol for the PUCCH transmission provided by higher layer parameter startingSymbolIndex.

A UE can be configured up to four sets of PUCCH resources. A PUCCH resource set is provided by higher layer parameter PUCCH-ResourceSet and is associated with a PUCCH resource set index provided by higher layer parameter pucch-ResourceSetId, with a set of PUCCH resource indexes provided by higher layer parameter resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by higher layer parameter maxPayloadMinus1. For the first PUCCH resource set, the maximum number of UCI information bits is 2. For the PUCCH resource set with the largest index, other than the first one, the maximum number of UCI information bits is 1706. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by higher layer parameter maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other PUCCH resource sets is 8.

If the UE transmits  UCI information bits, that include HARQ-ACK information bits, the UE determines a PUCCH resource set to be

-	a first set of PUCCH resources with pucch-ResourceSetId = 0 if  including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or

-	a second set of PUCCH resources with pucch-ResourceSetId = 1, if provided by higher layers, if  where  is provided by higher layer parameter maxPayloadMinus1 for the PUCCH resource set with pucch-ResourceSetId = 1, or

-	a third set of PUCCH resources with pucch-ResourceSetId = 2, if provided by higher layers, if  where  is provided by higher layer parameter maxPayloadMinus1 for the PUCCH resource set with pucch-ResourceSetId = 2, or

-	a fourth set of PUCCH resources with pucch-ResourceSetId = 3, if provided by higher layers, if

### 9.2.2	PUCCH Formats for UCI transmission

If a UE is not transmitting PUSCH, and the UE is transmitting UCI, the UE transmits UCI in a PUCCH using

-	PUCCH format 0 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of HARQ-ACK information bits with positive or negative SR (HARQ-ACK/SR bits) is 1 or 2

-	PUCCH format 1 if

-	the transmission is over 4 or more symbols,

-	the number of HARQ-ACK/SR bits is 1 or 2

-	PUCCH format 2 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of UCI bits is more than 2

-	PUCCH format 3 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2

-	the PUCCH resource does not include an orthogonal cover code

-	PUCCH format 4 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2,

-	the PUCCH resource includes an orthogonal cover code

A spatial setting for a PUCCH transmission is provided by higher layer parameter PUCCH-SpatialRelationInfo if the UE is configured with a single value for higher layer parameter pucch-SpatialRelationInfoId; otherwise, the spatial setting is provided by higher layer parameter PUCCH-SpatialRelationInfo from a set of multiple values provided by respective higher layer parameters pucch-SpatialRelationInfoId [11, TS 38.321]. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH 3 msec after the slot where the UE transmits HARQ-ACK information corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo.

-	If PUCCH-SpatialRelationInfo provides higher layer parameter ssb-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a SS/PBCH block with index provided by ssb-Index;

-	else if PUCCH-SpatialRelationInfo provides higher layer parameter csi-RS-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a CSI-RS with resource index provided by csi-RS-Index;

-	else PUCCH-SpatialRelationInfo provides higher layer parameter srs, and the UE transmits the PUCCH with a same spatial domain filter used as for a transmission of a SRS with resource index provided by srs.

A number of DMRS symbols for a PUCCH transmission using PUCCH format 3 or 4 is provided by higher layer parameter additionalDMRS.

Use of /2-PBSK, instead of QPSK, for a PUCCH transmission using PUCCH format 3 or 4 is indicated by higher layer parameter pi2BPSK.

### 9.2.3	UE procedure for reporting HARQ-ACK

A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot.

For DCI format 1_0, the PDSCH-to-HARQ-timing-indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For DCI format 1_1, if present, the PDSCH-to-HARQ-timing-indicator field values map to values for a set of number of slots provided by higher layer parameter dl-DataToUL-ACK as defined in Table 9.2.3-1.

For a SPS PDSCH reception ending in slot , the UE transmits the PUCCH in slot where  is provided by the PDSCH-to-HARQ-timing-indicator field in DCI format 1_0 or, if present, in DCI format 1_1 activating the SPS PDSCH reception.

If the UE detects a DCI format 1_1 that does not include a PDSCH-to-HARQ-timing-indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in slot , the UE provides  corresponding HARQ-ACK information in a PUCCH transmission within slot  where  is provided by higher layer parameter dl-DataToUL-ACK.

With reference to slots for PUCCH transmissions, if the UE detects a DCI format 1_0 or a DCI format 1_1 scheduling a PDSCH reception ending in slot  or if the UE detects a DCI format 1_0 indicating a SPS PDSCH release through a PDCCH reception ending in slot , the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot , where  is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by higher layer parameter dl-DataToUL-ACK. If the PDSCH subcarrier spacing is equal to or larger than the PUCCH subcarrier spacing or, in case of SPS PDSCH release, if the PDCCH subcarrier spacing is equal to or larger than the PUCCH subcarrier spacing,  corresponds to the slot of the PUCCH transmission that overlaps with the slot of the PDSCH reception or of the PDCCH reception in case of SPS PDSCH release. If the PDSCH subcarrier spacing is smaller than the PUCCH subcarrier spacing or, in case of SPS PDSCH release, if the PDCCH subcarrier spacing is smaller than the PUCCH subcarrier spacing,  corresponds to the slot of the PUCCH transmission that ends at a same time as the slot of the PDSCH reception or of the PDCCH reception in case of SPS PDSCH release.

A PUCCH transmission with HARQ-ACK information is subject to the limitations for UE transmissions described in Subclause 11.1 and Subclause 11.1.1.

Table 9.2.3-1: Mapping of PDSCH-to-HARQ_feedback timing indicator field values to numbers of slots

| PDSCH-to-HARQ_feedback timing indicator |  |  | Number of slots |
| --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |
| ‘0’ | ‘00’ | '000' | 1st value provided by dl-DataToUL-ACK |
| ‘1’ | ‘01’ | '001' | 2nd value provided by dl-DataToUL-ACK |
|  | ‘10’ | '010' | 3rd value provided by dl-DataToUL-ACK |
|  | ‘11’ | '011' | 4th value provided by dl-DataToUL-ACK |
|  |  | '100' | 5th value provided by dl-DataToUL-ACK |
|  |  | '101' | 6th value provided by dl-DataToUL-ACK |
|  |  | '110' | 7th value provided by dl-DataToUL-ACK |
|  |  | '111' | 8th value provided by dl-DataToUL-ACK |

For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources for  HARQ-ACK information bits, as described in Subclause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212] in a last DCI format 1_0 or DCI format 1_1, among the DCI formats 1_0 or DCI formats 1_1 that have a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes and are then indexed in an ascending order across PDCCH monitoring occasion indexes.

The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2, provided by higher layer parameter ResourceList for PUCCH resources from a set of PUCCH resources provided by higher layer parameter PUCCH-ResourceSet with a maximum of eight PUCCH resources.

For the first set of PUCCH resources and when the size  of higher layer parameter resourceList is larger than eight, when a UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a last DCI format 1_0 or DCI format 1_1 in a PDCCH reception, among DCI formats 1_0 or DCI formats 1_1 that indicate a same slot for the PUCCH transmission, the UE determines a PUCCH resource with index , , as

where  is a number of CCEs in control resource set  of the PDCCH reception for the DCI format 1_0 or DCI format 1_1 as described in Subclause 10.1,  is the index of a first CCE for the PDCCH reception, and  is a value of the PUCCH resource indicator field in the DCI format 1_0 or DCI format 1_1.

Table 9.2.3-2: Mapping of PUCCH resource indication field values to a PUCCH resource in a PUCCH resource set with maximum 8 PUCCH resources

| PUCCH resource indicator | PUCCH resource |
| --- | --- |
| '000' | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |
| '001' | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |
| '010' | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |
| '011' | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |
| '100' | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |
| '101' | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |
| '110' | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |
| '111' | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |

If a UE transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding HARQ-ACK information transmission is provided by higher layer parameter n1PUCCH-AN.

If a UE transmits HARQ-ACK information using PUCCH format 0, the UE determines values  and  for computing a value of cyclic shift  [4, TS 38.211] where  is provided by higher layer parameter initialCyclicShift of PUCCH-format0, and  is determined from the value of one HARQ-ACK information bit or from the values of two HARQ-ACK information bits as in Table 9.2.3-3 and Table 9.2.3-4, respectively.

Table 9.2.3-3: Mapping of values for one HARQ-ACK information bit to sequences for PUCCH format 0

| HARQ-ACK Value | 0 | 1 |
| --- | --- | --- |
| Sequence cyclic shift |  |  |

Table 9.2.3-4: Mapping of values for two HARQ-ACK information bits to sequences for PUCCH format 0

| HARQ-ACK Value | {0, 0} | {0, 1} | {1, 1} | {1, 0} |
| --- | --- | --- | --- | --- |
| Sequence cyclic shift |  |  |  |  |

If a UE transmits HARQ-ACK information using PUCCH format 1, the UE is provided a value for  by higher layer parameter initialCyclicShift of PUCCH-format1.

If a UE transmits HARQ-ACK information bits and  bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs  provided respectively by higher layer parameter nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and start from the first PRB from the number of PRBs, that results to  and, if , , where , , , and  are defined in Subclause 9.2.5.2. If , the UE transmits the PUCCH over  PRBs.

### 9.2.4	UE procedure for reporting SR

A UE is configured by higher layer paremater SchedulingRequestResourceConfig a set of configurations for SR transmission in a PUCCH using either PUCCH format 0 or PUCCH format 1.

The UE is configured a PUCCH resource by higher layer parameter SchedulingRequestResourceId providing a PUCCH format 0 resource or a PUCCH format 1 resource as described in Subclause 9.2.1. The UE is also configured a periodicity  in symbols or slots and an offset  in slots by higher layer parameter periodicityAndOffset for a PUCCH transmission conveying SR. If  is larger than one slot, the UE determines a SR transmission occasion in a PUCCH to be in a slot with number  [4, TS 38.211] in a frame with number  if .

If  is one slot, the UE expects that  and every slot is a SR transmission occasion in a PUCCH.

If  is smaller than one slot, the UE determines a SR transmission occasion in a PUCCH to start in a symbol with index  [4, TS 38.211] if  where  is the value of higher layer parameter startingSymbolIndex.

If the UE determines that, for an occasion of a SR transmission in a PUCCH, the number of symbols available for the PUCCH transmission in a slot is smaller than the value provided by higher layer parameter nrofSymbols, the UE does not transmit the PUCCH in the slot.

SR transmission occasions in a PUCCH are subject to the limitations for UE transmissions described in Subclause 11.1 and Subclause 11.1.1.

The UE transmits a PUCCH in the PUCCH resource for the corresponding SR configuration only when the UE transmits a positive SR. For a positive SR transmission using PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining  as described for HARQ-ACK information transmission in Subclause 9.2.3 and by setting . For a positive SR transmission using PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting .

### 9.2.5	UE procedure for reporting multiple UCI types

If a UE is configured with multiple PUCCH resources in a slot to transmit only semi-persistent or periodic CSI reports

-	If the UE is not provided higher layer parameter multi-CSI-PUCCH-ResourceList, the UE determines a first resource corresponding to a CSI report with the highest priority [6, TS38.214].

-	If the first resource includes PUCCH format 2, and if there are remaining resources in the slot that do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting.

-	If the first resource includes PUCCH format 3 or PUCCH format 4, and if there are remaining resources in the slot that include PUCCH format 2 and do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting.

-	If the UE is provided higher layer parameter multi-CSI-PUCCH-ResourceList, and if any of the multiple PUCCH resources overlap, the UE multiplexes all CSI reports in a resource from the resources provided by multi-CSI-PUCCH-ResourceList, as described in Subclause 9.2.5.2

If a UE would transmit multiple overlapping PUCCHs without repetitions in a slot or overlapping PUCCH(s) without repetitions and PUSCH(s) in a slot and, when applicable as described in Subclauses 9.2.5.1 and 9.2.5.2, the UE is configured to multiplex different UCI types in one PUCCH, and at least one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types if the following conditions are met.

If at least one of the PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE expects that the first symbol of the earliest PUCCH or PUSCH, among the overlapping PUCCHs and PUSCHs in the slot, is not before symbol  after a last symbol of any corresponding PDSCH reception and is not before symbol  after a last symbol of any corresponding PDCCH reception where  is obtained by adding one symbol to a number of symbols  corresponding to a PDSCH reception time for the UE PDSCH processing capability,  is obtained by adding one symbol to a number of symbols  corresponding to a PUSCH preparation time for the UE PUSCH processing capability, where , , , , , and  are defined in [6, TS 38.214]. A UE does not expect a PUCCH or a PUSCH that is in response to a DCI format detection to overlap with any other PUCCH or PUSCH that does not satisfy the above  and  timing conditions.

If a UE would transmit multiple PUCCHs in a slot that include HARQ-ACK, SR and CSI and any PUCCH with HARQ-ACK in the slot satisfies the above  and  timing conditions and does not overlap with any other PUCCH or PUSCH in the slot that does not satisfy the above  and  timing conditions, the UE multiplexes HARQ-ACK, SR and CSI and determines corresponding PUCCH(s) for transmission in the slot according to the following pseudo-code.

Set  to the set of resources for transmission of corresponding PUCCHs in a slot where

-	a resource with earlier first symbol is placed before a resource with later first symbol

-	for two resources with same first symbol, the resource with longer duration is placed before the resource with shorter duration

-	for two resources with same first symbol and same duration, the placement is arbitrary

-	the above three steps for the set  are according to a subsequent pseudo-code for a function

-	if the UE is not provided higher layer parameter simultaneousHARQ-ACK-CSI and resources for transmission of HARQ-ACK information include PUCCH format 0 or PUCCH format 2, resources that include PUCCH format 2, or PUCCH format 3, or PUCCH format 4 for transmission of CSI reports are excluded from the set  if they overlap with any resource from the resources for transmission of HARQ-ACK information

-	if the UE is not provided higher layer parameter simultaneousHARQ-ACK-CSI and at least one of the resources for transmission of HARQ-ACK information includes PUCCH format 1, PUCCH format 3, or PUCCH format 4

-	resources that include PUCCH format 3 or PUCCH format 4 for transmission of CSI reports are excluded from the set

-	resources that include PUCCH format 2 for transmission of CSI reports are excluded from the set  if they overlap with any resource from the resources for transmission of HARQ-ACK information

Set  to be the cardinality of

Set to be the first symbol of resource  in the slot

Set   to be the number of symbols of resource  in the slot

Set  - index of first resource in set

Set  - counter of overlapped resources

while

if any of resources  overlaps with resource

else

if

multiplex UCI for resources  in a single resource as described in Subclauses 9.2.5.1 and 9.2.5.2

set the index of the single resource to

% start from the beginning after reordering unmerged resources at next step

% function that re-orders resources in current set

else

end if

end if

end while

The function  performs the following pseudo-code

{

while  % the next two while loops are to re-order the unmerged resources

while

if  OR

end if

end while

end while

}

For each PUCCH resource in the set  that satisfies the aforementioned  and  timing conditions

-	the UE transmits a PUCCH using the resource if the resource does not overlap in time with a PUSCH transmission, or if the resource is associated with repetitions of a PUCCH transmission over multiple slots

-	the UE multiplexes HARQ-ACK information and/or CSI in a PUSCH if the resource overlaps in time with a PUSCH transmission, as described in Subclause 9.3, and does not transmit SR. In case the resource overlaps in time with multiple PUSCH transmissions, the PUSCH for multiplexing HARQ-ACK information and/or CSI is selected as described in Subclause 9.

Subclauses 9.2.5.1 and 9.2.5.2 assume the following

-	resources for transmissions of UCI types, prior to multiplexing or dropping, overlap in a slot

-	multiplexing conditions of corresponding UCI types in a single PUCCH are satisfied, and

-	the UE does not transmit any time-overlapping PUSCH in the slot

#### 9.2.5.1	UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH

In the following, a UE is configured to transmit  PUCCHs for respective  SRs in a slot, as determined by a set of higher layer parameters schedulingRequestResourceId, with transmission occasions that would overlap with a transmission of HARQ-ACK information from the UE in the slot or with periodic/semi persistent CSI transmission from the UE in the slot.

If a UE would transmit positive SR and at most two HARQ-ACK information bits in a resource using PUCCH format 0, the UE transmits a PUCCH in the resource using PUCCH format 0 in PRB(s) for HARQ-ACK information as described in Subclause 9.2.3. The UE determines a value of  and  for computing a value of cyclic shift  [4, TS 38.211] where  is provided by higher layer parameter initialcyclicshift of PUCCH-format0, and  is determined from the value of one HARQ-ACK information bit or from the values of two HARQ-ACK information bits as in Table 9.2.5-1 and Table 9.2.5-2, respectively.

If the UE would transmit negative SR and at most two HARQ-ACK information bits in a resource using PUCCH format 0, the UE transmits a PUCCH in the resource using PUCCH format 0 for HARQ-ACK information as described in Subclause 9.2.3.

Table 9.2.5-1: Mapping of values for one HARQ-ACK information bit and positive SR to sequences for PUCCH format 0

| HARQ-ACK Value | 0 | 1 |
| --- | --- | --- |
| Sequence cyclic shift |  |  |

Table 9.2.5-2: Mapping of values for two HARQ-ACK information bits and positive SR to sequences for PUCCH format 0

| HARQ-ACK Value | {0, 0} | {0, 1} | {1, 1} | {1, 0} |
| --- | --- | --- | --- | --- |
| Sequence cyclic shift |  |  |  |  |

If a UE would transmit positive or negative SR in a resource using PUCCH format 0 and HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the UE transmits only the HARQ-ACK information bits in the resource using PUCCH format 1.

If the UE would transmit positive SR in a first resource using PUCCH format 1 and at most two HARQ-ACK information bits in a second resource using PUCCH format 1 in a slot, the UE transmits a PUCCH with HARQ-ACK information bits in the firstresource using PUCCH format 1as described in Subclause 9.2.3. If a UE would transmit negative SR in a resource using PUCCH format 1 and at most two HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the UE transmits a PUCCH in the resource using PUCCH format 1 for HARQ-ACK information as described in Subclause 9.2.3.

If a UE would transmit a PUCCH with HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in Subclause 9.2.3,  bits representing a negative or positive SR, in ascending order of the values of schedulingRequestResourceId, are appended to the HARQ-ACK information bits and the UE transmits the combined UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for transmission of HARQ-ACK information bits. An all-zero value for the  bits represents a negative SR value across all  SRs.

If a UE would transmit a PUCCH with periodic/semi-persistent CSI in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot ,  bits representing corresponding negative or positive SR, in ascending order of the values of schedulingRequestResourceId , are prepended to the periodic/semi-persistent CSI information bits as described in Subclause 9.2.5.2 and the UE transmits a PUCCH with the combined UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. An all-zero value for the  bits represents a negative SR value across all  SRs.

If a UE transmits a PUCCH with  HARQ-ACK information bits,  SR bits, and  CRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs provided respectively by higher layer parameter nrofPRBs in PUCCH-format2  or nrofPRBs in PUCCH-format3 and starts from the first PRB from the number of PRBs, that results to  and, if , , where , , , and  are defined in Subclause 9.2.5.2. If , the UE transmits the PUCCH over the  PRBs.

#### 9.2.5.2	UE procedure for multiplexing HARQ-ACK/SR and CSI in a PUCCH

A UE multiplexes HARQ-ACK/SR and periodic/semi-persistent CSI in a same PUCCH if the UE is provided higher layer parameter simultaneousHARQ-ACK-CSI; otherwise, the UE drops the periodic/semi-persistent CSI report(s) and includes only HARQ-ACK/SR in the PUCCH.

For a single transmission occasion of periodic/semi-persistent CSI reports, a PUCCH resource is provided by higher layer parameter pucch-CSI-ResourceList. For multiple transmission occasions of periodic/semi-persistent CSI reports, corresponding PUCCH resources are provided by higher layer parameter multi-CSI-PUCCH-ResourceList.

If a UE is provided only one PUCCH resource set for transmission of HARQ-ACK information in response to PDSCH reception scheduled by a DCI format or in response to a SPS PDSCH release, the UE does not expect to be provided higher layer parameter simultaneousHARQ-ACK-CSI.

A UE is configured by higher layer parameter maxCodeRate a code rate for transmission of HARQ-ACK, SR and periodic/semi-persistent CSI report(s) in PUCCH format 2, PUCCH format 3, or PUCCH format 4.

If a UE transmits CSI reports using PUCCH format 2, the UE transmits only wideband CSI for each CSI report [6, TS 38.214]. In the following, a Part 1 CSI report refers either to a CSI report with only wideband CSI or to a Part 1 of a CSI report with wideband CSI and sub-band CSI.

If a UE transmits periodic/semi-persistent CSI reports that include Part 2 CSI reports, the UE determines a PUCCH resource and a number of PRBs in the PUCCH resource assuming that each of the periodic/semi-persistent CSI reports indicates rank 1. Let  be a total number of UCI bits and  be a total number of CRC bits a UE transmits in a PUCCH, where

-	 is a total number of HARQ-ACK bits, if any

-	 if there is no scheduling request bit; otherwise,  as described in Subclause 9.2.5.1

-	,  is a number of Part 1 CSI report bits for CSI report with priority level ,  is a number of Part 2 CSI report bits, if any, for CSI report with priority level  [6, TS 38.214], and  is a number of periodic/semi-persistent CSI reports

-	,  is a number of CRC bits, if any, for encoding HARQ-ACK, SR and Part 1 CSI report bits, and  is a number of CRC bits, if any, for encoding Part 2 CSI report bits

In the following

-	 is a code rate given by higher layer parameter maxCodeRate as in Table 9.2.5.2-1.

-	 is a number of PRBs for PUCCH format 2, or PUCCH format 3, or PUCCH format 4, respectively, where  is provided by higher layer parameter nrofPRBs in PUCCH-format2 for PUCCH format 2 or by higher layer parameter nrofPRBs in PUCCH-format3 for PUCCH format 3, and  for PUCCH format 4

-	 for PUCCH format 2,  for PUCCH format 3, and  for PUCCH format 4, where  is a number of subcarriers per resource block [4, TS 38.211]

-	 is equal to a number of PUCCH symbols  for PUCCH format 2 provided by higher layer parameter nrofSymbols in PUCCH-format2. For PUCCH format 3 or for PUCCH format 4,  is equal to a number of PUCCH symbols  for PUCCH format 3 or equal to a number of PUCCH symbols  for PUCCH format 4 provided by higher layer parameter nrofSymbols in PUCCH-format3 or nrofSymbols in PUCCH-format4, respectively, after excluding a number of symbols used for DM-RS transmission for PUCCH format 3 or for PUCCH format 4, respectively [4, TS 38.211]

-	 if pi/2-BPSK is the modulation scheme and  if QPSK is the modulation scheme as indicated by higher layer parameter pi2BPSK for PUCCH format 3 or PUCCH format 4. For PUCCH format 2,

If a UE has CSI reports and zero or more HARQ-ACK/SR information bits to transmit in a PUCCH where the HARQ-ACK, if any, is in response to a PDSCH reception without a corresponding PDCCH

-	if the UE is provided by higher layer parameter pucch-CSI-ResourceList or by higher layer parameter multi-CSI-PUCCH-ResourceList with  PUCCH resources, for PUCCH format 2 and/or PUCCH format 3 and/or PUCCH format 4, as described in Subclause 9.2.1, where the resources are indexed according to an ascending order for the product of a number of corresponding REs, modulation order , and configured code rate  where for PUCCH format 4 the number of REs is

-	if , the UE uses PUCCH format 2 resource , or the PUCCH format 3 resource , or the PUCCH format 4 resource ;

-	else if  and , , the UE transmits a PUCCH conveying HARQ-ACK, SR and periodic/semi-persistent CSI report(s) in a respective PUCCH where the UE uses the PUCCH format 2 resource , or the PUCCH format 3 resource , or the PUCCH format 4 resource ;

-	else the UE uses the PUCCH format 2 resource , or the PUCCH format 3 resource , or the PUCCH format 4 resource  and the UE selects  CSI report(s) for transmission together with HARQ-ACK and SR, when any, in ascending priority order as described in [6, TS 38.214].

If a UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports [6, TS38.214] to transmit and the UE determines a PUCCH resource with PUCCH format 3 or PUCCH format 4, where

-	the UE determines the PUCCH resource using the PUCCH resource indicator field [5, TS 38.212] in a last DCI format 1_0 or DCI format 1_1, from DCI formats 1_0 or DCI formats 1_1 that have a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and

-	the UE determines the PUCCH resource set as described in Subclause 9.2.1 and Subclause 9.2.3 for  UCI bits

and

-	if , the UE transmits the HARQ-ACK, SR and periodic/semi-persistent CSI reports bits by selecting the minimum number  of the  PRBs satisfying  as described in Subclauses 9.2.3 and 9.2.5.1;

-	else, the UE selects  CSI report(s) for transmission together with HARQ-ACK and SR in ascending priority order, where  the value of  satisfies  and , where  is a number of CRC bits corresponding to  UCI bits, and  is a number of CRC bits corresponding to  UCI bits.

If a UE has HARQ-ACK, SR and sub-band CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3 or PUCCH format 4, where

-	the UE determines the PUCCH resource using the PUCCH resource indicator field [5, TS 38.212] in a last DCI format 1_0 or DCI format 1_1, from DCI formats 1_0 or DCI formats 1_1 that have a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and

-	the UE determines the PUCCH resource set as described in Subclause 9.2.1 and Subclause 9.2.3 for  UCI bits

and

-	if , the UE transmits the HARQ-ACK, SR and the  periodic/semi-persistent CSI report bits by selecting the minimum number  of PRBs from the  PRBs satisfying  as described in Subclauses 9.2.3 and 9.2.5.1;

-	else,

-	if for  CSI part 2 report priority level(s), it is

and

,

the UE selects the first  CSI part 2 report priority level(s), according to [6, TS 38.214], for transmission together with the HARQ-ACK, SR and  CSI part 1 reports , where  is the number of CSI part 1 report bits for the  CSI report and  is the number of CSI part 2 report bits for the  CSI report priority level,  is a number of CRC bits corresponding to , and  is a number of CRC bits corresponding to ;

-	else, the UE drops all CSI part 2 reports and selects  CSI part 1 report(s), in ascending priority order, for transmission together with the HARQ-ACK and SR bits where the value of  satisfies  and , where is a number of CRC bits corresponding to  UCI bits, and  is a number of CRC bits corresponding to  UCI bits.

Table 9.2.5.2-1: Code rate  corresponding to higher layer parameter maxCodeRate

| maxCodeRate | Code rate |
| --- | --- |
| 0 | 0.08 |
| 1 | 0.15 |
| 2 | 0.25 |
| 3 | 0.35 |
| 4 | 0.45 |
| 5 | 0.60 |
| 6 | 0.80 |
| 7 | Reserved |

### 9.2.6	UCI repetition procedure

For PUCCH formats 1, 3, or 4, a UE can be configured a number of slots, , for a PUCCH transmission by respective higher layer parameters nrofSlots.

For ,

-	the UE repeats the UCI in the PUCCH transmission in the first slot of the  slots in the PUCCH transmission in each of the remaining  slots;

-	a PUCCH transmission has the same number of consecutive symbols, as provided by higher layer parameter nrofSymbols in PUCCH-format1, nrofSymbols in PUCCH-format3, or nrofSymbols in PUCCH-format4, in each of the  slots;

-	a PUCCH transmission has a same first symbol, as provided by higher layer parameter startingSymbolIndex in PUCCH-format1, startingSymbolIndex in PUCCH-format3, or startingSymbolIndex in PUCCH-format4, in each of the  slots;

-	the UE is configured by higher layer parameter interslotFrequencyHopping whether or not to perform frequency hopping for PUCCH transmissions in different slots.

-	If the UE is configured to perform frequency hopping for PUCCH transmissions across different slots,

-	the UE performs frequency hopping per slot;

-	the UE transmits the PUCCH starting from a first PRB provided by higher layer parameter startingPRB in slots with even number and starting from a second PRB provided by higher layer parameter secondHopPRB, in slots with odd number. The slot indicated to the UE for the first PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in slots is counted regardless of whether or not the UE transmits the PUCCH in the slot;

-	the UE does not expect to be configured to perform frequency hopping for a PUCCH transmission within a slot.

-	If the UE is not configured to perform frequency hopping for PUCCH transmissions across different slots and if the UE is configured to perform frequency hopping for PUCCH transmissions within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot.

If the UE determines that, for a PUCCH transmission in a slot, the number of symbols available for the PUCCH transmission is smaller than the value provided by higher layer parameter nrofSymbols for the corresponding PUCCH format, the UE does not transmit the PUCCH in the slot.

If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is additionally provided higher layer parameter tdd-UL-DL-ConfigDedicated for the slot format per slot over the number of slots, as described in Subclause 11.1, the UE determines the  slots for a PUCCH transmission starting from a slot indicated to the UE as described in Subclause 9.2.3 and having

-	an UL symbol or flexible symbol provided by higher layer parameter startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and

-	consecutive UL symbols or flexible symbols, starting from the first symbol, equal to or larger than a number of symbols provided by higher layer parameter nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4.

If a UE is not provided higher layer parameter tdd-UL-DL-ConfigurationCommon, the UE determines the  slots for a PUCCH transmission as the  consecutive slots starting from a slot indicated to the UE as described in Subclause 9.2.3.

If the UE would transmit PUCCH over a first number  of slots and the UE would transmit a PUSCH over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in Subclause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots.