3GPP TSG RAN WG1 Meeting #94																	R1-1809708

revision of R1-1808717

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	Intel Corporation

Title:	Reliability Evaluation for URLLC

Agenda item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

In this contribution, preliminary reliability evaluation results based on IMT-2020 methodology are presented. In section 2, the assumptions are outlined while in section 3 the DL reliability is discussed and evaluated by system level and then by link level modeling of PDCCH and PDSCH.

The IMT-2020 methodology for reliability defined in [1] is followed with the following steps:

- Step 1: Run downlink or uplink full buffer system-level simulations of candidate RITs/SRITs using the evaluation parameters of Urban Macro-URLLC test environment see § 8.4.1 below, and collect overall statistics for downlink or uplink SINR values, and construct CDF over these values. - Step 2: Use the CDF for the Urban Macro-URLLC test environment to save the respective 5th percentile downlink or uplink SINR value. - Step 3: Run corresponding link-level simulations for either NLOS or LOS channel conditions using the associated parameters in the Table 8-3 of this Report, to obtain success probability, which equals to (1-Pe), where Pe is the residual packet error ratio within maximum delay time as a function of SINR taking into account retransmission. - Step 4: The proposal fulfils the reliability requirement if at the 5th percentile downlink or uplink SINR value of Step 2 and within the required delay, the success probability derived in Step 3 is larger than or equal to the required success probability. It is sufficient to fulfil the requirement in either downlink or uplink, using either NLOS or LOS channel conditions.

Assumptions and Pathgain Statistics

First in this section we present the system-level assumptions taken for evaluation and the resulting total gain association statistics. The assumptions are summarized in Table 1.

Table 1. System-level evaluation parameters for SINR derivation.

| Parameters | Urban Macro |
| --- | --- |
| Carrier frequency for evaluation | Config A: 4 GHz Config B: 700 MHz |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 46 dBm per 10 MHz bandwidth |
| UE power class | 23 dBm |
| min UE power | -40 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500 m |
| Number of antenna elements per TRxP | 64 Tx/Rx, (M,N,P,Mg,Ng) = (4,8,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| Number of UE antenna elements | 2 Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°, 90° polarization |
| Device deployment | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor |
| Inter-site interference modeling | Explicitly modelled |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| BS antenna element pattern | According to TR 36.873 |
| UE antenna element gain | 0 dBi |
| UE antenna element pattern | Omni-directional |
| Thermal noise level | -174 dBm/Hz |
| Simulation bandwidth | 40 MHz |
| UE density | 10 UEs per TRxP |
| UE antenna height | 1.5 m |
| Channel model variant | Channel model B, IMT-2020 Urban Macro |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 99° in LCS |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees |
| UE attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min= 10m |
| Traffic model | Full buffer (Note: it is for SINR CDF distribution derivation) |

Further, following the listed assumptions we first derive the results coupling loss / pathgain statistics mainly for calibration purposes as showed in Figure 1 (for Model A on left hand side and Model B on right hand side).

Figure 1. UE useful pathgain statistics, IMT-2020 URLLC UMa Config A/B in channel model A/B.

Link-level evaluation assumptions are summarized in Table 2.

Table 2. Common link-level evaluation parameters

| Parameters | Values |
| --- | --- |
| Channel model | TDL-C, 300 ns delay spread |
| UE speed | 3 km/h |
| System BW | 40 MHz (106 PRB) |
| Numerology | 30 kHz |
| UE antenna | Config A: 4 RX, low correlation Config B: 2 RX, low correlation |
| BS antenna | 2 TX, low correlation |
| TX diversity | PDCCH and PDSCH - based on precoder cycling |
| Channel estimation | Practical |
| PDCCH | 1 symbol CORESET, AL 16 |
| DCI payload | 40 bit + 24 bit CRC |
| PDSCH | 6 symbols after PDCCH Mapping type B 65 PRB |
| PDSCH DMRS | Type 1 No additional DMRS |
| PDSCH MCS | MCS#0 of low SE table (QPSK, CR = 30/1024) TBS = 256 bit |

DL Reliability

## System Level Part (Steps 1-2)

In this section, following the evaluation assumptions in Table 1, the DL geometry for both cases of carrier frequency are derived and showed in Figure 2.

Figure 2. DL geometry SINR, IMT-2020 URLLC UMa Config. A/B in channel model A/B.

From the geometry presented above, the 5-% CDF point provides target SINR summarized in Table 3 below:

Table 3. DL SINR CDF

|  | Configuration A |  | Configuration B |  |
| --- | --- | --- | --- | --- |
|  | Model A | Model B | Model A | Model B |
| 5% SINR, dB | -2.091 | -1.965 | -1.936 | -1.902 |

There is no noticeable difference for two considered configurations (A and B) since overall geometry is interference limited. It should be noted, that this is a single-port SINR, wherein multi-antenna gains are assumed to be accounted in link level evaluations. For the link level part, the worst of the two values (i.e. -2.091 dB and -1.936 for Config A and B respectively) is further taken for analysis.

## Link Level Part (Steps 3-4)

It is assumed that 30 kHz subcarrier spacing is taken in 40 MHz system bandwidth. The required 1 ms latency budget needs to at least account for frame alignment delay, TX delay, and UE processing delay for single-shot transmission. Thus, dividing two slots of 30 kHz onto four parts of 7 symbols from UE perspective may result in maximum scheduling at most 7-symbol one-shot PDSCH, given that the first 7 symbols may be spent to frame alignment, and also N1 symbols may be spent for PDCCH+PSDCH processing, where N1 may be 10 or 4.5 symbols for regular and aggressive processing times respectively.

Figure 3. Sketch of DL transmission structure for reliability evaluation.

Assuming the 7 symbol transmission unit, wherein 1 symbol is allocated to PDCCH CORESET and one symbol is carrying DMRS plus data RE as showed in the figure, the resource allocation to carry TBS = 256 bit using MCS#0 from the low SE 64QAM table requires around 65 PRBs, which are taken for further LLS evaluation.

Figure 4. BLER vs SNR for PDCCH and PDSCH for Config A (4 RX) and Config B (2 RX)

It can be seen, that single-shot performance successfully achieves the requirements in case of both Config A and Config B. Config A obviously has superior performance due to high carrier frequency where 4 RX antennas are available at a UE.

Further, the total reliability (presented as joint BLER) is dominated by PDSCH performance and therefore not very different from single-shot PDSCH curve. Finally, we conclude that at 5-percentile point of DL SINR CDF derived at Step 2 the total DL reliability achieved within 1 ms is better than 99.999% for considered test cases (see Table 4).

Table 4. Summary table

|  | Configuration A |  | Configuration B |  |
| --- | --- | --- | --- | --- |
|  | Model A | Model B | Model A | Model B |
| Target SINR, dB | -2.091 | -1.965 | -1.936 | -1.902 |
| SINR at 99.999% reliability, dB | -7 | -7 | -3.8 | -3.8 |

Conclusions

In this contribution, initial reliability evaluation of Rel.15 NR URLLC based on IMT-2020 methodology for DL is provided. It is showed that the requirements are fulfilled with single-shot transmission of PDCCH+PDSCH for both Configuration A and B in both channel models A and B as summarized in the table below:

|  | Configuration A |  | Configuration B |  |
| --- | --- | --- | --- | --- |
|  | Model A | Model B | Model A | Model B |
| Target SINR, dB | -2.091 | -1.965 | -1.936 | -1.902 |
| SINR at 99.999% reliability, dB | -7 | -7 | -3.8 | -3.8 |

References

- Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”.

- R1-1807760, “Summary on offline discussion on URLLC evaluation method and parameters for IMT-2020 self-evaluation”, Busan, Korea, May 2018.