3GPP TSG RAN WG1 Meeting #94	 R1-1809749

Gothenburg, Sweden, August 20th – 24th 2018

Agenda Item:	7.1.2.4

Source:	Huawei, HiSilicon

Title:	Remaining issues on CSI-RS #1

Document for:	Discussion and decision

Introduction

This paper summarizes the proposals on CSI-RS submitted to RAN1#94.

Remaining issues on CSI-RS

Value range of time domain location l_0

Description:

It was agreed in RAN1#93 to change the value range of l1 as 2..12. When implementing the agreement, the value range of l0 was mistakenly updated as 2..12 as well. It was proposed to correct this typo.

TP #1:

Text proposal for Section 7.4.1.5.3 in 38.211:

< Start of text proposal > The time-domain locations l0∈2, 3, …, 12 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE and defined relative to the start of a slot. < End of text proposal >

Views from companies:

| Huawei, HiSilicon | TP #1 |
| --- | --- |
| Ericsson | TP #1 |
| ZTE | TP #1 |
| LG Electronics | TP #1 |
| Samsung | TP #1 |
| Spreadtrum | TP #1 |
| Qualcomm | TP #1 |
| OPPO | OK with TP1#1. |
| vivo | TP #1 |
| Nokia | TP #1 |
| Intel | OK with TP1#1 |
| CATT | TP #1 |

Possible agreement:

Text proposal for Section 7.4.1.5.3 in 38.211:

< Start of text proposal > The time-domain locations l0∈0, 1, 2, 3, …, 12, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE and defined relative to the start of a slot. < End of text proposal >

Time domain behavior for ZP CSI-RS

Description:

Previously, RRC parameter resourceType was used to indicate time domain behavior of ZP CSI-RS. This parameter has been removed from 38.331, and the time domain behavior is indicated through the parameter name. In addition, the RRC parameter ZP-CSI-RS-FreqBand has also been removed. It was proposed to update the corresponding descriptions in 38.214.

TP #1:

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > - < End of text proposal >

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > - The ZP-CSI-RS time domain behavior can be configured to periodic, semi-persistent or aperiodic by higher parameters within PDSCH-config. All the resources in a ZP CSI-RS resource set are configured with the same time domain behavior ('periodic', 'semi-persistent', 'aperiodic'). < End of text proposal >

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > For a UE configured with a list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList, is configured < End of text proposal >

TP #2 (Ericsson):

<-------------------------------- Begin of text proposal #5 Section 5.1.4.2 of 38.214 --------------------------------->

-	within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) with either aperiodic , semi-persistent or periodic time-domain behaviour (higher layer parameters zp-CSI-RS-ResourceToAddModList in ZP-CSI-RS-ResourceSet aperiodic-ZP-CSI-RS-ResourceSetsToAddModList, sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet), with each ZP-CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource  determines ZP CSI-RS resource configuration identity.

-	NrofPorts defines the number of CSI-RS ports, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	ZP-CSI-RS-FreqBand parameters enabling configuration of frequency occupancy of a ZP-CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211]. If the configured bandwidth is larger than the corresponding BWP, UE shall assume that the actual CSI-RS bandwidth is equal to the BWP size.

-	resourceMapping given by ZP-CSI-RS-Resource defines the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS.

-	resourceType in ZP-CSI-RS-ResourceSet defines the ZP-CSI-RS time domain behavior of ZP-CSI-RS resource configuration as described in Subclause 7.4.1.5 of [4, TS 38.211]. The ZP-CSI-RS-ResourceConfigType can be periodic, semi-persistent or aperiodic. All the resources in a ZP CSI-RS resource set are configured with the same ZP-CSI-RS-ResourceConfigType ('periodic', 'semi-persistent', 'aperiodic').

The UE may be configured with a DCI field for triggering the aperiodic ZP-CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config , is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s)configured (up to 2 bits). Each non-zero codepoint of ZP CSI-RS trigger in DCI triggers one aperiodic ZP-CSI-RS-ResourceSet in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetIds = 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetIds = 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetIds = 3. Codepoint '00' is reserved for not triggering aperiodic ZP CSI-RS.

For a The UE may be configured with the higher layer parameter resourceType set to 'semiPersistent ', a list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList, is configured

-	when the HARQ-ACK corresponding to the PDSCH carrying the activation command [10, TS 38.321] for ZP CSI-RS resource(s) transmitted in slot n, the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from slot n+3Nslotsubframe,µ+1.

-	when the HARQ-ACK corresponding to the PDSCH carrying the deactivation command [10, TS 38.321] for activated ZP CSI-RS resource(s) in slot n, the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from slot n+3Nslotsubframe,µ+1.

<-------------------------------- End of text proposal Section 5..1.4.2 of 38.214 --------------------------------->

Views from companies:

| Huawei, HiSilicon | TP #1 |
| --- | --- |
| Samsung | OK with TP #1 |
| ZTE | OK for TP#1 |
| LGE | Fine with TP#1 |
| Ericsson | TP#2 |
| Huawei, HiSilicon | @Mattias, as the majority view collected thus far seems ok with TP #1, can you please check whether TP #1 is ok to you? My guess is that you added TP #2 simply because TP #1 were not displayed properly and I have changed the display mode back to which the changes can be seen in lines. |
| CATT | Generally ok with TP2. We have two comments here: 1) The parameter freqBand is used to configure the frequency band of ZP CSI-RS in TS38.331. Therefore, the sentence shall be revised instead of removed. 2) The description about the periodic ZP CSI-RS should also be added into spec, such as ‘The UE may be configured with a higher layer parameter p-ZP-CSI-RS-ResourceSet for periodic ZP CSI-RS resources.” |

Possible agreement:

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > - < End of text proposal >

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > - The ZP-CSI-RS time domain behavior can be configured to periodic, semi-persistent or aperiodic by higher parameters within PDSCH-config. All the resources in a ZP CSI-RS resource set are configured with the same time domain behavior ('periodic', 'semi-persistent', 'aperiodic'). < End of text proposal >

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > For a UE configured with a list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList, is configured < End of text proposal >

Power ratio between CSI-RS and PDCCH

Description:

In Section 4.1 of 38.214, the description on assumed power ratio between CSI-RS and PDCCH is ambiguous and incomplete. The ‘PDCCH EPRE’ cannot be a ‘ratio’, and the purpose of having this assumed power ratio is not captured. It is proposed by some companies to refine the wording and also capture the purpose of having this assumption. It is alternatively proposed by some companies to remove the related statement and ask RAN4 to add such assumption in 38.133. It is also proposed by some companies to reintroduce the RRC parameter Pc-PDCCH or fix the power offset to 0 dB for all use cases.

TP #1:

Text proposal for Section 4.1 in 38.214:

< Start of text proposal > TThe ratio of the PDCCH EPRE to NZP CSI-RS EPRE is assumed to be 0 dB, which is used for calculation of hypothetical PDCCH BLER described in [11, TS 38.133]. < End of text proposal >

TP #2:

Text proposal for Section 4.1 in 38.214:

< Start of text proposal > The downlink PDCCH EPRE is assumed as the ratio of the PDCCH EPRE to NZP CSI-RS EPRE and takes the value of 0 dB. < End of text proposal >

Send an LS to RAN4 requesting that the 0 dB power offset assumption for computation of BLER on a hypothetical PDCCH is moved to 38.133.

Proposal #3:

- For the EPRE ratio of PDCCH DMRS and CSI-RS, take one of the following alternatives:

  - Alt. 1: Re-introduce the RRC parameter Pc-PDCCH (RRC impact). For the purpose of link recovery, the UE may assume that the ratio of downlink PDCCH EPRE to NZP CSI-RS EPRE takes the value of 0 dB.

  - Alt. 2: For all purposes, the UE may assume this ratio to be 0 dB.

Views from companies:

| Huawei, HiSilicon | TP #1 |
| --- | --- |
| Ericsson | TP #2. For TP#1, I wonder which CSI-RS resource is referred to? There could be multiple configured to the UE, each with its own power (Pc). Morover, the text describes an assumed power relation, so it should not be a proposal for Section 4.1. Proposal 3, Alt.1 could be a better solution for an assumed power relation. |
| Qualcomm | Proposal #3. If Alt. 2 is picked no change is in specification is needed. Technical Explanation of our position: From UE perspective, being able to explicitly or implicitly assume a known ratio for the PDCCH to CSI-RS is important at least for the following functionalities: - For calculation of BLER of hypothetical PDCCH - For driving the AGC based on the QCLed TRS, - For deriving an SNR estimate for the PDCCH channel estimation block. |
| Samsung | Fine with TP#2 (TP#1 as the second preference). In our understanding, we don’t need to clarify all the summptions for hypothetical PDCCH BLER in RAN1 specs. |
| ZTE | Proposal#3 or keep the current specification. |
| OPPO | OK with TP#2. |
| vivo | Keep the current specification. |
| Nokia | Proposal #3, no change |
| Intel | Proposal#3. We also think that the use case of power offset signalling should be captured. |
| CATT | Agree with Qualcomm that this still should be captured in RAN1 spec, but it is not appropriate to capture this in Power Control Section. As no RRC impact is expected, we suggest to add this to Section5.1.6 of CSI-RS reception procedure as “For the purpose of link recovery, the UE may assume that the ratio of downlink PDCCH EPRE to NZP CSI-RS EPRE takes the value of 0 dB.” |

Possible agreement:

Further offline on power ratio between CSI-RS and PDCCH

Common configuration among CSI-RS resources

Description:

It was agreed in RAN1#92b that all CSI-RS resources within a set are configured with the same starting RB, number of RBs and CDM-type. It is proposed to capture this agreement.

TP #1:

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > All CSI-RS resources within one set are configured with same bwp-Id, same nrofRBs, same startingRB, same cdm-Type, same density and same nrofPorts, except for the NZP CSI-RS resources used for interference measurement. < End of text proposal >

Views from companies:

| Huawei, HiSilicon | TP #1 |
| --- | --- |
| Qualcomm | TP #1 |
| ZTE | TP #1 |
| OPPO | TP #1 |
| LGE | TP #1 |
| vivo | TP #1 |
| Ericsson | TP #1 |
| Nokia | TP #1 |
| Intel | OK with TP#1 |
| CATT | In general, we are ok with TP #1. But current description seems to allow NZP CSI-RS resources for interference measurement to have different bwp-Id, nrofRBs, startingRB. It is not correct. We propose the following TP: All CSI-RS resources within one set are configured with same bwp-Id, same nrofRBs, same startingRB, same cdm-Type, same density. All CSI-RS resources within one set are configured with same nrofPorts, except for the NZP CSI-RS resources used for interference measurement. |

Possible agreement:

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > All CSI-RS resources within one set are configured with same bwp-Id, same nrofRBs, same startingRB, same cdm-Type, same density and same nrofPorts, except for the NZP CSI-RS resources used for interference measurement. < End of text proposal >

Transmit power indication for NZP CSI-RS

Description:

It is reported that there are conflicting descriptions on the definition/usage of Pc, i.e., powerControlOffset. For TRS (CSI-RS for tracking), Pc should be configured as described in Section 5.1.6.1.1 of 38.214:

-	same powerControlOffset and powerControlOffsetSS given by NZP-CSI-RS-Resource value across all resources.

However, as currently captured in Section 5.2.2.3.1 in 38.214, Pc is applicable only when “UE derives CSI feedback”:

powerControlOffset: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size.

To resolve the conflictions, it is proposed to remove the condition of “when UE derives CSI feedback” in the definition of powerControlOffset.

TP #1:

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > - powerControlOffset: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE and takes values in the range of [-8, 15] dB with 1 dB step size. < End of text proposal >

Views from companies:

| Huawei, HiSilicon | TP #1 |
| --- | --- |
| Qualcomm | TP #1. Reply to the comment from ZTE: From UE perspective, being able to explicitly or implicitly assume a known ratio for the PDxCH (PDSCH or PDCCH) to CSI-RS is crucial at least for the following functionalities: - For driving the PDxCH AGC based on the QCLed TRS, - For deriving an SNR estimate for the PDxCH channel estimation block. So, it is NOT true that the UE ignores this parameter if it is TRS. |
| Samsung | Prefer the following correction for clarification: - powerControlOffset: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback or performs time/frequency tracking and takes values in the range of [-8, 15] dB with 1 dB step size. |
| ZTE | Support keeping the current specification. The introduction of powerControlOffset is for calculation of CSI feedback. If we agree the TP, the motivation of this higher layer parameter will not be clear. Since TRS is not used for CSI feedback, the UE should just ignore this parameter for the use of tracking. Keeping the original description would make it clearer. |
| OPPO | Agree with wording from Samsung. |
| vivo | Fine with TP#1 |
| Nokia | TP #1 |
| Intel | Agree with ZTE view |

Possible agreement:

Further offline on transmit power indication for CSI-RS for tracking

Definition of frame number n_f

Description:

It is reported that definition for frame number n_f is missing from the current specs and proposed to add in corresponding sections.

TP #1:

Text proposal for Section 3.2 in 38.211:

< Start of text proposal > System frame number; see clause 4.3.1 < End of text proposal >

Text proposal for Section 4.3.1 in 38.211:

< Start of text proposal > Downlink and uplink transmissions are organized into frames numbered as which is the system frame number, with duration, each consisting of ten subframes of duration. The number of consecutive OFDM symbols per subframe is . Each frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 – 4 and half-frame 1 consisting of subframes 5 – 9. There is one set of frames in the uplink and one set of frames in the downlink on a carrier. < End of text proposal >

Views from companies:

| Huawei, HiSilicon | TP #1 |
| --- | --- |
| Qualcomm | OK |
| Samsung | OK |
| ZTE | TP #1 |
| OPPO | OK |
| LGE | OK |
| Spreadtrum | OK |
| vivo | Fine |
| Ericsson | OK with TP for Section 3.2. Not ok with Section 4.3.1 (since it is already defined elsewhere in specs) |
| Nokia | OK |
| CATT | TP #1 |

Possible agreement:

Text proposal for Section 3.2 in 38.211:

< Start of text proposal > System frame number; see clause 4.3.1 < End of text proposal >

Sharing CSI-RS for different functionalities

Description:

It is proposed not to allow CSI-RS resources in a CSI-RS resource sets with repetition “on” to be reused for other functionalities.

Proposal #1:

- NZP CSI-RS resource IDs contained in a CSI-RS resource set with repetition “on” should not be configured in other CSI-RS resource set(s).

Views from companies:

| vivo | Proposal #1 |
| --- | --- |
| Qualcomm | OK |
| Samsung | Seems non-essential. It is up to gNB implementation. |
| Spreadtrum | OK |
| Ericsson | Not ok. This doesn’t make sense. For example, a CSI-RS resource used for P2 or P3 are not any different. The same resource can be included in a set with repetition either “on” or “off” as long as the sets are triggered at different times. Forcing different resources to be configured creates extra RRC signaling overhead |
| Nokia | Would not be an essential correction/modification |
| CATT | It is not a necessary restriction. CSI-RS resources in a CSI-RS resource set with ‘repetition’ set to ‘ON’ can be used for other purposes(configured in other CSI-RS resource set) as long as they are not triggered/transmitted simultaneously. It is up to gNB implementation. |

Possible agreement:

Further offline on whether to restrict sharing CSI-RS for different functionalities or not

Descriptions of parameters for NZP CSI-RS

Description:

It is proposed to refine the descriptions of parameters for NZP CSI-RS as follows.

TP #1:

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > - repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether the CSI-RS resources within the NZP-CSI-RS ResourceSet are transmitted with the same downlink spatial domain transmission filter can be assumed at the UE side as described in Subclause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or 'none'. - qcl-InfoPeriodicCSI-RS contains a reference to a TCI-State indicating QCL source RS(s) and QCL type(s). If the TCI-State is configured with a reference to an RS with 'QCL-TypeD' association, that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic located in the same or different CC/DL BWP. - trs-Info in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and for which the UE can assume that the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same as described in Subclause 5.1.6.1.1 and can be configured when reporting setting is unconfigured or when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'none'. All CSI-RS resources within one set are configured with same bwp-Id, same density and same nrofPorts, except for the nrofPorts configured for NZP CSI-RS resources for interference measurement. < End of text proposal >

Views from companies:

| CATT | TP #1. |
| --- | --- |
| Huawei, HiSilicon | The last part is not needed, as the densities of NZP CSI-RS resources for IM can be different. |
| OPPO | TP #1 |
| vivo | Agree with HW. |
| Ericsson | Ok in principle but wording needs to be improved. For instance, I wold prefer to phrase it "the UE may assume that the resources.... transmitted with the same spatial filter" |
| Intel | Ok in general. Similar to E/// comment prefer to describe from UE perspective (e.g. same antenna port or QCL) |

Possible agreement:

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > - repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether UE can assume the CSI-RS resources within the NZP-CSI-RS ResourceSet are transmitted with the same downlink spatial domain transmission filter or not as described in Subclause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or 'none'. - qcl-InfoPeriodicCSI-RS contains a reference to a TCI-State indicating QCL source RS(s) and QCL type(s). If the TCI-State is configured with a reference to an RS with 'QCL-TypeD' association, that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic located in the same or different CC/DL BWP. - trs-Info in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and for which the UE can assume that the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same as described in Subclause 5.1.6.1.1 and can be configured when reporting setting is not configured or when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'none'. < End of text proposal >

Bandwidth configuration for NZP CSI-RS

Description:

It is observed that with the current description of CSI-RS bandwidth configuration, there may be cases where the CSI-RS bandwidth is smaller than min⁡(24, NBWPsize). It is then proposed to refine the description.

TP #1:

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > If startingRB<NBWPstart, or if startingRB>NBWPstart and NBWPsize+NBWPstart-startingRB<24, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize). < End of text proposal >

Views from companies:

| LG Electronics | TP #1 |
| --- | --- |
| ZTE | The case seems not practical. We propose to restrict this case, so we suggest the following TP: UE does not expect to be configured with startingRB>NBWPstart and NBWPsize+NBWPstart-startingRB<24, |
| vivo | Agree with ZTE |
| Huawei, HiSilicon | Following the comment from ZTE/vivo, is it correct understanding the last sentence of the paragraph quoted above already ruled out the case under discussion? |
| Ericsson | Not OK, There is no need for this text proposal since the last sentence specifically says that the CSI-RS bandwidth cannot be smaller than min(24, N_BWP^size) which is in-line with past agreements: “In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize).” |
| Nokia | Not needed. Agreed to Ericsson. |
| Intel | Same view as ZTE |
| CATT | It is not necessary. It can be treated as an error case. In addition, we suggest to apply the bandwidth definition of NZP CSI-RS (in section5.2.2.3.1) also to ZP CSI-RS (in section 5.1.4.2) The bandwidth and initial CRB index of a CSI-RS resource within a BWP, as defined in Subclause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize). |

Possible agreement:

No change is needed for CSI-RS bandwidth configuration

Sequence mapping for NZP CSI-RS

Description:

It is proposed to refine the description on sequence mapping of NZP CSI-RS, with different RB indexing methods for different configured densities.

TP #1:

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > For each CSI-RS configured, the UE shall assume the sequence being mapped to resources elements according to when the following conditions are fulfilled: - the resource element is within the resource blocks occupied by the CSI-RS resource for which the UE is configured The reference point for is subcarrier 0 in common resource block 0. The value of is given by the higher-layer parameter density in the CSI-RS-ResourceMapping IE and the number of ports X is given by the higher-layer parameter nrofPorts. If the UE is configured with a higher-layer parameter density equal to one or three in CSI-RS-ResourceMapping IE, If the UE is configured with a higher-layer parameter density equal to 0.5 in CSI-RS-ResourceMapping IE, - For evenPRBs, - For oddPRBs, < End of text proposal >

Views from companies:

| Intel | TP #1 |
| --- | --- |
| Huawei, HiSilicon | Not needed, already clarified in previous meeting. |
| Qualcomm | We also recall this was already clarified before. We can discuss further offline to check if this is not the case. |
| Samsung | Agree with Huawei. |
| LGE | Agree with Huawei |
| Ericsson | Not ok. |
| Nokia | Agree to Huawei |
| CATT | Not needed, for the reference point now is CRB0. |

Possible agreement:

No further clarification is needed for RB indexing in CSI-RS sequence mapping

ZP CSI-RS for multi-slot PDSCH scheduling

Description:

It is proposed to clarify whether ZP CSI-RS applies to all slots occupied by multi-slot PDSCH, when they are triggered together.

Proposal #1:

- For multi-slot PDSCH, the triggered ZP CSI-RS is applied to all the slots of scheduled PDSCH.

Views from companies:

| Spreadtrum | Proposal #1 |
| --- | --- |
| Qualcomm | OK |
| Samsung | Need joint discussion with scheduling agenda. |
| ZTE | OK |
| LGE | The motivation and benefits for Proposal#1 should first be clarified, since it rate-matches unnecessarily the same ZP CSI-RS REs across all the slots. If the motivation is to have multi-slot rate matching without additional DCI overhead, it seems better to reuse the current RRC parameter for slot offset per ZP CSI-RS resource in the set. |
| vivo | Fine with the proposal |
| Ericsson | Not clear. F2F discussion needed |
| Nokia | Further discussion needed. |
| Intel | Need more discussion |
| CATT | OK |

Possible agreement:

Further offline on rate-matching around ZP CSI-RS when it is triggered together with multi-slot PDSCH in a joint offline session with rate-matching colleagues

Duplicated description on common periodicity

Description:

It is observed that the agreement that all CSI-RS resources within one set should have the same periodicity has been captured in both Sections 5.2.2.3.1 and Section 5.1.6.1.2 of 38.214, and then proposed to remove the redundant description in Section 5.1.6.1.2.

TP #1:

Text proposal for Section 5.1.6.1.2 in 38.214:

< Start of text proposal > If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter. < End of text proposal >

Views from companies:

| OPPO | TP #1 |
| --- | --- |
| Qualcomm | OK |
| Samsung | Generally fine but is it essential? |
| ZTE | OK |
| LGE | OK |
| Spreadtrum | OK |
| vivo | OK |
| Ericsson | OK |
| Nokia | OK |
| Intel | OK with the proposal |
| CATT | Ok. |

Possible agreement:

Text proposal for Section 5.1.6.1.2 in 38.214:

< Start of text proposal > If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter. < End of text proposal >

Rate matching on NZP/ZP CSI-RS

Description:

It is reported the UE rate matching behavior around NZP/ZP CSI-RS has not been sufficiently discussed, and in the cases with cross-carrier PDSCH scheduling and same-slot CSI-RS scheduling, it can be difficult to perform rate matching properly.

Proposal #1:

- UE does not explicitly perform rate matching on a PDSCH

  - on the REs overlapped with an aperiodic NZP CSI-RS resource  unless

    - these resources are triggered by an UL DCI in a PDCCH whose last symbol is received at least 14 symbols before the first symbol of this PDSCH in the smallest SCS of the CC’s for the PDSCH and the PDCCH

  - on the REs overlapped with a ZP-CSIRS resource scheduled by a DL DCI other than the one which scheduled this PDSCH.

Views from companies:

| Qualcomm | Updated the proposal #1 with the new proposal in R1-1809504. We are also open for other simplifications (shown in blue in the above proposals), which effectively allow for a UE to perform the correct rate matching in on CC without having to decode all UL DCIs across all CCs, or at least while giving enough time to the UE to complete all the decoding. Note that cross-carrier PDSCH scheduling with (or even without) Search Spacing Sharing is an optional UE capability, and the current PDSCH rate matching behaviour when cross-carrier A-CSIRS triggering occurs is equally complex (without any related agreement in RAN1 to support it, or any associated capability) Question: Why this issue should be newly raised? Similar issue was there in LTE no? Answer: Because we never had an agreement on this for NR. The LTE Rel-14 agreement on this topic was For LTE Rel-14: RAN1#86 Agreement: - Definition of Aperiodic ZP CSIRS: - For PDSCH rate matching on BF CSIRS due to aperiodic CSI-RS and/or multi-shot CSI-RS, an aperiodic ZP CSIRS resource is indicated. - Aperiodic ZP CSIRS resource configuration is defined without Subframe_config UE conducts PDSCH rate matching on aperiodic ZP CSIRS at the subframe when the DCI is signaled. If companies consider NR doing the same as LTE, we are OK, but this needs an agreement, and it effectively translates to the proposal #1 above without the blue text, that is: - UE does not explicitly perform rate matching on a PDSCH on the REs overlapped with an aperiodic NZP CSI-RS resource. To ZTE: It is not about rate matching patterns; this is a separate UE capability completely different than what we describe here. What we are saying is that in the current spec, an UL DCI in some Search Space in some CC may be triggering a A-CSIRS transmission in some other CC in which the UE receives PDSCH, so the UE, in order to do correct rate matching, it has to decode all UL DCIs across all Search spaces in all CCs, so even if the UE is able to rate match 1 A-CSIRS resource, it still has to do all this time-consuming detection across all CCs. We don’t think that this was the intention, because, as we describe a feature of similar complexity, the cross-carrier PDSCH scheduling with/without search space sharing is optional. If the blue sublet is complicated, it could be removed as we explained above, and do what ever LTE is doing as we describe above. On the second bullet, a UE can get multiple PDCCH in the same slot (it’s a UE capability, what we were calling early in the study item a “mini-slot”), and can even get up to 7 TDMed PDSCH (2-symbol each) in one slot. Then, the question is: assume a ZPCSIRS resource set spanning symbols 1-4, and is triggered by DCI1 scheduling PDSCH1 in symbol 1-2, should the UE take this resource set into account for rate matching the PDSCH2 spanning symbols 3-4, or the rate matching of PDSCH2 should only depend on the ZP CSIRS resource set (if any) triggered in the associated DCI2? Proposal for further discussion With regards to the Aperiodic NZP CSI-RS resource rate matching: - Alt. 1: UE does not explicitly perform rate matching on a PDSCH of an Aperiodic NZP CSI-RS resource. - Alt. 2: If an Aperiodic NZP CSIRS resource is triggered by a DCI on the same CC with the scheduling DCI of a PDSCH, UE does not explicitly perform rate matching on this PDSCH unless - Alt. 2b.1: the CSIRS resource is triggered by a DCI in a PDCCH whose last symbol of the monitoring occasion is received no later than that of the PDCCH scheduling the PDSCH - Alt. 2b.2: the CSIRS resource is triggered by a DCI in a PDCCH whose last symbol of the monitoring occasion is received no later than the first symbol of this PDSCH. If the Aperiodic NZP CSIRS resource is triggered by a DCI on a different CC than the CC of the scheduling DCI of a PDSCH - Alt. 2a.1: UE does not explicitly perform rate matching on this PDSCH - Alt. 2a.2: UE does not explicitly perform rate matching on this PDSCH unless CSIRS is triggered by a PDCCH whose last symbol of the monitoring occasion is received at least 14 symbols before the first symbol of this PDSCH in the smallest SCS of the CC for the PDSCH and the PDCCH |
| --- | --- |
| Samsung | We think more discussions are needed (including why this issue should be newly raised up in NR although a similar one has been there from LTE) |
| ZTE | More discussion is needed for the motivation since the number of rate matching patterns is UE capability after all, and UE will reports its capability based on FG2-33a. For the second bullet, do we need this ? In the current 331 for AP ZP CSI-RS resource set, there is no configuration of CC/BWP and slot offset between DCI and ZP-CSI-RS, this means AP ZP CSI-RS scheduled by DCI#0 and PDSCH scheduled by DCI#1 may not be in the same CC and slot. For the first bullet, the sub-bullet seems complicated. |
| Spreadtrum | We have identified similar issue, and proposal was raised in AI 7.1.3.5 as copied below, the tdoc number is R1-1808803, Proposal: During PDSCH reception, UE is not expected to rate match around AP NZP CSI-RS triggered by PDCCH, if the monitoring occasion of the PDCCH is later than that of the PDCCH scheduling the PDSCH. Note that in our proposal, we discussed this issue under single CC case. For xCC case, stronger limitation seems reasonable. For the blue part of proposal #1, in addition to CSI request in UL DCI, A-CSI-RS can also be triggered by SRS request in UL/DL DCI for non-codebook based UL transmission. |
| vivo | Fine with the first sub-bullet. For the second bullet, it needs further clarification on the case ZP CSI-RS indicated in one DCI would influence PDSCH reception indicated in another DCI. |
| Ericsson | We acknowledge a potential problem, more discussion needed F2F. |
| Nokia | We do not sure the necessity, but O.K. to have more discussion. |
| Intel | Agree with the problem. Need more discussion on the solution. |
| CATT | Agree with Qualcomm that more detailed discussion is needed on this NZP/ZP CSI-RS rate matching topic. |

Possible agreement:

Further offline on rate-matching around aperiodic NZP or ZP CSI-RS when cross-carrier triggering is involved in a joint offline session with rate-matching colleagues

Notation/definition of OCC sequences

Description:

It is reported the existing notation/definition of OCC sequences in Table 7.4.1.5.3-2/3/4/5 are incorrect/misleading (what is provided in the table is wf or wt themselves, instead of the k'-th or l'-th elements of them) and proposed to revise the tables as below.

TP #1:

Text proposal for Section 5.1.6.1.2 in 38.214:

< Start of text proposal > Table 7.4.1.5.3-2: The sequences and for cdm-Type equal to 'no CDM'. Index; wf0; wt0. 0; 1; 1. Table 7.4.1.5.3-3: The sequences and for cdm-Type equal to 'FD-CDM2'. Index; wf0wf1; wt0. 0; 1. 1; 1. Table 7.4.1.5.3-4: The sequences and for cdm-Type equal to 'CDM4'. Index; wf0wf1; wt0wt1. 0. 1. 2. 3. Table 7.4.1.5.3-5: The sequences and for cdm-Type equal to 'CDM8'. Index; wf0wf1; wt0wt1wt2wt3. 0. 1. 2. 3. 4. 5. 6. 7. < End of text proposal >

Views from companies:

| Huawei, HiSilicon | TP #1 |
| --- | --- |
| Nokia | TP #1 |
| Intel | Seems OK |

Possible agreement:

Text proposal for Section 5.1.6.1.2 in 38.214:

< Start of text proposal > Table 7.4.1.5.3-2: The sequences and for cdm-Type equal to 'no CDM'. Index; wf0; wt0. 0; 1; 1. Table 7.4.1.5.3-3: The sequences and for cdm-Type equal to 'FD-CDM2'. Index; wf0wf1; wt0. 0; 1. 1; 1. Table 7.4.1.5.3-4: The sequences and for cdm-Type equal to 'CDM4'. Index; wf0wf1; wt0wt1. 0. 1. 2. 3. Table 7.4.1.5.3-5: The sequences and for cdm-Type equal to 'CDM8'. Index; wf0wf1; wt0wt1wt2wt3. 0. 1. 2. 3. 4. 5. 6. 7. < End of text proposal >

Agreements in RAN1#93 in CSI-RS agenda

Agreement (Impact to RAN2 specifications)

Text proposal #3 for Section 5.2.2.3.1 in TS38.214

< Unchanged parts are omitted >

The bandwidth and initial CRB index of a CSI-RS resource within a BWP, as defined in Subclause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min24, NBWPsize.

< Unchanged parts are omitted >

Include as part of LS to RAN2.

Agreement (Impact to RAN2 specifications)

Change the value range of nrofRBs in CSI-RS bandwidth configuration from 24 to maxNrofPhysicalResourceBlocks + 1 and include as part of an LS to RAN2. Ask RAN2 to update the relevant description.

Agreement

UE expects to be configured with same BWP_info for all CSI-RS resources within one set.

UE expects to be configured with same density for all CSI-RS resources within one set.

Agreement

- For P/SP-CSI-RS, UE expects to be configured with same periodicity for all CSI-RS resources within one set (slot offset can still be different for different CSI-RS resources).

Agreement

Text proposal for Section 5.1.6.1.2 in 38.214

If the UE is configured with a CSI-ReportConfig with reportQuantity set to "cri-RSRP", or "none" and if the CSI-ResourceConfig for channel measurement (higher layer parameter resourcesForChannelMeasurement) contains a NZP-CSI-RS-ResourceSet that is configured with the higher layer parameter repetition  and configured without the higher layer parameter trs-Info, the UE can only be configured with the same number (1 or 2) of  ports with the higher layer parameter nrofPorts for all CSI-RS resources within the set,

Agreement

UE expects to be configured with same number of ports for all CSI-RS resources within one set.

Agreement

Text proposal for Section 7.4.1.5.2 in TS38.211

< Unchanged parts are omitted >

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

at the start of each OFDM symbol where  is the slot number within a radio frame,  is the OFDM symbol number within a slot, and  equals the higher-layer parameter scramblingID.

< Unchanged parts are omitted >

Note: Inside the equation, the number of 14 is replaced as N_symb^slot.

Agreement

Suggest RAN2 to change the value range of firstOFDMSymbolInTimeDomain2 as from 2 to 12 and include as part of an LS to RAN2.

Agreement

Text proposal for Section 7.4.1.5.1 in 38.211

< Unchanged parts are omitted >

Zero-power and non-zero-power CSI-RS are defined

-	for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3

for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission. UE performs the same measurement/reception on channels/RSs except PDSCH regardless of whether they collide with ZP CSI-RS or not.

< Unchanged parts are omitted >

Working Assumption

Text proposal for TS38.214

If the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that a PDCCH DM-RS transmitted in the all search space sets associated with CORESET CORESET are quasi co-located with ‘QCL-Type D’, if ‘QCL-Type D’ is applicable. Furthermore, the UE shall not expect to be configured with CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s).

Agreements in RAN1#92b in CSI-RS agenda

Agreement:

Confirm the WA on CSI-RS sequence mapping for the case with 0.5 density

Agreement:

A UE may assume all CSI-RS ports within one CSI-RS resource are quasi co-located with respect to ‘QCL type A’ and ‘QCL type D’ when applicable.

Agreement:

Text proposal for Section 5.2.2.3.1 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211] , and the minimum bandwidth is the minimum between 24 and   resource blocks.

----------------------------------------------------------------------------------------------------------------------------------

Agreement

Text proposal for Section 5.1.6.1.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

If the UE is configured with a CSI-ReportConfig with reportQuantity set to “CRI/RSRP”, or “No Report” and if the Resource Setting for channel measurement contains a CSI-RS Resource Set that is configured with the higher layer parameter CSI-RS-ResourceRep and configured without the higher layer parameter TRS-Info, the UE can only be configured with 1 or 2 ports with the higher layer parameter NrofPorts for all CSI-RS resources within the set, and the UE may be configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, and the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with ‘QCL-TypeD’ if ‘QCL-TypeD‘ is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the SS/PBCH block, and the UE shall expect that the same subcarrier spacing is used for both the CSI-RS and the SS/PBCH block.

Agreement

The number of ports in a resource set for L1-RSRP

- Always have the same number of ports (1 or 2) across all resources

Agreement

For density ½, odd/even PRB allocations is respect to the common resource block grid

Include in LS to RAN2 for CSI-RS Xi (Huawei)

Agreement

Text proposal for Section 5.1.4.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

- ZP-CSI-RS-ResourceConfigId determines ZP CSI-RS resource configuration identity.

- NrofPorts defines the number of CSI-RS ports, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

- ZP-CSI-RS-Density defines the zero-power CSI-RS frequency density, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

- CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

----------------------------------------------------------------------------------------------------------------------------------

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211]. If the configured bandwidth is larger than the corresponding BWP, UE shall assume that the actual CSI-RS bandwidth is equal to the BWP size.

Agreement

- The UE expects that all the CSI resources of a resource set are configured with the same starting RB and number of RBs and the same CDM-type.

  - FFS: Other parameters

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214

--------------------------------------------------------------------------------------------------------------------------

-	CSI-RS-Density defines CSI-RS frequency density of each CSI-RS port per PRB and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. For density ½, odd/even PRB allocations is respect to the common resource block grid.

-	CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211].

-	Pc: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size.

-	Pc_SS: which is the assumed ratio of SS/PBCH block EPRE to NZP CSI-RS EPRE.

-	ScramblingID: defines scrambling ID of CSI-RS with length of 10 bits.

-	CC_Info defines which component carrier the configured CSI-RS is located in.

- 	BWP_Info defines which bandwidth part the configured CSI-RS is located in.

--------------------------------------------------------------------------------------------------------------------------

Agreement

Text proposal for Section 5.1.4.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

-	ZP-CSI-RS-ResourceMapping defines the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211].

----------------------------------------------------------------------------------------------------------------------------------

R1-1805740	[Draft] LS on NR CSI-RS	Huawei, HiSilicon

LS is endorsed in R1-1805626

Agreements in RAN1#92 in CSI-RS agenda

Agreement (RRC parameter update):

maxNrofCSI-IM-Resources                          INTEGER ::= 32        -- Maximum number of CSI-IM resources per CC. See CSI-IM-ResourceMax in 38.214.

Agreement (RRC parameter update):

Revise the higher-layer parameter CSI-RS-ResourceRep defined in TS 38.331 to be an optionally configurable parameter (not mandatory parameter as in the current specification), and include this into an LS to RAN2.

- CSI-RS-ResourceRep can only be configured for CSI-RS resource sets which are associated with CSI report with  report of L1 RSRP or “no report”

Agreement:

For the CSI-RS sequence mapping, the working assumption from RAN1 NR Ad Hoc 1801 for density 1 and 3 is confirmed

Agreement:

- By default, UE does not perform rate matching on REs overlapped with at least CSI-RS for mobility

  - Note: UE shall perform rate matching on REs overlapped with a CSI-RS for mobility only if ZP-CSI-RS covers the REs overlapped with the CSI-RS for mobility.

Conclusion:

UE performs the same measurement on NZP CSI-RS or CSI-IM regardless of whether they collide with ZP CSI-RS or not.

Agreement:

- The formula for SRS can be reused for CSI-RS:

It’s up to the editor how to reflect the above agreement into the specification.

Agreement (RRC parameter update):

maxNrofNZP-CSI-RS-Resources-1 is 191

Agreements in RAN1#AH1801 in CSI-RS agenda

Conclusion:

On the support of indicating two starting OFDM symbol positions for non-contiguous NZP CSI-RS mapping, include as part of an LS to RAN2 to inform them that the RRC signalling should support indication of up to two starting OFDM symbol positions and it is up to RAN2 to decide on the detailed signalling (relevant RRC parameter: CSI_RS_resourcemapping)

Agreement:

Remove Pc-PDCCH from RRC parameter list for NZP-CSI-RS and add clarification on 0dB power offset between CSI-RS for beam failure detection and PDCCH DMRS

Agreement:

- Indication mechanism of aperiodic ZP CSI-RS is RRC + DCI

- Each triggering state of “ZP CSI-RS trigger” in DCI is to trigger one ZP CSI-RS resource set

  - A state is reserved for not triggering aperiodic ZP CSI-RS

- All the resources in a ZP CSI-RS resource set are configured with the same ResourceConfigType (periodic or aperiodic)

- The bit-length of DCI field “ZP CSI-RS trigger” depends on the number of aperiodic ZP CSI-RS resource sets configured (up to 2 bits)

- The maximum number of aperiodic ZP CSI-RS resource sets configured per BWP is 3

Include as part of LS to RAN2

Agreement:

- Add the following RRC parameters for ZP CSI-RS

| PDSCH | 38.214 |  |  |  | ZP-CSI-RS-ResourceSetConfigList | New | ZP-CSI-RS-ResourceSetConfigList | Resource set configuration for ZP CSI-RS |  | NA | UE specific | 38.331 |  | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH | 38.214 |  |  |  | ZP-CSI-RS-ResourceSetConfigId | New | ZP-CSI-RS-ResourceSetConfigId | ZP CSI-RS Resource set configuration ID | 0 .. ZP-CSI-RS-ResourceSetMax-1 | NA | UE specific | 38.331 | Contained in ZP-CSI-RS-ResourceSetConfig | Y |

- Update the RRC parameter Aperiodic-ZP-CSI-RS-Resource-List as follows

| Aperiodic-ZP-CSI-RS-Resource-List | Contains list of ZP-CSI-RS resource set IDs for aperiodic triggering | Up to 3 ZP-CSI-RS resource set IDs per BWP |
| --- | --- | --- |

Include as part of LS to RAN2

Agreement:

- Introduce additional periodicities of {4,8,16,32,64} slots and the corresponding slot offsets to at least the following periodic/semi-persistent RS:

  - CSI-RS (includes ZP-CSI-RS and NZP-CSI-RS)

  - SRS

- FFS on whether and how to align to additional periodicities as agreed in the frame structure agenda

  - “The two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms”

- FFS on CSI-IM periodicity of 1 slot

Include as part of LS to RAN2

Working Assumption

Agree to the following text for TS.38.211 with the understanding that the 1 port rate=1/2 case needs to be checked. If there is an issue with the working assumption for 1 port rate=1/2, technical details will be modified.

7.4.1.5.3	Mapping to physical resources

For each CSI-RS component configured, the UE shall assume the sequence  being mapped to physical resources according to

under the condition that the resource elements indexed by are within the resource blocks occupied by the CSI-RS for which the UE is configured. The reference point for  is subcarrier 0 in common resource block 0. The value of ρ is given by the higher-layer parameter CSI-RS-Density.

Agreement:

- By default, UE does not perform rate matching on REs overlapped with at least CSI-RS for mobility, which is not configured with serving cell ID

  - UE shall perform rate matching on REs overlapped with a CSI-RS for mobility only if ZP-CSI-RS covers the REs overlapped with the CSI-RS for mobility.

  - FFS, rate matching on REs overlapped with CSI-RS for mobility configured with serving cell ID

Agreements in RAN1#91 in CSI-RS agenda

Agreement

Support reusing the allowed CSI-RS-to-PDSCH power offset values in LTE for NR.

Agreement:

- Support including the OFDM symbol index within a slot in the formula for c_init.

- Support including slot index within a radio frame in the formula for c_init.

Agreement:

Length-31 Gold sequence is used for CSI-RS

- Same polynomial as in LTE

- QPSK sequence modulation is used

- N_c and c_init are to be discussed separately

Agreement:

For NR CSI-RS sequence,

- CSI-RS scrambling ID has a length of 10 bits

- There is no default value for the scrambling ID

Agreement

Introduce the following RRC parameter for CSI-RS:

| CC/BWP-Info | Indication of which CC/BWP the configured CSI-RS is located in This parameter belongs within a CSI-RS resource configuration or in a BWP configuration (up to editor) | FFS |
| --- | --- | --- |

Agreement

Introduce the following RRC parameter for CSI-RS:

- Comb offset for D=1/2

  - 1 bit to indicate between odd and even RBs

Agreement

UE rate matches PDSCH around ZP-CSI-RS

Agreement

| CSI-RS-ResourceMapping | Include parameters to capture OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource FFS: how to configure CSI-RS in different slots for fine time/frequency tracking | Starting subcarrier: For 1 port CSI-RS, there is no restriction For Y=2, is constrained to be one among even subcarriers, in the given PRB (indexed from 0) For Y=4, is constrained to be one among subcarriers 0, 4, 8, in the given PRB (indexed from 0) Symbol location: {0,1,2,3,4,5,6,7,8,9,10,11,12,13}, where 2 is supported only when DL-DMRS-typeA-pos equals 3 - UE is not expected to receive CSI-RS and DMRS on overlapping REs - Only uniform RE pattern across all symbols for CSI-RS resource is supported |
| --- | --- | --- |
| CDMType | Includes parameters to capture CDM value (1, 2, 4, or 8), CDM pattern (freq only, time and freq, time only) | Agreed CDM types for different X and N {No CDM} for X = 1 and N = 1 {FD-CDM2} for X = 2 and N = 1 {FD-CDM2} for X = 4 and N = 1 {FD-CDM2} for X = 8 and N = 1 {FD-CDM2, CDM4 (FD2,TD2)} for X = 8 and N = 2 {FD-CDM2} for X = 12 and N = 1 {CDM4 (FD2,TD2)} for X = 12 and N = 2 {FD-CDM2, CDM4 (FD2,TD2)} for X = 16 and N = 2 {FD-CDM2, CDM4 (FD2, TD2), CDM8 (FD2, TD4)} for X = 24 and N = 4 {FD-CDM2, CDM4 (FD2, TD2), CDM8 (FD2, TD4)} for X = 32 and N = 4 |
| CSI-RS-FreqBand | Includes parameters to enable configuration of wideband and partial band CSI-RS | Combined indication methods from [90b-NR-19] Starting RB index and number of spanned RBs in the units of 4 - Minimum CSI-RS BW is min(24RBs, BWP for data) |
| Pc_SS | Power offset of NZP CSI-RS RE to SS RE Note: This parameter is optional | New parameter 2 bits in the range of [-3, 6] with step size of 3dB |
| ZP-CSI-RS-ResourceMapping | Include parameters to capture OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot | A list of NZP-CSI-RS resource mapping(s) by explicit configuration of time and frequency domain information |
| ZP-CSI-RS-timeConfig | Contains periodicity and slot offset for periodic ZP-CSI-RS | Same as NZP-CSI-RS |
| ZP-CSI-RS-FreqBand | Includes parameters to enable configuration of wideband and partial band ZP-CSI-RS | Same as NZP-CSI-RS |
| Aperiodic-ZP-CSI-RS-Resource-List | Contains list of ZP-CSI-RS resource IDs for aperiodic triggering |  |
| CC-Info | Indication of which CC the configured CSI-RS is located in. This parameter belongs within a CSI-RS resource configuration or in a CC configuration (up to editor) | How to capture this in the specification is up to the editor. This parameter applies to both NZP-CSI-RS and ZP-CSI-RS. |
| BWP-Info | Indication of which BWP the configured CSI-RS is located in. This parameter belongs within a CSI-RS resource setting configuration | How to capture this in the specification is up to the editor. This parameter applies to both NZP-CSI-RS and ZP-CSI-RS. |

Agreement

Introduce parameter PC-PDCCH which has a fixed value of 0dB and indicates the power offset of PDCCH and CSI-RS

Agreement

In addition to agreed RRC parameters for ZP-CSI-RS, following RRC parameters are added for ZP CSI-RS configuration.

| ZP-CSI-RS-Density | Density of ZP CSI-RS resource in frequency domain = RE pattern existence per PRB (1 PRB = 12 subcarriers and 1 sym) | Same as NZP CSI-RS resources |
| --- | --- | --- |
| ZP-CSI-RS-ResourceConfigId | ZP-CSI-RS resource configuration ID | 0 .. ZP-CSI-RS-ResourceMax - 1 |
| ResourceConfigType | Time domain behavior of resource configuration | aperiodic or periodic |

Agreements

- Support mapping CSI-RS sequence to the resource grid at RE-level granularity

- Same sequence for all CSI-RS ports on one symbol within a CSI-RS resource

Agreements:

- Support assigning CSI-RS port index across CDM groups first in frequency domain and then in time domain

Agreement

For NR CSI-RS sequence initialization,

    - Similar to LTE, the sequence initialization values should be different for all OFDM symbols within a frame (10ms)

cinit=210×14ns+l+12NID+1+NID mod 231,

where ns denotes slot index within a radio frame, and l is OFDM symbol index within a slot and NID∈{0,1,…,210-1} is UE specifically configured scrambling ID

Agreement:

- For Rel-15, the number of APs configured for one NZP CSI-RS resource (P) is equal to number of APs configured for CSI acquisition and reporting, i.e., N1*N2*2 = P (analogous to LTE)

Agreements:

- NR supports aperiodic ZP-CSI-RS for rate matching, with a separate DCI field for triggering aperiodic ZP-CSI-RS

- References

R1-1808144, Remaining issues on reference signals and QCL, Huawei, HiSilicon

R1-1808197, Maintenance for Reference signals and QCL, ZTE

R1-1808222, Remaining issues on reference signals and QCL, vivo

R1-1808312, Corrections on SRS antenna ports, Fujitsu

R1-1808376, Issues on reference signals, CATT

R1-1808488, Text proposals on reference signals, LG Electronics

R1-1808670, Corrections to reference signals and QCL, Intel Corporation

R1-1808751, Corrections on reference signals and QCL, Samsung

R1-1808801, Remaining issues on Reference signals, Spreadtrum Communications

R1-1808831, Discussion on remaining issues on SRS, CMCC

R1-1808881, Text proposals for RS and QCL, Guangdong OPPO Mobile Telecom

R1-1809107, Remaining issues on SRS, Sharp

R1-1809139, Maintenance for Reference signals and QCL, NTT DOCOMO, INC.

R1-1809198, Maintenance for RS and QCL, Ericsson

R1-1809238, Remaining issues on reference signals and QCL, Nokia, Nokia Shanghai Bell

R1-1809424, Maintenance for RS, Qualcomm Incorporated