3GPP TSG RAN WG1 Meeting #94		R1-1809785

# Gothenburg, Sweden, August 20th – 24th, 2018

Source:	Convida Wireless

Title:	Maintenance for Physical random access channel

Agenda Item:	7.1.1.2

# Document for:     Discussion and Decision

# Background

This document summarizes the submitted contributions for RAN1#94 for AI 7.1.1.2, " Maintenance for Physical random access channel and procedure".

Let us focus on the essential issues!

The key issues are listed below:

- Collision between SSB and PRACH Occasion (validation of ROs in unpaired spectrum)

- RACH configuration for handover

- Text proposals for 38.211

  - Corrections (editorial) to RACH configuration tables

  - Correction of PRACH signal generation

# Collision between SSB and PRACH Occasion (validation of ROs in unpaired spectrum)

## Validation of RO(s)

Two proposals regarding validation of RO(s) have been submitted. They propose to make some updates to the current wording in 38.213, section 8.1. As can be seen from the below proposals there are some contradictions. Ericsson propose that in general for unpaired spectrum RO(s) are invalid if an RO precedes an SSB within a RACH slot, while Mediatek propose that all RO(s) are valid if a UE is not provided with the higher layer parameter tdd-UL-DL_ConfigurationCommon.

Offline proposal:

Adopt the following text proposal for 38.213, section 8.1:

For paired spectrum all PRACH occasions are valid.

For unpaired spectrum, if a UE is not provided higher layer parameter tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least symbols after a last downlink symbol and at least symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Ericsson: For unpaired spectrum, a PRACH occasion in a PRACH slot is invalid if it precedes a SS/PBCH block in the PRACH slot or starts less than symbols after a last SS/PBCH block transmission symbol, where is provided in Table 8.2-2. If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, a PRACH occasion in a PRACH slot is also invalid if - it is not within UL symbols, orand - it does not precede a SS/PBCH block in the PRACH slot and starts at leastless than symbols after a last downlink symbol and at least symbols after a last SS/PBCH block transmission symbol, where is provided in Table 8.2-2.

Mediatek If a UE is not provided higher layer parameter tdd-UL-DL-ConfigurationCommon, all PRACH occasions are valid. If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, a PRACH occasion in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least symbols after a last downlink symbol and at least symbols after a last SS/PBCH block transmission symbol, where is provided in Table 8.2-2.

Based on the above proposals we have the following alternatives.

- Alt1: No need to update the current wording

  - Supported by:

- Alt2: Add MediaTeks wording

  - Supported by:

- Alt3: Use Ericsson’s proposed wording

  - Supported by:

## Clarification that only valid RO(s) are used when performing the SSB-RO association (handled in RACH procedure summary)

One proposal from Samsung regarding adding a note in 38.213 under table 8.1-2, the intention is to clarify that only valid RO(s) shall we used when performing the SSB-RO association. As in the current version of 38.213 section 8.1 (Random access preamble) the RO-SSB association is described before the RO validation rule. So, adding a note in the end of the section is proposed to reduce any potential misunderstandings when reading section 8.1.

++++++++++++++++++ Text proposals for 38.213, section 8.1 ++++++++++++++

<omitted>

For preamble format B4 [4, TS 38.211], .

Table 8.1-2:  values for different preamble subcarrier spacing configurations

| Preamble subcarrier spacing |  |
| --- | --- |
| 1.25 kHz or 5 kHz | 0 |
| 15 kHz or 30 kHz or 60 kHz or 120 kHz | 2 |

Note: only the valid RACH Occasions are used for performing SSB-RO association.

<omitted>

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

- Alt1: Adopt the text proposal from Samsung

- Alt2: No need to add the note

## Related text proposals (reference only)

The proposals from the companies are added below for reference:

MediaTek:

---------------------------- Text Proposal Part-2 begins ----------------------------------------------------------------------------

Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16} |
| 20 | {1, 2, 4, 8} |
| 40 | {1, 2, 4} |
| 80 | {1, 2} |
| 160 | {1} |

If a UE is not provided higher layer parameter tdd-UL-DL-ConfigurationCommon, all PRACH occasions are valid.

If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least symbols after a last downlink symbol and at least symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

---------------------------- Text Proposal Part-2 ends -------------------------------------------------------------------------------

Ericsson:

Currently, 3GPP TS 38.213 [1] specifies that a PRACH occasion colliding with an SSB is invalid only if semi-statically configured UL/DL is used (since the invalidation of PRACH occasions is in the specification text conditioned on the presence of the tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationCommon2). However, PRACH collisions with SSB should be generally prohibited whenever unpaired spectrum (TDD) is used, since in that case the TRP cannot listen to PRACH while transmitting SSB.

3GPP TS 38.213 V15.2.0 specifies that PRACH colliding with (or preceding) SSB is invalidated only if semi-statically configured UL/DL is used.

- PRACH occasions colliding with (or preceding) SSB should be invalid whenever unpaired spectrum (TDD) is used. To this end, the provided text proposal below for 38.213, Section 8.1, should be adopted.

------------------------------------------------ Start of TP to 38.213, Section 8.1 ----------------------------------

For unpaired spectrum, a PRACH occasion in a PRACH slot is invalid if it precedes a SS/PBCH block in the PRACH slot or starts less than symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, a PRACH occasion in a PRACH slot is also invalid if

-	it is not within UL symbols, orand

-	it does not precede a SS/PBCH block in the PRACH slot and starts at leastless than  symbols after a last downlink symbol and at least symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.2-2.

------------------------------------------------End of TP to 38.213, Section 8.1 --------------------------------------

Samsung:

SSB-RO association clarification

In 38.213, the SSB-RO association rule is introduced before the validation rule is presented. There might be a misunderstanding that the valid ROs is determined after the SSB-RO association, which is not the correct understanding. Thus the following proposal is given.

Proposal 3: add one note under following content in section 8.1 of 38.213:

<omitted>

For preamble format B4 [4, TS 38.211], .

Table 8.1-2:  values for different preamble subcarrier spacing configurations

| Preamble subcarrier spacing |  |
| --- | --- |
| 1.25 kHz or 5 kHz | 0 |
| 15 kHz or 30 kHz or 60 kHz or 120 kHz | 2 |

Note that only the valid RO(s) is used for performing SSB-RO association.

<omitted>

# RACH configuration for handover

## Summary of Monday offline

Offline discussion, no offline proposals was made. The following bullets summarize what was discussed but not concluded. More dicussions are needed to finalize handover conditions in unpaired spectrum.

- For handover purpose in unpaired spectrum and same frequency carrier there is no need to specify any conditions regarding PRACH configuration, timing assumption and association period.

- Further discuss for the inter frequency handover case in unpaired spectrum.

Comments:

For unpaired spectry and same frequencey carrier: There is an agreement, that UE may assume that in TDD bands useServingCellTimingForSync is assumed to be set to TRUE. For intra-frequency measurements, UE may use the serving cell timing to derive the SSB index of neighbor cells in the same frequency layer.

Way forward:

- Continue offline discussions using the following tables as a starting point (provided by LGE).

- At least make agreement for below 6GHz case within this meeting.

| To From |  | b3GHz |  | 3to6GHz | a6GHz |
| --- | --- | --- | --- | --- | --- |
|  |  | FDD | TDD | TDD | TDD |
| b3GHz | FDD | +/- 5ms | +/- 5ms | +/- 5ms | For SFN, +/- 5ms (frame boudnary information) How to obtain SSB index?? |
|  | TDD | +/- 5ms | . In same frequency layer: synchronized . In different frequency layer: +/- 5ms | +/- 2.5ms | For SFN, +/- 2.5ms (frame boudnary information) How to obtain SSB index?? |
| 3to6GHz | TDD | +/- 5ms | +/- 5ms | . In same frequency layer: synchronized . In different frequency layer: +/- 2.5ms | For SFN, +/- 2.5ms (frame boudnary information) How to obtain SSB index?? |
| a6GHz | TDD | +/- 5ms | +/- 5ms | +/- 2.5ms | . In same frequency layer: synchronized . In different frequency layer: 1) If success of PBCH decoding of certain a cell can be garantee, synchronized 2) Timing infomration of target cell could be obtained by PBCH decoding after HO |

|  |  | b3GHz |  | 3to6GHz | a6GHz |
| --- | --- | --- | --- | --- | --- |
|  |  | FDD | TDD | TDD | TDD |
| A) Same frequency layer | Sync assumption |  | TDD-TDD (same frequency layer): sync | TDD-TDD (same frequency layer): sync | TDD-TDD (same frequency layer): sync |
| B) Same frequency layer | 2) SSB index for SSB selection | SSB index can be obtained before HO | SSB index can be obtained before HO | SSB index can be obtained before HO | 1) If success of PBCH decoding of certain a cell can be guarantee, UE may obtain SSB index before HO. 2) Else, Timing infomration of target cell could be obtained by PBCH decoding after HO |
|  | 3-1) SFN index for SSB to RO mapping | Frame boundary of target cell can be obtained before HO (PBCH-DMRS) Also, gNB should guarantee the timing accuracy as belows: . FDD-FDD: +/- 5ms . TDD-FDD: +/- 5ms | Frame boundary of target cell can be obtained before HO (PBCH-DMRS) Also, gNB should guarantee the timing accuracy as belows: . FDD-TDD: +/- 5ms . TDD-TDD: +/-2.5ms | Frame boundary of target cell can be obtained before HO (CSI-RS or +/-2.5ms assumption) Also, gNB should guarantee the timing accuracy as belows: . FDD-TDD: +/- 5ms . TDD-TDD: +/-2.5ms | Frame boundary of target cell can be obtained before HO (CSI-RS or +/-2.5ms assumption) Also, gNB should guarantee the timing accuracy as belows: . FDD-TDD: +/- 5ms . TDD-TDD: +/-2.5ms --> But, this assumption may not be necessity when above assumption '2)' is considered. |
|  | 3-2) RACH Configuration | RACH configration period: 10ms Mapping pattern period: x = 1 | RACH configration period: 10ms Mapping pattern period: x = 1 | RACH configration period: 10ms Mapping pattern period: x = 1 | TDD-TDD (different frequency layer): RACH configration period: 10ms Mapping pattern period: x = 1 (Before HO, ssb-PositionsInBurst in ServingCellConfigCommon is used for SSB-to-RO mapping. In order for RACH transmission, SSB based measurement should be operated. However, SSB index of target cell may be or may be not obtained. ) --> This condition is not enough. |
|  |  | SSB to RO mapping is possible before HO (ssb-PositionsInBurst in ServingCellConfigCommon) SSB index should be obtained for RACH transmission. |  |  |  |

## Text proposals

In total three companies have submitted proposals regarding RACH configuration for handover.

The proposals from the companies are inserted below for reference:

Intel:

RAN1#93 reached to the following agreement for the motivation to avoid the decoding of PBCH of the target cell for the handover purpose.

Agreements: For paired spectrum and within the same frequency range, and L = 4, - the UE may for handover purpose assume an absolute value of the relative time difference between radio frame i in the current cell and the target cell is less than 153600∙TS, if the any of the following conditions of the PRACH configurations are met: - for PRACH configurations in which x is not equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3 - for PRACH configurations in which x is equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3 and the association period in Table 8.1-1 of [38.213] is not equal to 1

Similarly, we need to have the same restriction for unpaired spectrum. For unpaired spectrum, we need to consider not only the frequency band with but also the case of  and  since we already defined NR unpaired band above 3.5GHz.

In case of , the half frame bit is included in the PBCH DMRS, where UE can know the radio frame boundary before actual decoding of PBCH: UE anyway need to detect the PBCH DMRS for identifying SSB index. However, in case of   or , the half frame bit is included in the PBCH payload, where UE can just know the half radio frame boundary before actual decoding of PBCH. Therefore, if , UE may assume an absolute value of the relative time difference between radio frame  in the current cell and the target cell of less than  for handover purposes if the PRACH periodicity is larger than 10ms. Otherwise, UE may assume an absolute value of the relative time difference of less than 153600/2∙TS for all PRACH configuration.

Relevant text proposal for 38.211 [2] is proposed as given below.

-------------- Start of Text proposal for 38.211 ---------------------------

#### 6.3.3.2	Mapping to physical resources

The preamble sequence shall be mapped to physical resources according to

where  is an amplitude scaling factor in order to conform to the transmit power specified in [5, TS38.213], and  is the antenna port. Baseband signal generation shall be done according to clause 5.3 using the parameters in Table 6.3.3.1-1 or Table 6.3.3.1-2 with  given by Table 6.3.3.2-1.

Random access preambles can only be transmitted in the time resources given by the higher-layer parameter prach-ConfigurationIndex according to Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104].

Random access preambles can only be transmitted in the frequency resources given by the higher-layer parameter msg1-FrequencyStart. The PRACH frequency resources , where M equals the higher-layer parameter msg1-FDM, are numbered in increasing order within the initial active uplink bandwidth part during initial access, starting from the lowest frequency. Otherwise,  are numbered in increasing order within the active uplink bandwidth part, starting from the lowest frequency.

For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed:

-	15 kHz for FR1

-	60 kHz for FR2.

For handover purposes within the same frequency range in paired and unpaired spectrum with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For handover purposes within the same frequency range in unpaired spectrum with L>4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts.

-------------- End of Text proposal for 38.211 ---------------------------

LGE:

RACH configuration for handover

Observation 1:

- In handover case for TDD bands in the same frequency layer, it is not necessity to define any condition regarding PRACH configuration and the association period.

- For handover in different frequency layers, it is necessity to define the condition for RACH operation.

Observation 2:

- If network can guarantee to provide the relative time difference between cells in different frequency layer is less than 5ms, and if the UE can achieve frame boundary of the target cell, the same assumption for FDD band case can be applied to other cases (especially, for L=8 case).

Samsung

Proposal 4: adding the content under section 6.3.3.2 of 38.211:

<omitted>

For handover purposes within the same frequency range in paired spectrum with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x≠1

-	the entries in Tables 6.3.3.2-2 and 6.3.3.2-3 where x=1 and and the association period in Table 8.1-1 of [38.213] is not equal to 1

For handover purposes in unpaired spectrum or for handover purposes in paired spectrum within the different frequency range with L=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if any of the following conditions are fulfilled:

-	the determined association pattern period in [8.1 of 38.213] is not equal to 10ms

<omitted>

# Corrections to TS38.211

## TP for RACH configuration table (ZTE, Nokia)

Two minor editorial corrections are needed to the RACH configuration tables:

Proposal:  Adopt the following text proposal to 38.211, tables 6.3.3.2-3 and 6.3.3.2-4:

************************************* Start TS 38.211 *****************************************

#### 6.3.3.2	Mapping to physical resources

…

Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| 95 | A2 | 16 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 3 | 4 |

….

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| PRACH Config. Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| 134 | B4 | 1 | 0 | 3,5,7,9,11,13 | 02 | 1 | 1 | 12 |

*************************** TS 38.211 – End *******************************************

| PRACH Config. Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 78 | A3 | 1 | 0 | 9,11,13 | 2 | 1 | 2 | 6 |
| 127 | B4 | 1 | 0 | 19,39 | 2 | 2 | 1 | 12 |
| 128 | B4 | 1 | 0 | 3,5,7,9,11,13 | 02 | 1 | 1 | 1 |
| 133 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 1 | 12 |
| 135 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 1 | 12 |
| 136 | B4 | 1 | 0 | 3,5,7,9,11,13,15,17,19,21,23,25 | 2 | 1 | 1 |  |
| 138 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 1 | 12 |
| 141 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 | 12 |
| 144 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 146 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 193 | C2 | 1 | 0 | 23,27,31,35,39 | 7 | 2 | 1 | 6 |
| 195 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 2 | 1 | 6 |
| 198 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 |

## TP for correction of PRACH signal generation (ZTE, Sharp)

Two proposals from ZTE and Sharp (submitted in AI 7.1.1.1) to correct the PRACH signal generation with respect to the BWP and the higher layer parameters offsetToCarrier and subcarrierSpacing, see detailed motivation for the proposal below.

Proposal: Change  to  in the signal generation equation for PRACH in 38.211 section 5.3.2.

Based on offline discussions, the following companies support the proposal:

Supported by: ZTE, Samsung, Mediatek, CATT, Sharp, LGE, Ericsson, HW, HiSilicon, Intel

-----------Text Proposal 38.211 Section 5.3.2 -------

### 5.3.2 OFDM baseband signal generation for PRACH The time-continuous signal on antenna port for PRACH is defined by Current wording: New wording (the line starting with k1 is updated): where and - is given by clause 6.3.3; - is the subcarrier spacing of the initial active uplink bandwidth part during initial access. Otherwise, is the subcarrier spacing of the active uplink bandwidth part; - μ0 is the largest μ value among the subcarrier spacing configurations provided to the UE for this carrier; - is the lowest numbered resource block of the initial active uplink bandwidth part based on common resource block indexing and is derived by the higher-layer parameter initialUplinkBWP during initial access. Otherwise, is the lowest numbered resource block of the active uplink bandwidth part based on common resource block indexing and is derived by the higher-layer parameter UL-BWP;

-----------End of Text Proposal--------

Motivation for update:

From the description in TS 38.211, the  reference point is Point A. The starting frequency of BWP is shown in below figure.

Figure 1 starting frequency of BWP

When  determines the RACH signal generation, the  reference point is Point A but not the starting of grid , this will cause the PRACH signal with the additional frequency offset “offsetToCarrier” and map to the wrong frequency position.

In [2], the first PRB of BWP has been revised to the PRB offset relative to the PRB indicated by higher layer parameters offsetToCarrier and subcarrierSpacing. The reference point of first PRB of BWP is the .

a first PRB and a number of contiguous PRBs indicated by higher layer parameter locationAndBandwidth that is interpreted as RIV according to [4, TS 38.214], setting =275, and the first PRB is a PRB offset relative to the PRB indicated by higher layer parameters offsetToCarrier and subcarrierSpacing;

And then we suggest changing the  to  in the signal generation equation for PRACH

Sharp:

The term k1 is used to locate the lowest subcarrier of the lowest PRB of the PRACH transmission occasion in frequency domain with respect to the start of the corresponding resource grid, see e.g. Figure 1, where

- The term k0μ-Ngridsize,μNscRB/2 represents the start of the resource grid;

- The term nRAstartNscRB represents the start of the lowest PRACH transmission as a frequency offset with respect to the start of the active uplink BWP;

- The term nRANRBRANscRB represents the start of the target PRACH transmission as a frequency offset with respect to the start of the lowest PRACH transmission; and

- There should be another term (i.e. X in Figure 1) representing the start of the active uplink bandwidth part as a frequency offset with respect to the start of the resource grid. It can be derived from Figure 1 that X=(NBWP,istart-Ngridstart,μ)NscRB. However, the term NBWP,istartNscRB is currently used in [1] for this purpose which is defined as a frequency offset with respect to Point A.

Figure 1: Location of PRACH transmission occasion within the resource grid (k1)

Hence it is proposed to replace NBWP,istart with (NBWP,istart-Ngridstart,μ) in the term k1, i.e.

k1=k0μ+(NBWP,istart-Ngridstart,μ)NscRB+nRAstartNscRB+nRANRBRANscRB-Ngridsize,μNscRB/2

- In OFDM baseband signal generation for PRACH in subclause 5.3.2 of TS 38.211, replace NBWP,istart with (NBWP,istart-Ngridstart,μ) in the term k1, i.e.

k1=k0μ+(NBWP,istart-Ngridstart,μ)NscRB+nRAstartNscRB+nRANRBRANscRB-Ngridsize,μNscRB/2

# References

| 1 | R1-1808164 | Maintenance for Physical random access channel and procedure | ZTE |
| --- | --- | --- | --- |
| 2 | R1-1809512 (updated) | Remaining issues in PRACH design and RACH procedure | MediaTek Inc. |
| 3 | R1-1808483 | Maintenance for Physical random access channel and procedure | LG Electronics |
| 4 | R1-1808665 | Corrections to Random Access Channel and Procedures | Intel Corporation |
| 5 | R1-1808746 | Remaining Issues on Random Access | Samsung |
| 6 | R1-1808804 | Remaining issues on Physical random access channel and procedure | Spreadtrum Communications |
| 7 | R1-1808965 | Remaining details on NR Random Access | Nokia, Nokia Shanghai Bell |
| 8 | R1-1809257 | Maintenance for Physical random access channel and procedure | Ericsson |
| 9 | R1-189105 | Remaining issues on OFDM baseband signal generation | Sharp |