TSG-RAN WG1 #94	R1-1809997

Gothenburg, Sweden, August 20 – 24, 2018

Source:	Ericsson

Title:	Summary of offline discussion on LBRM

Agenda Item:	7.1.3.3

Document for:	Discussion and Decision

Based on further offline discussions, the following is proposed.

Offline Proposal for working assumption and prepare a TP for 38.214:

- The UE is not expected to handle any TBs in a 14 consecutive-symbol duration for normal CP (or 12 for extended CP) within a BWP on a component carrier whenever

2max⁡(0,μ-μ').i∈SCi'.Fi>1RLBRM.TBSLBRM

Where,

- S is the set of TBs belonging to PDSCH(s) that are contained in the consecutive-symbol duration

- for the ith TB

  - Ci’ is the number of scheduled code blocks for i-th TB, accounting for CBG-based retransmission. [38.212].

  - Fi=maxj=0,...,J-1(min(k0,ij+Eij, Ncb,i)), based on the values defined in Subclause 5.4.2.1 [38.212]

    - k0,ij starting location of RV for the jth transmission

    - Ei j=min(Er) of the transmitted code blocks for the jth transmission

    - Ncb,i is the circular buffer size

    - J-1 is the current (re)transmission for the i-th TB

  - μ' corresponds to the subcarrier spacing of the BWP (across all configured BWPs of a carrier) that has the largest configured number of PRBs

  - μ corresponds to the subcarrier spacing of the active BWP

  - RLBRM = 2/3 as defined in Subclause 5.4.2.1 [38.212].

  - TBSLBRM as defined in Subclause 5.4.2.1 [38.212].