3GPP TSG-RAN1 Meeting #94 	R1-1810035

Gothenburg, Sweden, August 20 – 24, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0462 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  |  | Date: |  | 2018-08-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inclusion of corrections agreed at RAN1#94. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of corrections agreed at RAN1#94. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete or incorrect NB-IoT specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.0.1.2, 10.1.3.6, 10.1.5, 10.1.6.1, 10.1.6.2, 10.2.2.4 (new), 10.2.3.4, 10.2.5.5, 10.2.6, 10.2.6B.1, 10.2.6B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | 36.212, 36.213 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 10	Narrowband IoT

## 10.0	General

### 10.0.1	Frame structure

#### 10.0.1.1	Frame structure type 1

Frame structure type 1 is applicable to FDD operation only.

#### 10.0.1.2	Frame structure type 2

Frame structure type 2 is applicable to TDD operation only.

The following restrictions apply:

-	Uplink-downlink configuration 0 and 6 are not supported.

-	UpPTS is not used for NPUSCH or NPRACH.

-	DwPTS and UpPTS in special subframe configuration 10 is not used for transmissions.

-	On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission.

## 10.1	Uplink

### 10.1.1	Overview

#### 10.1.1.1	Physical channels

The following narrowband physical channels are defined:

-	Narrowband Physical Uplink Shared Channel, NPUSCH

-	Narrowband Physical Random-Access Channel, NPRACH

#### 10.1.1.2	Physical signals

The following uplink narrowband physical signals are defined:

-	Narrowband demodulation reference signal

### 10.1.2	Slot structure and physical resources

#### 10.1.2.1	Resource grid

A transmitted physical channel or signal in a slot is described by one or several resource grids of  subcarriers and  SC-FDMA symbols. The resource grid is illustrated in Figure 10.1.2.1-1. The slot number within a radio frame is denoted  where  for  and  for .

Figure 10.1.2.1-1: Uplink resource grid for NB-IoT

The uplink bandwidth in terms of subcarriers , and the slot duration  are given in Table 10.1.2.1-1.

Table 10.1.2.1-1: NB-IoT parameters.

| Subcarrier spacing |  |  |
| --- | --- | --- |

A single antenna port  is used for all uplink transmissions.

#### 10.1.2.2	Resource elements

Each element in the resource grid is called a resource element and is uniquely defined by the index pair  in a slot where  and  are the indices in the frequency and time domains, respectively. Resource element  corresponds to the complex value . Quantities  corresponding to resource elements not used for transmission of a physical channel or a physical signal in a slot shall be set to zero.

#### 10.1.2.3	Resource unit

Resource units are used to describe the mapping of the NPUSCH to resource elements. A resource unit is defined as  SC-FDMA symbols in the time domain and consecutive subcarriers in the frequency domain, where  and  are given by Tables 10.1.2.3-1 and 10.1.2.3-2 for frame structure types 1 and 2, respectively.

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1.

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Table 10.1.2.3-2: Supported combinations of NscRU, NslotsUL, and NsymbUL for frame structure type 2.

| NPUSCH format |  | Supported uplink-downlink configurations |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1, 4 | 1 | 16 | 7 |
|  | 15 kHz | 1, 2, 3, 4, 5 | 1 | 16 |  |
|  |  |  | 3 | 8 |  |
|  |  |  | 6 | 4 |  |
|  |  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1, 4 | 1 | 4 |  |
|  | 15 kHz | 1, 2, 3, 4, 5 | 1 | 4 |  |

### 10.1.3	Narrowband physical uplink shared channel

The narrowband physical uplink shared channel supports two formats:

-	NPUSCH format 1, used to carry the UL-SCH

-	NPUSCH format 2, used to carry uplink control information

#### 10.1.3.1	Scrambling

Scrambling shall be done according to clause 5.3.1. The scrambling sequence generator shall be initialised with  where  is the first slot of the transmission of the codeword. In case of NPUSCH repetitions, the scrambling sequence shall be reinitialised according to the above formula after every  transmissions of the codeword with  and  set to the first slot and the frame, respectively, used for the transmission of the repetition. The quantity  is given by clause 10.1.3.6.

#### 10.1.3.2	Modulation

Modulation shall be done according to clause 5.3.2 resulting in a block of modulated symbols dq0, …, dqMsymb(q)-1. Table 10.1.3.2-1 specifies the modulation mappings applicable for the narrowband physical uplink shared channel.

The block of modulated symbols shall be multiplied with a code cSR resulting in a block of modulation symbols dq0, …, dqMsymbq-1 according to

d(q)i=cSRid(q)i

where

-	cSRi=-1i, i=0, 1, …, 15 in case a positive scheduling request according to [4] is to be transmitted using NPUSCH format 2

-	cSRi=1 otherwise

Table 10.1.3.2-1: NPUSCH modulation schemes

| NPUSCH format |  | Modulation scheme |
| --- | --- | --- |
| 1 | 1 | BPSK, QPSK |
|  | >1 | QPSK |
| 2 | 1 | BPSK |

#### 10.1.3.3	Layer mapping

Layer mapping shall be done according to clause 5.3.2A with  using dq0, …, dqMsymbq-1 instead of dq0, …, dqMsymb(q)-1.

#### 10.1.3.4	Transform precoding

Transform precoding shall be done according to clause 5.3.3 with  and  replaced by .

#### 10.1.3.5	Precoding

Precoding shall be done according to clause 5.3.3A assuming a single antenna port.

#### 10.1.3.6	Mapping to physical resources

NPUSCH can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of 3GPP TS 36.213 [4], each of which shall be transmitted  times.

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [4], and mapped in sequence starting with  to subcarriers assigned for transmission of NPUSCH. The mapping to resource elements  corresponding to the subcarriers assigned for transmission and not used for transmission of reference signals, shall be in increasing order of first the index , then the index, starting with the first slot in the assigned resource unit.

After mapping to slots, the  slots shall be repeated  additional times, before continuing the mapping of  to the following slot, where

For NPUSCH Format 1 and 2 on frame structure type 2 with ,

the NPUSCH transmission is carried out in the first set of  slots spanning over two contiguous uplink subframes not overlapping with any uplink subframe configured as invalid;

for TDD configuration 1 and 4, if the starting position for the NPUSCH is indicated as the second of the two contiguous uplink subframes, the NPUSCH transmission is postponed until the start of two consecutive uplink subframes.

If a mapping to  slots or a repetition of the mapping contains a resource element which overlaps with any configured NPRACH resource according to NPRACH-ConfigSIB-NB, or if it overlaps with any configured NPRACH resource according to nprach-ParametersList and the UE indicates multiCarrier-NPRACH as supported

-	for  the NPUSCH transmission in overlapped slots is postponed until the next  slots not overlapping with any configured NPRACH resource.

-	for  the NPUSCH transmission in overlapped  slots is postponed until the next  slots starting with the first slot satisfying  nsmod 2=0and not overlapping with any configured NPRACH resource.

NPRACH gaps as defined in section 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of  is then repeated until  slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, for frame structure type 1, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

When higher layer parameter npusch-AllSymbols is set to false, resource elements in SC-FDMA symbols overlapping with a symbol configured with SRS according to srs-SubframeConfig shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH. When higher layer parameter npusch-AllSymbols is set to true, all symbols are transmitted.

### 10.1.4	Demodulation reference signal

#### 10.1.4.1	Reference signal sequence

##### 10.1.4.1.1	Reference signal sequence for

The reference signal sequence for  is defined by

where the binary sequence  is defined by clause 7.2 and shall be initialised with  at the start of the NPUSCH transmission. The quantity  is given by Table 10.1.4.1.1-1 where  for NPUSCH format 2, and for NPUSCH format 1if group hopping is not enabled, and by clause 10.1.4.1.3 if group hopping is enabled for NPUSCH format 1.

Table 10.1.4.1.1-1: Definition of

| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 2 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 3 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 |
| 4 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 5 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 6 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 |
| 7 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 9 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 |
| 10 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 |
| 11 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 12 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 |
| 13 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 14 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 15 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 |

The reference signal sequence for NPUSCH format 1 is given by:

The reference signal sequence for NPUSCH format 2 is given by

where is defined in Table 5.5.2.2.1-2 with the sequence index chosen according to  with .

##### 10.1.4.1.2	Reference signal sequence for

The reference signal sequences for is defined by a cyclic shift  of a base sequence according to

,

where  is given by Table 10.1.4.1.2-1 for , Table 10.1.4.1.2-2 for  and Table 5.5.1.2-1 for .

If group hopping is not enabled, the base sequence index  is given by higher layer parameters threeTone-BaseSequence, sixTone-BaseSequence, and twelveTone-BaseSequence for , , and , respectively. If not signalled by higher layers, the base sequence is given by

If group hopping is enabled, the base sequence index  is given by clause 10.1.4.1.3.

The cyclic shift  for  and  is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 10.1.4.1.2-3. For , .

Table 10.1.4.1.2-1: Definition of  for

| u | ϕ0, ϕ1, ϕ2 |  |  |
| --- | --- | --- | --- |
| 0 | 1 | -3 | -3 |
| 1 | 1 | -3 | -1 |
| 2 | 1 | -3 | 3 |
| 3 | 1 | -1 | -1 |
| 4 | 1 | -1 | 1 |
| 5 | 1 | -1 | 3 |
| 6 | 1 | 1 | -3 |
| 7 | 1 | 1 | -1 |
| 8 | 1 | 1 | 3 |
| 9 | 1 | 3 | -1 |
| 10 | 1 | 3 | 1 |
| 11 | 1 | 3 | 3 |

Table 10.1.4.1.2-2: Definition of  for

| u | ϕ0, …, ϕ5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 3 | -3 |
| 1 | 1 | 1 | 3 | 1 | -3 | 3 |
| 2 | 1 | -1 | -1 | -1 | 1 | -3 |
| 3 | 1 | -1 | 3 | -3 | -1 | -1 |
| 4 | 1 | 3 | 1 | -1 | -1 | 3 |
| 5 | 1 | -3 | -3 | 1 | 3 | 1 |
| 6 | -1 | -1 | 1 | -3 | -3 | -1 |
| 7 | -1 | -1 | -1 | 3 | -3 | -1 |
| 8 | 3 | -1 | 1 | -3 | -3 | 3 |
| 9 | 3 | -1 | 3 | -3 | -1 | 1 |
| 10 | 3 | -3 | 3 | -1 | 3 | 3 |
| 11 | -3 | 1 | 3 | 1 | -3 | -1 |
| 12 | -3 | 1 | -3 | 3 | -3 | -1 |
| 13 | -3 | 3 | -3 | 1 | 1 | -3 |

Table 10.1.4.1.2-3: Definition of

| threeTone-CyclicShift |  |  |  | sixTone-CyclicShift |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  | 0 |  |  |  |
| 1 |  |  |  | 1 |  |  |  |
| 2 |  |  |  | 2 |  |  |  |
|  |  |  |  | 3 |  |  |  |

##### 10.1.4.1.3	Group hopping

For the reference signal for NPUSCH format 1, sequence-group hopping can be enabled where the sequence-group number  in slot  is defined by a group hopping pattern  and a sequence-shift pattern  according to

where the number of reference signal sequences available for each resource unit size, is given by Table 10.1.4.1.3-1.

Table 10.1.4.1.3-1: Definition of

| 1 | 16 |
| --- | --- |
| 3 | 12 |
| 6 | 14 |
| 12 | 30 |

Sequence-group hopping can be enabled or disabled by means of the cell-specific parameter groupHoppingEnabled provided by higher layers. Sequence-group hopping for NPUSCH can be disabled for a certain UE through the higher-layer parameter groupHoppingDisabled despite being enabled on a cell basis unless the NPUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

The group-hopping pattern  is given by

where  for  and  is the slot number of the first slot of the resource unit for . The pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with  at the beginning of the resource unit for and in every even slot for .

The sequence-shift pattern  is given by

where is given by higher-layer parameter groupAssignmentNPUSCH. If no value is signalled ,.

#### 10.1.4.2	Mapping to physical resources

The sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to the sub-carriers. 
The set of sub-carriers used in the mapping process shall be identical to the corresponding NPUSCH transmission as defined in clause 10.1.3.6. 
The mapping to resource elements  shall be in increasing order of first, then , and finally the slot number. The values of the symbol index  in a slot are given in Table 10.1.4.2-1.

Table 10.1.4.2-1: Demodulation reference signal location for NPUSCH.

| NPUSCH format | Values for |  |
| --- | --- | --- |
| 1 | 4 | 3 |
| 2 | 0,1,2 | 2,3,4 |

### 10.1.5	SC-FDMA baseband signal generation

For , the time-continuous signal  in SC-FDMA symbol  in a slot is defined by clause 5.6 with the quantity  replaced by .

φ=0φ

For , the time-continuous signal  for sub-carrier index in SC-FDMA symbol  in an uplink slot is defined by

for  where parameters for  and  are given in Table 10.1.5-1,  is the modulation value of symbol , φ=0φand the phase rotation  is defined by

where  is a symbol counter that is reset at the start of a transmission and incremented for each symbol during the transmission.

Table 10.1.5-1: SC-FDMA parameters for

| Parameter |  |  |
| --- | --- | --- |
|  | 8192 | 2048 |
| Cyclic prefix length | 256 |  |
| Set of values for | -24,-23,…,23 | -6,-5,…,5 |

The SC-FDMA symbols in a slot shall be transmitted in increasing order of , starting with , where SC-FDMA symbol starts at time  within the slot. For , the remaining  in  are not transmitted and used for guard period.

Only normal CP is supported for Narrowband IoT uplink in this release of the specification.

### 10.1.6	Narrowband physical random-access channel

#### 10.1.6.1	Time and frequency structure

The physical layer random access preamble is based on single-subcarrier frequency-hopping symbol groups. A symbol group is illustrated in Figure 10.1.6.1-1, consisting of a cyclic prefix of length  and a sequence of  identical symbols with total length. The total number of symbol groups in a preamble repetition unit is denoted by P. The number of time-contiguous symbol groups is given by G.

The parameter values for frame structures 1 and 2 are listed in Tables 10.1.6.1-1 and 10.1.6.1-2, respectively.

Figure 10.1.6.1-1: Random access symbol group

Table 10.1.6.1-1: Random access preamble parameters for frame structure type 1

| Preamble format |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 4 | 4 | 5 |  |  |
| 1 | 4 | 4 | 5 |  |  |
| 2 | 6 | 6 | 3 |  | 3∙24576Ts |

Table 10.1.6.1-2: Random access preamble parameters for frame structure type 2

| Preamble format | Supported uplink-downlink configurations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1, 2, 3, 4, 5 | 2 | 4 | 1 |  |  |
| 1 | 1, 4 | 2 | 4 | 2 |  |  |
| 2 | 3 | 2 | 4 | 4 |  |  |
| 0-a | 1, 2, 3, 4, 5 | 3 | 6 | 1 |  |  |
| 1-a | 1, 4 | 3 | 6 | 2 |  |  |

The preamble consisting of P symbol groups shall be transmitted  times. For frame structure type 2, when an invalid uplink subframe overlaps the transmission of G symbol groups without a gap, the G symbol groups are dropped. For frame structure type 2, the transmission of G symbol groups are aligned with the subframe boundary.

The transmission of a random-access preamble, if triggered by the MAC layer, is restricted to certain time and frequency resources.

A NPRACH configuration provided by higher layers contains the following:

-	NPRACH resource periodicity  (nprach-Periodicity),

-	frequency location of the first subcarrier allocated to NPRACH  (nprach-SubcarrierOffset),

-	number of subcarriers allocated to NPRACH  (nprach-NumSubcarriers),

-	number of starting sub-carriers allocated to UE initiated random access  (nprach-NumCBRA-StartSubcarriers),

-	number of NPRACH repetitions per attempt  (numRepetitionsPerPreambleAttempt),

-	NPRACH starting time  (nprach-StartTime),

-	Fraction for calculating starting subcarrier index for the range of NPRACH subcarriers reserved for indication of UE support for multi-tone msg3 transmission  (nprach-SubcarrierMSG3-RangeStart).

NPRACH transmission can start only  time units after the start of a radio frame fulfilling . For frame structure type 1, after transmissions of  time units for preamble formats 0 and 1, or 16∙6(TCP+TSEQ) time units for preamble format 2, a gap of  time units shall be inserted.

NPRACH configurations where  are invalid.

The NPRACH starting subcarriers allocated to UE initiated random access are split in two sets of subcarriers,  and , where the second set, if present, indicate UE support for multi-tone msg3 transmission.

The frequency location of the NPRACH transmission is constrained within  sub-carriers, and within NscRA=36 subcarriers when preamble format 2 as described in Table 10.1.6.1-1 is configured. Frequency hopping shall be used within the 12 subcarriers and 36 subcarriers when preamble format 2 as described in Table 10.1.6.1-1 is configured, where the frequency location of the ith symbol group is given by  where . The quantity nscRAi depends on the frame structure.

For frame structure type 1:

-	if G=4, P=4 for preamble formats 0 and 1 as described in Table 10.1.6.1-1:

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

-	if G=6, P=6 for preamble format 2 as described in Table 10.1.6.1-1:

nSCRAi=nSCRA0+f(i/6)modNscRAi mod 6=0 and i>0nSCRAi-1+1i mod 6=1, 5 and nSCRAi-1 mod 2=0nSCRAi-1-1i mod 6=1, 5 and nSCRAi-1 mod 2=0nSCRAi-1+3i mod 6=2, 4 and nSCRAi-13 mod 2=0nSCRAi-1-3i mod 6=2, 4 and nSCRAi-13 mod 2=0nSCRAi-1+18i mod 6=3 and nSCRAi-1<18nSCRAi-1-18i mod 6=3 and nSCRAi-1≥18

nSCRAi=nSCRA0+f(i/6)modNscRAi mod 6=0 and i>0nSCRAi-1+1i mod 6=1, 5 and nSCRAi-1 mod 2=0nSCRAi-1-1i mod 6=1, 5 and nSCRAi-1 mod 2=1nSCRAi-1+3i mod 6=2, 4 and nSCRAi-13 mod 2=0nSCRAi-1-3i mod 6=2, 4 and nSCRAi-13 mod 2=1nSCRAi-1+18i mod 6=3 and nSCRAi-1<18nSCRAi-1-18i mod 6=3 and nSCRAi-1≥18

ft=ft-1+n=10t+110t+9cn2n-10t+1modNscRA-1+1modNscRA

f-1=0

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

For frame structure type 2:

-	if G=2, P=4 for preamble formats 0, 1, and 2 as described in Table 10.1.6.1-2:

nSCRAi=             Yi mod 8=0, 2 and i>0nSCRAi-1+1i mod 8=1 and nSCRAi-1=0, 2, 4, 6, 8, 10nSCRAi-1-1i mod 8=1 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+6i mod 8=3 and nSCRAi-1=0, 1, 2, 3, 4, 5nSCRAi-1-6i mod 8=3 and nSCRAi-1=6, 7, 8, 9, 10, 112Y mod 6+1i mod 8=4 and nSCRAi-4=0, 2, 4, 6, 8, 102Y mod 6i mod 8=4 and nSCRAi-4=1, 3, 5, 7, 9, 11nSCRAi-1-1i mod 8=5 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+1i mod 8=5 and nSCRAi-1=0, 2, 4, 6, 8, 10Y mod 6+6i mod 8=6 and nSCRAi-4=0, 1, 2, 3, 4, 5Y mod 6i mod 8=6 and nSCRAi-4=6, 7, 8, 9, 10, 11nSCRAi-1-6i mod 8=7 and nSCRAi-1=6, 7, 8, 9, 10, 11nSCRAi-1+6i mod 8=7 and nSCRAi-1=0, 1, 2, 3, 4, 5

nSCRAi=             Yi mod 8=0, 2 and i>0nSCRAi-1+1i mod 8=1 and nSCRAi-1=0, 2, 4, 6, 8, 10nSCRAi-1-1i mod 8=1 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+6i mod 8=3 and nSCRAi-1=0, 1, 2, 3, 4, 5nSCRAi-1-6i mod 8=3 and nSCRAi-1=6, 7, 8, 9, 10, 11 2Y2+1i mod 8=4 and nSCRAi-4=0, 2, 4, 6, 8, 10 2Y2i mod 8=4 and nSCRAi-4=1, 3, 5, 7, 9, 11nSCRAi-1-1i mod 8=5 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+1i mod 8=5 and nSCRAi-1=0, 2, 4, 6, 8, 10Y mod 6+6i mod 8=6 and nSCRAi-4=0, 1, 2, 3, 4, 5Y mod 6i mod 8=6 and nSCRAi-4=6, 7, 8, 9, 10, 11nSCRAi-1-6i mod 8=7 and nSCRAi-1=6, 7, 8, 9, 10, 11nSCRAi-1+6i mod 8=7 and nSCRAi-1=0, 1, 2, 3, 4, 5

Y= ñSCRA0+f(i/2)modNscRA

ft=ft-1+n=10t+110t+9cn2n-10t+1modNscRA-1+1modNscRA

f-1=0

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

-	if G=3, P=6 for preamble formats 0-a, 1-a, as described in Table 10.1.6.1-2:

nSCRAi=nSCRA0+f(i/3)modNscRAi mod 6=0, 3 and i>0nSCRAi-1+1i mod 6=1 and nSCRAi-1=0, 2, 4, 6, 8, 10nSCRAi-1-1i mod 6=2 and nSCRAi-2=0, 2, 4, 6, 8, 10nSCRAi-1-1i mod 6=1 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+1i mod 6=2 and nSCRAi-2=1, 3, 5, 7, 9, 11nSCRAi-1+6i mod 6=4 and nSCRAi-1=0, 1, 2, 3, 4, 5nSCRAi-1-6i mod 6=5 and nSCRAi-2=0, 1, 2, 3, 4, 5nSCRAi-1-6i mod 6=4 and nSCRAi-1=6, 7, 8, 9, 10, 11nSCRAi-1+6i mod 6=5 and nSCRAi-2=6, 7, 8, 9, 10, 11

ft=ft-1+n=10t+110t+9cn2n-10t+1modNscRA-1+1mod NscRA

f-1=0

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

#### 10.1.6.2	Baseband signal generation

The time-continuous random-access signal  for symbol group  is defined by

sit=βNPRACHej2πnSCRAi+Kk0+12+φ∆fRAt-TCP∆fRAt-TCP

where 0≤t≤TSEQ+TCP,  is an amplitude scaling factor in order to conform to the transmit power  specified in clause 16.3.1 in 3GPP TS 36.213 [4], ,  accounts for the difference in subcarrier spacing between the random access preamble and uplink data transmission, and the location in the frequency domain controlled by the parameter  is derived from clause 10.1.6.1. The variable is given by Table 10.1.6.2-1。φ=0φ

Table 10.1.6.2-1: Random access baseband parameters

| Preamble format |  |  |
| --- | --- | --- |
|  | Frame Structure Type 1 | Frame Structure Type 2 |
| 0, 1, | 3.75 kHz |  |
| 0-a, 1-a |  | 3.75 kHz |
| 2 | 1.25 kHz | 3.75 kHz |

### 10.1.7	Modulation and upconversion

Modulation and upconversion to the carrier frequency of the complex-valued baseband signal or the complex-valued NPRACH baseband signal is shown in Figure 5.8-1. The filtering required prior to transmission is defined by the requirements in 3GPP TS 36.101 [7].

## 10.2	Downlink

### 10.2.1	Overview

#### 10.2.1.1	Physical channels

A downlink narrowband physical channel corresponds to a set of resource elements carrying information originating from higher layers and is the interface defined between 3GPP TS 36.212 [3] and the present document 3GPP TS 36.211.

The following downlink physical channels are defined:

-	Narrowband Physical Downlink Shared Channel, NPDSCH

-	Narrowband Physical Broadcast Channel, NPBCH

-	Narrowband Physical Downlink Control Channel, NPDCCH

#### 10.2.1.2	Physical signals

A downlink narrowband physical signal corresponds to a set of resource elements used by the physical layer but does not carry information originating from higher layers. The following downlink physical signals are defined:

-	Narrowband reference signal, NRS

-	Narrowband synchronization signal

-	Narrowband positioning reference signal, NPRS

-	Narrowband wake up signal, NWUS

### 10.2.2	Slot structure and physical resource elements

#### 10.2.2.1	Resource grid

The transmitted signal on one antenna port in each slot is described by a resource grid of size one resource block as defined in clause 6.2.3.

Only  is supported.

Narrowband positioning reference signals are transmitted on antenna port . The channel over which a symbol on antenna port  is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only within  consecutive subframes where

-	if the higher layer parameter nprsBitmap is configured,  equals the length of the nprsBitmap;

-	if the higher layer parameter nprsBitmap is not configured,  where  is configured by higher layers.

#### 10.2.2.2	Resource elements

Resource elements are defined according to clause 6.2.2.

#### 10.2.2.3	Guard period for half-duplex FDD operation

Only type-B half-duplex FDD operation is supported.

### 10.2.2.4			Guard period for TDD operation

For frame structure type 2, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig, a guard period is created by the UE by

- not receiving the first part of the first OFDM symbol of a downlink subframe immediately following an uplink subframe from the same UE for 15-kHz subcarrier spacing on an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates guardband or standalone, or higher-layer parameter inbandCarrierInfo is not present.

### 10.2.3	Narrowband physical downlink shared channel

#### 10.2.3.1	Scrambling

Scrambling shall be done according to clause 6.3.1. If the NPDSCH is carrying the BCCH, the scrambling sequence generator shall be initialised with . Otherwise, the scrambling sequence generator shall be initialised with  whereis the first slot of the transmission of the codeword.

In case of NPDSCH repetitions and the NPDSCH carrying the BCCH, the scrambling sequence generator shall be reinitialized according to the expression above for each repetition.

In case of NPDSCH repetitions and the NPDSCH is not carrying the BCCH, the scrambling sequence generator shall be reinitialized according to the expression above after every minMrepNPDSCH,4 transmission of the codeword withand set to the first slot and the frame, respectively, used for the transmission of the repetition.

#### 10.2.3.2	Modulation

Modulation shall be done according to clause 6.3.2 using one of the modulation schemes in Table 10.2.3-1

Table 10.2.3-1: Modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| NPDSCH | QPSK |

#### 10.2.3.3	Layer mapping and precoding

Layer mapping and precoding shall be done according to clause 6.6.3 using the same set of antenna ports as the NPBCH.

#### 10.2.3.4	Mapping to resource elements

NPDSCH can be mapped to one or more than one subframes, , as given by clause 16.4.1.3 of 3GPP TS 36.213 [4], each of which shall be transmitted MrepNPDSCH times.

For each of the antenna ports used for transmission of the physical channel, the block of complex-valued symbols  shall be mapped to resource elements  which meet all of the following criteria in the current subframe:

-	the subframe is not used for transmission of NPBCH, NPSS, or NSSS, and

-	except in a special subframe when MrepNPDSCH>1, they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.4.1.4 of 3GPP TS 36.213 [4] ], and

-	in addition, for frame structure Type 2

-	in a special subframe, if MrepNPDSCH=1, they are in DwPTS

-	in a special subframe, if MrepNPDSCH>1, they are not NRS locations in subframes which are not special subframes.

The mapping of  in sequence starting with  to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. For NPDSCH not carrying BCCH, after mapping to a subframe, the subframe shall be repeated for minMrepNPDSCH,4-1 additional subframes, before continuing the mapping of  to the following subframe.

MrepNPDSCH=1

MrepNPDSCH>1

l= NsymbDL-2, NsymbDL-1

minMrepNPDSCH,4-1The resource elements in a special subframe that are not part of DwPTS are counted but not used in the mapping if MrepNPDSCH>1. When l= NsymbDL-5, NsymbDL-4, the resource elements in a special subframe assumed by the UE for NRSs are counted but not used in the mapping if MrepNPDSCH>1.

For frame structure type 1, for NPDSCH associated with C-RNTI when interferenceRandomisationConfig is configured, or NPDSCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or NPDSCH associated with G-RNTI or SC-RNTI, or for frame structure type 2, for NPDSCH not carrying the BCCH, define as the block of complex-valued symbols mapped to  subframe number ns/2 and radio frame number . Each complex-valued symbol  shall be multiplied with before its transmission, with

where the scrambling sequence is given by clause 7.2, and shall be initialized at the start of each subframe with .

The mapping of  is then repeated until MrepNPDSCHNSF subframes have been transmitted. For frame structure type 2 , the resource elements in a special subframe that are not part of DwPTS are counted but not used in the repetition. When l= NsymbDL-5, NsymbDL-4, the resource elements in a special subframe assumed by the UE for NRSs are counted but not used in the repetition.

For NPDSCH carrying BCCH, the  yp(k-1)Msymbap, …, ypkMsymbap-1 is mapped to  subframes in sequence and then repeated until MrepNPDSCHNSF subframes have been transmitted, where

-	k=2 for mapping NPDSCH carrying SystemInformationBlockType1-NB to subframe #3 for frame structure type 2;

-	k=1 otherwise.

The NPDSCH transmission can be configured by higher layers with transmission gaps where the NPSDCH transmission is postponed. There are no gaps in the NPDSCH transmission if where  is given by the higher layer parameter dl-GapThreshold and  is given by [4]. The gap starting frame and subframe is given by  where the gap periodicity,, is given by the higher layer parameter dl-GapPeriodicity. The gap duration in number of subframes is given by , where  is given by the higher layer parameter dl-GapDurationCoeff. For NPDSCH carrying the BCCH there are no gaps in the transmission.

The UE shall not expect NPDSCH in subframe  if it is not a NB-IoT downlink subframe, except for transmissions of NPDSCH carrying SystemInformationBlockType1-NB in

-	subframes 3 and 4 for frame structure type 1; and

-	subframes 0, 4, and 5 for frame structure type 2.

In case of NPDSCH transmissions, in subframes that are not NB-IoT downlink subframes, the NPDSCH transmission is postponed until the next NB-IoT downlink subframe.

### 10.2.4	Narrowband physical broadcast channel

#### 10.2.4.1	Scrambling

Scrambling shall be done according to clause 6.6.1 with  denoting the number of bits to be transmitted on the NPBCH.  equals 1600 for normal cyclic prefix. The scrambling sequence shall be initialised with  in radio frames fulfilling.

#### 10.2.4.2	Modulation

Modulation shall be done according to clause 6.6.2 using the modulation scheme in Table 10.2.4.2-1

Table 10.2.4.2-1: Modulation schemes for NPBCH

| Physical channel | Modulation schemes |
| --- | --- |
| NPBCH | QPSK |

#### 10.2.4.3	Layer mapping and precoding

Layer mapping and precoding shall be done according to clause 6.6.3 with . The UE shall assume antenna ports 2000 and 2001 are used for the transmission of the narrowband physical broadcast channel.

#### 10.2.4.4	Mapping to resource elements

The block of complex-valued symbols for each antenna port is transmitted in subframe 0 for frame structure type 1 or subframe 9 for frame structure type 2 during 64 consecutive radio frames starting in each radio frame fulfilling . The quantity Msymb=800 for normal cyclic prefix. Define as the block of complex-valued symbols to be transmitted in subframe 0 of radio frame , as ,  with  for normal cyclic prefix, and

where the scrambling sequence ,  is given by clause 7.2, and shall be initialized at the start of each radio frame with . The block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements . The mapping to resource elements  not reserved for transmission of reference signals shall be in increasing order of first the index, then the index . The first three OFDM symbols in a subframe shall not be used in the mapping process.

For the purpose of the mapping, the UE shall assume cell-specific reference signals for antenna ports 0-3 and narrowband reference signals for antenna ports 2000 and 2001 being present irrespective of the actual configuration. The frequency shift of the cell-specific reference signals shall be calculated by replacing with  in the calculation of in clause 6.10.1.2.

### 10.2.5	Narrowband physical downlink control channel

#### 10.2.5.1	NPDCCH formats

The narrowband physical downlink control channel carries control information. A narrowband physical control channel is transmitted on an aggregation of one or two consecutive narrowband control channel elements (NCCEs), where a narrowband control channel element corresponds to 6 consecutive subcarriers in a subframe where NCCE 0 occupies subcarriers 0 through 5 and NCCE 1 occupies subcarriers 6 through 11. The NPDCCH supports multiple formats as listed in Table 10.2.5.1-1. For NPDCCH format 1, both NCCEs belong to the same subframe.

One or two NPDCCHs can be transmitted in a subframe.

Table 10.2.5.1-1: Supported NPDCCH formats

| NPDCCH format | Number of NCCEs |
| --- | --- |
| 0 | 1 |
| 1 | 2 |

#### 10.2.5.2	Scrambling

Scrambling shall be done according to clause 6.8.2. The scrambling sequence shall be initialised at the start of subframe k0 according to [4] Subclause 16.6 and after every 4th NPDCCH subframe with  where is the first slot of the NPDCCH subframe in which scrambling is (re-)initialized.

#### 10.2.5.3	Modulation

Modulation shall be done according to clause 6.8.3 using the modulation scheme in Table 10.2.5.3-1

Table 10.2.5.3-1: Modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| NPDCCH | QPSK |

#### 10.2.5.4	Layer mapping and precoding

Layer mapping and precoding shall be done according to clause 6.6.3 using the same set of antenna ports as the NPBCH.

#### 10.2.5.5	Mapping to resource elements

The block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the NCCE(s) assigned for the NPDCCH transmission, and

-	they are not used for transmission of NPBCH, NPSS, or NSSS , and

-	except in a special subframe when NPDCCH is transmitted in more than one subframe, they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.6.1 of 3GPP TS 36.213 [4],

-	in addition, for frame structure Type 2,

-	in a special subframe where the NPDCCH is transmitted in one subframe, they are in DwPTS

-	in a special subframe where the NPDCCH is transmitted in more than one subframe, they are not NRS locations when the subframe is not a special subframe.

The mapping to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. Denote  as the complex-valued symbols that are mapped to resource elements meeting the criteria above in subframe ns/2, with the insertion of <NIL> elements in the locations of resource elements which are not part of the NCCE(s) assigned for the NPDCCH transmission.

If the NPDCCH is transmitted in more than one subframe, the resource elements in a special subframe that are not part of DwPTS are counted but not used in the mapping. When l= NsymbDL-5, NsymbDL-4, the resource elements in a special subframe assumed by the UE for NRSs are counted but not used in the mapping if the NPDCCH is transmitted in more than one subframe.

For frame structure type 1, for NPDCCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or NPDCCH associated with G-RNTI or SC-RNTI, or for NPDCCH associated with C-RNTI when interferenceRandomisationConfig is used according to [11], or for frame structure type 2, each complex-valued symbol , shall be multiplied with ,where

where the scrambling sequence  is given by clause 7.2, and shall be initialized at the start of each subframe with .

The NPDCCH transmission can be configured by higher layers with transmissions gaps where the NPDCCH transmission is postponed. The configuration is the same as described for NPDSCH in clause 10.2.3.4.

The UE shall not expect NPDCCH in subframe  if it is not a NB-IoT downlink subframe. In case of NPDCCH transmissions, in subframes that are not NB-IoT downlink subframes, the NPDCCH transmission is postponed until the next NB-IoT downlink subframe.

### 10.2.6	Narrowband reference signal (NRS)

Before a UE obtains operationModeInfo:

-	If frame structure type 1 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #9 and in subframes #0 not containing NSSS.

On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating guardband or standalone.

-	If frame structure type 1 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, and in subframes #0 not containing NSSS, and in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions.

-	If frame structure type 1 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes.

-	If frame structure type 2 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, subframes #0 not containing NSSS, in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions, and in NB-IoT downlink subframes.

On an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo-NB indicates non-anchor for frame structure type 2, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0 and #5. After the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #5, and in NB-IoT downlink subframes indicated by tdd-SI-SubframesBitmap.

On an NB-IoT carrier for which DL-CarrierConfigCommon-NB is present and no inbandCarrierInfo is present.

-	When an NB-IoT UE is configured by higher layers to decode NPDCCH with CRC scrambled by the P-RNTI, the UE may assume NRSs are transmitted in the NPDCCH candidate where the UE finds a DCI with CRC scrambled by the P-RNTI. The UE may also assume NRSs are transmitted in 10 NB-IoT DL subframes before and in 4 NB-IoT DL subframes after the NPDCCH candidate where the UE finds a DCI with CRC scrambled by the P-RNTI, where NB-IoT DL subframes without NRS are not counted. If the DCI with CRC scrambled by the P-RNTI schedules a NPDSCH, the UE may assume NRSs are transmitted in the NB-IoT DL subframes carrying the NPDSCH as well as in 4 NB-IoT DL subframes before and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted.

-	During the window controlled by higher layers where the UE shall attempt to decode the NPDCCH with DCI scrambled by RA-RNTI (see [8], subclause 5.1.4), the UE may assume NRSs are transmitted in the Type-2 CSS configured by higher layers, as well as in 10 NB-IoT DL subframes before and in 4 NB-IoT DL subframes after each Type-2 CSS, where NB-IoT DL subframes without NRS are not counted. If a DCI scrambled by the RA-RNTI is detected, the UE may assume NRSs are transmitted in the NPDSCH scheduled by the DCI scrambled by the RA-RNTI, as well as in 4 NB-IoT DL subframes before and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted. In addition, when the UE attempts to decode a DCI with CRC scrambled by the RA-RNTI as well as receiving the NPDSCH scheduled by the DCI scrambled by the RA-RNTI, the UE may assume NRSs are transmitted in subframes #0, #1, #3, #4 and #9.

-	During random access procedure, when an NB-IoT UE is configured by higher layers to decode NPDCCH with CRC scrambled by the temporary C-RNTI and/or the C-RNTI, before the the DCI scrambled by temporary C-RNTI and/or C-RNTI is detected, the UE may assume NRSs are transmitted in the Type-2 CSS configured by higher layers, as well as in 10 NB-IoT DL subframes before the start of each Type-2 CSS and in 4 NB-IoT DL subframes after the end of each Type-2 CSS until the mac-ContentionResolutionTimer expires, where NB-IoT DL subframes without NRS are not counted. If a DCI scrambled by the temporary C-RNTI or C-RNTI is detected, the UE may assume NRSs are transmitted in the NPDSCH scheduled by the DCI scrambled by the temporary C-RNTI or C-RNTI as well as in 4 NB-IoT DL subframes before and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted.

-	An NB-IoT UE may assume NRSs are transmitted in NB-IoT DL subframes that are used for Type1A-NPDCCH common search space, and Type2A-NPDCCH common search space, as well as in 10 NB-IoT DL subframes prior and in 4 NB-IoT DL subframes after each Type1A-NPDCCH common search space and Type2A-NPDCCH common search space. A UE may assume NRSs are transmitted in NB-IoT DL subframes carrying NPDSCH scheduled by DCI CRC scrambled by G-RNTI or SC-RNTI as well as 4 NB-IoT DL subframes prior and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted.

-	In other cases, if frame structure typ1 is used, the UE may assume NRSs are transmitted in subframes #0, #1, #3, #4, #9, and in NB-IoT downlink subframes and shall not expect NRSs in other downlink subframes.

On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating inband-SamePCI or inband-DifferentPCI.

-	If frame structure type 1 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #4 and in subframes #9 not containing NSSS, and in subframes #3 which contain SystemInformationBlockType1-NB when additionalTransmissionSIB1 is configured as TRUE.

-	If frame structure type 2 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, and in subframes #0 not containing NSSS, and in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions.

-	If frame structure type 1 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #4, subframes #9 not containing NSSS, subframes #3 which contain SystemInformationBlockType1-NB when additionalTransmissionSIB1 is configured as TRUE, and in NB-IoT downlink subframes.

-	If frame structure type 2 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, subframes #0 not containing NSSS, in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions, and in NB-IoT downlink subframes

On an NB-IoT carrier for which DL-CarrierConfigCommon-NB is present and inbandCarrierInfo is present:

-	When an NB-IoT UE is configured by higher layers to decode NPDCCH with CRC scrambled by the P-RNTI, the UE may assume NRSs are transmitted in the NPDCCH candidate where the UE finds a DCI with CRC scrambled by the P-RNTI. The UE may also assume NRSs are transmitted in 10 NB-IoT DL subframes before and in 4 NB-IoT DL subframes after the NPDCCH candidate , where NB-IoT DL subframes without NRS are not counted. If the DCI with CRC scrambled by the P-RNTI schedules a NPDSCH, the UE may assume NRSs are transmitted in the NB-IoT DL subframes carrying the NPDSCH as well as 4 NB-IoT DL subframes before and after the scheduled NPDSCH , where NB-IoT DL subframes without NRS are not counted.

-	During random access procedure, during the window controlled by higher layers where the UE shall attempt to decode the NPDCCH with DCI scrambled by RA-RNTI (see [8], subclause 5.1.4), before the DCI scrambled by RA-RNTI is detected, the UE may assume NRSs are transmitted in the Type-2 CSS configured by higher layers, as well as in 10 NB-IoT DL subframes before and in 4 NB-IoT DL subframes after the start of each Type-2 CSS, where NB-IoT DL subframes without NRS are not counted. If a DCI scrambled by the RA-RNTI is detected, the UE may assume NRSs are transmitted in the NPDSCH scheduled by the DCI scrambled by the RA-RNTI, as well as in 4 NB-IoT DL subframes before and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted. In addition, when the UE attempts to decode a DCI with CRC scrambled by the RA-RNTI as well as receiving the NPDSCH scheduled by the DCI scrambled by the RA-RNTI, the UE may assume NRSs are transmitted in subframes #0, #4 and #9.

-	During random access procedure, when an NB-IoT UE is configured by higher layers to decode NPDCCH with CRC scrambled by the temporary C-RNTI and/or the C-RNTI, before the DCI scrambled by temporary C-RNTI and/or C-RNTI, is detected, the UE may assume NRSs are transmitted in the Type-2 CSS configured by higher layers, as well as in 10 NB-IoT DL subframes before the start of each Type-2 CSS and in 4 NB-IoT DL subframes after the end of each Type-2 CSS until the mac-ContentionResolutionTimer expires, where NB-IoT DL subframes without NRS are not counted. If a DCI scrambled by the temporary C-RNTI or C-RNTI is detected, the UE may assume NRSs are transmitted in the NPDSCH scheduled by the DCI scrambled by the temporary C-RNTI or C-RNTI as well as in 4 NB-IoT DL subframes before and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted.

-	An NB-IoT UE may assume NRSs are transmitted in NB-IoT DL subframes that are used for Type1A-NPDCCH common search space, and Type2A-NPDCCH common search space, as well as in 10 NB-IoT DL subframes prior and in 4 NB-IoT DL subframes after each Type1A-NPDCCH common search space and Type2A-NPDCCH common search space, where NB-IoT DL subframes without NRS are not counted. A UE may assume NRSs are transmitted in NB-IoT DL subframes carrying NPDSCH scheduled by DCI CRC scrambled by G-RNTI or SC-RNTI as well as in 4 NB-IoT DL subframes prior and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted.

-	In other cases, if frame structure type 1 is used, the UE may assume NRSs are transmitted in subframes #0, #4, #9, and in NB-IoT downlink subframes and shall not expect NRSs in other downlink subframes.

On an NB-IoT carrier for which DL-CarrierConfigDedicated-NB is present and no inbandCarrierInfo is present:

-	If frame structure type 1 is used, the UE may assume NRSs are transmitted in subframes #0, #1, #3, #4, #9, and in NB-IoT downlink subframes and shall not expect NRSs in other downlink subframes.

On an NB-IoT carrier for which DL-CarrierConfigDedicated-NB is present and inbandCarrierInfo is present:

-	If frame structure type 1 is used, the UE may assume NRSs are transmitted in subframes #0, #4, #9, and in NB-IoT downlink subframes and shall not expect NRSs in other downlink subframes.

An NB-IoT UE may assume NRSs are not transmitted in subframes that are configured by higher layer parameter nprsBitmap for narrowband positioning reference signal transmission.

#### 10.2.6.1	Sequence generation

The narrowband reference sequence shall be initialised according to clause 6.10.1.1 where  is replaced with .

#### 10.2.6.2	Mapping to resource elements

Narrowband reference signals are transmitted on one or two antenna ports .

If the higher layer indicates UE may assume that  is equal to , UE may assume

-	the number of antenna ports for the cell-specific reference signals as defined in clause 6.10.1 is the same as for the narrowband reference signals,

-	the antenna ports for cell-specific reference signals {0, 1} are equivalent to antenna ports for narrowband reference signals {2000, 2001}, respectively, and

-	the cell-specific reference signals are available in all subframes where the narrowband reference signals are available.

If the higher layer does not indicate UE may assume that  is equal to , UE may assume

-	the number of antenna port for the cell-specific reference signals as defined in clause 6.10.1 is obtained from the higher layer parameter eutra-NumCRS-Ports,

-	the cell-specific reference signals are available in all subframes where the narrowband reference signals are available, and

the cell-specific frequency shift for cell-specific reference signals as defined in clause 6.10.1.2 is given by .

The reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference symbols for antenna port  in slot  according to

where

When frame structure type 2 is used, the following values of l apply for the generation of NRSs in special subframes

-	l= NsymbDL-5, NsymbDL-4 in each slot for special subframe configurations {3, 4, 8}

-	l= NsymbDL-5, NsymbDL-4 in the first slot for special subframe configurations {9, 10}

-	l= NsymbDL-2, NsymbDL-1 in the first slot for special subframe configurations {1, 2, 6, 7}.

The variables ν and  define the position in the frequency domain for the different reference signals where ν is given by

The cell-specific frequency shift is given by .

Resource elements  used for transmission of narrowband reference signals on any of the antenna ports in a slot shall not be used for any transmission on any other antenna port in the same slot and set to zero.

Narrowband reference signals shall not be transmitted in subframes containing NPSS or NSSS.

For frame structure type 2, narrowband reference signals shall not be transmitted in special subframe for configurations 0 and 5.

Figure 10.2.6.2-1 illustrates the resource elements used for reference signal transmission according to the above definition. The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 10.2.6.2-1. Mapping of downlink narrowband reference signals (normal cyclic prefix)

### 10.2.6A	Narrowband positioning reference signal (NPRS)

Narrowband positioning reference signals (NPRSs) shall only be transmitted in resource blocks in NB-IoT carriers configured for NPRS transmission. In a subframe configured for NPRS transmission, the starting positions of the OFDM symbols configured for NPRS transmission shall be identical to those in a subframe in which all OFDM symbols have the same cyclic prefix length as the OFDM symbols configured for NPRS transmission. NPRS are defined for and normal CP only.

NPRSs are transmitted on antenna port 2006.

#### 10.2.6A.1	Sequence generation

The NPRS sequence  is defined by

where  is the slot number within a radio frame,  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each OFDM symbol where  equals  unless configured by higher layers and where

#### 10.2.6A.2	Mapping to resource elements

For an NB-IoT carrier which is configured for NPRS transmission, the reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference signal for antenna port  in slot  according to

where

-	when the higher layer parameter when the higher layer parameter operationModeInfoNPRS for the configured NB-IoT carrier is set to in-band

where  is signalled by higher layers nprs-SequenceInfo, and  if the higher layer parameter nprs-SequenceInfo indicates is odd, and  if the higher layer parameter nprs-SequenceInfo indicates is even.

-	when the higher layer parameter operationModeInfoNPRS for the configured NB-IoT carrier is set to standalone or guard-band

and where . If is not configured by higher layers, . The number of PBCH antenna ports is signalled by higher layers.

If higher layer parameter nprsBitmap is not configured, resource elements in OFDM symbols 5 and 6 in each slot shall not be used for transmission of NPRS.

Figure 10.2.6A.2-1: Mapping of NPRS (operationModeInfoNPRS is set to in-band, nprsBitmap configured)

Figure 10.2.6A.2-2: Mapping of NPRS (operationModeInfoNPRS is set to standalone or guard-band, nprsBitmap configured)

#### 10.2.6A.3	NPRS subframe configuration

On a NB-IoT DL carrier configured for NPRS transmission, an NB-IoT UE can assume NPRSs are transmitted in DL subframes configured by all higher layer parameters nprsBitmap, the NB-IoT carrier-specific subframe configuration period  the NB-IoT-carrier-specific starting subframe offset  and the number of consecutive downlink subframes  where NPRS shall be transmitted. If frame structure type 2 is used, the UE shall not assume NPRSs are transmitted in special subframes.

-	If ,  and  are not configured for an NB-IoT downlink carrier configured for NPRS transmission, an NB-IoT UE shall assume NPRSs are transmitted in downlink subframes configured by higher layer parameter nprsBitmap.

-	If nprsBitmap is not configured for an NB-IoT downlink carrier configured for NPRS transmission, an NB-IoT UE shall assume NPRSs are transmitted in downlink subframes configured by the higher layer parameters ,  and .

-	If the higher layer parameter operationModeInfoNPRS for the configured NB-IoT carrier is set to in-band, the higher layer parameters nprsBitmap shall be configured.

-	If ,  and  are configured, the NPRS instances in the first subframe of the  downlink subframes, shall satisfy .

The NPRSs shall not be mapped to resource elements  allocated to resource blocks of NPBCH, NPSS, NSSS, or SystemInformationBlock-Type1-NB regardless of their antenna port .

### 10.2.6B	Narrowband wake up signal (NWUS)

#### 10.2.6B.1	Sequence generation

The NWUS sequence w(m) in subframe x=0, 1, …, M-1 is defined by

wm=θnf,nsm'∙e-jπunn+1131

m=0, 1,…, 131

m'=m+132x

n=m mod 132

θnf,nsm'=1, if cnf,ns2m'=0 and  cnf,ns2m'+1=0-1, if cnf,ns2m'=0 and  cnf,ns2m'+1=1j, if cnf,ns2m'=1 and  cnf,ns2m'+1=0-j, if cnf,ns2m'=1 and  cnf,ns2m'+1=1

u=NIDNcellmod 126+3

where M is the actual duration of NWUS 1≤M≤MNWUSmaxMNWUSmaxas defined in  [4].

The scrambling sequence cnf,nsi, i=0, 1, …, 2∙132M-1 is given by clause 7.2, and shall be initialized at the start of the NWUS with

cinit_WUS=(NIDNcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell

where nf_start_PO is the first frame of the first PO to which the NWUS is associated, and ns_start_PO is the first slot of the first PO to which the NWUS is associated.

#### 10.2.6B.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the NWUS within a subframe. The UE shall not assume that the NWUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. If only one NRS port is configured by the eNB, the UE may assume the transmission of all NWUS subframes is using the same antenna port; otherwise, the UE may assume the same antenna port is used for NWUS transmission in DL subframes w0+2n and w0+2n+1, where w0 is the first DL subframe of the NWUS transmission as specified in [4], and n=0,1,…. .

The NWUS sequence is mapped to the set of subframes in the actual NWUS duration as defined in [4], where in a subframe #4 in which SystemInformationBlockType1-NB is transmitted or a subframe in which an SI message is transmitted, the subframe is counted in the NWUS mapping but not used for transmission of NWUS.

On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating inband-SamePCI, inband-DifferentPCI, guardband or standalone or on an NB-IoT carrier for which DL-CarrierConfigCommon-NB is present, the NWUS sequence wm shall be mapped to resource elements (k,l) in sequence, starting with w(0) in increasing order of first the index k=0,1,…,NscRB-1,  over the 12 assigned subcarriers and then the index l=3, 4, …, 2NsymbDL-1 in each subframe in which NWUS is transmitted.

Additionally, on an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating guardband or standalone, or on an NB-IoT carrier for which DL-CarrierConfigCommon-NB is present and no inbandCarrierInfo is present, the resource mapping for the first three OFDM symbols in the subframe is performed as follows:

-	The resource element (k,7) is mapped to resource element (k,0) of every index k over 12 assigned subcarriers

-	The resource element (k,8) is mapped to resource element (k,1) of every index k over 12 assigned subcarriers

-	The resource element (k,9) is mapped to resource element (k,2) of every index k over 12 assigned subcarriers

A resource element (k,l) overlapping with resource elements where cell-specific reference signals according to clause 6.10 are transmitted or NRSs according to clause 10.2.6 are transmitted shall not be used for NWUS transmission but is counted in the mapping process.

### 10.2.7	Synchronization signals

There are 504 unique physical-layer cell identities indicated by the narrowband secondary synchronization signal.

#### 10.2.7.1	Narrowband primary synchronization signal (NPSS)

##### 10.2.7.1.1	Sequence generation

The sequence  used for the narrowband primary synchronization signal is generated from a frequency-domain Zadoff-Chu sequence according to

where the Zadoff-Chu root sequence index  and for different symbol indices  is given by Table 10.2.7.1.1-1.

Table 10.2.7.1.1-1: Definition of .

| Cyclic prefix length |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 |

##### 10.2.7.1.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the narrowband primary synchronization signal within a subframe.

UE shall not assume that the narrowband primary synchronization signal is transmitted on the same antenna port as any of the downlink reference signals. The UE shall not assume that the transmissions of the narrowband primary synchronization signal in a given subframe use the same antenna port, or ports, as the narrowband primary synchronization signal in any other subframe.

The sequences  shall be mapped to resource elements  in increasing order of first the index  and then the index  in subframe 5 in every radio frame. For resource elements  overlapping with resource elements where cell-specific reference signals according to clause 6.10 are transmitted, the corresponding sequence element  is not used for the NPSS but counted in the mapping process.

#### 10.2.7.2	Narrowband secondary synchronization signal (NSSS)

##### 10.2.7.2.1	Sequence generation

The sequence  used for the narrowband secondary synchronization signal is generated from a frequency-domain Zadoff-Chu sequence according to

where

The binary sequence  is given by Table 10.2.7.2.1-1. The cyclic shift in frame number  is given by

.

Table 10.2.7.2.1-1: Definition of .

| 0 | [1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1] |
| --- | --- |
| 1 | [1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1] |
| 2 | [1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1] |
| 3 | [1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1] |

##### 10.2.7.2.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the narrowband secondary synchronization signal within a subframe.

The UE shall not assume that the narrowband secondary synchronization signal is transmitted on the same antenna port as any of the downlink reference signals. The UE shall not assume that the transmissions of the narrowband secondary synchronization signal in a given subframe use the same antenna port, or ports, as the narrowband secondary synchronization signal in any other subframe.

If indicated by higher layer, a UE may assume different precoders are applied for NSSS transmission in a number of consecutive NSSS occasions signalled by higher layer.

The sequence  shall be mapped to resource elements  in sequence starting with  in increasing order of first the index  over the 12 assigned subcarriers and then the index over the assigned last  symbols of subframe 9 for frame structure type 1 or subframe 0 for frame structure type 2  in radio frames fulfilling , where  is given by Table 10.2.7.2.2-1.

Table 10.2.7.2.2-1: NSSS number of symbols

| Cyclic prefix length |  |
| --- | --- |
| Normal | 11 |

For resource elements  overlapping with resource elements where cell-specific reference signals according to clause 6.10 are transmitted, the corresponding sequence element  is not used for the NSSS but counted in the mapping process.

### 10.2.8	OFDM baseband signal generation

For an NB-IoT carrier

-	for which the higher layer parameter operationModeInfo indicates 'inband-DifferentPCI ' and for all NB-IoT downlink physical channels and signals except NPRS,

-	for which the higher layer parameter operationModeInfo indicates 'Guardband ' or 'Standalone ',

-	for an NB-IoT carrier for which the higher layer parameter CarrierConfigDedicated-NB or DL-CarrierConfigCommon-NB is present and no inbandCarrierInfo is present, or

-	for an NB-IoT carrier for which the higher layer parameters CarrierConfigDedicated-NB or DL-CarrierConfigCommon-NB is present and inbandCarrierInfo is present and the higher layers do not indicate  is the same as  and for all NB-IoT downlink physical channels and signals except NPRS,

the time-continuous signal  on antenna port  in OFDM symbol  in a downlink slot is defined by

for where , N=2048,  and  is the content of resource element  on antenna port .

Otherwise, the time-continuous signal  on antenna port  in OFDM symbol , where  is the OFDM symbol index from the start of the last even-numbered subframe, is defined by

for  where  and,  if resource element is used for Narrowband IoT except for NPRS, and 0 otherwise including NPRS. The quantity fNB-IoT is the frequency location of the center of the Narrowband IoT PRB minus the frequency location of the center of the LTE signal.

Only normal CP is supported for Narrowband IoT downlink in this release of the specification.

### 10.2.9	Modulation and upconversion

Modulation and upconversion to the carrier frequency of the complex-valued OFDM baseband signal for each antenna port is shown in Figure 6.13-1. The filtering required prior to transmission is defined by the requirements in 3GPP TS 36.104 [6].