3GPP TSG RAN WG1 Meeting #94bis                       R1-1810257

Chengdu, China, October 8th – 12th, 2018

Agenda Item:	7.1.3.2

Source: 	LG Electronics

Title: 	Correction on PUCCH and UCI multiplexing for NR

Document for:	Discussion and decision

- Introduction

In RAN plenary meeting, NR specifications TS38.211, TS38.212, TS38.213, and TS38.214 had been approved [1 – 4]. In this contribution, we discuss some remaining issues on PUCCH and UCI multiplexing.

- Discussion

  - UCI multiplexing among overlapping PUCCH(s)

In the previous RAN1#94 meeting, the followings were agreed for the UCI multiplexing rule on overlapping PUCCH resources in a slot [5].

Agreements: - When a UE is not configured with multi-CSI-PUCCH report, - When SR occasion and/or DL SPS HARQ-ACK and two non-overlapping CSI PUCCH resources contained in Resource A and Set X (as defined in RAN1 #93 meeting) decided by the pseudo code (as in 9.2.5 of TS38.213) in a slot - DL SPS HARQ-ACK/SR is multiplexed on the CSI PUCCH resource with higher priority, and CSI on the CSI PUCCH resource with lower priority is dropped - In case of presence of DL SPS HARQ-ACK, multiplexing occurs if the corresponding timeline requirement is met, otherwise, it is considered as an error case

In addition to the SPS HARQ-ACK case, this behavior can be extended to any case where there are two or more non-overlapping CSI PUCCH resources in Resource A and Set X (as defined in RAN1 #93 meeting). More specifically, when two or more non-overlapping CSI PUCCH resources are included in Resource A and Set X, only the CSI report having the highest priority among the plurality of CSI reports can be subjected to UCI multiplexing and the remaining CSI reports can be dropped, even in case when a HARQ-ACK corresponding to the PDSCH scheduled by DL grant DCI is contained in the same Resource A and Set X. In addition, the case when a UE is configured with multi-CSI PUCCH report can also be considered. In the case, the UE can consider that all CSI reports in Resource A and Set X are subject to UCI multiplexing, also even in case when a HARQ-ACK corresponding to the PDSCH scheduled by DL grant DCI is contained in the same Resource A and Set X.

Proposal #1: When two non-overlapping CSI PUCCH resources and HARQ-ACK PUCCH resource corresponding to the PDSCH scheduled by DL grant DCI are contained in Resource A and Set X (as defined in RAN1 #93 meeting) decided by the pseudo code (as in 9.2.5 of TS38.213) in a slot:

- When a UE is not configured with multi-CSI-PUCCH report,

  - Only the highest priority CSI report is subject to UCI multiplexing on the HARQ-ACK PUCCH resource, and the remaining lower priority CSI report(s) are dropped

- When a UE is configured with multi-CSI-PUCCH report,

  - All CSI reports are subject to UCI multiplexing on the HARQ-ACK PUCCH resource

  - Tie-breaking between multiple PUCCH resources for CSI reporting

In RAN1 #92 meeting, it was agreed that up to 2 multi-CSI PUCCH resources can be configured to a UE per UL BWP and the basic principle behind the PUCCH resource determination when more than one multi-CSI PUCCH resource is configured is to choose the multi-CSI PUCCH resource with the smallest capacity but such that the CSI payload of all colliding CSI reports is not larger than the capacity. And if no such resource is configured, choose the largest multi-CSI PUCCH resource and apply dropping rules. As we understand it, there is no such restriction that the configured two multi-CSI PUCCH resources shall not have the same capacity. If configuring two multi-CSI PUCCH resources with the same capacity is allowed for some reasons, e.g., same capacity but different code rates or different PUCCH durations, the UE and gNB should have the common understanding on which multi-CSI PUCCH resource the UE is expected to transmit the CSI reports to avoid unnecessary BD trials at the gNB side.

The tie-breaking rule that we see a merit is to choose the PUCCH format with the higher maximum code rate for efficiency because the smaller number of REs are occupied for the same CSI payload bits. If the configured maximum code rates of two different PUCCH formats are same, then the PUCCH resource having the PUCCH format with the shorter duration can be prioritized in favour of short latency. For applications where the low latency is of top priority, the PUCCH format with shorter duration should be prioritized over that of the higher maximum code rate. However, if the application-dependent priority control is needed, it may be more desirable that the gNB takes control of the priority e.g., by letting the UE select the PUCCH resource based on the order in the RRC configuration list.

Proposal #2: If the supportable maximum UCI payload size (or capacity) of different PUCCH resources configured for multiple CSI reports are the same, the PUCCH resource is selected based on the following priority order:

- PUCCH resource with the higher maximum code rate (R)

- PUCCH resource having the PUCCH format with shorter duration (if the configured maximum code rates are the same)

  - Power offset considering multiple SR configuration

When K (> 1) SR and HARQ-ACK are multiplexed and transmitted using PUCCH format 1, the hypothesis for channel selection at the base station may be increased in proportion to the value of K, resulting in degraded performance. In order to solve the above problem, an operation of applying an additional power offset for the PUCCH transmission according to the K value may need to be considered.

Proposal #3: When K (> 1) SR and HARQ-ACK are multiplexed and transmitted using PUCCH format 1, applying power offset in proportion to K can be considered.

- Conclusion

In this contribution, we provided our views on some remaining issues on PUCCH and UCI multiplexing and the followings were proposed:

Proposal #1: When two non-overlapping CSI PUCCH resources and HARQ-ACK PUCCH resource corresponding to the PDSCH scheduled by DL grant DCI are contained in Resource A and Set X (as defined in RAN1 #93 meeting) decided by the pseudo code (as in 9.2.5 of TS38.213) in a slot:

- When a UE is not configured with multi-CSI-PUCCH report,

  - Only the highest priority CSI report is subject to UCI multiplexing on the HARQ-ACK PUCCH resource, and the remaining lower priority CSI report(s) are dropped

- When a UE is configured with multi-CSI-PUCCH report,

  - All CSI reports are subject to UCI multiplexing on the HARQ-ACK PUCCH resource

Proposal #2: If the supportable maximum UCI payload size (or capacity) of different PUCCH resources configured for multiple CSI reports are the same, the PUCCH resource is selected based on the following priority order:

- PUCCH resource with the higher maximum code rate (R)

- PUCCH resource having the PUCCH format with shorter duration (if the configured maximum code rates are the same)

Proposal #3: When K (> 1) SR and HARQ-ACK are multiplexed and transmitted using PUCCH format 1, applying power offset in proportion to K can be considered.

- Reference

- 3GPP TS38.211 V15.2.0 (2018-06): “NR; Physical channels and modulation”.

- 3GPP TS38.212 V15.2.0 (2018-06): “NR; Multiplexing and channel coding”.

- 3GPP TS38.213 V15.2.0 (2018-06): “NR; Physical layer procedures for control”.

- 3GPP TS38.214 V15.2.0 (2018-06): “NR; Physical layer procedures for data”.

- RAN1 chairman’s notes, RAN1 #94.