3GPP TSG RAN WG1 Meeting #94bis	R1-1810858

Chengdu, China, October 8th – 12th, 2018

Agenda item:	7.2.2.3.1

Source:	Samsung

Title:	Downlink signal and channel design for NR-U

Document for:	Discussion and Decision

# Introduction

In RAN1#94 [1], the following agreements were made:

Agreement: - Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for - Meeting OCB requirement - Compacting signals in time domain to limit the required number of channel access and for short channel occupancy - Support of stand-alone NR-U deployments - Support of automatic neighbour relations (ANR) functionality in an NR-U deployment - Resolution of PCI confusion in an NR-U deployment - Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals - The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial - Note: This does not imply that RMSI-CORESET+PDSCH and CSI-RS can only be transmitted as part of the NR-U DRS, and does not imply that these are necessarily part of all NR-U DRS transmissions.

This contribution discusses the aspects related to the downlink signal and channel design for NR-U:

- NR-U discovery signals and channels (DSCH)

- Indication of COT structure

- Signal for burst detection

- Adaptation of PDCCH monitoring periodicity

# Discovery Signals and Channels (DSCH)

## Terminology

In the last RAN1 meeting [1], it was agreed that the NR-U DRS can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals. However, it was also noted that “NR-U DRS” may not be a proper terminology to reflect the signals and channels with the supported functionality. Hence, we propose to use an alternative terminology as “Discovery Signals and Channels (DSCH)” for NR-U, although it is still convenient to use “NR-U DRS” in the offline discussion.

Proposal 1: Support the terminology of “Discovery Signals and Channels (DSCH)” for NR-U DRS.

## Numerology of DSCH

In NR FR1, 15 kHz and 30 kHz are the two candidate SCS values for SS/PBCH block, and 15 kHz and 30 kHz are also the candidate SCS values for RMSI/OSI/paging. One of the discussion points is whether or not to support 60 kHz subcarrier spacing for SS/PBCH block and RMSI/OSI/paging, for NR-U sub7 GHz. In the last meeting [1], it was agreed to be identified beneficial to support using the same numerology for all DL signals and channels at least for standalone operation, and it was further identified beneficial to support using the same numerology for all DL and UL signals and channels for standalone operation in our accompany contribution [2]. We study the following aspects on comparing using 30 kHz or 60 kHz as the supported SCS for DSCH:

1. Cell search performance.

In NR FR1, 30 kHz SCS of SS/PBCH block has been proved to be sufficient for resisting initial CFO for all the carrier frequency range, and it was further testified by RAN4 that 30 kHz SCS of SS/PBCH block can be used for carrier frequency range 3 GHz to 5 GHz (e.g. band n77, n78, n79). Hence, from the perspective of initial cell search performance, both 30 kHz and 60 kHz SCS of SS/PBCH block can perform well enough to resist initial CFO.

2. OCB requirement.

SS/PBCH block with 60 kHz SCS and 20 PRB BW occupies 72% of the nominal bandwidth of 20 MHz. SS/PBCH block with 30 kHz SCS and 20 PRB BW occupies 36% of the nominal bandwidth of 20 MHz, but in standalone operation, it can be further multiplexed with other channels or signals in DSCH (e.g. PDSCH of RMSI/OSI/paging) to meet the OCB requirement.

3. Transmission duration.

60 kHz SCS can provide an extra benefit of shortening the beam sweeping time of SS/PBCH burst set by assuming using the same number of SS/PBCH blocks within a burst set as 30 kHz SCS, however, if considering multiplexed SS/PBCH burst set with the associated PDCCHs/PDSCHs of RMSI/OSI/paging in the DSCH, there is no gain in transmission duration by using 60 kHz SCS at least for standalone case.

4. Coverage for PDCCH of RMSI/OSI/paging.

If using 60 kHz as the SCS of DSCH, the CORESET BW can only be configured as 24 PRBs based on the specified channel utilization of 20 MHz carrier bandwidth. In contrast, the CORESET BW can be configured as large as 48 PRBs, if using 30 kHz as the SCS of DSCH. Hence, the coverage for PDCCH of RMSI/OSI/paging may be degraded by using 60 kHz SCS comparing with using 30 kHz SCS.

5. Frequency offset configuration.

For 20 MHz channel BW, the channel utilization is 51 RBs by using 30 kHz SCS and 24 RBs by using 60 kHz SCS. Comparing to the supported CORESET BW (e.g. 48 RBs for 30 kHz SCS and 24 RBs for 60 kHz SCS), using 30 kHz SCS is more robust to the uncertainty of location on channel raster, such that smaller number of configurations on the frequency offset indication is required, which is beneficial in potentially saving bit in PBCH content for NR-U.

6. Specification impact.

NR FR1 has fully supported using 30 kHz for all the DL and UL signals and channels, hence, using 30 kHz for NR-U has minimized specification impact, comparing to using 60 kHz. For example, there is no need to further discuss sub-PRB interlace waveform for standalone operation if 30 kHz is the only supported SCS for all DL and UL signals and channels.

From above discussions, we didn’t observe a clear benefit of supporting 60 kHz as the SCS of DSCH at least for standalone operation. Hence, for 5 GHz unlicensed spectrum, 30 kHz should be adopted as the SCS of DSCH for standalone operation, and 60 kHz can be considered as one of the candidate SCSs of DSCH for non-standalone operation. Same proposal can be applicable to 6 GHz unlicensed spectrum as well, if the minimum channel bandwidth of 6 GHz unlicensed spectrum is also 20 MHz.

Proposal 2: For 5 GHz unlicensed spectrum, 30 kHz shall be supported as the SCS of DSCH for standalone operation, and 60 kHz can be considered as one of the candidate SCSs of DSCH for non-standalone operation.

- Same proposal can be supported for 6 GHz unlicensed spectrum, if the minimum channel bandwidth is also 20 MHz.

- Further study the mapping of SS/PBCH blocks if 60 kHz is supported as one candidate SCS of DSCH for non-standalone operation.

## Multiplexing in DSCH

In the last meeting, it was agreed that it is identified beneficial to multiplex PDCCHs/PDSCHs of RMSI/OSI/paging into DSCH with their associated SS/PBCH block. One remaining detail is the multiplexing method of PDCCH/PDSCH of RMSI/OSI/paging with the associated SS/PBCH block.

When the SCS of DSCH is 30 kHz for standalone operation, examples of the multiplexing of SS/PBCH block, its associated CORESET, and PDSCH are illustrated in Figure 1, wherein the CORESET is multiplexed with the associated SS/PBCH block using multiplexing Pattern 1 with group offset 0 (e.g. TDMed within the same slot), and PDSCH is FDMed with the associated SS/PBCH block (e.g. frequency offset is up to the configuration in PBCH content).

Figure 1 Examples of multiplexing pattern for DSCH.

The multiplexing Pattern 2 and Pattern 3 were also proposed and discussed in the last meetings. However, based on a previous agreement that the initial active DL BWP is 20 MHz, wherein the initial active DL BWP is same as CORESET BW by its definition, the SS/PBCH block and its associated CORESET cannot be confined within the same nominal channel of 20 MHz in Pattern 2 or Pattern 3, which is not preferable in initial cell search procedure and should not be supported in NR-U.

Observation 1: SS/PBCH block and its associated CORESET cannot be confined within the same nominal channel of 20 MHz, if using multiplexing Pattern 2 or Pattern 3.

Proposal 3: Multiplexing Pattern 1 is sufficient for DSCH in NR-U, and Pattern 2 and Pattern 3 are not needed.

## Enhancement on SS/PBCH block

In NR, one SS/PBCH block is consisted from 4 consecutive symbols with a BW of 20 PRBs, wherein the central 12 PRBs of the first and third symbols are mapped for PSS and SSS, respectively, and the second and fourth symbols as well as the remaining PRBs in the third symbol other than SSS are mapped for PBCH. An illustration of the composition of SS/PBCH block is shown in Figure 2.

Figure 2 Illustration of SS/PBCH block composition in NR.

Based on this composition pattern, it has been observed that the cell ID detection performance in NR can achieve 90% and the PBCH detection error (conditioning on accurate cell ID) can be within 10-2 at SNR -6 dB, based on up to 4 combinations of SS/PBCH blocks.

In NR-U, the transmission of SS/PBCH blocks can be subject to the sensing result of Listen-Before-Talk (LBT), such that the UE cannot always expect to receive the SS/PBCH blocks periodically, which challenges the receiver implementation on combining multiple SS/PBCH blocks and degrades the performance of cell search and system information delivery.

Hence, potential enhancement on the SS/PBCH block structure, e.g. larger transmission BW and/or larger number of symbols should be studied in NR-U, in order to enhance the one-shot detection performance to composite the loss due to reduced channel access opportunities subject to LBT, especially when larger periodicity of SS/PBCH block is considered for NR-U. For example, by adding another symbols for extra PSS, SSS, or PBCH, the performance of one-shot detection can be improved.

Proposal 4: Enhancement to the synchronization signal and PBCH block structure is observed beneficial for improving the one-shot detection performance and compensating reduced channel access opportunities subject to LBT, and should be studied for NR-U.

# Indication of COT structure

In LTE, subframe configuration for LAA in current and next subframe can be indicated by a PDCCH with CRC scrambled by CC-RNTI. That is, if a UE detects the PDCCH with CRC scrambled by CC-RNTI in subframe n-1 or subframe n of a LAA SCell, the UE may assume that the configuration of occupied OFDM symbols in subframe n according to the 'Subframe configuration for LAA' field in the detected DCI in subframe n-1 or subframe n. For eLAA, a UL offset (l) and UL duration (d) can also be indicated by the PDCCH with CRC scrambled by CC-RNTI, together with the subframe configuration in subframe n-1 or subframe n. If the UE detects the PDCCH with CRC scrambled by CC-RNTI in subframe n, the UE is not required to receive any downlink physical channels and/or physical signals in subframe(s) n+l+i with i=0, 1, … , d-1, which may save UE power consumption to some extent by eliminating unnecessary PDCCH monitoring in uplink subframe(s). Furthermore, the UE may use channel access Type 2 for transmissions in subframes n+l+i where i=0, 1, … , d-1, irrespective of the channel access Type signalled in the UL grant for those subframes, if the end of UE transmission occurs in or before subframe n+l+d-1. Since the subframe configuration and UL duration are indicated by the number of occupied OFDM symbols for downlink and the number of subframes for uplink, these can be indicated by at least one of SFI values specified in NR. Therefore, Rel-15 SFI can be used to indicate COT structure (e.g. DL/UL direction of COT) for NR-U, which is beneficial for for CSI measurement in DL portion, PDCCH monitoring skipping in UL portion, periodic DL/UL signal transmission, etc.

Proposal 5: Rel-15 SFI can be used to indicate COT structure for NR-U.

In Rel-15, autonomous UL (AUL) transmission has been introduced to increase uplink performance by eliminating control channel signalling for uplink transmission which requires additional LBT for transmission. In addition to subframe(s) configured for AUL transmission, AUL transmission can be performed in subframe(s) within the indicated UL duration. That is, AUL sharing field is included in the PDCCH with CRC scrambled by CC-RNTI to inform to AUL UE about availability of AUL transmission within the indicated UL duration. If NR-U also supports dynamic AUL sharing mechanism, some enhancements on DCI format 2-0 or defining new DCI format to indicate slot format would be expected. Therefore, further study would be required to clarify necessary information to be informed for NR-U operation. If additional information other than slot format information (e.g. AUL sharing) is required, efficient signalling mechanism should also be further investigated.

Proposal 6: Clarify necessary information to be informed together with SFI for NR-U operation.

# Signal for Detection

In NR phase 1, a UE shall monitor a set of PDCCH candidates in one or more control resource sets (CORESET) according to corresponding search spaces, wherein the search spaces may appear periodically according to the configured periodicity by higher-layer signaling. For NR-U operation, the periodicity of PDCCH is likely to be broken, e.g., due to LBT failure. Furthermore, if the configured periodicity is relatively large, e.g., 20 slot, the DL/UL transmission opportunity would be dramatically reduced if gNB fails LBT for PDCCH search space slot. On the other hand, if the configured periodicity is small, e.g., 1 slot or less than a slot, more transmission opportunities are provided at the cost of more frequent PDCCH detection. In LTE, UE may skip PDCCH decoding if UE does not detect the CRS. In NR, CSI-RS for tracking or DMRS for PDCCH may not achieve the same robust detection performance as the CRS, thus UE may not skip PDCCH decoding in slots which gNB does not transmit. For example, wideband DMRS for a PDCCH can be considered to facilitate burst detection, which can provide similar detection performance as the CRS.

Proposal 7: Utilizing existing RS (e.g. wideband DMRS) for power-efficient burst detection in NR-U.

In the last meeting, using Wi-Fi preamble as the indication of burst detection was discussed. However, Wi-Fi preamble is constructed by a single unified sequence, which may not be properly utilized for the indication of burst detection in a cell-specific or UE-specific transmission. Meanwhile, the information on COT (e.g. packet duration) is carried by a separate associated field to the Wi-Fi preamble, which cannot be decodable using currently supported channel decoder in NR, such that using Wi-Fi preamble as the indication of burst detection may not be beneficial for co-existence at all.

Proposal 8: Wi-Fi preamble shall not be considered as a candidate signal for burst detection in NR-U.

# Adaptation of PDCCH monitoring periodicity

For supporting start of DL transmission immediately after gNB’s channel access, the time granularity for PDCCH monitoring periodicity should be finer (e.g., every symbol), which results in significant UE power consumption for PDCCH monitoring. Therefore, for the purpose of UE power saving while supporting flexible and fast DL transmission, adaptation of PDCCH monitoring periodicity according to the acquisition of COT would be desirable. For instance, larger PDCCH monitoring periodicity within a gNB’s COT than that for outside the gNB’s COT can be considered.

Proposal 9: NR-U support adaptation of PDCCH monitoring periodicity (e.g., larger PDCCH monitoring periodicity within a gNB’s COT than that for outside the gNB’s COT)

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: Support the terminology of “Discovery Signals and Channels (DSCH)” for NR-U DRS.

Proposal 2: For 5 GHz unlicensed spectrum, 30 kHz shall be supported as the SCS of DSCH for standalone operation, and 60 kHz can be considered as one of the candidate SCSs of DSCH for non-standalone operation.

- Same proposal can be supported for 6 GHz unlicensed spectrum, if the minimum channel bandwidth is also 20 MHz.

- Further study the mapping of SS/PBCH blocks if 60 kHz is supported as one candidate SCS of DSCH for non-standalone operation.

Proposal 3: Multiplexing Pattern 1 is sufficient for DSCH in NR-U, and Pattern 2 and Pattern 3 are not needed.

Proposal 4: Enhancement to the synchronization signal and PBCH block structure is observed beneficial for improving the one-shot detection performance and compensating reduced channel access opportunities subject to LBT, and should be studied for NR-U.

Proposal 5: Rel-15 SFI can be used to indicate COT structure for NR-U.

Proposal 6: Clarify necessary information to be informed together with SFI for NR-U operation.

Proposal 7: Utilizing existing RS (e.g. wideband DMRS) for power-efficient burst detection in NR-U.

Proposal 8: Wi-Fi preamble shall not be considered as a candidate signal for burst detection in NR-U.

Proposal 9: NR-U support adaptation of PDCCH monitoring periodicity (e.g., larger PDCCH monitoring periodicity within a gNB’s COT than that for outside the gNB’s COT)

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #94, Gothenburg, Sweden, August 20th - 24th, 2018.

[2] R1-1810857, Frame structure for NR-U, Samsung

[3] R1-1810859, Uplink signal and channel design for NR-U, Samsung

[4] R1-1810860, Channel access procedure for NR-U, Samsung

[5] ETSI EN 302 567 V2.0.22.