3GPP TSG RAN WG1 Meeting #95                                                              R1-1812175

Spokane, USA, November 12th – 16th, 2018

# Source: 	ZTE, Sanechips

# Title:	System level simulation results for NOMA

# Agenda Item:	7.2.1.4

# Document for:	Discussion/Decision

# Introduction

The evaluation methodologies, performance metrics, traffic models and simulation assumptions for system-level evaluations for NOMA were discussed and agreed in RAN1 #92bis, #93 and #94 meetings[1][2][3]. And the coupling loss and downlink geometry were calibrated in the email discussion after RAN1 #94 meeting.

In this contribution, system level evaluation results for NOMA in mMTC, URLLC, and eMBB scenarios are further updated and discussed, on the basis of the contribution for last meeting [4].

# SLS results for NOMA

In the evaluation, the performance comparison between baseline scheme and MUSA in mMTC, URLLC and eMBB scenarios are conducted with the agreed assumptions shown in Table A1 in the Appendix. It should be noted that some parameters do not have mandated values, should be reported by companies, e.g. number of UEs per cell, power control, and HARQ/repetition, etc.  The values used for the simulations in this contribution are given in Table 1 for mMTC, URLLC and eMBB scenarios respectively. In addition, some details of the baseline and MUSA can be also found in the table.

Table 1 Assumptions reported for MUSA system level performance evaluation in this contribution

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| Carrier Frequency + ISD | 700MHz + 1732m | 4GHz + 200m 700MHz + 500m | 4GHz + 200m |
| Simulation bandwidth | 6 PRBs | 12 PRBs | 12 PRBs |
| Resource allocation | Case 1: 1 PRB per UE for baseline, 1 PRB + 4ms per UE for MUSA; Case 2: 6 PRBs per UE for baseline and MUSA; spreading codes of length 4 are used [5] | 3 PRBs per UE for baseline, 12 PRBs per UE for MUSA; spreading codes of length 4 are used [5] | Case 1: 3 PRBs per UE for baseline, 12 PRBs per UE for MUSA Case 2: 12 PRBs per UE for baseline and MUSA; spreading codes of length 4 are used [5] |
| Configured grant details | 24 DMRS is assumed for each RB; Periodicity = 4 ms for baseline and MUSA for Case 1; Periodicity = 5 ms for baseline and MUSA for Case 2 | 24 DMRS is assumed for each RB; For 60 bytes packet size: periodicity = 0.5 ms for baseline and MUSA For 200 bytes packet size: periodicity = 2 ms for baseline and MUSA | 24 DMRS is assumed for each RB; Periodicity = 5 ms for baseline and MUSA for both Case 1 and Case 2 |
| Packet size | 20~200 bytes Pareto distribution with shaping parameter alpha = 2.5, and 29 bytes protocol overhead | 60 bytes 200 bytes | 50~600 bytes Pareto distribution with shaping parameter alpha = 1.5, no protocol overhead |
| Segmentation | Yes, 5 bytes packet segmentation overhead is added for each TB | No | Yes, 5 bytes packet segmentation overhead is added for each TB |
| HARQ/repetition | For configured grant, the maximum number of HARQ transmissions is 8, and non-adaptive re-transmissions are assumed; For grant-free with random selection, UEs are classified to 3 groups, different repetitions are assumed respectively | Number of transmission(s) = 1 No HARQ/repetition | Number of transmission(s) = 1 No HARQ/repetition |
| ARQ retransmission | Not modelled, if one TB is not received correctly after HARQ/ repetition, the packet is dropped | Not modelled | Not modelled, if one TB is not received correctly after HARQ transmission, the packet is dropped |
| Number of UEs per cell | 100 | 20 | 100 |
| UE power control | Open loop power control: P0 = -100 dBm, alpha = 1 for configured grant; P0 = -95 dBm, alpha = 1 for grant-free with random selection | Open loop power control, P0 = -90 dBm, alpha = 1 | Open loop power control, P0 = -95 dBm, alpha = 1 |
| Number of BS antennas | 2Rx | 4Rx | 4Rx |
| BS antenna downtilt | 92 | 102 for 4GHz + 200m 98 for 700MHz + 500m | 102 |
| Channel estimation | Realistic | Realistic | Realistic |
| BS receiver | For configured grant: Baseline: MMSE-IRC or MMSE-PIC MUSA: MMSE-PIC For grant-free with random selection: MMSE-IC for baseline and MUSA | Baseline: MMSE-IRC or MMSE-PIC (max times of decoding for a UE = 2) MUSA: same as baseline | Baseline: MMSE-IRC or MMSE-PIC MUSA: MMSE-PIC |

## mMTC (configured grant)

In mMTC configured grant scenario, the transmission resources including time/frequency resource and DMRS are preconfigured, and there is no DMRS collision. The maximum number of HARQ transmissions is 8.

- Case 1: 1 PRB per UE for baseline, 1 PRB + 4ms per UE for MUSA

In this case, resource allocation with FDM mode is used. For fair comparison, the energy of spreading codes for MUSA is normalized to 1.

Figure 1(a) shows the packet drop rate (PDR) performances for baseline with MMSE-IRC receiver or MMSE-PIC receiver, and MUSA with MMSE-PIC receiver. From the figure, we can observe that the supported packet arrival rate (PAR) at PDR = 1% for MUSA is about 2 times of that for baseline with MMSE-IRC receiver, and is more than 1.5 times of that for baseline with MMSE-PIC receiver.

The distributions of maximum number of UEs per transmission resource and resource utilization are shown in Figure 1(b) and 1(c) respectively for baseline and MUSA. It can be observed that the maximum number of UEs per transmission resource and resource utilization are increasing with the traffic load, and these metrics for MUSA are relatively higher than baseline because that in this case multiple basic resources would be used after spreading and more users would use the same transmission resource. It should be noted that multiple times of HARQ transmissions also have impact on these metrics. The maximum number of UEs per transmission resource for baseline and MUSA would be some specific values at high traffic load due to the configured grant mechanism.

| (a) Packet drop rate | (b) Max number of UEs per Tx resource | (c) Resource utilization |
| --- | --- | --- |

Figure 1 Simulation results for mMTC configured grant Case 1

- Case 2: 6 PRBs per UE for baseline and MUSA

In this case, lower code rate would be used for baseline relative to Case 1.

Figure 2(a) shows the PDR performances for baseline with MMSE-IRC receiver or MMSE-PIC receiver, and MUSA with MMSE-PIC receiver. From the figure, we can observe that the supported PAR at PDR = 1% for MUSA is more than 2 times of that for baseline with MMSE-IRC receiver or MMSE-PIC receiver.

Figure 2(b) shows the maximum number of UEs per transmission resource for baseline and MUSA, the distributions are similar since 6 PRBs per UE are used for both baseline and MUSA. The CDF of the number of UEs per transmission resource for MUSA at PAR = 200 packet/s/cell and 500 packet/s/cell are also shown in Figure 2(c). More users would share the same transmission resource with the increasing of traffic load.

Figure 2(d) shows the resource utilization for baseline and MUSA, similar distributions can be also observed.

Figure 2(e) and 2(f) shows the CDF of IoT and SINR for MUSA at PAR = 200 packet/s/cell and 500 packet/s/cell, where large scale channel is used, and only inter-cell interference is involved. With the increasing of traffic load, the impact of inter-cell interference would be more serious.

| (a) Packet drop rate | (b) Max number of UEs per Tx resource | (c) CDF of the number of UEs per Tx resource |
| --- | --- | --- |
| (d) Resource utilization | (e) CDF of IoT | (f) CDF of SINR |

Figure 2 Simulation results for mMTC configured grant Case 2

## mMTC (grant free with random selection)

In this section, we demonstrate some simulation results for grant-free with random selection in mMTC scenario, where transmission resource and MA signature (including DMRS and/or spreading code) are randomly selected by each UE, and MA signature collision exists.

As for the baseline scheme, 24 orthogonal ports for the enhanced DMRS configuration is assumed without spreading, while for DMRS based MUSA scheme, 64 orthogonal ports via configuration is assumed, and symbol spreading with spreading code of length 4 is used. The number of available spreading codes is 64 which are one-to-one mapping with the 64 DMRS. The 64 spreading codes are also used for data-only based MUSA.

In the simulation, UEs in a cell are classified to 3 groups, different resource pools are configured for these groups respectively, and 1 PRB would be randomly selected by a UE from the corresponding resource pool for transmission, i.e. mMTC resource allocation case 1 as described above. In addition, 1, 4 and 16 repetitions are configured for the 3 groups respectively for baseline, while for MUSA, since spreading codes of length 4 are used, 1, 1 and 4 repetitions are configured for the 3 groups respectively, and the energy of spreading codes used by UEs in the first group is normalized to 1.

It should be noted that blind MMSE-IC receivers are used for these three schemes, the respective PHY abstraction methods of the receivers can be found in [6].

Figure 3(a) shows the PDR performances for baseline, DMRS based MUSA, and data-only based MUSA. From the figure, we can observe that the supported PAR at PDR = 1% for DMRS based MUSA is obviously better than that for baseline, and data-only based MUSA has better performance at low traffic load.

The distributions of maximum number of UEs per transmission resource and resource utilization are also shown in Figure 3(b) and 3(c) respectively for baseline and MUSA. Similar distributions can be observed for the two MUSA schemes.

| (a) Packet drop rate | (b) Max number of UEs per Tx resource | (c) Resource utilization |
| --- | --- | --- |

Figure 3 Simulation results for mMTC grant-free with random selection

## URLLC (configured grant)

For URLLC, configured grant scenario is simulated, the transmission resources are preconfigured, and there is no DMRS collision. For fair comparison, the power consumption for baseline and MUSA are kept to be the same. And in the evaluation, MMSE-IRC receiver and MMSE-PIC receiver are both used for baseline and MUSA.

Figure 4 shows the percentages of users satisfying reliability and latency requirements for baseline and MUSA with 60 bytes packet size, from which we can observe that the supported PAR at target percentage = 95% for MUSA would be almost 3 times of that for baseline, even with MMSE-IRC receiver. With MMSE-PIC receiver, the packet error rate can be further reduced, and the percentage of users satisfying requirements is increased for baseline, however, for MUSA, due to the packet error rate is very low with MMSE-IRC receiver, the performance improvement on the percentage of users is very small.

Figure 5 shows the percentages of users satisfying reliability and latency requirements for baseline and MUSA with 200 bytes packet size. Similar observations as above can be derived, and the performances are decreased relative to that with 60bytes packet size.

| (a) Percentage of UEs satisfying requirements for 4GHz + 200m and packet size = 60bytes | (b) Percentage of UEs satisfying requirements for 700MHz + 500m and packet size = 60bytes |
| --- | --- |

Figure 4 Simulation results for URLLC (packet size = 60bytes)

| (a) Percentage of UEs satisfying requirements for 4GHz + 200m and packet size = 200bytes | (b) Percentage of UEs satisfying requirements for 700MHz + 500m and packet size = 200bytes |
| --- | --- |

Figure 5 Simulation results for URLLC (packet size = 200bytes)

## eMBB (configured grant)

For eMBB, the scenario of configured grant with no DMRS collision is also firstly simulated.

- Case 1: 3 PRBs per UE for baseline, 12 PRBs per UE for MUSA

In this case, resource allocation with FDM mode is used for baseline. For fair comparison, the power consumption for baseline and MUSA are kept to be the same.

Figure 6(a) shows the PDR performances for baseline with MMSE-IRC receiver or MMSE-PIC receiver, and MUSA with MMSE-PIC receiver. From the figure, we can observe that the supported PAR at PDR = 1% for MUSA is more than 2 times of that for baseline with MMSE-IRC receiver, and is about 1.5 times of that for baseline with MMSE-PIC receiver.

Figure 6(b) shows the maximum number of UEs per transmission resource for baseline and MUSA, more users would share the same transmission resource with the increasing of traffic load, and the maximum number of UEs per transmission resource for baseline and MUSA would be some specific values at high traffic load due to the configured grant mechanism. For MUSA, the maximum number of UEs per transmission resource would be 16 at PAR = 2000 packet/s/cell, where PDR = 1% can be reached. It means that DMRS enhancement is necessary to support more users transmitted on the same resources. The CDF of the number of UEs per transmission resource for MUSA at PAR = 2000 packet/s/cell is also shown in Figure 6(c).

Figure 6(d) shows the resource utilization for baseline and MUSA. Figure 6(e) and 6(f) shows the CDF of IoT and SINR for MUSA at PAR = 2000 packet/s/cell, where large scale channel is used, and only inter-cell interference is involved. These curves can be used for reference.

| (a) Packet drop rate | (b) Max number of UEs per Tx resource | (c) CDF of the number of UEs per Tx resource (PAR = 2000 packet/s/cell) |
| --- | --- | --- |
| (d) Resource utilization | (e) CDF of IoT (PAR = 2000 packet/s/cell) | (f) CDF of SINR (PAR = 2000 packet/s/cell) |

Figure 6 Simulation results for eMBB configured grant Case 1

- Case 2: 12 PRBs per UE for baseline and MUSA

In this case, lower code rate would be used for baseline relative to Case 1.

Figure 7(a) shows the PDR performances for baseline with MMSE-IRC receiver or MMSE-PIC receiver, and MUSA with MMSE-PIC receiver. From the figure, we can observe that the supported PAR at PDR = 1% for MUSA is about 3 times of that for baseline with MMSE-IRC receiver, and is about 2 times of that for baseline with MMSE-PIC receiver.

Figure 7(b) shows the maximum number of UEs per transmission resource for baseline and MUSA, the distributions are similar since 12 PRBs per UE are used for both baseline and MUSA. The CDF of the number of UEs per transmission resource for MUSA at PAR = 2000 packet/s/cell is also shown in Figure 7(c). More than 12 users would share the same transmission resource when the PAR is larger than 1500 packet/s/cell, where PDR = 1% can be reached with MUSA.

Figure 7(d) shows the resource utilization for baseline and MUSA, similar distributions can be also observed.

Figure 7(e) and 7(f) shows the CDF of IoT and SINR for MUSA at PAR = 2000 packet/s/cell, where large scale channel is used, and only inter-cell interference is involved. These curves can be used for reference.

| (a) Packet drop rate | (b) Max number of UEs per Tx resource | (c) CDF of the number of UEs per Tx resource (PAR = 2000 packet/s/cell) |
| --- | --- | --- |
| (d) Resource utilization | (e) CDF of IoT (PAR = 2000 packet/s/cell) | (f) CDF of SINR (PAR = 2000 packet/s/cell) |

Figure 7 Simulation results for eMBB configured grant Case 2

Observation 1: For mMTC configured grant scenario with FDM resource allocation, the supported packet arrival rate (PAR) at packet drop rate (PDR) = 1% for MUSA is about 2 times of that for baseline with MMSE-IRC receiver, and is more than 1.5 times of that for baseline with MMSE-PIC receiver.

Observation 2: For mMTC configured grant scenario with 6 PRBs per UE for baseline and MUSA, the supported PAR at PDR = 1% for MUSA is more than 2 times of that for baseline with MMSE-IRC receiver or MMSE-PIC receiver.

Observation 3: For mMTC grant free with random selection scenario, the supported PAR at PDR = 1% for DMRS based MUSA is obviously better than that for baseline MUSA has better performance at low traffic load.

Observation 4: For URLLC configured grant scenario, the supported PAR at target percentage of users satisfying requirements = 95% for MUSA would be almost 2~3 times of that for baseline, even with MMSE-IRC receiver. And the performances with 200 bytes packet size are decreased relative to that with 60bytes packet size.

Observation 5: For eMBB configured grant scenario with FDM resource allocation for baseline, the supported PAR at PDR = 1% for MUSA is more than 2 times of that for baseline with MMSE-IRC receiver, and is about 1.5 times of that for baseline with MMSE-PIC receiver.

Observation 6: For eMBB configured grant scenario with 12 PRBs per UE for baseline and MUSA, the supported PAR at PDR = 1% for MUSA is about 3 times of that for baseline with MMSE-IRC receiver, and is about 2 times of that for baseline with MMSE-PIC receiver.

Observation 7: The number of UEs per transmission resource and resource utilization are increasing with the traffic load, and multiple times of re-transmissions have impact on these metrics.

Observation 8: DMRS enhancement is necessary to support more users transmitted on the same resources.

# Conclusions

In this contribution, system level evaluation results for NOMA in mMTC, URLLC, and eMBB scenarios are further demonstrated and discussed.

Based on this contribution, we make the following observations:

Observation 1: For mMTC configured grant scenario with FDM resource allocation, the supported packet arrival rate (PAR) at packet drop rate (PDR) = 1% for MUSA is about 2 times of that for baseline with MMSE-IRC receiver, and is more than 1.5 times of that for baseline with MMSE-PIC receiver.

Observation 2: For mMTC configured grant scenario with 6 PRBs per UE for baseline and MUSA, the supported PAR at PDR = 1% for MUSA is more than 2 times of that for baseline with MMSE-IRC receiver or MMSE-PIC receiver.

Observation 3: For mMTC grant free with random selection scenario, the supported PAR at PDR = 1% for DMRS based MUSA is obviously better than that for baseline, and data-only based MUSA has better performance at low traffic load.

Observation 4: For URLLC configured grant scenario, the supported PAR at target percentage of users satisfying requirements = 95% for MUSA would be almost 2~3 times of that for baseline, even with MMSE-IRC receiver. And the performances with 200 bytes packet size are decreased relative to that with 60bytes packet size.

Observation 5: For eMBB configured grant scenario with FDM resource allocation for baseline, the supported PAR at PDR = 1% for MUSA is more than 2 times of that for baseline with MMSE-IRC receiver, and is about 1.5 times of that for baseline with MMSE-PIC receiver.

Observation 6: For eMBB configured grant scenario with 12 PRBs per UE for baseline and MUSA, the supported PAR at PDR = 1% for MUSA is about 3 times of that for baseline with MMSE-IRC receiver, and is about 2 times of that for baseline with MMSE-PIC receiver.

Observation 7: The number of UEs per transmission resource and resource utilization are increasing with the traffic load, and multiple times of re-transmissions have impact on these metrics.

Observation 8: DMRS enhancement is necessary to support more users transmitted on the same resources.

# References

- 3GPP, Draft Report of 3GPP TSG RAN WG1 #92bis v0.1.0

- 3GPP, Draft minutes report RAN1#93 v020

- 3GPP, Draft Report of 3GPP TSG RAN WG1 #94 v0.1.0

- 3GPP, R1-1810207, Detailed system-level simulation results for NOMA, ZTE, RAN1 #94bis

- 3GPP, R1-1808155, Link-level simulation results for mMTC scenario, ZTE, RAN1#94

- 3GPP, R1-1810208, Physical layer abstraction for NOMA SLS, ZTE, RAN1#94bis

# Appendix 1 Agreed system-level assumptions for NOMA

Table A1 System-level assumptions for NOMA evaluation

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |
| Inter-BS distance | 1732m | 200m for 4GHz 500m for 700MHz | 200m |
| Carrier frequency | 700MHz | 4GHz or 700MHz | 4GHz |
| Simulation bandwidth | 6 PRBs as starting point | 12 PRBs | 12 PRBs |
| Number of UEs per cell | Companies report |  |  |
| Channel model | UMa in TR 38.901; The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz. |  |  |
| UE Tx power | Max 23 dBm |  |  |
| BS antenna configurations | 2 Rx or 4 Rx for 700MHz; 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; dH = dV = 0.5λ; BS antenna downtilt: companies to report, FFS a single value 4 Rx or 16 Rx for 4GHz; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; 16 ports: (M, N, P, Mg, Ng) = (10, 8, 2, 1, 1), 16 TXRU; dH = 0.5λ, dV = 0.8λ; BS antenna downtilt: companies to report, FFS a single value |  |  |
| BS antenna height | 25m |  |  |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |  |  |
| BS receiver noise figure | 5dB |  |  |
| UE antenna configuration | 1Tx as starting point |  |  |
| UE antenna height | Follow the modelling of TR 38.901 |  |  |
| UE antenna gain | 0dBi as starting point |  |  |
| UE distribution | For mMTC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell For URLLC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell; Note: Other option(s) not precluded, e.g., 500m ISD, 80% of users are outdoors (3km/h), 20% of users are indoor (3km/h). For eMBB: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell |  |  |
| UE power control | Open loop PC for mMTC. Companies report the PC mechanisms used for eMBB and URLLC. |  |  |
| HARQ/repetition | Companies report (including HARQ mechanisms). |  |  |
| Channel estimation | Realistic |  |  |
| BS receiver | Advanced receiver, with baseline scheme is MU-MIMO (e.g., has the capability of spatial differentiation) Companies to provide analysis of complexity between baseline vs. advanced receivers |  |  |