3GPP TSG RAN WG1 Meeting #95																R1-1812449

Spokane, USA, November 12th – 16th, 2018

Source: 	Intel Corporation

Title:	Discussion on collision handling between PSS/SSS/PBCH and UE-specific RS

Agenda item:	5

Document for:	Discussion and Decision

# 1 Introduction

In RAN1#94bis the issue of collision handling between PSS/SSS/PBCH and UE-specific RS was raised for transmission modes based on UE-specific RS [1]. Two possible UE interpretation were raised for pre Rel-13 UEs based on the puncturing of the PRBs colliding with PSS/SSS/PBCH and PRBs rate matching. In this contribution we provide our views on this issue.

# Discussion

The issue of collision handling was originally brought for antenna port 5. Based on the conclusion, it was clarified that PRBs overlapping with PSS/SSS/PBCH is an error case, therefore, UE can discard the whole subframe [2]. A similar discussion happened at RAN1#82, #82bis and #83 meetings [3], where the same interpretation was confirmed that PRBs overlapping with PSS/SSS/PBCH is an error case [4]. It was also agreed to specify new UE behaviour for Rel-13, where UE rate matches only around the colliding PRBs in the RBG and processes the non-colliding PRBs in the RBG [5]. Considering the discussion above it is not worth introducing a new interpretation including RRC signalling, wherein the UE punctures the colliding PRBs in the RBGs as proposed in RAN1#94bis meeting.

Observations:

- Up to Rel-12 UE, PDSCH based on UE-specific RS antenna ports 5, 7-14 with PRBs overlapping with PSS/SSS/PBCH should be considered as an error case

- Rel-13 and onwards, depending on UE capability PDSCH based on antenna ports 5, 7-14 with PRBs overlapping with PSS/SSS/PBCH is allowed, where colliding PRB in RBG are rate matched and UE process only non-colliding PRBs

- No new UE behavior and higher layer signalling is required to support collision handling between PSS/SSS/PBCH and UE-specific RS antenna ports port 5, 7-14

# Conclusion

In this contribution we provide our views on collision handling between PSS/SSS/PBCH and UE-specific RS antenna ports 5, 7-14. Based on the discussion the following observations were made:

- Up to Rel-12 UE, PDSCH based on UE-specific RS antenna ports 5, 7-14 with PRBs overlapping with PSS/SSS/PBCH should be considered as an error case

- Rel-13 and onwards, depending on UE capability PDSCH based on antenna ports 5, 7-14 with PRBs overlapping with PSS/SSS/PBCH is allowed, where colliding PRB in RBG are rate matched and UE process only non-colliding PRBs

- No new UE behavior and higher layer signalling is required to support collision handling between PSS/SSS/PBCH and UE-specific RS antenna ports port 5, 7-14

# References

[1] RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting #94bis, Chengdu, China, October 5-10, 2018.

[2] R1-084660, 6.213 CR0167R1 (Rel-8, F), Transmission overlapping of physical channels/signals with PDSCH for transmission mode 7, Nokia, Nokia Siemens Networks, Qualcomm Europe, Texas Instruments, Marvell, NEC Group, Motorola.

[3] R1-155702, Discussion on PDSCH collision with PSS/SSS/PBCH, 3GPP TSG RAN WG1 #82bis, Qualcomm Incorporated, October 5th- 9th, 2015, Malmo, Sweden.

[4] RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting #82bis, Beijing, China, 24th - 28th August 2015.

[5] RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting #83, Anaheim, USA, 15th - 22th November 2015.