3GPP TSG RAN WG1 Meeting #95 									R1-1812913

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.2.4.1

Source:	Apple Inc.

Title:	Channel Access Procedures for NR-U

Document for:	Discussion/Decision

# Introduction

LTE based unlicensed operations, i.e. LAA/eLAA, were introduced starting from Rel-13. A Rel-15 study item [1] on NR-based unlicensed access has been approved to study the solutions applicable to potential spectrum candidates both below and above 6Ghz, as well as various deployment scenarios, including CA or DC based LAA and standalone based unlicensed operations.

In RAN1 #92bis, it was agreed that [2]:

Agreement:

- Baseline for study: If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer multiple of 20MHz

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

- Study whether or not the following techniques enhance performance beyond the baseline LBT mechanisms

  - Techniques to cope with directional antennas/transmissions

  - Receiver assisted LBT : RTS/CTS type mechanism

    - On-demand receiver assisted LBT: For example receiver assisted LBT enabled only when needed

  - Techniques to enhance spatial reuse

  - Preamble detection

  - Enhancements to baseline LBT mechanisms above 7 GHz

- Note: LTE-LAA LBT mechanism are assumed as baseline for evaluations for 5GHz.

- Note: Other aspects are not precluded from being included

In RAN1 #93, it was further agreed that

- Agreement:

- Benefits of using a signal that facilitates its detection with low complexity can be investigated including all/part of the following scenarios/use cases:

  - UE power saving

  - Improved coexistence

  - Spatial reuse at least within the same operator network

  - Serving cell transmission burst acquisition

  - FFS: further usage scenarios

In this contribution, we briefly discuss the possible mechanism of wideband channel access for NR-U and some potential enhancements for NRU coexistence with other RATs, as well as UE power saving. In a companion contribution [6][7], we also discuss how new NR channel designs can benefit the NR-based unlicensed access, as well as potential further enhancements to PRACH.

# Wideband operation for NR-U

As shown in section 1, it has been agreed that for bands where absence of Wi-Fi can’t be guaranteed, NR-U operation should support bandwidth in multiple of 20Mhz, and that the LBT can be performed in unit of 20Mhz. This helps fair coexistence with existing Wi-Fi, where channel occupancy can be in form or 20Mhz, 40Mhz, 80Mhz and 160Mhz as in 802.11ac. In addition, LBT in unit of 20Mhz also allows for higher channel usage efficiency and flexibility, as gNB may potentially have faster access to the available subchannels depending on the implementation.

## Bandwidth adaptation with sub-channel LBT

Implementation and challenges of wideband operation in NR-U has been extensively discussed in the past meetings (e.g., in [8][9]). Due to the variations in channel availability in unlicensed spectrum, both gNB and UE will need to dynamically adapt the operation bandwidth based on LBT results. This dynamic bandwidth adaptation can potentially be achieved using two alternative mechanisms under the existing NR framework:

- BWP approach: using BWP switching mechanism to switch between different BWP’s with different BW

- CA approach: using component carrier activation/deactivation mechanism

Configuring different 20Mhz sub-channels as independent component carriers to the UE gives the network more flexibility to dynamically expand to wider BW based on channel availability regardless whether the available subchannels are contiguous or not, as UE may use separate RF chains for different carriers. The CA approach (as adopted in LAA) will be supported in NR by default. However, this approach has implications on HW architecture and availability. In addition, existing CC activation procedure relies on MAC-CE, which is not be fast enough. It is desirable to support faster CC activation mechanism, e.g., using DCI.

The BWP approach simplifies the UE HW architecture, as wider bandwidth (in integer multiples of 20Mhz) is achieved with a single Rx chain. Four candidate options for BWP based implementation were discussed in RAN1 #94bis:

Agreement:

- NR-U should support that a serving cell can be configured with bandwidth larger than 20 MHz.

  - For DL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

    - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on one or more BWPs

    - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on single BWP

    - Option 2: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on a single BWP if CCA is successful at gNB for the whole BWP

    - Option 3: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB

  - Note: CCA is declared to be successful or not in multiples of 20 MHz.

  - FFS for UL operation including some or all of above options can be applied

  - Note: Capture the following in TR only after further discussion for down-selecting from the options in RAN1#95

For option 3, gNB can transmit data to UE even only part of the BWP is available. For example, UE is monitoring 40Mhz BWP, while network may still transmit to the UE on 20Mhz subchannel when the other 20Mhz subchannel is not available. Although this allows more efficient usage of the channel, UE might suffer from strong in-band interferer inside a wideband BWP, as shown in Figure 2-1 (a), due to the following reasons:

- The signal from the other occupied sub-channel can potentially be close to the receiver, injecting strong signal that can de-sense the receiver.

- The signal from the other occupied sub-channel is likely not synchronous or orthogonal to the serving gNB, leading to potential ICI, unless large enough guard tones reserved within the BWP

Figure 2-1 gNB performing sub-channel based LBT

In addition to the de-sense issue, UE reception at a wider bandwidth BWP while only part of the active BWP can be used by the gNB is not desirable from UE power saving perspective.

For option 2, this potential issue of in-band interferer is largely avoided, as gNB only transmit to the UE if the whole BWP monitored by the UE is fully available.

For option 1a/1b, the definition and the corresponding UE behavior and implementation are not clear. For example, does UE use separate Rx chains for the multiple active BWPs? If so, that is essentially CA. If multiple active BWPs are supported on a single Rx chain, then a similar issue of receiver de-sense from in-band interferer would occur as with option 3.

Proposal 1: NR-U supports BWP based wide bandwidth operation using option 2.

## Primary sub-channel monitoring from UE power saving perspective

From gNB perspective, monitoring wideband channel availability using (20Mhz) sub-band based LBT allows for more efficient channel utilization. However, from UE perspective, keep monitoring the scheduling information across wideband (i.e., multiple sub-channels) can be power consuming. For example, monitoring a BWP of 40MHz can cost 40% more power than monitoring 20Mhz subchannel, and monitoring a BWP of 80MHz leads to more than twice the power consumption compared to a monitoring 20Mhz subchannel. Therefore, it is desirable that UE only monitors a “primary” sub-channel (e.g., 20MHz), as configured by the gNB, and can be changed to wider bandwidth dynamically based on gNB LBT results.

Specifically, this includes the following steps:

- gNB configures a “primary” (or default) sub-channel (e.g., 20Mhz) for each UE individually.

- UE will only monitor on the primary sub-channel for scheduling by the serving gNB.

- gNB always starts scheduling UE on the primary sub-channel, but can dynamically switch/activate wider bandwidth for the UE during the CoT, based on LBT results.

However, to avoid frequent BW retuning overhead at UE side, during extensive data transaction (e.g. when downloading a large size file), gNB can tentatively configure the primary channel of the UE to be a larger BW (e.g. 40Mhz, or 80Mhz). This way, the UE doesn’t need to retune its RF BW for every CoT during the period of extensive data transaction.

The network can further support low complexity “initial signal” to reduce UE power consumption, as discussed in more details in [6].

### BWP based dynamic adaptation to wider BW

Figure 2-2 illustrates how the network can incrementally increase the UE BW by changing to wider BWP when more than one sub-channel can be accessed. To further balance the load and have more efficient utilization of different sub-channels, gNB can configure different primary sub-channel to different UEs.

Figure 2-2 UE sub-band monitoring and wider bandwidth BWP switching

For dynamic bandwidth adaptation using BWP mechanism, network should use contiguous sub-channels. Further, for fair coexistence with other NR-U network or RATs (e.g. 802.11ac), it is also desirable to adopt certain multi-channel bonding rules, as shown in Figure 2-3

Figure 2-3 channel bonding rule for BWP of different bandwidth

The channel bonding rule makes two BWPs of the same size either non-overlapping or fully overlapping, and can avoid the situation where a single BWP blocking 2 other BWPs with same size (e.g. partial overlapping with both 2 BWPs).

### Component carrier based dynamic adaptation to wider BW

Carrier aggregation-based BW adaptation is generally more flexible, as each sub-channel can be activated or deactivated independently. For the case of non-contiguous sub-channels, as shown in Figure 2-1 (a), the UE may use different RF chains to achieve wider BW and avoid potential in band jamming/interference discussed in section 2.1.

A common challenge for both the BWP based and CA based BW adaptation is the required RF retuning time, either to retuning the center frequency of BWP or to activate a new carrier, which is generally on the order of several hundred micro seconds. During such a transition time, the channel availability could potentially change and a new LBT may be needed by the gNB. One possible solution is to let gNB send reservation signal before the UE is ready to receive on the new sub-channels.

Observation 1: dynamic BW adaptation in NR-U can be supported by both BWP switching and component carrier activation/deactivation together with sub-channel based LBT at gNB side.

Proposal 2: NR-U supports narrow band monitoring from UE side, where UE only monitors scheduling information on single “primary” sub-channel configured by the network.

Proposal 3: For dynamic BW adaptation in NR-U using BWP switching, consider introducing sub-channel bonding rule.

# UE Assisted Channel Selection

One mechanism that enables a fair spectrum sharing is dynamic channel selection, where the transmitter (e.g. LAA base station) can scan the spectrum for available channels both at initial power up as well as periodically after initial power up. This dynamic channel selection can be helpful in regions where LBT are not mandated. The measurement is typically done at the transmitter side, and typically uses energy-based measurement making it RAT agnostic.

As discussed in earlier sections, in certain scenarios, the interference or channel utilization observed at the transmitter does not always reflect the interference seen at the receiver side, which is often more relevant. Therefore, it could be helpful that UEs can periodically (or occasionally) report their own measurement to the gNB scheduler to assist the channel selection procedure.

In addition, for the UE-assisted channel selection, the UE measurement could be energy-based if scheduler want to be RAT agnostic, or potentially RAT specific if LAA cells want to only avoid interference from a particular RAT (e.g. UE can detect based on Wi-Fi pre-amble if want to avoid interference from Wi-Fi devices).

Proposal 4: NR-U to support UE-assisted channel selection.

# Receiver assisted LBT

LBT is an effective coexistence mechanism that, together with appropriate back-off configuration, allows different devices and RATs to share the usage of the spectrum in TDM fashion.

However, there is also certain limitations associated with LBT, specifically in the following scenarios:

- Hidden nodes: where transmitter does not detect the same level of channel occupancy as the receiver

- Highly directional communication: where substantial spectrum reuse can be achieved when Tx and Rx beamforming are used.

The “hidden nodes” issue is well known for CSMA type of multi-access for unlicensed spectrum, even for sub-6GHz spectrum where transmission/reception are less directional. It is observed that existing commercial Wi-Fi networks can suffer from hidden-node issue in some deployment and use cases, where the transmitter could not precisely detect the interference level at the receiver side.

NR-U could potentially support larger cell size than Wi-Fi due to lower operating point, which could lead to even more serious hidden-node issue. Specifically, for residential deployment where gNB’s are positioned without careful cell planning, an NR-U device could be placed much closer to an interfering gNB than the serving gNB. An optional receiver-assisted LBT technique would be very beneficial in this scenario.

Proposal 5: NR-U to support optional receiver-assisted LBT.

# Conclusions

In this contribution, we briefly discuss how NR-U could achieve dynamic bandwidth adaptation with reduced UE complexity, as well as a few potential enhancements to NR based unlicensed channel access. We have the following observation and proposals:

Observation 1: dynamic BW adaptation in NR-U can be supported by both BWP switching and component carrier activation/deactivation together with sub-channel based LBT at gNB side.

Proposal 1: NR-U supports BWP based wide bandwidth operation using option 2.

Proposal 2: NR-U supports narrow band monitoring from UE side, where UE only monitors scheduling information on single “primary” sub-channel configured by the network.

Proposal 3: For dynamic BW adaptation in NR-U using BWP switching, consider introducing sub-channel bonding rule.

Proposal 4: NR-U to support UE-assisted channel selection.

Proposal 5: NR-U to support optional receiver-assisted LBT.

# References

- RP-172021, “Revised SID on NR-based Access to Unlicensed Spectrum”, 3GPP RAN #77, Sapporo, Japan, 11th – 14th September 2017.

- “Chairman’s notes RAN1 92bis final”, 3GPP RAN1 #92bis, Sanya, China, 16th – 20th, April, 2018.

- “Chairman’s notes RAN1 93 final”, 3GPP RAN1 #93, Busan, Korea, 21st – 25th, May, 2018.

- ETSI EN 301 893, “5GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

- ETSI EN 302 567, “Multiple-Gigabit radio equipment operating in the 60GHz band”.

- R1-1811120, “On DL signals and channels for NR-U”, Apple Inc., 3GPP RAN1 #94b, Chengdu, China, 8th – 12th, October 2018.

- R1-1811121, “On UL signals and channels for NR-U”, Apple Inc., 3GPP RAN1 #94b, Chengdu, China, 8th – 12th, October 2018

- R1-1808825, “On wideband operation for NR-U”, Nokia, NSB, 3GPP RAN1 #94, Gothenburg, Sweden, 20th – 24th, August 2018

- R1-1808768, “Channel access procedure for NR-U”, Samsung, 3GPP RAN1 #94, Gothenburg, Sweden, 20th – 24th, August 2018