3GPP TSG-RAN WG1 Meeting #95	R1- 1812924

# Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.9.1

Source:	Apple Inc.

Title:	Evaluation Methodology for UE Power Saving

Document for:	Discussion, Decision

# Introduction

In RAN1#94b meeting, UE power consumption model for UE Power Saving SI[1] was agreed capturing following aspect of model[2].

- Power states and transition

- Power/energy consumption numbers with a reference configuration

- Power scaling factor for scaling model in bandwidth, # of carriers, # of antennas, tx power

But, there are still many undecided factors which may be required for proposal evaluation. But, those undecided factors could be different for different proposals since not all the models could use the same model. For example, one proposal may require a new state modeling self-contained slot, but other proposals may not. However, the power model agreed in the RAN1#94b contains important reference numbers which could be basis number for further extension. In this contribution, we introduce additional power states which are useful for proposal evaluation.

Additionally, for simple evaluation of proposals, we introduce statistical grant arrival model.

# Power Model

## Breakdown of Power Numbers

In this section, we determine a few power numbers which we can be derived from the already agreed power numbers. The newly determined power numbers could be used to predict the power consumption numbers of new states which will be discussed in the next section.

Here we describe our understanding on power states agreed in [2].

| State | Description | Average Power Consumption in a slot (FR1) |
| --- | --- | --- |
| PDCCH-only (same-slot scheduling) | PDCCH decoding and PDSCH buffering are performed. It is assumed that UE enters Micro Sleep state in the later part of the slot. | 100 |
| PDCCH-only (cross-slot scheduling) | Only PDCCH decoding is performed. Since there is no PDSCH buffering, UE can enter Micro Sleep earlier than the case of PDCCH-only (same-slot scheduling) | 70 |
| PDCCH + PDSCH | DL grant is processed, i.e., both PDCCH and PDSCH are decoded. Note that ACK/NACK is not included here. | 300 |
| UL | Long PUCCH for 14 OFDM symbols | 250 (0dBm) 700 (23dBm) |
| Micro Sleep | It is assumed that UE can move to Micro sleep state with zero transition time and zero transition energy. | 45 |

Based on above agreed numbers, we want to determine following numbers.

- X : DL RF power consumption contribution per symbol

- Y : PDCCH baseband processing power contribution

- Z : PDSCH baseband processing power contribution per symbol

Note that X and Z are defined as power contributor per symbol. Once we characterize X and Z, then we can use those numbers to scale to compute power consumption for variable RF on/off duration and variable PDSCH lengths.

From PDCCH-only(same-slot scheduling) state, we get

5X + Y + 45*9/14 = 100,							(1)

where 45 is micro-sleep power and we assume that PDCCH decoding finishes 5th OFDM symbol. It is assumed that RF is turned off after 5th OFDM symbol. From PDCCH-only (cross-slot scheduling) state, we get

2X + Y + 45*12/14 = 70. 							(2)

From PDCCH + PDSCH state, we get

14X + Y + 12Z = 300. 							(3)

Solving the linear equations (1)~(3), we obtain the power breakdown number X = 13.2, Y=5, and Z=110/12.

## New Power States

In this section, we introduces following new states and associated power numbers. The notation of state name is given a sequence of XXX(N), where XXX(N) denotes that the consecutive N symbols are used for XXX. For example, PDCCH(2) means that two symbols are used for PDCCH. Using X, Y, Z values from above, we compute power consumption of following new states.

Table 1 New States and Power Consumption Calculation

| State | Power Calculation |
| --- | --- |
| MS(2)PDSCH(12) | 14X + MS*2/14 + 12Z = 274.8 |
| PDCCH(2)PDSCH(9)MS(1)PUCCH0dBm(2) | 12X + Y + 9Z + 45/14 + 250*2/14 = 284.8 |
| PDCCH(2)PDSCH(9)MS(1)PUCCH23dBm(2) | 12X + Y + 9Z + 45/14 + 700*2/14 = 349.1 |
| MS(12)PUCCH0dBm(2) | 45*12/14 + 250*2/14 = 74.3 |
| MS(2)PDSCH(9)MS(1)PUCCH0dBm(2) | 9X + 45*2/14 + Z*9 + 45/14 + 250*2/14 = 246.7 |
| MS(2)PDSCH(9)MS(1)PUCCH23dBm(2) | 9X + 45*2/14 + Z*9 + 45/14 + 700*2/14 = 310.9 |
| PDCCH(2)PDSCH(9)MS(3) | 11X + Y + 9Z + 45*3/14 = 242.3 |
| MS(2)PDSCH(9)MS(3) | 9X + 9Z + 45*5/14 = 217.3 |
| PDCCH(2)MS(10)PDSCH(2) | 4X + Y + 45*10/14 + 2Z = 108.3 |
| PDCCH(2)PDSCH(2)MS(12) | 2X + Y + 2Z + 45*12/14 = 88.3 |

In our companion paper[5], we use these new states and power consumption numbers to evaluate our proposals.

Proposal 1

- For UE power saving SI, consider the power number breakdown method for deriving power numbers for new states.

- New states and power numbers considered in Table1 is used for proposal evaluation in UE PS SI.

# Traffic Model

In addition to other traffic models, we propose to consider Grant Arrival Model (GAM) for simple evaluation of proposals. GAM is a simple mathematical model for modeling dynamic grant arrivals. In this model, we consider a slotted system, where, in each slot, grant arrives with a probability p. Then, the grant inter-arrival time becomes a random variable with geometric distribution. This model may be useful to model UE operation with self-contained slot operation, where PDCCH, PDSCH, and ACK/NACK are transmitted in the same slot. During inter-grant arrival time, UE can either continuously monitor PDCCH or sleep. Sleep power during inter-grant arrival time can be also modeled as a random variable as a function of inter-grant time D. The traffic load can be controlled by changing probability p.

The agreed power consumption number can be used to compute power consumption during grant processing. The scaling factor could be also easily applied. Our companion paper [4] uses this model to show potential gain of sleep indication mechanism.

Proposal 2

- For UE power saving SI evaluation, consider grant arrival model.

# Conclusions

In this contribution, we have proposed new states and a new method for computing power consumption of these new states. The method breakdown the total power into RF, PDCCH, and PDSCH and allow one to compute power number for new state. The method is systematic and consistent in the sense that it generate reasonable trend in power numbers w.r.t number of symbols for PDSCH, number of symbols for Micro Sleep, number of symbols with RF ON.

Proposal 1

- For UE power saving SI, consider the power number breakdown method for deriving power numbers for new states.

- New states and power numbers considered in Table1 is used for proposal evaluation in UE PS SI.

Proposal 2

- For UE power saving SI evaluation, consider grant arrival model.

# References

[1] RP-181432 Study on UE Power Saving in NR, CATT

[2] R1-xxxxxx Chairman’s Note RAN1 94b, Cheng-du, Oct, 2018

[3] R1-1808615 Power Model for Evaluating UE Power Saving Technique, Apple, Oct, 2018

[4] R1-1812926 Power Saving Techniques based on Explicit Indication, Apple, Nov, 2018

[5] R1-1812925 Power Saving Techniques based on UE Adaptation, Apple, Nov, 2018