3GPP TSG RAN WG1 Meeting #95	 		R1-1813005

# Spokane, USA, 12th – 16th November 2018

Agenda item:	7.2.8.4

Source: 	Samsung

Title: 	View on full power UL transmission

Document for:	Discussion and Decision

# Introduction

In RAN1#94bis, the following agreement was made.

Agreement Consider the following potential solutions and other solutions (such as combination of the solutions below) for UL full power transmission. Decision will be made in RAN1#95: Option 1: Refinement/adjustment of UL codebook is supported - 1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs - 1-2: Introduce additional scaling factor for uplink codebook Option 2: UE transparently apply a small cyclic or linear delay Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s) - Note: Full rated PA refers to a PA having power not lower than that of the power class Option 4: Up to UE implementation (no specification impact)

This contribution discusses the four options in the agreement, and provides a solution for UL full power transmission.

# Discussion on four options

The two options, Option 2 and Option 4, on their own are out of scope in our view since they correspond to spec-transparent (e.g. implementation based) solutions, which is in contradiction with the following objective included in Rel. 16 NR MIMO WID [2], according to which the speciation should support UL full power transmission non-transparently.

- Extend specification support in the following areas [RAN1] - … - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class) - …

Regarding Option 1-1, a new codebookSubset is introduced to allow PC and NC capable UEs to be indicated with a TPMI that corresponds to a precoding matrix with number of non-zero (NZ) entries (ports) no less than number of antenna ports. With this, the full (total) power transmission can be achieved for all rank values, but it assumes certain UE implementations such as that the UE applies a specific transmission (e.g. Option 2). Such assumptions, however, not only increase UE complexity but in order to ensure good performance, it may also require additional spec support, for example, to introduce some kind of delay in SRS transmissions from antenna ports in order for the gNB to measure channel with some delay that matches with the delay considered in the specific transmission scheme (Option 2). Without any additional spec support, the performance of such solutions is questionable.

Regarding Option 1-2, the idea is to scale (increase) power of NZ entries (ports) of the precoding matrix to a value so that their sum corresponds to full (total) power. Note that the scaling value needs to be selected such that the power after UL power control corresponds to full (total) power. This option, however, requires the UE to be able to scale power of NZ antenna ports depending on the TPMI.

Between Option 1-1 and Option 1-2, our preference is Option 1-2 since it is simpler and does not rely of assumptions on a specific transmission scheme at the UE, and also incurs less UE implementation complexity.

Finally, Option 3 can achieve the same goal as Option 1-2 (either alone or together with Option 1-2) by bringing the precoder scaling in the UL power control mechanism.

Proposal 1: Support a solution for UL full power transmission based on either Option 1-2 or Option 3 or their combination.

- If Option 1-2 is supported, the scaling factor should be such that full power transmission is achieved after uplink power control.

# Power issue in Rel. 15 UL MIMO

For codebook based UL transmission in Rel. 15, a pre-coding matrix  is indicated to a UE (via TPMI) from an UL codebook that comprises pre-coding matrices of one of the three coherent types, nonCoherent (NC), partialAndNonCoherent (PC+NC), fullAndPartialAndNonCoherent (FC+PC+NC), where the UL codebook is configured to the UE depending on the capability reported by the UE. Based on the UE capability, the antenna ports at the UE are grouped into coherent port groups. As an example, for 4 antenna ports, the coherent port groups are illustrated in Table 1 for the three coherent types.

Table 1: Coherent port groups for 4 antenna ports

The UL codebook in NR comprises pre-coding matrices of these three coherent types. The total power of the pre-coding matrices for different ranks and coherence types is summarized in Table 2.

Table 2: Total power of precoding matrix for 4 antenna ports

| Rank | NC TPMIs |  | PC TPMIs |  | FC TPMIs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TPMI indices | Total power | TPMI indices | Total power | TPMI indices | Total power |
| 1 | 0-3 | ¼ | 4-11 | ½ | 12-27 | 1 |
| 2 | 0-5 | ½ | 6-13 | 1 | 14-21 | 1 |
| 3 | 0 | ¾ | 1-2 | 1 | 3-6 | 1 |
| 4 | 0 | 1 | 1-2 | 1 | 3-4 | 1 |

We can observe the following two issues.

- Issue 1: for PC and NC TPMIs, total power increases as rank increases, which implies that the TPMI indication/selection for PUSCH transmission is biased to higher rank. The main reason of this issue is the fact that the total number of non-zero (NZ) entries in the precoding matrix is less than the number of antenna ports for rank 1 PC and rank 1-3 NC TPMIs.

- Issue 2: for a given rank, total power of NC TPMIs ≤ total power of PC TPMIs ≤ total power of FC TPMIs. The reason for this trend is that the power of NZ antenna ports does not changes across three types of TPMIs.

Issue 1 will affect UL performance for NC and PC UEs significantly. Issue 2 on the other hand can be argued to be necessary/beneficial for UE power saving by turning off antenna(s), similar to LTE UL codebook.

For non-codebook based UL transmission in Rel. 15, Issue 1 is also present.

The solution for UL full power transmission in Rel. 16 should at least address Issue 1, and allow full power transmission regardless of the rank value.

# Proposed Solution

A (Option 3 based) solution to address Issue 1 is proposed in [3], which is copied below.

>>>>>>>>>>>> Start text proposal in [3] >>>>>>>>>>>>

For PUSCH, a UE first scales a linear value  of the transmit power  on UL BWP , as described in Subclause 12, of carrier  of serving cell , with parameters as defined in Subclause 7.1.1, by β and the resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted, where β=1 for single antenna port transmission, and for multi-antenna port transmission:

- β=min{1,α} with α=Kρ0ρ where

  - ρ is the number of antenna ports  according to 38.211 6.3.1.5

  - ρ0 be the number of non zero antenna ports in according to 38.211 6.3.1.5.

  - For non-codebook based transmission K=1. For codebook based transmission K is given from the table below, where ULCodebookSubset is a higher layer parameter.

| ULCodebookSubset | Number of UE antenna ports | K |
| --- | --- | --- |
| fullAndPartialAndNonCoherent | 2 | 1 |
| fullAndPartialAndNonCoherent | 4 | 1 |
| partialCoherent | 4 | 2 |
| nonCoherent | 2 | 2 |
| nonCoherent | 4 | 4 |

>>>>>>>>>>>> End text proposal in [3] >>>>>>>>>>>>

However, there is one issue with this solution. For 4 antenna ports and PC+NC coherence type, as highlighted (red) in Table 3, the power per NZ port changes across NC and PC TPMIs (for a fixed rank of 2 or 3). In our view, it is better to keep power per NZ ports fixed for a given rank. This is to ensure that the UE doesn’t need to change its power setting if only TPMI changes (and TRI does not change), and may have to change power only when TRI value changes, and TRI value change is expected to be slower than TPMI change.

Table 3: power per NZ port according to proposal in [3]

| Coherence type | Rank | NC TPMIs |  |  | PC TPMIs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TPMI | β | Power/NZ ports | TPMI | β | Power/NZ ports |
| PC+NC | 2 | 0-5 | 1 | 1/2 | 6-13 | 1 | ¼ |
|  | 3 | 0 | 1 | 1/3 | 1-2 | 1 | ¼ |

In particular, the solution to address Issue 1 should meet the following constraints.

- Constraint 1: at least for the “most coherent” TPMIs, total power should not change across rank (to solve Issue 1)

  - for FC+PC+NC, the most coherent TPMIs = FC TPMIs;

  - for PC+NC, the most coherent TPMIs = PC TPMIs; and

  - for NC, the most coherent TPMIs = NC TPMIs.

- Constraint 2: For a given rank, power per NZ antenna port should not change (to save UE power) across all TPMIs.

For codebook-based UL transmission, a solution which satisfies the two constraints is the scaling factor β=β1β2, where

- β1 scales the transmit power equally across the coherent port groups (K in proposal [3]) on which a non-zero PUSCH is transmitted, i.e., β1=1K0, and

- for each coherent port group with a non-zero PUSCH, β2 further scales the transmit power equally across the ports within the coherent port group on which a non-zero PUSCH is transmitted, i.e., β2=k=0K-1ρ0,kρk=Kk=0K-1ρ0,kρ=Kρ0ρ=α in proposal [3]

  - K = #coherent port groups (for the “most coherent” TPMIs) as in Table 4

  - K0 = #coherent port groups (for the “most coherent” TPMIs) with a non-zero PUSCH

  - ρk = #configured ports for the transmission in the k-th coherent port group,

  - ρ0,k = #ports with a non-zero PUSCH transmission in the k-th coherent port group,

  - ρ = #configured ports for the transmission, and

  - ρ0 = #ports with a non-zero PUSCH transmission.

Table 4: Number of coherent antenna port groups (for the “most coherent” TPMIs) (K)

| Coherence type | Number of antenna ports = 2 | Number of antenna ports = 4 |
| --- | --- | --- |
| nonCoherent | 2 | 4 |
| partialAndNonCoherent |  | 2 |
| fullAndPartialAndNonCoherent | 1 | 1 |

Effectively, this solution replaces the min operation in proposal [3] with another scaling β1 to distribute power over NZ coherent port groups. We can verify that the power per NZ port is fixed for a given rank. For example, for 4 antenna ports and PC+NC coherence type, as highlighted in Table 5, the power per NZ port is fixed across NC and PC TPMIs (for a fixed rank of 2 or 3).

Table 5: power per NZ port according to proposal in this contribution

| Coherence type | Rank | β1 | NC TPMIs |  |  |  | PC TPMIs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TPMI | β2 | β | Power/NZ ports | TPMI | β2 | β | Power/NZ ports |
| PC+NC | 2 | ½ | 0-5 | 1 | ½ | ¼ | 6-13 | 2 | 1 | ¼ |
|  | 3 | ½ | 0 | 3/2 | ¾ | ¼ | 1-2 | 2 | 1 | ¼ |

An example of the proposed solution for 4 antenna ports is shown in Table 6. As shown, both constraints (Constraint 1 and 2) are satisfied with the proposed solution.

For non-codebook based UL transmission, β=1.

In an alternate solution (Option 1-2 based), the proposed β scaling can also be applied to the UL codebooks. In particular, the additional scaling β1 is applied to the UL codebook, and the scaling β2 is applied to the UL power control mechanism.

Table 6: An example of proposed solution for 4 antenna ports (ρ=4)

Proposal 2: Support one of the following solutions

- (Option 3 based solution) For UL power control, PUSCH is scaled by a factor β and the resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted, where

  - for non-codebook based UL transmission, β=1

  - for codebook based UL transmission, β=β1β2

    - β1=1K0, and

    - β2=Kρ0ρ, where

      - K = #coherent port groups (for the “most coherent” TPMIs) as in Table 4,

      - K0 = #coherent port groups (for the “most coherent” TPMIs) with a non-zero PUSCH,

      - ρ = #configured ports for the transmission, and

      - ρ0 = #ports with a non-zero PUSCH transmission.

- (Option 1-2 + Option 3 based solution) the scaling β1 is applied to the UL codebook, and the scaling β2 is applied to the UL power control mechanism.

# Conclusion

In this contribution, the following proposals are made regarding full power UL transmission.

- Support a solution for UL full power transmission based on either Option 1-2 or Option 3 or their combination.

  - If Option 1-2 is supported, the scaling factor should be such that full power transmission is achieved after uplink power control.

- Support one of the following solutions

  - (Option 3 based solution) For UL power control, PUSCH is scaled by a factor β and the resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted, where

    - for non-codebook based UL transmission, β=1

    - for codebook based UL transmission, β=β1β2

      - β1=1K0, and

      - β2=Kρ0ρ, where

        - K = #coherent port groups (for the “most coherent” TPMIs) as in Table 4,

        - K0 = #coherent port groups (for the “most coherent” TPMIs) with a non-zero PUSCH,

        - ρ = #configured ports for the transmission, and

        - ρ0 = #ports with a non-zero PUSCH transmission.

  - (Option 1-2 + Option 3 based solution) the scaling β1 is applied to the UL codebook, and the scaling β2 is applied to the UL power control mechanism.

# Reference

- Chairman’s notes, RAN1#94bis

- RP-182067, “Revised WID: Enhancements on MIMO for NR,” Samsung

- R1-1811546, Enabling full TX power UL transmission for PUSCH, Ericsson