3GPP TSG RAN WG1 Meeting #95	R1-1813275

Spokane, U.S., November 12th - 16th, 2018

Source:	Ericsson

Title:	On virtual cell ID for SRS

Agenda Item:	6.2.3.1.2

Document for:	Discussion and Decision

# Discussion

In RAN1#94 and RAN1#94bis, the following agreements were made with regards to virtual cell ID for SRS in LTE Rel-16 [1]-[2]:

Agreement

For SRS, nIDRS=nIDSRS if nIDSRS nIDSRS is configured by higher layer, otherwise nIDRS=NIDcell nIDRS=NIDcell, where nIDRS is within the range from 0 to 503.

Agreement

A single virtual cell ID is configured per UE per CC

With the introduction of the virtual cell ID for SRS, this virtual cell ID can now be used to select the sequence group number u and the number v within the group for ZC sequence root associated with SRS.  In [3], a proposal was made to introduce time-varying SRS sequence for more than one SRS symbol in an UL normal subframe.  The reason given in [3] for this proposal is to improve interference randomization.

In our view, the performance benefits of introducing time-varying SRS sequences for more than one SRS symbol in an UL normal subframe still need to be quantified via system level evaluations.  Unless performance benefits are demonstrated via evaluations, our preference is not to introduce time-varying SRS sequences for multi-symbol SRS in LTE Rel-16.

Time-varying SRS sequences for multi-symbol SRS in normal UL subframes is not introduced in LTE Rel-16, unless the performance benefits of this feature are quantified via evaluations.

# References

[1] RAN1 Chairman’s Notes, RAN1#94

[2] RAN1 Chairman’s Notes, RAN1#94bis

[3] R1-1810930, ‘Virtual cell ID for SRS’, Qualcomm Incorporated