3GPP TSG RAN WG1 Meeting #95	R1-1813610

Spokane, USA, 12-16 November 2018

Source:	Ericsson

Title:	A comparison between NC-JT with coordinated vs. independent scheduling in indoor scenario

Agenda Item:	7.2.8.6

Document for:	Discussion and Decision

# Introduction

NC-JT was studied and specified in LTE Rel-14 [1] and Rel-15, respectively.  Some NC-JT gains were observed with 2Tx antenna ports at each TRP and 4Rx at the UE. Most of the gains occur at lower loads where active UEs are sparse and with NC-JT, full rank 4 operation is possible from the UE’s perspective.  NC-JT has been proposed as a MIMO enhancement in Rel-16 NR [2].  In this contribution, we present some preliminary simulation results to study the impact of different scheduling approaches on the performance on NC-JT in indoor scenario.

# Coordinated vs independent scheduling

In this section, we compare two different scheduling approaches shown in Figure 1 with the cluster size fixed at 2 as described below:

Coordinated scheduling:  In the coordinated scheduling approach shown in Figure 1a, 2-TRP clusters are pre-formed, and the clustering remains static independent of which UEs are served by the cluster.  The TRPs within a cluster are assumed to be scheduled by a coordinated scheduler.  As shown in Figure 1a, UEs 1 and 3 are associated with cluster 1 constituting TRPs 1-2 and UE 2 is associated with cluster 2 constituting TRPs 3-4.  Within the cluster, a UE may be served with either NC-JT or DPS depending on the coordinated scheduler’s decision.  Since the clusters are pre-formed and static, it is assumed that a single scheduler can control the scheduling decisions in the cluster without considering backhaul delay.

Independent scheduling:  In the independent scheduling approach shown in Figure 1b, 2-TRP clusters are formed on a UE specific basis, and each TRP scheduled by an independent scheduler.  In the example shown in Figure 1b, UE 1 is associated with cluster 1 constituting TRPs 1-2, while UE 2 is associated with cluster 2 constituting TRPs 1 and 3.  In the simulation results reported in the paper, the cluster associated with each UE is formed using the two strongest TRPs as seen by the UE.

| (a) | (b) |
| --- | --- |

Figure 1 An illustration of  (a) coordinated scheduling vs. (b)  independent scheduling approaches

To compare the performance of the two types of clustering approaches, we performed system level evaluations using the agreed evaluation assumptions for the indoor-hotspot scenario at 4 GHz.  The system level simulation assumptions are given in the Appendix.  The system level evaluation results are summarized in Table 1.  We simulated both single TRP and DPS as baselines.  Note that for DPS, a coordinated scheduler within the cluster is assumed.  In Table 1, cases where NC-JT outperforms DPS are marked in green and cases where NC-JT performs worse than DPS are marked in red.

From these results, it is noted that in the indoor office scenario with 2 Tx antenna ports, NC-JT with coordinated scheduling outperforms NC-JT with independent scheduling.  This is because of the benefit from coordinated scheduling due to the presence of a single scheduler per cluster.  It is furthermore noted that NC-JT with coordinated scheduling can provide notable mean throughput performance gains at lower offered load values (corresponding to 10-20% RU of single TRP) over both DPS and single TRP approaches.  However, this gain vanishes at higher offered load (corresponding to 40% RU).  Hence, we make the following observations:

In the indoor office scenario with 2 Tx per TRP and 4 Rx in the UE, NC-JT with coordinated scheduling provides notable mean throughput performance gains over single TRP and DPS at low offered load.

In the indoor office scenario with 2 Tx per TRP and 4 Rx in the UE, NC-JT with coordinated scheduling outperforms NC-JT with independent scheduling.

Table 1:  Performance comparison between coordinated vs independent scheduling.

|  | Single TRP | DPS | NC-JT with coordinated scheduling | NC-JT with independent scheduling |
| --- | --- | --- | --- | --- |
| RU [%] | 10 |  |  |  |
| 5% user throughput gain [%] | 0 | 26 | 18 | 10 |
| Mean throughput gain [%] | 0 | 3 | 36 | 32 |
| RU [%] | 20 |  |  |  |
| 5% user throughput gain [%] | 0 | 10 | -2 | -12 |
| Mean throughput gain [%] | 0 | 3 | 14 | 10 |
| RU [%] | 40 |  |  |  |
| 5% user throughput gain [%] | 0 | 1 | -17 | -32 |
| Mean throughput gain [%] | 0 | -2 | -6 | -22 |

# Conclusion

In this contribution, we present some preliminary simulation results to study the impact of different scheduling approaches on the performance on NC-JT in indoor scenario.  Based on the results presented in the contribution, we make the following observations:

- Observation 1	In the indoor office scenario with 2 Tx per TRP and 4 Rx in the UE, NC-JT with coordinated scheduling provides notable mean throughput performance gains over single TRP and DPS at low offered load.

- Observation 2	In the indoor office scenario with 2 Tx per TRP and 4 Rx in the UE, NC-JT with coordinated scheduling outperforms NC-JT with independent scheduling.

# References

TR36.741, “Study on further enhancements to Coordinated Multi-Point (CoMP) operation for LTE”

RP-181453 WID on NR MIMO.

# Appendix:  System level evaluation assumptions

For system level evaluations, the agreed assumptions from RAN1#94bis are used.  The remaining evaluation assumptions are given in the table below.

| Parameter | Indoor-hotspot |
| --- | --- |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 10 MHz |
| Subcarrier spacing | 15 kHz |
| BS antenna configuration | (M, N, P, Mg, Ng) = (1,1,2,1,1) |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1,2,2,1,1); |
| UE receiver | MMSE-IRC |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes Results reported for 10%, 20%, and 40% RU for baseline scheme |
| OLLA | On |
| Channel Estimation and Feedback assumptions | Ideal channel estimation |
| Rank hypothesis | 1 or 2 rank transmission per TRP (rank adaptation enabled) |
| Coordination cluster size | 2 TRPs per cluster for results in Section 2 |